3GPP TSG-RAN WG2 Meeting #123                           R2- 2307740

Toulouse, France, 21st Aug – 25th Aug 2023

Agenda item:	7.7.4.1.2

Source:	TCL

Title:	Discussion on NTN to NTN cell reselection enhancements

Document for:	Discussion and Decision

- Introduction

In Rel-18 WID[1], Following objectives are specified for mobility enhancements.

- For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4]

- Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signaling overhead. [RAN2, RAN3]

- Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized). [RAN2, RAN3, RAN4]

- Study and, if needed, specify enhancement to Xn[/NG] signaling to support feeder link switch-over, CHO, e.g., exchange of necessary information between gNBs. [RAN3]

Regarding the issue of NTN- NTN cell reselection, during last two meeting, the following agreements have been reached.

RAN2#121b:

- RAN2 understands that for earth-moving cell reselection, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (FFS whether a new epochTime IE is needed). RAN2 understanding is that both PVT and orbital parameters can be used for this. FFS if additional information is needed to allow more accurate measurements.

- For earth-moving cell, new IE is introduced to indicate the reference location of serving cell.

- For cell (re)selection in earth-moving system, a distance threshold is introduced for location-based measurement initiation, which reuses distanceThresh in SIB19.

- For cell (re)selection in earth-moving system, time-based measurement initiation is used to address feeder-link switch case.

RAN2#122:

Re-use t-Service-r17 format for the IE used to trigger UE neighbour cell measurements prior to cell replacement due to feeder link switch. FFS whether we reuse exactly the same IE name as in R17 (updating the field description) or a new one.

In this contribution, we will further provide the discussion on NTN – NTN cell reselection enhancements.

- Discussion

RAN2#121bis-e concluded a UE can roughly derive the serving cell trajectory based on the serving satellite ephemeris and epoch time, with the assumption that the serving cell reference location broadcast by the network is the one at epoch time. Using this information a UE can track the reference location as the satellite moves overhead, which is necessary to support location-based measurement initiation in earth-moving cells.

The following two aspects remain FFS:

- FFS whether a new epochTime IE is needed (for the associated reference location of earth moving cell)

- FFS if additional information is needed to allow more accurate measurements.

In Rel-17 NTN the IE referencLocation is for NTN quasi-Earth fixed system, and the reference location of serving cell is fixed. However the reference location for earth-moving cell is moving. Therefore in Rel-18 it has been agreed that for earth-moving cell, new IE is introduced to indicate the reference location of serving cell.

However regarding IE epochTime, in Rel-17 NTN the IE epochTime is used by UE to derive and track the reference location movement, and there is no difference between quasi-earth fixed cell and earth-moving cell in this sense. The simplest approach is to reuse the existing epochTime IE for the reference location.

Proposal 1: Reusing the existing epochtime IE for earth-moving cell.

Regarding the issue that whether additional information is needed to allow more accurate measurements, our view is it is unnecessary. For the mobility enhancement, lots of aspects have been discussed and supported in Rel-17 and Rel-18. For example, time-based/location-based measurement triggering and CHO for NTN-NTN mobility, and TN coverage information broadcast in NTN cell for NTN-TN IDLE/INACTIVED UE mobility.

There shall be a trade off between the information signalling and the accuracy. From the information above, we can see that the enhancement for NTN mobility is just for a rough accuracy. With this intention in mind, in Rel-18 for NTN-TN cell reselection enhancements, it has been agreed that no more accurate information for TN coverage area information is introduced except for cell center and radius.

Considering one of the main objectives for the enhancements of NTN mobility is signalling overhead reduction, it is unnecessary to introduce additional information for more accurate measurements.

Proposal 2: Additional information is not needed for reference location derivation in earth moving cell.

Regarding the issue that for the IE used to trigger UE neighbour cell measurements prior to cell replacement due to feeder link switch, whether we reuse exactly the same IE name as in R17 which is t-Service-r17 (updating the field description) or a new one, our perspective is we can reuse t-Service-r17.

In R18 for enhancements of NTN-NTN HO, RAN2 has made the following agreements for unchanged PCI scenario.

- t-Service in SIB19 can also be interpreted by Rel-18 UE in Connected mode to know that a satellite change or feeder link change happens.

- In hard switch unchanged PCI scenario (i.e. no handover), the UE needs to know the time the UE attempts to re-synchronize. (FFS whether a new “t-Start” / a t-gap is needed or whether t-Service can be reused (i.e. no other IE) if the gap is very short/zero).

We can see that unchanged PCI scenario includes feeder link change scenario. We assume RAN2 has the common sense that t-Service-r17 can be interpreted by Rel-18 UE in connected mode to know that feeder link switch. Moreover RAN2 is also discussing the time the UE attempts to re-synchronize to the “Target Cell”, and reusing t-Service-r17 is one of the options on the table. Therefore for the scenario of feeder link switch, reusing t-Service-r17 can align the understanding for UEs in idle/inactive state and connected state.

Proposal 3: For the IE used to trigger UE neighbour cell measurements prior to cell replacement due to feeder link switch, we reuse exactly the same IE name as in R17 which is t-Service-r17.

- Conclusion

Based on the above discussion, we recommend RAN2 to discuss and adopt the following proposal:

Proposal 1: Reusing the existing epochtime IE for earth-moving cell.

Proposal 2: Additional information is not needed for reference location derivation in earth moving cell.

Proposal 3: For the IE used to trigger UE neighbour cell measurements prior to cell replacement due to feeder link switch, we reuse exactly the same IE name as in R17 which is t-Service-r17.

Reference

- RP-221819, WID: NR NTN (Non-Terrestrial Networks) enhancements