3GPP TSG RAN WG2 #123	R2-2307741

Toulouse, France, August 21st – 25th, 2023

Source:	Panasonic

Title:		Discussion on satellite switch with unchanged PCI

Agenda Item:	7.7.4.2	Handover enhancements

Document for:	Discussion, decision

# 1. Introduction

For satellite switch (or feeder link switch), unchanged PCI solution (i.e. no handover) is under discussion. Satellite switch with unchanged PCI is beneficial especially to save L3 mobility procedure such as RRC reestablishment and security update. The following agreements were made in the last RAN2 meeting.

Agreements:

- t-Service in SIB19 can also be interpreted by Rel-18 UE in Connected mode to know that a satellite change or feeder link change happens

- In hard switch unchanged PCI scenario (i.e. no handover), the UE needs to know the time the UE attempts to re-synchronize. (FFS whether a new “t-Start” / a t-gap is needed or whether t-Service can be reused (i.e. no other IE) if the gap is very short/zero).

This document provide our view on the satellite switch with unchanged PCI.

# 2. Discussion on unchanged PCI

# 2.1 Timing related information

For satellite switch, the beam direction of the ground station is switched from the current serving satellite to the new satellite (Figure 1). The switching time depends on the antenna configuration. For example, if the ground station is equipped with electrical beam steering antenna, e.g. phased array antenna, beam switch with almost zero gap would be possible. On the other hand, if the ground station is equipped with mechanical beam steering antenna, e.g. dish type antenna, relatively long gap (e.g. > 100ms) would be required for the satellite switch. The similar would be applied to the antenna located in the satellite. In these longer gap cases, UE needs an information when to start the feeder link with new satellite.

Figure 1 Satellite switch

Observation 1: The very short/zero gap would not be feasible depending on the antenna configuration. UE needs an information on when to start the transmission via new satellite.

Proposal 1: The network should provide information on when to start the transmission via new satellite.

For electrical steering antenna, various operational choices of the beam switch for DL and UL can be considered. One example is to switch the DL and UL beams towards the satellites simultaneously as illustrated in Figure 2. In this case, there is a long time period where UL data from old satellite can not be received due to the long RTT of NTN. Note that during this period, gNB does not receive any UL data via new satellite because UE does not receive DL from new satellite and the synchronization is not yet established. UE should not transmit UL data including periodic transmission (e.g. Configured grant PUSCH, periodic SRS, periodic PUCCH) during this period to reduce interferences to other cells.

Observation 2: If gNB switches the DL and UL beams toward the satellites simultaneously, there is a long time period where UL data from old satellite can not be received due to the long RTT of NTN.

Proposal 2: UE should not transmit UL after the stop timing of old satellite until DL resynchronization to new satellite is completed.

Figure 2 Satellite switch with simultaneous DL and UL

Another example is to switch the DL and UL beams independently as illustrated in Figure 3. In this case, UL data via old satellite can be received until UL stop timing which would be typically set to RTT after DL stop timing. UE can keep UL transmission via old satellite until the UL switching time. Because DL switch timing and UL switch timing are different, it should be clarified that the indication of stop/start timing is DL timing in the specification. DL switching time is essential for UE to determine the timing of re-synchronization. Information on UL switching time is beneficial but may not be essential for UE to know until when gNB can receive UL data via old satellite.

Observation 3: If gNB switches the DL and UL beams toward the satellites independently, UL data via old satellite can be received until UL stop timing.

Proposal 4: It should be clarified that the indication of stop/start timing is DL timing in the specification.

Proposal 5: To indicate UL switching time to allow UE to know until when gNB can receive UL data via old satellite.

Figure 3 Satellite switch with independent DL and UL

2.2 Issues on UE neighbor cell management

Another issue is neighbor cell status for the neighbor cell measurement. UE performs neighbor cell search and manages the detected neighbor cells’ information such as PCI and detection timing for neighbor cell measurement. The detected timing is also used for handover. When the satellite switch occurs on the serving cell, neighbor cell’s satellite link may or may not change depending on whether the neighbor cell is served by the same satellite. If the detected neighbour cells is served by another satellite, neighbor cell’s satellite link may change independently from the serving cell. Therefore, when neighbour cell's satellite link is changed, UE would reset detected neighbor cells’ timing and re-synchronize the neighbor cell. Because UE does not know when the neighbor cell’s satellite link is changed, one possible implementation method is UE periodically resets all detected neighbour cells' timing and re-synchronizes the neighbour cell but it increases the power consumption of UE and maintained neighbour list is inefficient. Therefore, if UE can obtain information on the neighbor cell status, e.g. whether neighbor cells’ satellite are changed, UE can know whether re-synchronization to neighbor cells is needed and can avoid unnecessary trials of the neighbor cell search.

Proposal 6: UE is informed of the information on neighbor cell status whether neighbor cells’ satellite are changed.

# 3. Conclusion

In this document, we discussed the issues on satellite switch with unchanged PCI. The following observations and proposals were made.

Observation 1: The very short/zero gap would not be feasible depending on the antenna configuration in the ground station. UE needs an information on when to start the transmission via new satellite.

Observation 2: If gNB switches the DL and UL beams toward the satellites simultaneously, there is a long time period where UL data from old satellite can not be received due to the long RTT of NTN.

Observation 3: If gNB switches the DL and UL beams toward the satellites independently, UL data via old satellite can be received until UL stop timing.

Proposal 1: The network should provide information on when to start the transmission via new satellite.

Proposal 2: UE should not transmit UL after the stop timing of old satellite until DL resynchronization to new satellite is completed.

Proposal 4: It should be clarified that the indication of stop/start timing is DL timing in the specification.

Proposal 5: To indicate UL switching time to allow UE to know until when gNB can receive UL data via old satellite.

Proposal 6: UE is informed of the information on neighbor cell status whether neighbor cells’ satellite are changed.