3GPP TSG-RAN WG2 Meeting #123	R2-2307742

# Toulouse, France, August 21 – August 25, 2023

Source:	Qualcomm Incorporated

Title:	Common part and Layer-2 specific part on U2U Relay

Agenda Item:	7.9 Enhanced NR Sidelink Relay

Document for:	Discussion and Decision

# Introduction & Background

This contribution discusses the open issues on discovery and Relay UE (re)selection and Layer-2 specific part for U2U relay based on RAN2 and SA2 progress.

# Discussion

### 2.1 Open issues on discovery and Relay (re)selection

In last RAN2 meeting, there are some open issues were captured in [2], but not discussed.

Issue 1: whether indication needs to be introduced when the link quality between the relay UE and peer remote UE is below a threshold

several companies proposed that an indication needs to be introduced when the link quality between the relay UE and peer remote UE is below a threshold. In U2N relay, Relay UE does not inform Remote UE when the Uu link is below a threshold. Similarly, in U2U relay, the Relay UE does not need to inform the Remote UE.

Proposal 1: No indication is to be introduced when the link quality between the relay UE and peer remote UE is below a threshold.

Issue 2: Whether AS criterion needs to be introduced for switching from indirect path to direct path

There was proposal in previous meeting that AS criterion needs to be introduced for switching from indirect path to direct path, e.g. when the direct path link quality (i.e. SL-RSRD) is above a threshold and indirect path link quality is below a threshold, UE can switch from indirect path to direct path. However, in existing peer UE discovery and direct path communication, there is no AS layer criterion for the UE to determine whether to trigger PCR message. As long as the source UE can discover the target UE, the source UE can determine to switch from indirect path to direct path, no AS criterion is needed to this type of path switching.

Proposal 2: AS criterion is not needed for switching from indirect path to direct path. UE can switch to direct path once discovering the target remote UE.

Issue 3: How to coordinate the final Relay UE between the source and target Remote UE

There was discussion whether/how to handle the case that remote UE and its peer remote UE may select two different relay UEs simultaneously for communicating with each other. This is already covered SA2 specification. For Modal A discovery, it is the source Remote UE to select the final Relay UE among the candidate Relay UEs announcing the target Remote UE; for Modal B discovery, it is the target Remote UE to select the final Relay UE according to the signal strength, local policy, operator policy per Relay Service Code if any; for integrated discovery, it is the target Remote UE to select the final Relay UE.

Proposal 3: RAN2 follows SA2 and CT1 specification on how to coordinate the final Relay UE between the source and target Remote UE in ProSe layer.

### 2.1 Layer-2 specific

#### 2.2.1 Identification of the S-Remote UE/D-Remote UE

In last RAN2 meeting, there were some discussion and agreement on identification of Remote UE in SRAP layer.

Agreements:

For the possible use of a short ID in U2U relay, RAN2 will downselect between the following options for identifying the source and destination remote UEs at the SRAP layer:

a)	Single ID, identifying the source and destination remote UEs

b)	Source ID and Destination ID

For the possible use of a short ID in U2U relay, the U2U relay UE performs the ID assignment. FFS if this ID should be assigned hop-by-hop or globally.

These agreements do not imply agreement to use a short ID.

Based on the discussion and progress, there are the following possible options to be considered.

Option 1: Single local ID, globally assignment

In this option, single local ID is used in each hop, and the single local ID is assigned globally, that means the local ID is same on each hop.

This option has collision issue in case of multi-hop Relays, if there is no centralized node to coordinate the global local ID assignment, then the local ID could be collide. Taking the following as an example, when the D-UE1 receives traffics from the shared link, D-UE1 does not know the traffic is from S-UE1 or S-UE2.

- S-UE1/D-UE1 communication share the same PC5 link with S-UE2/D-UE1      communication

- Local ID #1 is assigned for S-UE1/D-UE1 connection by Relay UE #1

- Local ID #1 is assigned for S-UE2/D-UE1 connection by Relay UE #2

- There is no coordination between Relay UE #1 and Relay UE #2

For single-hop relay, this is not issue. Taking the following figure as example, Relay UE1 assigns Local ID #1 for S-UE1/D-UE1 communication, when Relay UE2 assign local ID for S-UE2/D-UE2, Relay UEs will avoid assigning local ID #1. Then there is no collision issue when only single-hop relay is considered.

Observation 1: If using single global local ID assignment, there could be collision issue in case of multi-hop relays; there is no collision issue in case of single-hop relay.

One benefit of option 1 is, the existing SRAP header and Relay UE SRAP behavior in U2N relay can be almost reused into U2U relay.

Option 2: Two local IDs, globally assignment

In this option, two global local IDs are added in SRAP layer on each hop and each ID is assigned for source Remote UE or target Remote UE. The same collision issue as option 1 exists in this option for multi-hop relays.

Option 3: Single local ID, per-hop assignment

In this option, the per-hop local ID is to identify S-UE/D-UE pair uniquely on each hop and can be different on different hops. The Relay UE maintains the mapping of the two local IDs and replace the local ID using the one on the next hop in SRAP layer. Since the per-hop local ID is unique within the current hop, then there is no collision issue in case of multi-hop relays.. One drawback is the Relay UE needs to maintain the mapping of the two local ID and replace it. But it is not big issue since anyway Relay UE needs to maintain the U2U relay configuration.

Observation 2: Single per-hop local ID has no collision issue in case of multi-hop relays

Option 4: Two local IDs, per-hop assignment

This option is similar with option 3, and the only difference is that two per-hop local IDs assigned for source Remote UE or target Remote UE. The Relay UE maintains the mapping of the two IDs on previous hop and next hop and replace the two local IDs using the ones assigned to the next hop in SRAP layer. Relay UE behavior is same as option 3 but need to maintain two local IDs.

Option 5: both of source Layer-2 ID and target Layer-2 ID

For Layer-2 included in SRAP layer, there are some drawbacks. The size of Layer-2 ID is 24bits, and the ID header in adaptation layer should be 48bits if considering future-compatible to multi-hop relay. Layer-2 ID is link ID, not UE ID, and there could be multiple Layer-2 IDs for one UE and could be collision, especially after multiple hops relay. According to SA2 specification, Remote UE layer-2 ID is not included in discovery message, and the Remote UE does not know the Layer-2 ID of the peer Remote UE ID. And from security point of view, if privacy is required, Layer-2 ID will be updated every 5 minutes, then PC5-S/RRC signalling will be costed every time Layer-2 ID is updated from each Remote UE.

Observation 3: Layer-2 ID has the following disadvantages:

- Is link ID instead of UE ID, and could be collision after multi-hop relay,

- The Layer-2 ID size is 24 (bits), and the adaptation layer header overhead is large

(48bits).

- The Remote UE does not know the Layer-2 ID of the peer Remote UE ID.

- If privacy is required, Layer-2 ID will be updated every 5 minutes, then PC5-S/RRC

signalling will be costed every time Layer-2 ID is updated from each Remote UE.

Evaluation of options can be summarized into the following table.

Table 1: Evaluation of identification options

| Options | Relay UE | Applicability | Performance | SRAP spec impact |
| --- | --- | --- | --- | --- |
| Option 1 (Single global local ID) | Assign the local ID Routing according to the target local ID | Only applicable for single-hop relay Collision issue for multi-hop relay | Less low header cost (8bits) | Almost reuse existing SRAP layer |
| Option 2 (Two global local ID) | Assigns two local IDs Routing according to target local ID. | Only applicable for single-hop relay Collision issue for multi-hop relay | Low header cost (16bits) | Change SRAP header Change Relay UE behavior to cover two local IDs |
| Option 3 (Single per-hop local ID) | Assign the local ID Maintain the mapping of previous hop local ID and next hop local ID Replace local ID with the next hop local ID | Applicable for multi-hop relay | Less low header cost (8bits) | No change SRAP header Change Relay UE behavior for traffic routing based on local ID |
| Option 4 (Two per-hop local ID) | Assign the local IDs Maintain the mapping of previous two per-hop local IDs and next two per-hop local IDs Replace local IDs with the next two per-hop local IDs | Applicable for multi-hop relay | Low header cost (16bits) | Change SRAP header Change Relay UE behavior for traffic routing based on local ID |
| Option 5 (Two Layer-2 IDs) | Obtain Layer-2 ID of target Remote UE Routing according to the target Layer-2 ID | Collision issue for multi-hop relay | High header cost (48bits) PC5-S/RRC signalling on all hops will be costed every time Layer-2 ID is updated from each Remote UE | Change SRAP header Change Relay UE behavior for traffic routing based on Layer-2 IDs |

Based on the evaluation, it can be found that if only single-hop relay is considered in Rel-18, Option 1 is the best solution, which does not require much specification change. If multi-hop relay needs to be considered, then option 3 is preferred. Option 2 and Option 4 have more UE implementation complexity compared to option 1 and option 3, but there is no additional benefit.

Proposal 4: If only single-hop relay is considered, Option 1 (i.e. single global local ID) is adopted.

Proposal 5: If multi-hop relay is considered, Option 3 (i.e. single per-hop local ID) is adopted.

Considering the drawback of Layer-2 ID, it is proposed not to pursue Layer-2 ID as ID format in adaptation layer.

Proposal 6: RAN2 does not pursue the Layer-2 ID as ID format in SRAP layer.

#### 2.2.2 E2E SL-SRB configuration

In last RAN2 meeting, there is FFS issue on how to configure SRAP and PC5 RLC channel configuration for SL-SRB as following highlight.

Agreements:

For the E2E SL-SRB configuration of U2U relay, specified PDCP configuration is used. FFS for the SRAP and PC5 RLC channel configuration for SL-SRB.

Currently, for PC5 direct communication, default configuration for SL SRB is used. For U2U relay, the current mechanism can be taking as baseline.

Proposal 7: For the E2E SL-SRB configuration of U2U relay, specified per-hop RLC Channel configuration is used.

Proposal 8: Reuse the existing specified SL-SRB1/2/3 RLC Channel configuration to RLC Channel configuration of U2U relay E2E SL-SRB1/2/3.

As proposal 6, for each of E2E SL-SRB1/2/3, the RLC Channel is already specified, then no need to SRAP configuration for bearer and channel mapping.

Proposal 9: No SRAP configuration is needed for E2E SL-SRB bear and RLC Channel mapping.

#### 2.2.3 QoS handling and E2E SL-DRB configuration

For per-hop RLC Channel configuration, there could be different issues for the first hop and second hop.

It was agreed that the Relay UE splits E2E QoS parameters, one issue is how the Relay UE obtain E2E QoS profiles. There are a lot of QoS parameters to be sent to Relay UE, the QoS parameters are shown in Anexx. In Layer-3 based U2U relay, the source Remote UE sends E2E QoS profiles and QoS flow ID(s) to the Relay UE using PC5-S message. There could be a lot of QoS parameters, and the PC5-S message including E2E QoS parameters can be reused for Layer-2 based U2U relay, and RAN2 does not need to introduce QoS parameters in PC5-RRC message.

Proposal 10: The source Remote UE reuses the existing PC5-S message to send E2E QoS profiles and QoS flow ID(s) to Relay UE.

Firstly, considering the second hop RLC Channel configuration, it should be Relay UE to derive the configuration. But since SDAP is E2E configuration and the Relay UE does not know the E2E SL-DRB for each QoS flow, then the Relay UE cannot does not know how to generate and map the RLC Channel configuration to the E2E SL-DRB on the second hop.

Proposal 11: RAN2 discusses how to associate the RLC Channel configuration on the second hop with the E2E SL-DRB ID.

The Relay UE has already known the QoS flow ID(s) as proposed in proposal 10, then one simple way is that the Remote UE sends the mapping of E2E-SL-DRB and QoS flows to the Relay UE. Relay UE knows the per-hop QoS profiles on the second hop for the QoS flows, then the Relay UE can derive the SL-DRB info and corresponding QoS profiles on the second hop, and then can derive the RLC Channel for the second hop.

Proposal 12: For RLC Channel configuration on the second hop, the Remote UE sends the mapping of E2E-SL-DRB and QoS flow ID(s) to the Relay UE; the Relay UE derives SL-DRB ID for the QoS flow ID(s); and the Relay UE derives the RLC Channel configuration on the second hop for the SL-DRB ID.

For the first hop RLC Channel configuration, there could be two options on how to generate SL-DRB configurations based on the split QoS profiles.

Option 1: Relay UE sends back the split QoS profiles and QoS flow ID(s) to source Remote UE, and the source Remote UE derives first hop RLC Channel configuration for the E2E-DRB. This option can reuse existing PC5-S message to send QoS profiles from Relay UE to source Remote UE.

Option 2: Relay UE generates per-hop configuration on the two hops based on the split QoS profiles and configures to the source UE and target UE. This option does not require QoS signaling from Relay UE to source Remote UE.

Option 1 is more aligned with existing procedure that TX UE derives the sidelink configuration.

Proposal 13: For RLC Channel configuration on the first hop, Relay UE sends back the split QoS profiles and QoS flow ID(s) to source Remote UE; and the source Remote UE derives first hop RLC Channel configuration for the E2E-DRB.

For E2E SL-DRB configurations, the source Remote UE can derive SDAP and PDCP configuration.

Proposal 14: The source Remote UE derives SDAP and PDCP configuration for a E2E SL-DRB.

SA2 sent LS to RAN2 in [3] and asks RAN2 to inform SA2 if any impact.

SA2 asks RAN2 to define the AS solution/signalling to address QoS splitting, and if RAN2 identify any impact to SA2 to inform SA2.

Then it is proposed to send LS reply to SA2 that PC5-S message is reused to transmit QoS parameters between the source Remote UE and the Relay UE.

Proposal 15: Send LS reply to SA2 that PC5-S message is reused to transmit QoS parameters between the source Remote UE and the Relay UE.

#### 2.2.4 RLF handling and recovery

Currently, the sidelink RLF is detected in the following conditions, and when any condition is met, UE will release PC5-RRC connection.

1>	upon indication from sidelink RLC entity that the maximum number of retransmissions for a specific destination has been reached; or

1>	upon T400 expiry for a specific destination; or

1>	upon indication from MAC entity that the maximum number of consecutive HARQ DTX for a specific destination has been reached; or

1>	upon integrity check failure indication from sidelink PDCP entity concerning SL-SRB2 or SL-SRB3 for a specific destination

In U2U relay, since there per-hop connection and E2E connection, the RLF could be detected on current hop (e.g. the condition on RLC or MAC entity maximum retransmission number) or on E2E connection (e.g. the condition on T400 expiry and PDCP integrity failure).

When the per-hop PC5 RLF is detected on first hop, it could be wasteful to release and establish again the per-hop PC5 connection on the second hop and the E2E PC5 connection. One reasonable way is to keep the second hop connection and E2E connection unaffected, let the Remote UE try to discover and reselect a candidate Relay UE. If the old Relay UE is selected, then the PC5 link on the second hop is not impacted, and if a new Relay UE is selected, then the second hop will be released by the peer Remote UE once the new PC5 link is established. During the procedure the E2E connection is not impacted.  This can be shown in the following procedure.

In case that the source Remote UE is indicated per-hop PC5 RLF by the Relay UE, the Remote UE assumes the old relay path is not available, then the source Remote UE should release the per-hop PC5 link for the S-Remote UE/D-Remote UE pair, and try to discover and reselect a candidate Relay UE.

Proposal 16: When per-hop RLF is detected on one hop, E2E PC5 connection can be kept during Relay reselection and per-hop PC5 connection recovery.

Proposal 17: When per-hop RLF is detected on one hop, per-hop PC5 connection on another hop can be kept if the old Relay UE is reselected; if a new Relay UE is reselected, the destination Remote UE release the old per-hop PC5 connection.

Proposal 18: If E2E PC5 RLF is detected, the Remote UE should release the E2E link connection.

# Conclusion

This contribution discusses RAN2 part for UE-to-UE relay discovery and (re-) selection and provides the following proposals.

Open issues on discovery and Relay (re)selection,

Proposal 1: No indication is to be introduced when the link quality between the relay UE and peer remote UE is below a threshold.

Proposal 2: AS criterion is not needed for switching from indirect path to direct path. UE can switch to direct path once discovering the target remote UE.

Proposal 3: RAN2 follows SA2 and CT1 specification on how to coordinate the final Relay UE between the source and target Remote UE in ProSe layer.

Identification of the S-Remote UE/D-Remote UE,

Observation 1: If using single global local ID assignment, there could be collision issue in case of multi-hop relays; there is no collision issue in case of single-hop relay.

Observation 2: Single per-hop local ID has no collision issue in case of multi-hop relays

Observation 3: Layer-2 ID has the following disadvantages:

- Is link ID instead of UE ID, and could be collision after multi-hop relay,

- The Layer-2 ID size is 24 (bits), and the adaptation layer header overhead is large

(48bits).

- The Remote UE does not know the Layer-2 ID of the peer Remote UE ID.

- If privacy is required, Layer-2 ID will be updated every 5 minutes, then PC5-S/RRC

signalling will be costed every time Layer-2 ID is updated from each Remote UE.

Proposal 4: If only single-hop relay is considered, Option 1 (i.e. single global local ID) is adopted.

Proposal 5: If multi-hop relay is considered, Option 3 (i.e. single per-hop local ID) is adopted.

Proposal 6: RAN2 does not pursue the Layer-2 ID as ID format in SRAP layer.

E2E SL-SRB configuration,

Proposal 7: For the E2E SL-SRB configuration of U2U relay, specified per-hop RLC Channel configuration is used.

Proposal 8: Reuse the existing specified SL-SRB1/2/3 RLC Channel configuration to RLC Channel configuration of U2U relay E2E SL-SRB1/2/3.

Proposal 9: No SRAP configuration is needed for E2E SL-SRB bear and RLC Channel mapping.

QoS handling and E2E SL-DRB configuration,

Proposal 10: The source Remote UE reuses the existing PC5-S message to send E2E QoS profiles and QoS flow ID(s) to Relay UE.

Proposal 11: RAN2 discusses how to associate the RLC Channel configuration on the second hop with the E2E SL-DRB ID.

Proposal 12: For RLC Channel configuration on the second hop, the Remote UE sends the mapping of E2E-SL-DRB and QoS flow ID(s) to the Relay UE; the Relay UE derives SL-DRB ID for the QoS flow ID(s); and the Relay UE derives the RLC Channel configuration on the second hop for the SL-DRB ID.

Proposal 13: For RLC Channel configuration on the first hop, Relay UE sends back the split QoS profiles and QoS flow ID(s) to source Remote UE; and the source Remote UE derives first hop RLC Channel configuration for the E2E-DRB.

Proposal 14: The source Remote UE derives SDAP and PDCP configuration for a E2E SL-DRB.

Proposal 15: Send LS reply to SA2 that PC5-S message is reused to transmit QoS parameters between the source Remote UE and the Relay UE.

For RLF handling,

Proposal 16: When per-hop RLF is detected on one hop, E2E PC5 connection can be kept during Relay reselection and per-hop PC5 connection recovery.

Proposal 17: When per-hop RLF is detected on one hop, per-hop PC5 connection on another hop can be kept if the old Relay UE is reselected; if a new Relay UE is reselected, the destination Remote UE release the old per-hop PC5 connection.

Proposal 18: If E2E PC5 RLF is detected, the Remote UE should release the E2E link connection.

# Reference

[1]    RP-221262: New WID on NR sidelink relay enhancements

[2]    R2-2306555 Summary of AI 7.9.2 on UE-to-UE relay	ZTE, Sanechips

[3]    R2-2304652 Reply LS on 5G ProSe Layer-2 UE-to-UE Relay QoS enforcement (S2-2305915; contact: Qualcomm)	SA2	LS in

# Annex

QoS parameters sent to Relay UE in PC5-S message (TS 24.554)

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC5 QoS flow descriptions IEI |  |  |  |  |  |  |  | octet 1 |
| Length of PC5 QoS flow descriptions contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| PC5 QoS flow description 1 |  |  |  |  |  |  |  | octet 4 octet u |
| PC5 QoS flow description 2 |  |  |  |  |  |  |  | octet u+1 octet v |
| ... |  |  |  |  |  |  |  | octet v+1 octet w |
| PC5 QoS flow description n |  |  |  |  |  |  |  | octet w+1 octet x |

Figure 11.3.5.1: PC5 QoS flow descriptions information element

| 8 | 7 |  | 6 |  | 5 |  | 4 |  | 3 | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare |  | 0 Spare | PQFI |  |  |  |  |  |  |  |  |  | octet 4 |
| Operation code |  |  |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare | PII |  | octet 5 |
| 0 Spare |  | E | Number of parameters |  |  |  |  |  |  |  |  |  | octet 6 |
| Associated ProSe identifiers |  |  |  |  |  |  |  |  |  |  |  |  | octet 7* octet k* |
| Parameters list |  |  |  |  |  |  |  |  |  |  |  |  | octet k+1* octet u* |

Figure 11.3.5.2: PC5 QoS flow description