3GPP TSG-RAN WG2 Meeting #123                                                        R2-2307749

Toulous, France, 21 August – 25 August, 2023

Source:                Qualcomm Incorporated

Title:	Discussion on UE QoE capabilities

Agenda Item:	7.14

Document for:	Discussion and Decision

# Introduction & Background

This contribution discuss UE capabilities for Rel-18 QoE features, including QoE collection in IDLE and Inactive state, QoE collection in NR-DC, enhancemnets for Rel-17 leftover issues.

# Discussion

UE capabilities on IDLE/Inactive state QoE

For IDLE and Inactive state QoE, UE is required to support QoE data collection in IDLE and Inactive state, then UE capability needs to be introduced. And UE needs to report the capability to gNB for gNB selecting the right UEs for QoE collection in IDLE and Inactive state.

- Introduce UE capability of supporting QoE collection in IDLE/Inactive with radio access capability parameters.

Since IDLE and Inactive state QoE collection is for MBS broadcast service, then only those UEs supports MBS broadcast service can report of supporting QoE collection in IDLE/Inactive state

- UE can report capability of supporting IDLE and Inactive state QoE collection only when UE supports MBS broadcast.

Another issue is whether to introduce UE capability for CONNECTE state for MBS service, but this depends on SA4/SA5 feedback on whether to treat MBS as a new service type or communication mode.

- Waiting for SA4/SA5 feedback on whether to introduce UE capability of supporting CONNECTED state QoE for MBS service type.

For AS buffer size, reuse existing 64KB buffer size for IDLE and Inactive state QoE data buffering if the UE supports QoE collection in IDLE and Inactive state.

- UE is required to buffer 64KB size for QoE data if the UE supports QoE collection for IDLE and Inactive state.

In last RAN2 meetings, there are some proposals on additional buffer sizes (e.g. 256KB, 512KB). How much QoE data could be collected depends on the service type using MBS broadcast and the time duration the application layer lasts for. RAN2 can assume some UE capabilities to support different buffer size, and ask SA4 to confirm the UE capabilities.

- RAN2 assume to introduce independent UE capabilities of supporting 128KB i.e. 256KB, 512KB buffer size. Send LS to SA4 to confirm the UE capabilites.

UE capabilities on NR-DC QoE

NR-DC QoE collection is new feature with introducing QoE configuration via SRB3, then UE capability should be introduced for QoE collection in NR-DC.

- Introduce UE capability of supporting NR-DC configuration via SRB3 with radio access capability parameter.

SRB5 is agreed to be introduced for UE reporting QoE data to SN, this is new SRB and it should be optional for the UE to support the new SRB.

- Introduce UE capability of supporting SRB5 for QoE reporting with radio access capability parameters.

There could be other impact to support NR-DC QoE collection, e.g. configuration or reporting RRC message enhancements, RVQoE configuration and reporting. But this needs to decided based on further progress on NR-DC QoE collection.

- It is FFS on other NR-DC QoE capabilities e.g. whether to support new message or IEs.

Other UE capabilities

RAN2 and RAN3 agreed to introduce QoS Flow ID in RVQoE measurement reporting. The QFI should be provided by application layer, then it should be optional to provide QFI together with RVQoE measurement.

- RAN2 assumes it is optional to report QoS Flow ID when reporting RVQoE measurements.

For threshold based buffer level reporting, since SA4 is evaluating whether it can be supported in Rel-18, it needs to be postponed until SA4 has progress.

- Wait for SA4 progress on UE capability of supporting threshold-based.

The following UE feasures and capabilities are related to application layer, it is suggest to send LS to SA4 for confirmation.

- Send LS to SA4 to check the following UE capabilities

- Independent UE capabilities of supporting 128KB i.e. 256KB, 512KB buffer size

- UE capability of supporting QoS Flow ID reporting

# Conclusion

This contribution discusses the MBS broadcast service QoE configuration and report for IDLE and Inactive state.

UE capabilities on IDLE/Inactive state QoE

- Introduce UE capability of supporting QoE collection in IDLE/Inactive with radio access capability parameters.

- UE can report capability of supporting IDLE and Inactive state QoE collection only when UE supports MBS broadcast.

- Waiting for SA4/SA5 feedback on whether to introduce UE capability of supporting CONNECTED state QoE for MBS service type.

- UE is required to buffer 64KB size for QoE data if the UE supports QoE collection for IDLE and Inactive state .

- RAN2 assume to introduce independent UE capabilities of supporting 128KB i.e. 256KB, 512KB buffer size. Send LS to SA4 to confirm the UE capabilites .

UE capabilities on NR-DC QoE

- Introduce UE capability of supporting NR-DC configuration via SRB3 with radio access capability parameter .

- Introduce UE capability of supporting SRB5 for QoE reporting with radio access capability parameters .

- It is FFS on other NR-DC QoE capabilities e.g. whether to support new message or IEs.

Other UE capabilities

- RAN2 assumes it is optional to report QoS Flow ID when reporting RVQoE measurements .

- Wait for SA4 progress on UE capability of supporting threshold-based

- Send LS to SA4 to check the following UE capabilities

- Independent UE capabilities of supporting 128KB i.e. 256KB, 512KB buffer size

- UE capability of supporting QoS Flow ID reporting

# Reference

[1]    RP-221803: Enhancement on NR QoE management and optimizations for diverse services