3GPP TSG-RAN WG2 Meeting #123	R2-2307762

Toulouse, France, August 21-25, 2023

Agenda item:		7.5.4.1

Source:		Futurewei

Title:	Discussions on new Buffer Status table design for XR

Document for:		Discussion and Decision

# Introduction

In RAN2 meeting #122, RAN2 agreed on the following with respect to the design of new BS table for XR [1]:

Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases).

We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

In this contribution, we discuss the detailed design of the new BS table for XR.

# Discussions

## Data to be reported using the new BS table

In our companion paper [2] submitted in this meeting, we have proposed that the UE classifies the PDUs buffered at an LCG as urgent PDUs or non-urgent PDUs, e.g., by comparing the remaining time associated with the PDUs to a pre-configured time threshold. Then, the UE reports the data volumes of the urgent PDUs and the non-urgent PDUs separately in the report for the LCG, where the data volume of the urgent PDUs is reported using a new XR BSR MAC CE while the data volume of the non-urgent PDUs is reported using one of the legacy BSR MAC CEs.

The purpose of the new BS table is to provide finer granularity (with less quantization error) for the buffer size reported. However, there is no strong need to provide very accurate reporting of the data volume for the non-urgent data because the non-urgent data can be transmitted opportunistically as long as they remain non-urgent. Therefore, we propose that the new BS table is only used for the BS field in the new XR BSR MAC CE, which is used exclusively for reporting the urgent data volume. The BS field(s) in the legacy BSR MAC CEs will continue to use the legacy BS table, no matter the legacy BSR MAC CE is used for an LCG configured for XR or not.

Proposal 1. The new BS table is only used for the BS field in the new XR BSR MAC CE. The BS field(s) in the legacy BSR MAC CEs will continue to use the legacy BS table, no matter the legacy BSR MAC CE is used for an LCG configured for XR or not.

## Suitable range and step size of the BS values covered by the new Buffer Size table

In [3], the AR UL video traffic is assumed to have a frame rate of 60 Hz and a data rate of 10Mbps (baseline) or 20Mbps (optional). Therefore, the average frame size is about 20 Kbytes for the baseline scenario and 40 KB for the optional scenario. However, the average frame size computed based on the average data rate and the frame rate mostly reflects the size of P frames, because P frame appears more frequently than I frame. Meanwhile, the upper bound of the new Buffer Size table should be designed based on the size of I frames. The average size of I frames can be 4 to 5 times of the average size of P frames. Therefore, we consider an upper bound of the size of a single frame for AR UL video can be 100 Kbytes for the baseline scenario and 200 Kbytes for the optional scenario. Given the 30 msec PDB [3] and 16.666 msec traffic periodicity, there can be at most two consecutive video frames being buffered at a time. Any older frames may be discarded by the discard operation and hence should not be counted for the reporting.

Between the two consecutive video frames (PDU Sets) possibly buffered for the same XR LCG, the UE should report only the earlier frame as the urgent data. The second frame, which arrived 16.666 msec later, should be reported as the non-urgent data. Therefore, the new Buffer Size table should cover a buffer size up to the size of a single I frame for the optional video data rate scenario, or 200 Kbytes as previously derived.

If the new BS table starts from 0 Kbytes and has a linear quantization step size of 1024 bytes, it can cover up to a size of 260,095 bytes, which is sufficient for the range that we have discussed above. Table 1 below illustrates the values in such table using a single linear quantization step size of 1024 bytes.

Table 1. New Buffer Size Table with a single linear region

| Index | BS value (bytes) | Index | BS value (bytes) | Index | BS value (bytes) | Index | BS value (bytes) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 64 | ≤ 65535 | 128 | ≤ 131071 | 192 | ≤ 196607 |
| 1 | ≤ 1023 | 65 | ≤ 66559 | 129 | ≤ 132095 | 193 | ≤ 197631 |
| 2 | ≤ 2047 | 66 | ≤ 67583 | 130 | ≤ 133119 | 194 | ≤ 198655 |
| 3 | ≤ 3071 | 67 | ≤ 68607 | 131 | ≤ 134143 | 195 | ≤ 199679 |
| 4 | ≤ 4095 | 68 | ≤ 69631 | 132 | ≤ 135167 | 196 | ≤ 200703 |
| 5 | ≤ 5119 | 69 | ≤ 70655 | 133 | ≤ 136191 | 197 | ≤ 201727 |
| 6 | ≤ 6143 | 70 | ≤ 71679 | 134 | ≤ 137215 | 198 | ≤ 202751 |
| 7 | ≤ 7167 | 71 | ≤ 72703 | 135 | ≤ 138239 | 199 | ≤ 203775 |
| 8 | ≤ 8191 | 72 | ≤ 73727 | 136 | ≤ 139263 | 200 | ≤ 204799 |
| 9 | ≤ 9215 | 73 | ≤ 74751 | 137 | ≤ 140287 | 201 | ≤ 205823 |
| 10 | ≤ 10239 | 74 | ≤ 75775 | 138 | ≤ 141311 | 202 | ≤ 206847 |
| 11 | ≤ 11263 | 75 | ≤ 76799 | 139 | ≤ 142335 | 203 | ≤ 207871 |
| 12 | ≤ 12287 | 76 | ≤ 77823 | 140 | ≤ 143359 | 204 | ≤ 208895 |
| 13 | ≤ 13311 | 77 | ≤ 78847 | 141 | ≤ 144383 | 205 | ≤ 209919 |
| 14 | ≤ 14335 | 78 | ≤ 79871 | 142 | ≤ 145407 | 206 | ≤ 210943 |
| 15 | ≤ 15359 | 79 | ≤ 80895 | 143 | ≤ 146431 | 207 | ≤ 211967 |
| 16 | ≤ 16383 | 80 | ≤ 81919 | 144 | ≤ 147455 | 208 | ≤ 212991 |
| 17 | ≤ 17407 | 81 | ≤ 82943 | 145 | ≤ 148479 | 209 | ≤ 214015 |
| 18 | ≤ 18431 | 82 | ≤ 83967 | 146 | ≤ 149503 | 210 | ≤ 215039 |
| 19 | ≤ 19455 | 83 | ≤ 84991 | 147 | ≤ 150527 | 211 | ≤ 216063 |
| 20 | ≤ 20479 | 84 | ≤ 86015 | 148 | ≤ 151551 | 212 | ≤ 217087 |
| 21 | ≤ 21503 | 85 | ≤ 87039 | 149 | ≤ 152575 | 213 | ≤ 218111 |
| 22 | ≤ 22527 | 86 | ≤ 88063 | 150 | ≤ 153599 | 214 | ≤ 219135 |
| 23 | ≤ 23551 | 87 | ≤ 89087 | 151 | ≤ 154623 | 215 | ≤ 220159 |
| 24 | ≤ 24575 | 88 | ≤ 90111 | 152 | ≤ 155647 | 216 | ≤ 221183 |
| 25 | ≤ 25599 | 89 | ≤ 91135 | 153 | ≤ 156671 | 217 | ≤ 222207 |
| 26 | ≤ 26623 | 90 | ≤ 92159 | 154 | ≤ 157695 | 218 | ≤ 223231 |
| 27 | ≤ 27647 | 91 | ≤ 93183 | 155 | ≤ 158719 | 219 | ≤ 224255 |
| 28 | ≤ 28671 | 92 | ≤ 94207 | 156 | ≤ 159743 | 220 | ≤ 225279 |
| 29 | ≤ 29695 | 93 | ≤ 95231 | 157 | ≤ 160767 | 221 | ≤ 226303 |
| 30 | ≤ 30719 | 94 | ≤ 96255 | 158 | ≤ 161791 | 222 | ≤ 227327 |
| 31 | ≤ 31743 | 95 | ≤ 97279 | 159 | ≤ 162815 | 223 | ≤ 228351 |
| 32 | ≤ 32767 | 96 | ≤ 98303 | 160 | ≤ 163839 | 224 | ≤ 229375 |
| 33 | ≤ 33791 | 97 | ≤ 99327 | 161 | ≤ 164863 | 225 | ≤ 230399 |
| 34 | ≤ 34815 | 98 | ≤ 100351 | 162 | ≤ 165887 | 226 | ≤ 231423 |
| 35 | ≤ 35839 | 99 | ≤ 101375 | 163 | ≤ 166911 | 227 | ≤ 232447 |
| 36 | ≤ 36863 | 100 | ≤ 102399 | 164 | ≤ 167935 | 228 | ≤ 233471 |
| 37 | ≤ 37887 | 101 | ≤ 103423 | 165 | ≤ 168959 | 229 | ≤ 234495 |
| 38 | ≤ 38911 | 102 | ≤ 104447 | 166 | ≤ 169983 | 230 | ≤ 235519 |
| 39 | ≤ 39935 | 103 | ≤ 105471 | 167 | ≤ 171007 | 231 | ≤ 236543 |
| 40 | ≤ 40959 | 104 | ≤ 106495 | 168 | ≤ 172031 | 232 | ≤ 237567 |
| 41 | ≤ 41983 | 105 | ≤ 107519 | 169 | ≤ 173055 | 233 | ≤ 238591 |
| 42 | ≤ 43007 | 106 | ≤ 108543 | 170 | ≤ 174079 | 234 | ≤ 239615 |
| 43 | ≤ 44031 | 107 | ≤ 109567 | 171 | ≤ 175103 | 235 | ≤ 240639 |
| 44 | ≤ 45055 | 108 | ≤ 110591 | 172 | ≤ 176127 | 236 | ≤ 241663 |
| 45 | ≤ 46079 | 109 | ≤ 111615 | 173 | ≤ 177151 | 237 | ≤ 242687 |
| 46 | ≤ 47103 | 110 | ≤ 112639 | 174 | ≤ 178175 | 238 | ≤ 243711 |
| 47 | ≤ 48127 | 111 | ≤ 113663 | 175 | ≤ 179199 | 239 | ≤ 244735 |
| 48 | ≤ 49151 | 112 | ≤ 114687 | 176 | ≤ 180223 | 240 | ≤ 245759 |
| 49 | ≤ 50175 | 113 | ≤ 115711 | 177 | ≤ 181247 | 241 | ≤ 246783 |
| 50 | ≤ 51199 | 114 | ≤ 116735 | 178 | ≤ 182271 | 242 | ≤ 247807 |
| 51 | ≤ 52223 | 115 | ≤ 117759 | 179 | ≤ 183295 | 243 | ≤ 248831 |
| 52 | ≤ 53247 | 116 | ≤ 118783 | 180 | ≤ 184319 | 244 | ≤ 249855 |
| 53 | ≤ 54271 | 117 | ≤ 119807 | 181 | ≤ 185343 | 245 | ≤ 250879 |
| 54 | ≤ 55295 | 118 | ≤ 120831 | 182 | ≤ 186367 | 246 | ≤ 251903 |
| 55 | ≤ 56319 | 119 | ≤ 121855 | 183 | ≤ 187391 | 247 | ≤ 252927 |
| 56 | ≤ 57343 | 120 | ≤ 122879 | 184 | ≤ 188415 | 248 | ≤ 253951 |
| 57 | ≤ 58367 | 121 | ≤ 123903 | 185 | ≤ 189439 | 249 | ≤ 254975 |
| 58 | ≤ 59391 | 122 | ≤ 124927 | 186 | ≤ 190463 | 250 | ≤ 255999 |
| 59 | ≤ 60415 | 123 | ≤ 125951 | 187 | ≤ 191487 | 251 | ≤ 257023 |
| 60 | ≤ 61439 | 124 | ≤ 126975 | 188 | ≤ 192511 | 252 | ≤ 258047 |
| 61 | ≤ 62463 | 125 | ≤ 127999 | 189 | ≤ 193535 | 253 | ≤ 259071 |
| 62 | ≤ 63487 | 126 | ≤ 129023 | 190 | ≤ 194559 | 254 | ≤ 260095 |
| 63 | ≤ 64511 | 127 | ≤ 130047 | 191 | ≤ 195583 | 255 | > 260095 |

One advantage of choosing a step size of a power of 2 is that the transmitter may determine the BS index for a given non-zero BS value by truncating certain number of LSBs of the BS value and adding 1 (due to index 0 being used for indicating zero BS) to the remaining MSBs, which is much simpler for UE implementation than using a searching algorithm to search within a loop-up table or performing a logarithmic calculation, as in the case of the legacy BS table. We consider a quantization step of 1024 bytes as acceptable, noting that in the legacy 8-bit Buffer Size table, the quantization step of the last buffer size that is within the size of 200 Kbytes is about 12 Kbytes.

If it is desirable for RAN2 to specify a single 8-bit BS table with piecewise linearity, e.g., by using two different quantization step sizes to cover different linear regions, Table 2 below illustrates the values in such table where the quantization step size of 32 bytes is used, primarily for the pose traffic, to cover up to a BS level of 1023 bytes, which is sufficient for up to 10 pose packets being buffered at a time (100 bytes pose packet size, 10 msec PDB, and up to 1kHz packet rate). After 1023 bytes, the quantization step size of 1024 bytes is used, primarily for the video traffic. With two linear regions, the UE still can use truncating bits to derive the BS index from a given non-zero BS value, by truncating different numbers of LSBs and then adding different offset values to the remaining MSBs for the different linear regions.

Table 2. New Buffer Size Table with two linear regions

| Index | BS value (bytes) | Index | BS value (bytes) | Index | BS value (bytes) | Index | BS value (bytes) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 64 | ≤ 33791 | 128 | ≤ 99327 | 192 | ≤ 164863 |
| 1 | ≤ 31 | 65 | ≤ 34815 | 129 | ≤ 100351 | 193 | ≤ 165887 |
| 2 | ≤ 63 | 66 | ≤ 35839 | 130 | ≤ 101375 | 194 | ≤ 166911 |
| 3 | ≤ 95 | 67 | ≤ 36863 | 131 | ≤ 102399 | 195 | ≤ 167935 |
| 4 | ≤ 127 | 68 | ≤ 37887 | 132 | ≤ 103423 | 196 | ≤ 168959 |
| 5 | ≤ 159 | 69 | ≤ 38911 | 133 | ≤ 104447 | 197 | ≤ 169983 |
| 6 | ≤ 191 | 70 | ≤ 39935 | 134 | ≤ 105471 | 198 | ≤ 171007 |
| 7 | ≤ 223 | 71 | ≤ 40959 | 135 | ≤ 106495 | 199 | ≤ 172031 |
| 8 | ≤ 255 | 72 | ≤ 41983 | 136 | ≤ 107519 | 200 | ≤ 173055 |
| 9 | ≤ 287 | 73 | ≤ 43007 | 137 | ≤ 108543 | 201 | ≤ 174079 |
| 10 | ≤ 319 | 74 | ≤ 44031 | 138 | ≤ 109567 | 202 | ≤ 175103 |
| 11 | ≤ 351 | 75 | ≤ 45055 | 139 | ≤ 110591 | 203 | ≤ 176127 |
| 12 | ≤ 383 | 76 | ≤ 46079 | 140 | ≤ 111615 | 204 | ≤ 177151 |
| 13 | ≤ 415 | 77 | ≤ 47103 | 141 | ≤ 112639 | 205 | ≤ 178175 |
| 14 | ≤ 447 | 78 | ≤ 48127 | 142 | ≤ 113663 | 206 | ≤ 179199 |
| 15 | ≤ 479 | 79 | ≤ 49151 | 143 | ≤ 114687 | 207 | ≤ 180223 |
| 16 | ≤ 511 | 80 | ≤ 50175 | 144 | ≤ 115711 | 208 | ≤ 181247 |
| 17 | ≤ 543 | 81 | ≤ 51199 | 145 | ≤ 116735 | 209 | ≤ 182271 |
| 18 | ≤ 575 | 82 | ≤ 52223 | 146 | ≤ 117759 | 210 | ≤ 183295 |
| 19 | ≤ 607 | 83 | ≤ 53247 | 147 | ≤ 118783 | 211 | ≤ 184319 |
| 20 | ≤ 639 | 84 | ≤ 54271 | 148 | ≤ 119807 | 212 | ≤ 185343 |
| 21 | ≤ 671 | 85 | ≤ 55295 | 149 | ≤ 120831 | 213 | ≤ 186367 |
| 22 | ≤ 703 | 86 | ≤ 56319 | 150 | ≤ 121855 | 214 | ≤ 187391 |
| 23 | ≤ 735 | 87 | ≤ 57343 | 151 | ≤ 122879 | 215 | ≤ 188415 |
| 24 | ≤ 767 | 88 | ≤ 58367 | 152 | ≤ 123903 | 216 | ≤ 189439 |
| 25 | ≤ 799 | 89 | ≤ 59391 | 153 | ≤ 124927 | 217 | ≤ 190463 |
| 26 | ≤ 831 | 90 | ≤ 60415 | 154 | ≤ 125951 | 218 | ≤ 191487 |
| 27 | ≤ 863 | 91 | ≤ 61439 | 155 | ≤ 126975 | 219 | ≤ 192511 |
| 28 | ≤ 895 | 92 | ≤ 62463 | 156 | ≤ 127999 | 220 | ≤ 193535 |
| 29 | ≤ 927 | 93 | ≤ 63487 | 157 | ≤ 129023 | 221 | ≤ 194559 |
| 30 | ≤ 959 | 94 | ≤ 64511 | 158 | ≤ 130047 | 222 | ≤ 195583 |
| 31 | ≤ 991 | 95 | ≤ 65535 | 159 | ≤ 131071 | 223 | ≤ 196607 |
| 32 | ≤ 1023 | 96 | ≤ 66559 | 160 | ≤ 132095 | 224 | ≤ 197631 |
| 33 | ≤ 2047 | 97 | ≤ 67583 | 161 | ≤ 133119 | 225 | ≤ 198655 |
| 34 | ≤ 3071 | 98 | ≤ 68607 | 162 | ≤ 134143 | 226 | ≤ 199679 |
| 35 | ≤ 4095 | 99 | ≤ 69631 | 163 | ≤ 135167 | 227 | ≤ 200703 |
| 36 | ≤ 5119 | 100 | ≤ 70655 | 164 | ≤ 136191 | 228 | ≤ 201727 |
| 37 | ≤ 6143 | 101 | ≤ 71679 | 165 | ≤ 137215 | 229 | ≤ 202751 |
| 38 | ≤ 7167 | 102 | ≤ 72703 | 166 | ≤ 138239 | 230 | ≤ 203775 |
| 39 | ≤ 8191 | 103 | ≤ 73727 | 167 | ≤ 139263 | 231 | ≤ 204799 |
| 40 | ≤ 9215 | 104 | ≤ 74751 | 168 | ≤ 140287 | 232 | ≤ 205823 |
| 41 | ≤ 10239 | 105 | ≤ 75775 | 169 | ≤ 141311 | 233 | ≤ 206847 |
| 42 | ≤ 11263 | 106 | ≤ 76799 | 170 | ≤ 142335 | 234 | ≤ 207871 |
| 43 | ≤ 12287 | 107 | ≤ 77823 | 171 | ≤ 143359 | 235 | ≤ 208895 |
| 44 | ≤ 13311 | 108 | ≤ 78847 | 172 | ≤ 144383 | 236 | ≤ 209919 |
| 45 | ≤ 14335 | 109 | ≤ 79871 | 173 | ≤ 145407 | 237 | ≤ 210943 |
| 46 | ≤ 15359 | 110 | ≤ 80895 | 174 | ≤ 146431 | 238 | ≤ 211967 |
| 47 | ≤ 16383 | 111 | ≤ 81919 | 175 | ≤ 147455 | 239 | ≤ 212991 |
| 48 | ≤ 17407 | 112 | ≤ 82943 | 176 | ≤ 148479 | 240 | ≤ 214015 |
| 49 | ≤ 18431 | 113 | ≤ 83967 | 177 | ≤ 149503 | 241 | ≤ 215039 |
| 50 | ≤ 19455 | 114 | ≤ 84991 | 178 | ≤ 150527 | 242 | ≤ 216063 |
| 51 | ≤ 20479 | 115 | ≤ 86015 | 179 | ≤ 151551 | 243 | ≤ 217087 |
| 52 | ≤ 21503 | 116 | ≤ 87039 | 180 | ≤ 152575 | 244 | ≤ 218111 |
| 53 | ≤ 22527 | 117 | ≤ 88063 | 181 | ≤ 153599 | 245 | ≤ 219135 |
| 54 | ≤ 23551 | 118 | ≤ 89087 | 182 | ≤ 154623 | 246 | ≤ 220159 |
| 55 | ≤ 24575 | 119 | ≤ 90111 | 183 | ≤ 155647 | 247 | ≤ 221183 |
| 56 | ≤ 25599 | 120 | ≤ 91135 | 184 | ≤ 156671 | 248 | ≤ 222207 |
| 57 | ≤ 26623 | 121 | ≤ 92159 | 185 | ≤ 157695 | 249 | ≤ 223231 |
| 58 | ≤ 27647 | 122 | ≤ 93183 | 186 | ≤ 158719 | 250 | ≤ 224255 |
| 59 | ≤ 28671 | 123 | ≤ 94207 | 187 | ≤ 159743 | 251 | ≤ 225279 |
| 60 | ≤ 29695 | 124 | ≤ 95231 | 188 | ≤ 160767 | 252 | ≤ 226303 |
| 61 | ≤ 30719 | 125 | ≤ 96255 | 189 | ≤ 161791 | 253 | ≤ 227327 |
| 62 | ≤ 31743 | 126 | ≤ 97279 | 190 | ≤ 162815 | 254 | ≤ 228351 |
| 63 | ≤ 32767 | 127 | ≤ 98303 | 191 | ≤ 163839 | 255 | > 228351 |

Proposal 2. RAN2 discuss and consider adopting one of Table 1 and Table 2 above for XR.

## On how the network knows which table to use

The UE and the gNB always use the new BS table for the urgent data volume, where the refined granularity is needed. As the urgent data volume can only be reported using the new XR BSR MAC CE, the gNB always uses the new BS table to interpret the BS field in the XR BSR MAC CE. Meanwhile, the non-urgent data volume is reported using one of the legacy BSR MAC CEs and the BS field in it is interpreted using the legacy BS table, because the coarse quantization is not that critical for the non-urgent data.

Therefore, the UE determines which BS table to use only based on the urgency of the data being reported. And, there is no need for the UE to identify the table to the gNB, other than the LCID of the (legacy or new) BSR MAC CE used for the report.

Proposal 3. The UE determines which BS table to use only based on the urgency of the data being reported.

Proposal 4. There is no need for the UE to identify the table to the gNB, other than the LCID of the (legacy or new) BSR MAC CE used for the report.

# Conclusions

The followings are proposed:

Proposal 1. The new BS table is only used for the BS field in the new XR BSR MAC CE. The BS field(s) in the legacy BSR MAC CEs will continue to use the legacy BS table, no matter the legacy BSR MAC CE is used for an LCG configured for XR or not.

Proposal 2. RAN2 discuss and consider adopting one of Table 1 and Table 2 above for XR.

Proposal 3. The UE determines which BS table to use only based on the urgency of the data being reported.

Proposal 4. There is no need for the UE to identify the table to the gNB, other than the LCID of the (legacy or new) BSR MAC CE used for the report.

# References

R2-2306542, RAN2-122 Report on LTE MUSIM QoE XR, Vice Chairman (Nokia).

R2-2307761, Discussions on delay status reporting in BSR for XR, Futurewei.

TR 38.838 V17.0.0.