3GPP TSG-RAN WG2 Meeting #123	R2- 2307768

Toulouse,  21 – 25  August 2023

Agenda item:	7.11.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Control plane aspects of multicast reception in RRC_INACTIVE state

WID/SID:	NR_MBS_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The Rel-18 work item on Enhancements of NR Multicast and Broadcast services has been agreed in [1] with the objectives including:

- Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3]

  - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2]

  - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE.  (Seamless/lossless mobility is not required) [RAN2, RAN3]

Rel-17 work specified multicast reception for UEs in RRC_CONNECTED state, whereas Rel-18 WID targets especially cells with a large number of UEs. In addition to the varied limitations on the network side for keeping all UEs receiving a multicast service in RRC_CONNECTED state, receiving multicast transmission in RRC_INACTIVE state by the UEs is assumed to be more power efficient [1].

In this paper, we provide an analysis of the configuration principles to operate the feature of multicast reception in RRC_INACTIVE state, in addition to the principles for mobility aspects. Furthermore, we provide an analysis of the RRC state transitions, and the details on notifications to be provided to the UEs. Previous agreements in RAN2#119-e, RAN2#119bis-e, RAN2#120, RAN2#121, RAN2#121bis-e and RAN2#122 meetings [2][3][4][5][6][7] are considered and open issues are targeted.

# 2	Principles for MBS reception in RRC_INACTIVE state

### 2.1	Configuration of the UEs for Receiving Multicast Services in RRC_INACTIVE State

The following related agreements were made in RAN2#121 and RAN2#121bis regarding the information delivered to the UE to receive multicast in RRC_INACTIVE state, and the means to provide such information to the UE [5][6]:

- When network configures UE to receive multicast in INACTIVE state, RRCRelease message with suspendconfig can be used to deliver the PTM configuration. Other dedicated RRC messages will not be used to provide PTM configuration for MBS multicast for INACTIVE.

- If network finds it useful, the PTM configuration for the (single) serving cell can be configured to UE before the session activation, and UE stores the configuration. When session is activated, UE can receive multicast in INACTIVE state by applying the configuration without going back to RRC_CONNECTED, if not updated by MCCH after being configured.

- Similar to Rel-17 broadcast reception procedure, UE acquires new SIB and multicast MCCH to get PTM configuration after cell reselection.

Moreover, there is the following FFSs in the Running RRC CR and Running Stage-2 CR:

Editor’s Note: FFS if “configured with MBS multicast reception in RRC_INACTIVE” should be replaced with other description or terminology.

Editor’s Note:	FFS that the above description of PTM configuration(s) delivery will be revised according to future conclusions.

Editor’s Note: 	FFS whether the UE can get the initial PTM configuration via multicast MCCH.

In our understanding, PTM configuration in the above agreements refer to the content of MCCH, i.e., information needed by the UE to receive multicast in RRC_INACTIVE state excluding the information about how to receive MCCH. The latter is referred as Multicast MCCH configuration. With this understanding, above agreements lead to:

- RRC release with suspendConfig can be used to deliver PTM configuration to the UE, while sending the UE to RRC_INACTIVE state, for the serving cell, while being not mandatory.

- PTM configuration of a specific service can be provided in RRC release with suspendConfig independent from multicast session’s activation state.

- When the UE reselects to a new cell, it checks the SIBx/MCCH of the cell to receive the multicast MCCH configuration and PTM configuration to receive multicast, if available.

Based on the above agreements and running CR the following issue is still open: What is the condition at the UE to start monitoring MCCH after being sent to RRC_INACTIVE state from RRC_CONNECTED state?

In our understanding, RRC release is not an enable/disable signal for RRC_INACTIVE reception at the UE. Current text in RRC running CR talks about “UE receiving configuration to receive multicast in RRC_INACTIVE state”, but such configuration needs clarifications, at least due to following reasons:

- gNB is not mandated to provide PTM configuration of any multicast service in RRC release with suspendConfig. gNB may decide not to provide the initial PTM configuration in RRC release, rather the UE would receive it via MCCH.

- Since RRC_INACTIVE multicast reception is per cell decision, UE may get the “initial PTM configuration” for RRC_INACTIVE reception in a neighbor cell for a service.

- UE may be sent to RRC_INACTIVE state in a cell where a service is not provided, i.e., out of service area. But such UE may come to a cell in RRC_INACTIVE where the service is provided to UEs in RRC_INACTIVE. This UE needs to receive MCCH to learn about configuration.

- A service may be deactive in a cell, where the UE is sent to RRC_INACTIVE. UE may go to a neighbor cell and it would be beneficial for the UE to receive MCCH for that service for future activation (note that PTM configuration of a deactive service is given only if gNB finds it useful, i.e., not mandatory).

Observation 1: RRC release providing PTM configuration is not an enable/disable signal for RRC_INACTIVE reception at the UE and cannot be used as such for several reasons:

- gNB is not mandated to provide PTM configuration of any multicast service in RRC release with suspendConfig. gNB may decide not to provide the initial PTM configuration in RRC release, rather the UE would receive it via MCCH.

- Since RRC_INACTIVE multicast reception is per cell decision, UE may get the “initial PTM configuration” for RRC_INACTIVE reception in a neighbor cell for a service, although the service is not provided to UEs in RRC_INACTIVE in the cell that sent the UE to RRC_INACTIVE.

- UE may be sent to RRC_INACTIVE state in a cell where a service is not provided, i.e., out of service area. But such UE may come to a cell in RRC_INACTIVE where the service is provided to UEs in RRC_INACTIVE. This UE needs to receive MCCH to learn about configuration.

- A service may be deactive in a cell, where the UE is sent to RRC_INACTIVE. UE may go to a neighbor cell and it would be beneficial for the UE to receive MCCH for that service for future activation  (note that PTM configuration of a deactive service is given only if gNB finds it useful, i.e., not mandatory).

Proposal 1: RRC release providing PTM configuration is not an enable/disable signal for RRC_INACTIVE reception at the UE for a multicast service.

Some companies have proposed to include a flag in RRC release to indicate the UE that the service will be provided in RRC_INACTIVE state, for the UE to start monitoring MCCH. In our view, this approach is not feasible, as transmitting data to the UEs in RRC_INACTIVE state is per cell decision. For example, if such flag is not given to the UE for the serving cell while sending the UE to RRC_INACTIVE, would this UE not monitor MCCH for a particular multicast service also in a neighbor cell? Note that information regarding inter-gNB cells will not be provided in RRC release message.

For those reasons, we propose that a UE in RRC_INACTIVE state that joined successfully via NAS signalling to an MBS session(s) shall monitor SIBx/MCCH for that respective TMGI(s).

Proposal 2: A UE in RRC_INACTIVE state that joined successfully to an MBS multicast session monitors SIBx/MCCH for that particular service, irrespective of what UE receives in RRC release with suspendConfig.

### 2.2	Suspension/Continuation of MRBs (and Multicast Service Reception) when Transitioning from RRC_CONNECTED to RRC_INACTIVE State

A UE can be given the PTM configuration via RRC release. If network finds it useful, this could even be done for a deactive session. However, currently, there is no way for UE to know the session activation status except checking out MCCH. To avoid unnecessary power consumption due to blind decoding for a deactive multicast session, the UE should be provided with an indication for “not suspending/continuing” MRBs (or MRBs of the sessions that are active) while being sent to RRC_INACTIVE state.

The UE monitors such MRBs as soon as it is in RRC_INACTIVE state, and suspends the other MRBs (and not monitor the corresponding G-RNTIs).

Proposal 3: The UE is provided with an indication for “not suspending/continuing” MRBs while being sent to RRC_INACTIVE state, and UE monitors such MRBs as soon as it is in RRC_INACTIVE state, while suspending the other MRBs (and not monitor the corresponding G-RNTIs) for which such indication is not received.

- This indication is per TMGI.

Proposal 4: If RRC release does not provide any PTM configuration, UE suspends the MRBs when going from RRC_CONNECTED to RRC_INACTIVE (as per Rel-17 behavior).

- Note that UE still checks MCCH for the PTM configuration.

Also, currently the MRB IDs are not provided over MCCH (and also not in PTM config in RRC release), as for broadcast such IDs were chosen by the UE itself. However, for service continuity during RRC state transition, UE should have a way to map MRBs in RRC_CONNECTED that are to be continued in RRC_INACTIVE.

For that reason, UE can use the Logical Channel IDs for the MRBs that is also included in the MCCH. As LCID has 1-to-1 mapping with MRBs, the UE can map the MRBs in RRC_INACTIVE to MRBs in RRC_CONNECTED to achieve service continuity.

Proposal 5: To provide the service continuity during RRC state changes from RRC_CONNECTED to RRC_INACTIVE (and vice-versa), as LCID has 1-to-1 mapping with MRBs, the UE can map the MRBs in RRC_INACTIVE to MRBs in RRC_CONNECTED via LCIDs to achieve service continuity.

### 2.3			MRB Type for UEs in RRC_INACTIVE State Receiving Multicast Services

Initial version of the running RRC CR introduced a new MRB type called Multicast-inactive MRB, which is not aligned with any agreements made in RAN2.

Observation 2: A new MRB type for multicast reception in RRC_INACTIVE state was not agreed by RAN2, although introduced in the current running RRC CR.

In our view, there is no need for introducing a new MRB type for multicast services just for the sake of using such MRB type in RRC_INACTIVE state. For broadcast, no such MRB type distinction exists for different RRC states.

Observation 3: For broadcast, no such MRB type distinction exists for different RRC states.

Moreover, RAN2 agreed that as a baseline assumption, the same transmission of the multicast service will be receivable by the UEs in both RRC_CONNECTED and RRC_INACTIVE states. Furthermore, discussions are being held to achieve service continuity during RRC state changes, suspension/continuity of MRBs during state changes etc. In case we introduce a new MRB type without a clear technical reason, the specifications will get unnecessarily complicated at least for those reasons.

Observation 4: There is no technical reason to introduce a new MRB type for UEs in RRC_INACTIVE state receiving multicast, compared to MRB type we have for UEs in RRC_CONNECTED state and such introduction would create more complications for various reasons explained in this paper.

Proposal 6: RAN2 does not introduce a new MRB type “multicast-inactive MRB” contrary to what is proposed by the Running RRC CR.

# 3	Principles of Multicast Reception in RRC_INACTIVE State at Mobility

The following table summarizes all mobility and state transition cases and necessary actions for a UE receiving a multicast transmission in one delivery mode from the source cell and in same or different delivery mode from the target cell:

| Target Cell Source Cell | RRC_CONNECTED (Rel-17 multicast delivery mechanism) | RRC_INACTIVE |
| --- | --- | --- |
| RRC_CONNECTED (Rel-17 multicast delivery mechanism) | Scenario 1 Example use case: Source and target cells are providing multicast to the UE in RRC_CONNECTED state, i.e., Rel-17 multicast reception. Action: Rel-17 handover of MRBs. | Scenario 2 Example use case: Source cell is providing multicast to the UE in RRC_CONNECTED state, target cell is providing multicast to the UE in RRC_INACTIVE state and no other unicast service exists for this UE to be handed over. Action: A handover command sending the UE to RRC_INACTIVE, releasing source MRB(s) and indicating the target SIB/MCCH/MTCH configuration. |
| RRC_INACTIVE | Scenario 3 Example use case: Source cell provides multicast to the UE in RRC_INACTIVE state and target cell provides in RRC_CONNECTED state. Action: UE reselects the target cell, checks SIB/MCCH and cannot find the service provided to UEs in RRC_INACTIVE. UE shall reconnect unless session is inactive. Warning: means shall be provided to enable RRC_INACTIVE UE to differentiate the cases where: - Target cell does not currently provide service to UEs in RRC_INACTIVE state, but rather only UEs in RRC_CONNECTED state - Where the session is deactivated. For that, we provide more details in Section 4.3. | Scenario 4 Example use case: Source and target cells are providing multicast to the UE in RRC_INACTIVE state. Action: UE reselects the target cell, UE reads new broadcasted SIB/MCCH/MTCH configuration and continues reception. |

Scenario 1 is already supported. For scenario 2, we propose that RAN2/3 further studies the mechanisms and the details to provide the necessary enhancements for the needed mobility and state transitions. A target node of the handover should be able to directly send the UE to RRC_INACTIVE state, rather than first accepting UE in RRC_CONNECTED state, i.e., conventional handover of the UE, and then sending another RRC release with suspendConfig to send the UE to RRC_INACTIVE state, as such process would require redundant signalling. Rather than that, a target node should be able to directly send the UE to RRC_INACTIVE state via handover command.

In our view, target cell can directly provide the PTM configuration and multicast MCCH configuration to the UE during handover, e.g., either via RRC reconfiguration command including the handover command or RRC release to be produced by the source cell that would guide UE to RRC_INACTIVE.

Proposal 7: RAN2/3 further studies the details to provide necessary enhancements for the needed mobility and state transitions, where the UE is in RRC_CONNECTED state receiving a multicast service in a source cell and to be handed over to a target cell that can serve the UE in RRC_INACTIVE state.

For scenario 3 to properly function, a notification in SIB/MCCH is needed, and this is provided in Section 4.3. Scenario 4 is rather simple, however, means shall be provided to guide the UE in RRC_INACTIVE state to the best possible neighbor frequency, which is discussed in the next section.

### 3.1	Cell Reselection Optimization for UEs in RRC_INACTIVE State Receiving Multicast Services

Frequency prioritization may be provided to the UE for cell reselection for multicast reception in RRC_INACTIVE, detailed mechanism on how to identify the frequency info (e.g., SAI, USD, or frequency info directly provided by network) is FFS.

No need to define a mechanism other than the frequency prioritization, i.e., per cell based prioritization in cell re-selection, to help UE choose the right cell to camp on.

With the above agreements, RAN2 agreed define a mechanism to guide the UE to a frequency/cell that provides the service for service continuity purposes, similar to Rel-17 broadcast frequency prioritization feature. Furthermore, RAN2 agreed not to use a cell-based prioritization, e.g., cell reselection should not be guided by neighbor cell list.

In our view, FSAI-based mechanism from broadcast cell reselection is to be reused, as the mechanism is well-known by the companies and significant amount of effort was spent in Rel-17 to define such complete behavior:

  - RAN nodes are configured with FSAI to frequency mapping, and this information is exchanged among neighbor gNBs.

  - UEs are informed about FSAI to MBS service mapping via service announcement (UEs can also be directly given frequencies rather than FSAIs mapping to a multicast service).

  - RAN node signals SIBx (possibly similar to broadcast SIB21) including FSAI to neighbor frequencies (intra-frequency neighbors as well).

  - UE prioritizes neighbor frequencies during cell reselection in RRC_INACTIVE state.

For that reason, coordination with SA2 for the provision of FSAIs (as AF/MB-SMF configuration/involvement would be needed) is required.

Proposal 8: For multicast service continuity, the UE in RRC_INACTIVE state uses FSAI-based frequency prioritization mechanism of Rel-17 broadcast.

Proposal 9: RAN2 sends an LS to SA2 regarding the feasibility of provisioning of FSAIs also for multicast services.

### 3.2	Neighbor Cell List for UEs in RRC_INACTIVE State Receiving Multicast Services

How would gNB get to know of neighbor gNBs’ delivery, i.e., whether multicast setup is established with the core network and whether the service is provided to the UEs in RRC_INACTIVE? Currently, there are no means for such information exchange. Therefore, RAN3 should specify the procedure regarding neighbor gNBs’ information exchange on multicast delivery to UEs in RRC_INACTIVE state.

Proposal 10: An LS is sent to RAN3 to define the signalling for information exchange on multicast delivery to UEs in RRC_INACTIVE state between neighbor gNBs.

In the draft running CR:

This seems to imply UE behaviour is mandated to reconnect to RRC_CONNECTED although it was agreed:

The neighbor cell list mechanism for multicast reception in RRC_INACTIVE may be configured e.g. it can be used by UE to resume RRC connection if service is not available in the re-selected cell by NCL, without reading MCCH in the re-selected cell, in some aspects similar to Rel-17 NCL mechanism in MBS broadcast.

For the broadcast handling UE behaviour is in fact captured in stage-2 and we think similar handling should be done also for RRC_INACTIVE. Problem with mandating UE always to reconnect if the mbs-NeighbourCellList  is included and the UE session is not in the list. would require that if the list is sent then NW has to coordinate every single service although this is not likely feasible. It will also trigger further discussions that we held in Rel-17 regarding the existence etc. of the TMGI in the list. That’s why we think one should not mandate this behaviour for the UE as UE could use some cleverness to understand when the reconnection is actually required.

Proposal 11: Remove the UE behaviour from stage-3 regarding mbs-NeighbourCellList. as the UE behaviour is already captured in stage-2.

The current text in running stage-2 CR regarding this is as follows:

The gNB may indicate in the multicast MCCH of current serving cell the list of neighbour cells providing the same MBS multicast service(s) for UEs in RRC_INACTIVE state as provided in the serving cell. This allows the UE, e.g., to resume RRC connection without reading MCCH of the re-selected cell, if service is not available to the UE in RRC_INACTIVE state.

### 3.3	Multicast Service Continuity After Cell Reselection in RRC_INACTIVE State

Rel. 17 multicast operation supports synchronization of PDCP COUNT/Sequence number (SN) among gNBs, so that when the UE moves from one cell to another in the RRC_CONNECTED state, i.e., handover, then the UE can continue receiving the same multicast service.  However, this synchronization of the PDCP COUNT/SN is not mandatory, resulting that UEs are unaware of whether the PDCP COUNT/SN is synchronized among the gNBs.  This is not a problem in Rel. 17 multicast reception as the multicast in Rel-17 is supported only for RRC_CONNECTED UEs, and network takes care of mobility. This is however a problem for Rel-18 RRC_INACTIVE UEs, because these RRC_INACTIVE UEs should manage themselves the minimization of packet loss during the mobility, by continuing the ongoing PDCP COUNT/SN, when applicable, i.e., when the network achieves PDCP COUNT/SN synchronization among the gNBs.

Observation 5: Rel. 18 RRC_INACTIVE UEs are unaware whether PDCP COUNT/SN synchronization is applied in the network among the gNBs. Therefore, UE in RRC_INACTIVE state would not know whether it can assume PDCP COUNT/SN continuation when selecting a new cell for camping.

If the PDCP COUNT/SN is synchronized among gNBs, the UE could ideally continue receiving the same multicast session in a newly selected cell in the RRC_INACTIVE state without re-establishing the PDCP entity in the new cell. However, the UE is unaware of which MBS radio bearer (MRB) in the source cell maps to which MRB in the target cell.

Observation 6: The UE in RRC_INACTIVE state is unaware which MRB in the source cell maps to which MRB in the target cell. Although PDCP COUNT/SN synchronization is applied across gNBs, the UE cannot achieve service continuity without such knowledge.

Proposal 12: To provide the service continuity when PDCP COUNT/SN synchronization is applied across cells, mapping of Logical Channel IDs (LCIDs) in the source and neighbor cells can be indicated to the UE via RRC release with suspendConfig and/or MCCH. As LCID has 1-to-1 mapping with MRBs, the UE can map the source cell’s MRBs to the target cell’s MRBs to achieve service continuity.

Proposal 13: RAN2 to discuss whether such information should be exchanged over the Xn interface among gNBs. Information exchange can include the list of QFIs (QoS flow identifiers) which maps to LCID as an identifier for this LCID.

Proposal 14: If the PDCP COUNT/SN synchronization is not applied among the gNBs, the RRC_INACTIVE UEs can follow Rel. 17 MBS broadcast principle for achieving the service continuity.

# 4	Management of Multicast Delivery to UEs in RRC_INACTIVE State

With the understanding explained in previous sections on how the multicast delivery to the UEs in RRC_INACTIVE state is configured, several elements are needed to be considered to manage the delivery of the multicast sessions to the UEs in RRC_INACTIVE state, which will be discussed in the upcoming sections.

### 4.1	Details of MCCH for Multicast Reception in RRC_INACTIVE State

New MCCH logical channel was agreed to be used for multicast reception in RRC_INACTIVE state, along with a new SIBx. This avoids the usage of the same MCCH-RNTI for broadcast to be used also for multicast. Thus, a new MCCH-RNTI for multicast is needed.

Proposal 15: A new MCCH-RNTI is specified to receive the new MCCH for multicast.

### 4.2	Details of DCI to Be Used for Multicast Reception in RRC_INACTIVE State

RAN1 replied the RAN2 LS R2-2304330 on multicast reception in RRC_INACTIVE with R1-2306243. Based on the reply, there is no consensus to support DCI format 4_2 for multicast MTCH reception in RRC INACTIVE in RAN1. The main reason was that the support of DCI format 4_2 is optional for a Rel-17 UE (and RAN1 expects that to be optional also for Rel-18 UE).

Observation 7: There is no consensus to support DCI format 4_2 for multicast MTCH reception in RRC INACTIVE in RAN1, where the main reason is the DCI format 4_2 being optional for a Rel-17 UE.

Previously, RAN2 agreed to be able to use same DCI to schedule both RRC_INACTIVE and RRC_CONNECTED UEs. For that reason, in light of RAN1 reply, there are the following options for the system design:

- DCI format 4_2 is not mandatorily supported by Rel-18 UEs (Rel-17 behavior).

Option A-1): As soon as gNB enables transmission to RRC_INACTIVE UEs, gNB uses DCI format 4_1 for all UEs.

Disadvantage would be not being able to use more functionality offered by DCI format 4_2 (e.g., more HARQ detailed options, MIMO options on top of 4_1) for UEs in RRC_CONNECTED state.

Option A-2) gNB uses DCI format 4_1 for RRC_INACTIVE UEs (+ possibly CONNECTED UE not supporting 4_2) and 4_2 for RRC_CONNECTED UEs to schedule the same data.

Disadvantage of this option would be more PDCCH overhead due to 2 DCI scheduling the same data.

- All Rel-18 UEs mandatorily support DCI format 4_2 for reception in RRC_INACTIVE state. In this case, gNB can freely choose based on the capability of Rel-17 UEs to transmit either DCI format 4_1 or 4_2 for UEs in RRC_INACTIVE and RRC_CONNECTED states.

Proposal 16: Regarding the system design for DCI operation, RAN2 discusses the following options:

- DCI format 4_2 is not mandatorily supported by Rel-18 UEs (Rel-17 behavior):

Option A-1): As soon as gNB enables transmission to RRC_INACTIVE UEs, gNB uses DCI format 4_1 for all UEs.

Option A-2) gNB uses DCI format 4_1 for RRC_INACTIVE UEs (+ possibly CONNECTED UE not supporting 4_2) and 4_2 for RRC_CONNECTED UEs to schedule the same data.

- All Rel-18 UEs mandatorily support DCI format 4_2 for reception in RRC_INACTIVE state. In this case, gNB can freely choose based on the capability of Rel-17 UEs to transmit either DCI format 4_1 or 4_2 for UEs in RRC_INACTIVE and RRC_CONNECTED states.

### 4.3	Indication of Multicast Session Activation/Deactivation to UEs in RRC_INACTIVE State

Aspects of running CR in 5.3.2.3

In the current running CR in case UE is paged with both unicast (UE_ID) and group paging (TMGI), UE will first forward UE identity and then TMGIs later to the upper layers. This could lead to situation that upper layers respond to both pagings although UE should only respond to unicast paging. Either we could change the RAN2 CR in such a way that only unicast identity is forward in this scenario or then we send a LS to CT1 and ask whether in this scenario there is a problem identified. To us it would be just simpler to change RAN2 CR

Proposal 17: Ensure that UE only responds to unicast paging (either I-RNTI or upper layer allocated identity) if paged with both unicast and group paging

Then regarding this bullet in 5.3.2.3:

Based on 1, UE enters to check the conditions if UE has joined any multicast sessions indicated by TMGIs in pagingGroupList. Then, based on this newly added 2, if the UE is not configured with multicast reception in RRC_INACTIVE, or if new flag to stay in RRC_INACTIVE is NOT included for "at least one MBS session that the UE has joined". This does not check further if those joined sessions are the sessions in pagingGroupList  and as they could be also other sessions. Thus we see that in fact bullet 1 and 2 should be combined to be proper:

1>	if in RRC_INACTIVE and the UE has joined one or more MBS session(s) indicated by the TMGI(s) included in the pagingGroupList

2>	if the UE is not configured with multicast reception in RRC_INACTIVE or if inactiveReceptionAllowed is not included for at least one of the MBS session (s) that the UE has joined, whose TMGI was included in the pagingGroupList:

Proposal 18: Ensure that UE only responds to group paging if the TMGI of the session UE has joined was in the pagingGroupList (e.g. by doing yellow highlight change shown above).

Additionally, there seems to be obsolete “stay in RRC_INACTIVE” statement in the CR as well:

This seems to be not required and should be deleted.

Proposal 19: Remove “stay in RRC_INACTIVE and” from the bullet in the 5.3.2.3.

Enhanced Group Paging and Special UEs

The following agreements were made in last meetings on multicast session activation/deactivation:

Rel-18 UE can stay in RRC_INACTIVE and start monitoring corresponding G-RNTI upon an enhanced group paging (e.g., upon session activation or data transmission resumed). Details FFS.

Introduce a new indication per tmgi in the group paging which informs Rel-18 UEs having a valid PTM configuration to receive the multicast in RRC_INACTIVE.

Therefore, RAN2 agreed to enhance group paging, such that the Rel-18 UEs in RRC_INACTIVE state do not go to RRC_CONNECTED state after group paging, in case the gNB decides to use the delivery mode that enables the reception of a multicast MBS session in a cell by the UEs in RRC_INACTIVE state.

Moreover, following Editor’s Notes are there in the running RRC specification:

Editor’s note: FFS whether a UE shall apply the multicast MCCH information acquisition procedure upon receiving paging with inactiveReceptionAllowed included for the concerned TMGI(s).

Currently, if the UE in RRC_INACTIVE state does not have the PTM configuration already (either via RRC release or MCCH) when receiving enhanced group paging, this UE goes to RRC_CONNECTED state. This would mean that the PTM configuration of the service should be signalled to the UEs before the session gets activated, in case gNB desires UE to stay in RRC_INACTIVE.

To avoid this limitation, for a Rel-18 UE without any PTM configuration, enhanced group paging can lead UE to start reading MCCH in the next modification period. In case UE cannot find the PTM configuration in MCCH, that can lead UE to transition to RRC_CONNECTED state.

Proposal 20: For a Rel-18 UE without any PTM configuration for a service, enhanced group paging for the respective service leads UE to receive MCCH in the next modification period (via MCCH change notification that is already specified). In case UE cannot find the PTM configuration in MCCH in the next modification period, UE to transitions to RRC_CONNECTED state.

In addition, the following agreement was made in the last meetings:

A "special UE" identified by MBS assistance information from 5GC can be released to RRC_INACTIVE (e.g., when the session is deactivated). FFS how can network enable such UE to resume to RRC_CONNECTED upon session activation

As agreed by SA2 WG, there may be some UEs, e.g., frequent talkers of a public safety session or platoon leaders, that are preferably kept in RRC_CONNECTED state when receiving a multicast session. A gNB can be informed about such UEs by the core network via different means. In that case, even though some other Rel-18 UEs are to be staying in RRC_INACTIVE state when the session is activated, such UEs would still be desired to go to RRC_CONNECTED state. When such UE receives a group paging message, i.e., either Rel-17 group paging or enhanced group paging as proposed above, it should go into RRC_CONNECTED state. For the Rel-17 group paging, anyway the UE would reconnect via current means. However, for the enhanced group paging using the extra paging indication to keep other UEs in RRC_INACTIVE, such UEs are to be informed to still go to RRC_CONNECTED state. Otherwise, extra unicast paging would be required towards such UEs. For such informing, while releasing UE to RRC_INACTIVE, the UE can be configured with the information that it should always reconnect in case of a specific multicast session activation.

Observation 8:  gNB based on information from core and from some other internal information may desire to get only some of the Rel-18 UEs, e.g., special UEs, to RRC_CONNECTED from RRC_INACTIVE, when the session gets activated.

Proposal 21: Special UEs, e.g., the ones preferred by 5GC to be served in RRC_CONNECTED for a multicast session, can be configured when UEs are released to RRC_INACTIVE state, e.g., to always come to RRC_CONNECTED state in case of session activation.

MCCH Enhancements for Session Activation Status

In last RAN2 meeting, RAN2 made the following agreement to signal session deactivation to the UEs in RRC_INACTIVE state:

- MCCH is used for notifying MC session deactivation for multicast reception in RRC_INACTIVE to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI.

Moreover, additional mechanisms are needed in addition to the group paging for indication of activation of a multicast session to UEs in RRC_INACTIVE state. For example, a UE in RRC_INACTIVE state may be out of the service area of the multicast session initially, but then may go into a cell within the service area. This UE may have missed the group paging performed by the gNB (as above).

Observation 9: A UE in RRC_INACTIVE state may be out of the service area of the multicast session initially, but then may go into a cell within the service area. This UE may have missed the group paging performed by the gNB for session activation.

Furthermore, the following was agreed in the last RAN2 meeting:

When a UE reselects to a cell for which PTM configuration is not available in multicast MCCH, the UE initiates RRC resumption procedure for an active multicast session it is interested to receive or continue receiving.

When camping in a new cell, a UE can check broadcasted SIBx/MCCH to see whether the configuration of RRC_INACTIVE multicast session reception is provided. There are two possibilities:

- MCCH contains the configuration of RRC_INACTIVE multicast session reception

In this case, the UE would understand the session is active and it can receive the service in RRC_INACTIVE state.

- MCCH does not contain the configuration of RRC_INACTIVE multicast session reception

Either the service is inactive, or the service is provided only to the UEs in RRC_CONNECTED state, e.g., audience size is small, therefore, gNB decides on Rel-17 multicast delivery method. In the former case, the UE does not need to reconnect, whereas in the latter, the UE should reconnect to receive the multicast session.

To overcome above mentioned ambiguity in (2), an indication can be provided in MCCH, indicating whether a service is active/deactive. A session status indication in MCCH would also solve the problem of missed group paging due to UE being out of the service area, the ambiguity mentioned above after cell reselection and the problem of indication of session deactivation to the UEs.

Observation 10: A session activation status in MCCH solves the problem of missed group paging due to UE being out of the service area, the ambiguity mentioned above after cell reselection and the problem of indication of session deactivation to the UEs.

Proposal 22: An indication is provided in MCCH indicating whether a service is active or deactive.

### 4.4	QoS of UEs Receiving a Multicast Session in RRC_INACTIVE State

Due to lack of feedback in the RRC_INACTIVE state (e.g., no HARQ feedback), the gNB cannot monitor/estimate the QoS of the multicast session for UEs in RRC_INACTIVE state. RAN2 considered how can UE/network recover from the scenario that UE in RRC_INACTIVE state does not receive multicast session with sufficient QoS and agreed the following:

UE may trigger RRC connection resumption if the reception quality of the multicast data is below a configured threshold, FFS how to specify the threshold/reception quality.

In our view, the decision to configure a threshold and reception quality should be left to network implementation. However, one thing to be considered is that such threshold should not lead the UE to always to move to RRC_CONNECTED without making reselection to a new cell. Thus, there should be some control in the network that in some scenario if QoS of the service gets bad UE needs resume the connection, e.g., some services may require very high QoS. But in most scenarios the services are such that QoS tolerates easily any interruptions caused by reselections and then one should not require UE to resume the connection.

The main motivation to have such a threshold is to ensure appropriate QoS is achieved, e.g. BLER, as a method to trigger for connection resumption. Therefore, metrics for such threshold should be QoS measurements, rather than radio measurements. It is not clear how the gNB map, e.g., the RSRP measurements into, e.g., BLER, perceived by the UE.

Observation 11: QoS (e.g., BLER) threshold for connection resumption should not lead the RRC_INACTIVE UE to reconnect before each cell reselection, as otherwise, RAN2 agreement on UE mobility without reconnections for reception of multicast service in RRC_INACTIVE state would be violated.

Proposal 23: The configured threshold(s) for controlling UE trigger for RRC connection resumption is QoS threshold(s) (e.g., BLER), rather than radio measurements, whose value is left to network implementation and those threshold(s) are per TMGI. FFS details on configuration.

### 4.5	Beam Management for Multicast Sessions

- For Rel-17 MBS multicast, since all the UEs receiving the multicast are in RRC_CONNECTED state, the network can transmit the PTM transmission only to those beams under which there are multicast UEs. When some UEs are released to RRC_INACTIVE, the network does not anymore know under which beams there are multicast UEs and therefore, the MBS multicast has to be transmitted to all the beams of the cell. RAN2 therefore agreed that the multicast transmission in RRC_INACTIVE is performed via beam sweeping based on SSB index like broadcast MBS (i.e., beam information is not needed in DCI.)

When the multicast PTM transmission is performed via beam sweeping, it is important that the UEs receiving the transmission know the mapping between a given beam and certain PDCCH monitoring occasions. This is especially beneficial from UE power consumption point of view as the UE need not monitor all the PDCCH monitoring occasions to receive the MBS multicast transmission. Therefore, it would be beneficial to indicate to both RRC_INACTIVE and RRC_CONNECTED UEs when beam sweeping is applied as well as the mapping between the beams and the PDCCH monitoring occasions.

- Proposal 24: For power saving purposes, indicate to both RRC_INACTIVE and RRC_CONNECTED MBS multicast UEs receiving the PTM transmission when beam sweeping is applied as well as the mapping pattern between the beams and the PDCCH monitoring occasions.

# 5	Conclusion

Observation 1: RRC release providing PTM configuration is not an enable/disable signal for RRC_INACTIVE reception at the UE and cannot be used as such for several reasons:

Proposal 1: RRC release providing PTM configuration is not an enable/disable signal for RRC_INACTIVE reception at the UE for a multicast service.

Proposal 2: A UE in RRC_INACTIVE state that joined successfully to an MBS multicast session monitors SIBx/MCCH for that particular service, irrespective of what UE receives in RRC release with suspendConfig.

Proposal 3: The UE is provided with an indication for “not suspending/continuing” MRBs while being sent to RRC_INACTIVE state, and UE monitors such MRBs as soon as it is in RRC_INACTIVE state, while suspending the other MRBs (and not monitor the corresponding G-RNTIs) for which such indication is not received.

Proposal 4: If RRC release does not provide any PTM configuration, UE suspends the MRBs when going from RRC_CONNECTED to RRC_INACTIVE (as per Rel-17 behavior).

Proposal 5: To provide the service continuity during RRC state changes from RRC_CONNECTED to RRC_INACTIVE (and vice-versa), as LCID has 1-to-1 mapping with MRBs, the UE can map the MRBs in RRC_INACTIVE to MRBs in RRC_CONNECTED via LCIDs to achieve service continuity.

Observation 2: A new MRB type for multicast reception in RRC_INACTIVE state was not agreed by RAN2, although introduced in the current running RRC CR.

Observation 3: For broadcast, no such MRB type distinction exists for different RRC states.

Observation 4: There is no technical reason to introduce a new MRB type for UEs in RRC_INACTIVE state receiving multicast, compared to MRB type we have for UEs in RRC_CONNECTED state and such introduction would create more complications for various reasons explained in this paper.

Proposal 6: RAN2 does not introduce a new MRB type “multicast-inactive MRB” contrary to what is proposed by the Running RRC CR.

Proposal 7: RAN2/3 further studies the details to provide necessary enhancements for the needed mobility and state transitions, where the UE is in RRC_CONNECTED state receiving a multicast service in a source cell and to be handed over to a target cell that can serve the UE in RRC_INACTIVE state.

Proposal 8: For multicast service continuity, the UE in RRC_INACTIVE state uses FSAI-based frequency prioritization mechanism of Rel-17 broadcast.

Proposal 9: RAN2 sends an LS to SA2 regarding the feasibility of provisioning of FSAIs also for multicast services.

Proposal 10: An LS is sent to RAN3 to define the signalling for information exchange on multicast delivery to UEs in RRC_INACTIVE state between neighbor gNBs.

Proposal 11: Remove the UE behaviour from stage-3 regarding mbs-NeighbourCellList. as the UE behaviour is already captured in stage-2

Observation 5: Rel. 18 RRC_INACTIVE UEs are unaware whether PDCP COUNT/SN synchronization is applied in the network among the gNBs. Therefore, UE in RRC_INACTIVE state would not know whether it can assume PDCP COUNT/SN continuation when selecting a new cell for camping.

Observation 6: The UE in RRC_INACTIVE state is unaware which MRB in the source cell maps to which MRB in the target cell. Although PDCP COUNT/SN synchronization is applied across gNBs, the UE cannot achieve service continuity without such knowledge.

Proposal 12: To provide the service continuity when PDCP COUNT/SN synchronization is applied across cells, mapping of Logical Channel IDs (LCIDs) in the source and neighbor cells can be indicated to the UE via RRC release with suspendConfig and/or MCCH. As LCID has 1-to-1 mapping with MRBs, the UE can map the source cell’s MRBs to the target cell’s MRBs to achieve service continuity.

Proposal 13: RAN2 to discuss whether such information should be exchanged over the Xn interface among gNBs. Information exchange can include the list of QFIs (QoS flow identifiers) which maps to LCID as an identifier for this LCID.

Proposal 14: If the PDCP COUNT/SN synchronization is not applied among the gNBs, the RRC_INACTIVE UEs can follow Rel. 17 MBS broadcast principle for achieving the service continuity.

Proposal 15: A new MCCH-RNTI is specified to receive the new MCCH for multicast.

Observation 7: There is no consensus to support DCI format 4_2 for multicast MTCH reception in RRC INACTIVE in RAN1, where the main reason is the DCI format 4_2 being optional for a Rel-17 UE.

Proposal 16: Regarding the system design for DCI operation, RAN2 discusses the following options:

- DCI format 4_2 is not mandatorily supported by Rel-18 UEs (Rel-17 behavior):

Option A-1): As soon as gNB enables transmission to RRC_INACTIVE UEs, gNB uses DCI format 4_1 for all UEs.

Option A-2) gNB uses DCI format 4_1 for RRC_INACTIVE UEs (+ possibly CONNECTED UE not supporting 4_2) and 4_2 for RRC_CONNECTED UEs to schedule the same data.

- All Rel-18 UEs mandatorily support DCI format 4_2 for reception in RRC_INACTIVE state. In this case, gNB can freely choose based on the capability of Rel-17 UEs to transmit either DCI format 4_1 or 4_2 for UEs in RRC_INACTIVE and RRC_CONNECTED states.

Proposal 17: Ensure that UE only responds to unicast paging (either I-RNTI or upper layer allocated identity) if paged with both unicast and group paging.

Proposal 18: Ensure that UE only responds to group paging if the TMGI of the session UE has joined was in the pagingGroupList (e.g. by doing yellow highlight change shown above).

Proposal 19: Remove “stay in RRC_INACTIVE and” from the bullet in the 5.3.2.3.

Proposal 20: For a Rel-18 UE without any PTM configuration for a service, enhanced group paging for the respective service leads UE to receive MCCH in the next modification period (via MCCH change notification that is already specified). In case UE cannot find the PTM configuration in MCCH in the next modification period, UE to transitions to RRC_CONNECTED state.

Observation 8:  gNB based on information from core and from some other internal information may desire to get only some of the Rel-18 UEs, e.g., special UEs, to RRC_CONNECTED from RRC_INACTIVE, when the session gets activated.

Proposal 21: Special UEs, e.g., the ones preferred by 5GC to be served in RRC_CONNECTED for a multicast session, can be configured when UEs are released to RRC_INACTIVE state, e.g., to always come to RRC_CONNECTED state in case of session activation.

Observation 9: A UE in RRC_INACTIVE state may be out of the service area of the multicast session initially, but then may go into a cell within the service area. This UE may have missed the group paging performed by the gNB for session activation.

Observation 10: A session activation status in MCCH solves the problem of missed group paging due to UE being out of the service area, the ambiguity mentioned above after cell reselection and the problem of indication of session deactivation to the UEs.

Proposal 22: An indication is provided in MCCH indicating whether a service is active or deactive.

Observation 11: QoS (e.g., BLER) threshold for connection resumption should not lead the RRC_INACTIVE UE to reconnect before each cell reselection, as otherwise, RAN2 agreement on UE mobility without reconnections for reception of multicast service in RRC_INACTIVE state would be violated.

Proposal 23: The configured threshold(s) for controlling UE trigger for RRC connection resumption is QoS threshold(s) (e.g., BLER), rather than radio measurements, whose value is left to network implementation and those threshold(s) are per TMGI. FFS details on configuration.

- Proposal 24: For power saving purposes, indicate to both RRC_INACTIVE and RRC_CONNECTED MBS multicast UEs receiving the PTM transmission when beam sweeping is applied as well as the mapping pattern between the beams and the PDCCH monitoring occasions.

# References

- RP-221458, “Revised WID: Enhancements of NR Multicast and Broadcast Services”.

- R2-2208707, “Report from MBS breakout session”, RAN2#119-e.

- R2-2210807, “Report from MBS breakout session”, RAN2#119bis-e.

- R2-2213007, “Report from MBS breakout session”, RAN2#120.

- R2-2301907 “Report from MBS breakout session”, RAN2#121.

- R2-2304207 “Report from MBS breakout session”, RAN2#121bis-e

- R2-2306547 “Report from MBS breakout session”, RAN2#122.