3GPP TSG-RAN WG2 Meeting #123	R2- 2307769

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.25.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Cross-RRH TCI state switch

WID/SID:	NR_HST_FR2_Enh - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN4 has indicated in R4-2306399 that:

RAN WG4 has introduced the support for High Speed Train (HST) scenarios in NR Frequency Range (FR2) in Release 17 NR_HST_FR2 WI.

As a result Release 17 discussion, a RAN4 requirement on One shot large UL timing adjustment for FR2 Power Class 6 UE was introduced in Clause 7.1.2.3 of TS 38.133.

RAN WG4 is working on the Release 18 NR_HST_FR2_enh WI on enhanced support for HST scenario in FR2. One of the objectives of the Release 18 WI [WID, RP-220985] is to

- Specify UL timing adjustment solution, including explicit NW signalling assistance, for FR2 HST scenario with large UL/DL propagation delay difference from different RRHs/TRPs to UE [RAN4, RAN2].

RAN4 has concluded that NW signalling assistance is needed to indicate when the UE shall apply one-shot large uplink timing adjustment at TCI state switch. The following was agreed at RAN4#106bis-e:

- Introduce MAC-CE based solution with 1bit indication to inform UE on the TCI state switch across RRHs

In this paper we discuss implications to RAN2.

# 2	Discussion

## 2.1	Background

RAN WG4 has originally introduced the support for High Speed Train (HST) scenarios in NR Frequency Range (FR2) in Release 17 NR_HST_FR2 WI.

Following the TR 38.854, in HST FR2 deployments, 1 to 4 non-collocated RRHs can be connected to the same BBU sharing the same PCI. Therefore, a large jump in propagation delay happens when the TCI state is switched in between non-collocated RRHs of the same cell.

During the Release 17 WI the following key agreements were achieved in RAN4 in relation to UL TX timing adjustment in HST FR2 scenarios:

- Introduction of a mechanism for one shot large uplink timing adjustment for FR2 HST scenarios with UE allowed to adjust uplink timing beyond Tq [RAN4#101-e, R4-2120416]

- Network configuration to enable one shot large uplink timing adjustment mechanism [RAN4#101-e, R4-2120416]

- UE shall apply one shot large timing adjustment, if enabled, on TCI switching occasion if UE measurement on DL received timing difference is larger than a threshold. [RAN4#101-bsi-e, R4-2202767]

- When one shot large uplink timing adjustment mechanism not in use, initial transmit timing requirement in Clause 7.1.2 is applied after inter-RRH TCI state switch. [RAN4#106, R4-2303158]

As a result of the agreements above, in Release 17, a new requirement on One shot large UL timing adjustment for FR2 Power Class 6 UE was introduced in Clause 7.1.2.3 of TS 38.133.

Now RAN2 received LS regarding the Release 18 NR_HST_FR2_enh WI on enhanced support for HST scenario in FR2. One of the objectives of the Release 18 WI [WID, RP-220985] is to

- Specify UL timing adjustment solution, including explicit NW signalling assistance, for FR2 HST scenario with large UL/DL propagation delay difference from different RRHs/TRPs to UE [RAN4, RAN2].

RAN4 has concluded that NW signalling assistance is needed to indicate when the UE shall apply one-shot large uplink timing adjustment at TCI state switch. The following was agreed at RAN4#106bis-e:

- Introduce MAC-CE based solution with 1bit indication to inform UE on the TCI state switch across RRHs

in RAN2#122 The LS from RAN4 was discussed and reply LS was sent in R2-2306865 with following content:

RAN2 discussed the RAN4 LS (R4-2306399) on MAC-CE based indication for cross-RRH TCI state switch. RAN2 agreed that It is feasible to specify requested MAC CE. But before progressing with the CRs RAN2 would like to understand and ask for few clarifications.

Therefore, RAN2 would like to seek RAN4’s input for the below questions:

- Question 1: Is it correct RAN2 understanding that the RAN4 LS only affects the MAC CEs intended for indicating target TCI state for PDCCH in 6.1.3.15 in TS 38.321?

- Question 2: Whether the enhanced TCI state indication in 6.1.3.44 of TS 38.321 (i.e., the MAC CE indicates two target TCI states) or the unified TCI state indication in 6.1.3.47 (i.e., the MAC CE indicating a unified states for UL and DL) is intended to be supported for cross-RRH TCI state switch ?

- Question 3: What is the intended UE behavior (e.g. regarding timing advance handling) upon reception of the MAC CE with indication on the TCI state switch across RRHs? For example, Does UE behavior also depend on the existing RRC parameter highSpeedDeploymentTypeFR2? Is it possible to update timing advance upon reception of the MAC CE with indication on the TCI state switch across RRHs and if not should UE stop uplink transmissions?

When cross RRH TCI state switch indication is sent to the UE it means that NW knows that UL timing difference between RRHs is large

Figure 2: Large UL timing difference between RRHs

but as the indication is purely “1-bit” indication it means NW does not know actual timing difference between RRHs and is not signaling those to the UE. Thus we see that there are at least two feasible situations that may occur:

- UE supports and NW has configured highSpeedLargeOneStepUL-TimingFR2-r17 (one shot uplink timing adjustment)

  - UE performs one shot large uplink timing adjustment if configured by NW (and UE supports the R17 corresponding feature)

- else

  - As the UE cannot make autonomous UL timing adjustment it seems UE needs to setup new timing advance via existing mechanisms i.e. PDCCH order or UE autonomous RACH procedure. It seems necessary to stop UL transmissions until new TA has been acquired in this situation

Observation 1: After receiving cross-RRH TCI state switch MAC CE UE should stop UL transmissions unless uplink timing is updated

It seems that in scenario where UE does cannot autonomously update UL timing (one shot large uplink timing adjustment) one needs to wait uplink timing adjustment before one can continue with uplink transmissions.In such a scenario UE autonomous handling should just stop UL transmissions but not delete RRC configuration regarding SRS/PUCCH etc. From RAN2 point of view one way to capture this behaviour is to model it via timeAlignmentTimer handling i.e. UE will not transmit anything in UL until it has acquired appropriate UL timing i.e. until timeAlignmentTimer is running. From 38.321 one can derive that:

- timeAlignmentTimer should be running only when UL time is assumed to be aligned

- timeAlignmentTimer is started or restarted when time alignment adjustment is received either in RAR message or in MAC CE.

- timeAlignmentTimer is not running, MAC entity shall not perform any UL transmission on a Serving Cell

Problem with this approach is that UE would also do following as TAT is considered as expired if stopped:

3>	flush all HARQ buffers for all Serving Cells;

3>	notify RRC to release PUCCH for all Serving Cells, if configured;

3>	notify RRC to release SRS for all Serving Cells, if configured;

3>	clear any configured downlink assignments and configured uplink grants;

3>	clear any PUSCH resource for semi-persistent CSI reporting;

3>	consider all running timeAlignmentTimers as expired;

3>	maintain NTA (defined in TS 38.211 [8]) of all TAGs.

In this scenario this should not be done i.e. we would need some other behaviour e.g. which does not trigger all of above points. Thus we propose to that timeAlignmentTimer is stopped in the case of cross-RRH TCI state switch MAC CE is received but it would not trigger other TAT expiry actions.

Proposal 1: UE shall stop timeAlignmentTimer when cross-RRH TCI state switch MAC CE is received and highSpeedLargeOneStepUL-TimingFR2-r17 is not configured but does not trigger other TAT Expiry actions but prevents UL transmissions

In our view this should be all we need to do in RAN2 i.e. make new TCI state switch MAC CE that indicates this state switch is cross RRH. Simplest in our view is just ”copy-paste” existing ”TCI State Indication for UE-specific PDCCH MAC CE” whose reception would indicate cross-RRH state switch.

Proposal 3: Introduce new MAC CE that is similar to existing “TCI State Indication for UE-specific PDCCH MAC CE”

Proposal 4: Take TP from Annex as baseline for MAC CR

# 3	Conclusion

Observation 1: After receiving cross-RRH TCI state switch MAC CE UE should stop UL transmissions unless uplink timing is updated

Proposal 1: UE shall stop timeAlignmentTimer when cross-RRH TCI state switch MAC CE is received and highSpeedLargeOneStepUL-TimingFR2-r17 is not configured but does not trigger other TAT Expiry actions but prevents UL transmissions

Proposal 3: Introduce new MAC CE that is similar to existing “TCI State Indication for UE-specific PDCCH MAC CE”

Proposal 4: Take TP from Annex as baseline for MAC CR

# Annex: TP

### 5.18.1	General

This clause specifies the requirements upon reception of the following MAC CEs:

-	SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE;

-	Aperiodic CSI Trigger State Subselection MAC CE;

-	TCI States Activation/Deactivation for UE-specific PDSCH MAC CE;

-	TCI State Indication for UE-specific PDCCH MAC CE;

-	Cross RRH TCI State Indication for UE-specific PDCCH;

-	SP CSI reporting on PUCCH Activation/Deactivation MAC CE;

-	SP SRS Activation/Deactivation MAC CE;

-	PUCCH spatial relation Activation/Deactivation MAC CE;

-	Enhanced PUCCH spatial relation Activation/Deactivation MAC CE;

-	SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE;

-	Recommended Bit Rate MAC CE;

-	Enhanced SP/AP SRS Spatial Relation Indication MAC CE;

-	SRS Pathloss Reference RS Update MAC CE;

-	PUSCH Pathloss Reference RS Update MAC CE;

-	Serving Cell set based SRS Spatial Relation Indication MAC CE;

-	SP Positioning SRS Activation/Deactivation MAC CE;

-	Timing Delta MAC CE;

-	Guard Symbols MAC CEs;

-	Positioning Measurement Gap Activation/Deactivation Command MAC CE;

-	PPW Activation/Deactivation Command MAC CE;

-	PUCCH spatial relation Activation/Deactivation for multiple TRP PUCCH repetition MAC CE;

-	PUCCH Power Control Set Update for multiple TRP PUCCH repetition MAC CE;

-	Unified TCI States Activation/Deactivation for UE-specific PDSCH MAC CE;

-	Differential Koffset MAC CE;

-	Case-7 Timing advance offset MAC CE;

-	DL TX Power Adjustment MAC CE;

-	Child IAB-DU Restricted Beam Indication MAC CE;

-	Timing Case Indication MAC CE.

---------------------- NEXT MODIFIED SECTION----------------------

### 5.18.5	Indication of TCI state for UE-specific PDCCH

The network may indicate a TCI state for PDCCH reception for a CORESET of a Serving Cell or a set of Serving Cells configured in simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2 by sending the TCI State Indication for UE-specific PDCCH MAC CE described in clause 6.1.3.15 or cross-RRH UE-specific PDCCH MAC CE described in clause 6.1.3.XX. The network may also indicate two TCI states for PDCCH reception for a CORESET of a Serving Cell or a set of Serving Cells configured in simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2 by sending the Enhanced TCI States Indication for UE-specific PDCCH MAC CE described in clause 6.1.3.44.

The MAC entity shall:

1>	if the MAC entity receives a TCI State Indication for UE-specific PDCCH MAC CE or  Cross RRH TCI State Indication for UE-specific PDCCH MAC CE on a Serving Cell:

2>	indicate to lower layers the information regarding the TCI State Indication for UE-specific PDCCH MAC CE or  Cross RRH TCI State Indication for UE-specific PDCCH MAC CE.

1>	if the MAC entity receives an Enhanced TCI States Indication for UE-specific PDCCH MAC CE on a Serving Cell:

2>	indicate to lower layers the information regarding the Enhanced TCI States Indication for UE-specific PDCCH MAC CE.

### 5.18.5a	Cross RRH Indication of TCI state for UE-specific PDCCH

In addition to UE behaviour described in 5.18.5:

The MAC entity shall:

stop timeAlignmentTimer associated with the TAG where TCI state MAC CE was received and not perform any uplink transmission on the Serving Cells of associated with TAG;

allow uplink transmission once UE receives a Timing Advance Command;

---------------------- NEXT MODIFIED SECTION----------------------

#### 6.1.3.XX	Cross RRH TCI State Indication for UE-specific PDCCH MAC CE

The Cross RRH TCI State Indication for UE-specific PDCCH MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size of 16 bits with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits. If the indicated Serving Cell is configured as part of a simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2 as specified in TS 38.331 [5], this MAC CE applies to all theServing Cells in the set simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2, respectively;

-	CORESET ID: This field indicates a Control Resource Set identified with ControlResourceSetId as specified in TS 38.331 [5], for which the TCI State is being indicated. In case the value of the field is 0, the field refers to the Control Resource Set configured by controlResourceSetZero as specified in TS 38.331 [5]. The length of the field is 4 bits;

-	TCI State ID: This field indicates the TCI state identified by TCI-StateId as specified in TS 38.331 [5] applicable to the Control Resource Set identified by CORESET ID field. If the field of CORESET ID is set to 0, this field indicates a TCI-StateId for a TCI state of the first 64 TCI-states configured by tci-StatesToAddModList and tci-StatesToReleaseList in the PDSCH-Config in the active BWP or by dl-OrJoint-TCI-State-ToAddModList and dl-OrJoint-TCI-State-ToReleaseList in the PDSCH-Config in the active BWP or the reference BWP. If the field of CORESET ID is set to the other value than 0, this field indicates a TCI-StateId configured by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList in the controlResourceSet identified by the indicated CORESET ID. The length of the field is 7 bits.

Figure 6.1.3.XX-1: Cross RRH TCI State Indication for UE-specific PDCCH MAC CE

---------------------- NEXT MODIFIED SECTION----------------------

## 6.2	Formats and parameters

### 6.2.1	MAC subheader for DL-SCH and UL-SCH

The MAC subheader consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1, 6.2.1-1c and 6.2.1-2 for the DL-SCH and UL-SCH respectively. There is one LCID field per MAC subheader. The size of the LCID field is 6 bits. If the LCID field is set to 34, one additional octet is present in the MAC subheader containing the eLCID field and follow the octet containing LCID field. If the LCID field is set to 33, two additional octets are present in the MAC subheader containing the eLCID field and these two additional octets follow the octet containing LCID field;

NOTE 1:	For MBS broadcast, a logical channel is identified based on G-RNTI and LCID if the same LCID is allocated for logical channels corresponding to different G-RNTIs.

-	eLCID: The extended Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE as described in tables 6.2.1-1a, 6.2.1-1b, 6.2.1-2a and 6.2.1-2b for the DL-SCH and UL-SCH respectively. The size of the eLCID field is either 8 bits or 16 bits.

NOTE 2:	The extended Logical Channel ID space using two-octet eLCID and the relevant MAC subheader format is used, only when configured, on the NR backhaul links between IAB nodes or between IAB node and IAB Donor, or for multicast MTCHs.

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to fixed-sized MAC Ces, padding, and MAC SDUs containing UL CCCH. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	R: Reserved bit, set to 0.

The MAC subheader is octet aligned.

Table 6.2.1-1 Values of LCID for DL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | CCCH |
| 1–32 | Identity of the logical channel of DCCH, DTCH and multicast MTCH |
| 33 | Extended logical channel ID field (two-octet eLCID field) |
| 34 | Extended logical channel ID field (one-octet eLCID field) |
| 35–46 | Reserved |
| 47 | Recommended bit rate |
| 48 | SP ZP CSI-RS Resource Set Activation/Deactivation |
| 49 | PUCCH spatial relation Activation/Deactivation |
| 50 | SP SRS Activation/Deactivation |
| 51 | SP CSI reporting on PUCCH Activation/Deactivation |
| 52 | TCI State Indication for UE-specific PDCCH |
| 53 | TCI States Activation/Deactivation for UE-specific PDSCH |
| 54 | Aperiodic CSI Trigger State Subselection |
| 55 | SP CSI-RS/CSI-IM Resource Set Activation/Deactivation |
| 56 | Duplication Activation/Deactivation |
| 57 | Scell Activation/Deactivation (four octets) |
| 58 | Scell Activation/Deactivation (one octet) |
| 59 | Long DRX Command |
| 60 | DRX Command |
| 61 | Timing Advance Command |
| 62 | UE Contention Resolution Identity |
| 63 | Padding |

Table 6.2.1-1a Values of two-octet eLCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to (216 – 1) | 320 to (216 + 319) | Identity of the logical channel |

Table 6.2.1-1b Values of one-octet eLCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 226 | 64 to 290 | Reserved |
| 227 | 291 | Serving Cell Set based SRS TCI State Indication MAC CE |
| 228 | 292 | SP/AP SRS TCI State Indication MAC CE |
| 229 | 293 | BFD-RS Indication MAC CE |
| 230 | 294 | Differential Koffset |
| 231 | 295 | Enhanced Scell Activation/Deactivation MAC CE with one octet Ci field |
| 232 | 296 | Enhanced Scell Activation/Deactivation MAC CE with four octet Ci field |
| 233 | 297 | Unified TCI States Activation/Deactivation MAC CE |
| 234 | 298 | PUCCH Power Control Set Update for multiple TRP PUCCH repetition MAC CE |
| 235 | 299 | PUCCH spatial relation Activation/Deactivation for multiple TRP PUCCH repetition MAC CE |
| 236 | 300 | Enhanced TCI States Indication for UE-specific PDCCH |
| 237 | 301 | Positioning Measurement Gap Activation/Deactivation Command |
| 238 | 302 | PPW Activation/Deactivation Command |
| 239 | 303 | DL Tx Power Adjustment |
| 240 | 304 | Timing Case Indication |
| 241 | 305 | Child IAB-DU Restricted Beam Indication |
| 242 | 306 | Case-7 Timing advance offset |
| 243 | 307 | Provided Guard Symbols for Case-6 timing |
| 244 | 308 | Provided Guard Symbols for Case-7 timing |
| 245 | 309 | Serving Cell Set based SRS Spatial Relation Indication |
| 246 | 310 | PUSCH Pathloss Reference RS Update |
| 247 | 311 | SRS Pathloss Reference RS Update |
| 248 | 312 | Enhanced SP/AP SRS Spatial Relation Indication |
| 249 | 313 | Enhanced PUCCH Spatial Relation Activation/Deactivation |
| 250 | 314 | Enhanced TCI States Activation/Deactivation for UE-specific PDSCH |
| 251 | 315 | Duplication RLC Activation/Deactivation |
| 252 | 316 | Absolute Timing Advance Command |
| 253 | 317 | SP Positioning SRS Activation/Deactivation |
| 254 | 318 | Provided Guard Symbols |
| 255 | 319 | Timing Delta |
| 256 | 320 | Cross RRH TCI State Indication for UE-specific PDCCH |

Table 6.2.1-1c Values of LCID for MBS broadcast on DL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | MCCH |
| 1–32 | Identity of the logical channel of broadcast MTCH |
| 33–63 | Reserved |

Table 6.2.1-2 Values of LCID for UL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]), except for a RedCap UE |
| 1–32 | Identity of the logical channel of DCCH and DTCH |
| 33 | Extended logical channel ID field (two-octet eLCID field) |
| 34 | Extended logical channel ID field (one-octet eLCID field) |
| 35 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]) for a RedCap UE |
| 36 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]) for a RedCap UE |
| 37–42 | Reserved |
| 43 | Truncated Enhanced BFR (one octet Ci) |
| 44 | Timing Advance Report |
| 45 | Truncated Sidelink BSR |
| 46 | Sidelink BSR |
| 47 | Reserved |
| 48 | LBT failure (four octets) |
| 49 | LBT failure (one octet) |
| 50 | BFR (one octet Ci) |
| 51 | Truncated BFR (one octet Ci) |
| 52 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]), except for a RedCap UE |
| 53 | Recommended bit rate query |
| 54 | Multiple Entry PHR (four octets Ci) |
| 55 | Configured Grant Confirmation |
| 56 | Multiple Entry PHR (one octet Ci) |
| 57 | Single Entry PHR |
| 58 | C-RNTI |
| 59 | Short Truncated BSR |
| 60 | Long Truncated BSR |
| 61 | Short BSR |
| 62 | Long BSR |
| 63 | Padding |

Table 6.2.1-2a Values of two-octet eLCID for UL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to (216 – 1) | 320 to (216 + 319) | Identity of the logical channel |

Table 6.2.1-2b Values of one-octet eLCID for UL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 228 | 64 to 292 | Reserved |
| 229 | 293 | Enhanced Multiple Entry PHR for multiple TRP (four octets Ci) |
| 230 | 294 | Enhanced Multiple Entry PHR for multiple TRP (one octets Ci) |
| 231 | 295 | Enhanced Single Entry PHR for multiple TRP |
| 232 | 296 | Enhanced Multiple Entry PHR (four octets Ci) |
| 233 | 297 | Enhanced Multiple Entry PHR (one octets Ci) |
| 234 | 298 | Enhanced Single Entry PHR |
| 235 | 299 | Enhanced BFR (one octet Ci) |
| 236 | 300 | Enhanced BFR (four octet Ci) |
| 237 | 301 | Truncated Enhanced BFR (four octet Ci) |
| 238 | 302 | Positioning Measurement Gap Activation/Deactivation Request |
| 239 | 303 | IAB-MT Recommended Beam Indication |
| 240 | 304 | Desired IAB-MT PSD range |
| 241 | 305 | Desired DL Tx Power Adjustment |
| 242 | 306 | Case-6 Timing Request |
| 243 | 307 | Desired Guard Symbols for Case 6 timing |
| 244 | 308 | Desired Guard Symbols for Case 7 timing |
| 245 | 309 | Extended Short Truncated BSR |
| 246 | 310 | Extended Long Truncated BSR |
| 247 | 311 | Extended Short BSR |
| 248 | 312 | Extended Long BSR |
| 249 | 313 | Extended Pre-emptive BSR |
| 250 | 314 | BFR (four octets Ci) |
| 251 | 315 | Truncated BFR (four octets Ci) |
| 252 | 316 | Multiple Entry Configured Grant Confirmation |
| 253 | 317 | Sidelink Configured Grant Confirmation |
| 254 | 318 | Desired Guard Symbols |
| 255 | 319 | Pre-emptive BSR |