3GPP TSG-RAN WG2 Meeting #123	R2-2307771

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.4.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Further analysis on S-CPAC signalling procedures, Configurations and security issues

WID/SID:	Mobility Enhancements

Document for:	Discussion and Decision

# Introduction

In the following, the previous RAN2 meeting agreements are shared and the agreements relevant to the content of this paper are highlighted below.

RAN2#121 meeting

Assume to support the following scenarios of SCG selective activation:

    - SN initiated intra-SN SCG selective activation

    - MN initiated inter-SN SCG selective activation

    - SN initiated inter-SN SCG selective activation

For inter-SN CPC, MN should provide the reference configuration to all candidate T-SNs (in order to generate the T-SN candidate configuration).

In RAN2#121 bis-e meeting

For MN initiated inter-SN SCG selective activation, source MN generates the execution conditions for the initial CPAC.

- FFS on the following options for subsequent CPC:

- Option 1: Source MN generates the execution conditions for all subsequent CPC.

- Option 2: Candidate SN may generate execution conditions for subsequent CPC.

For SN initiated inter-SN SCG selective activation, source SN generates the execution conditions for the initial CPC. 
FFS if Candidate SN may generate/modify execution conditions for subsequent CPC

Assume for now that there is only one reference configuration.

The following may be included in the initial RRC reconfiguration message containing the Rel-18 CPC configurations:

- Reference SCG configuration (Optionality FFS). Assume as for LTM Reference configuration may be empty.

- FFS whether MCG configuration is included.

- FFS RRC model for the reference configuration.

- Initial List of candidate target PSCells (this list can be updated by the network, e.g., cells may be added or removed) with associated target SCG configurations. FFS whether the MCG configurations associated with the target SCG configurations are included.

- 3. The execution conditions associated with each candidate target PSCell.

- a.	For MN initiated procedure, execution conditions based on event A4 are supported. FFS whether A3/A5 are supported.

RAN2#122 meeting

For SN-initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC.

FFS if it shall be possible to do something like MN-initiated CPA/CPC where Candidate SN generate execution conditions for subsequent CPC

The reference configuration is provided to all candidates involved in the preparation, FFS which node initially generates it. Assume it can be provided in MN-initiated and in SN initiated procedures.

In this document we further analyze the remaining issues related to measurements for subsequent execution, configuration aspects, security key management and failure handling for S-CPAC.

# 2	Discussion

## 2.1	RRC Configuration Structure for S-CPAC

In the running CR, the conditional-reconfiguration structure of Rel-16 is extended to include variations for S-CPAC. The S-CPAC configurations require many additional parameters and changes that are unique to S-CPAC configurations only. Extending the conditional-reconfiguration structure for all these parameters with conditional applicability for S-CPAC requires additional clarifications in the specification. Moreover, S-CPAC may need to be evolved for other scenarios like MCG in later release.  Hence we propose separate information element at the level of CHO and DAPS is introduced for S-CPAC.

Proposal 1:  New Information element is introduced that contains all the configurations related to S-CPAC is introduced in the running CR. The proposed RRC Configuration model is given below.

## 2.2 	Measurement Configuration and Execution Conditions for S-CPAC

The CPC configurations of the prepared target PSCells are associated with CPC execution conditions that are set by the initiating node for the first CPAC execution. The CPC condition refers to measurement ID associating a measurement object with a reporting configuration. The reporting configuration defines the parameters, i.e., offset, TTT, of the conditional event, e.g., condeventA3 or condeventA5, whereas the measurement object defines the Cell Individual Offset (CIO) that are considered in the evaluation of the CPC conditions.

In case of S-CPAC, the CPC conditions are determined by the source SN and the parameters of the conditional events as well as the CIOs will be set (by the network) to optimize the mobility of the UE from source PSCell to other target PSCells. After the PSCell change, for the subsequent CPAC  the measurement configuration and reporting configurations for CPAC evaluation should use different CIOs and parameters for the conditional events given that the radio propagations between the new serving PSCell and its neighbours are different from those corresponding to previous source PSCell.

Observation 1: After the PSCell change, the measurements and reporting configurations for the conditional events (such as CIO ,TTT, Event thresholds) should be determined by the new serving cell  given that the radio propagations between the new serving PSCell and its neighbours are different from those corresponding to previous source PSCell.

There could be three alternative ways to configure the UE with proper measurement configurations and conditions for subsequent PSCell change.

One solution would be to update the CPC conditions after the UE performs the PSCell change. That is the new serving PSCell updates the CPC conditions associated with the retained CPC configurations. In this way, the new serving PSCell ensures that the UE uses the optimal CIOs and parameters for the conditional events at the expense of delayed measurements and reconfiguring the UE by means of RRC Reconfiguration message after each PSCell change. As it was agreed in RAN2#119bis-e meeting, after finishing the PSCell addition or change, the UE should continue evaluating the execution conditions of other candidate PSCells.

Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PScells.

Observation 2: Updating the CPC conditions by means of RRC reconfiguration after each cell change causes delayed evaluation and radio signalling overhead.  This option is possible using existing signalling procedures.

Alternative 1: One alternative would be to provide the measurement configurations and the execution conditions outside of each CPAC preparation configuration (outside of the condRRCReconfig). Each candidate cell provides update to the common measurement configuration as required as part of S-CPAC preparation. This information is included as additional parameter for each candidate cell in the S-CPAC configuration.

In that way, the UE stores the S-CPAC measurement configuration and execution conditions in UE variables and associates them with each condReconfigId. This can be achieved by the coordination of MN among the SNs. This will provide the following benefits

- The UE can easily activate/deactivate the measurement configurations and conditions and continue evaluating the conditions right after finishing one CPAC execution (as the configurations will be associated with the condReconfigId).

- When a new S-CPAC PSCell is prepared, the measurement configurations and conditions of the previous preparations can be updated without updating the whole CPAC preparation configurations.

- It will reduce the size of the overall preparation messaging as the MN can create a common list of configurations and share that with UE. In that case, the MN will configure the UE only with the differences between each S-CPAC preparation (i.e., common list of MeasObj and ReportConfig can be used but the association of those with each CPAC preparation after each CPAC execution will be different).

Alternative 2: Delta configuration over existing preparation configuration (the preparation configuration can be either delta or complete) is another alternative to configure the UE with proper measurement configurations and execution conditions among the S-CPAC. Herein, the target SNs’ of the previously prepared PSCells can be requested to provide measurement configurations and the conditions for newly prepared PSCell. Those configurations will be provided to the UE in association with the previous S-CPAC preparations in the form of delta configuration. Once the UE executes a S-CPAC preparation,

- If the S-CPAC preparation configuration is complete configuration, the UE will apply the delta configuration (received for the newly added cell) over the complete configuration to update the measurement configurations and conditions inside the executed S-CPAC preparations.

- If the S-CPAC preparation configuration is a delta configuration (to be used along with the reference configuration), the UE will apply the delta configuration (received for the newly added cell) over the existing delta configuration to update the measurement configurations and conditions inside the executed S-CPAC preparations.

Considering the observations above, alternative 1 and alternative 2 are the solutions that can be considered as promising solutions. Due to the fact that the alternative 1 provides more flexibility and less complexity on the UE side, alternative 1 can be considered as favourable solution for S-CPAC measurement and execution condition configurations. RAN2 should consider the advantages and disadvantages of each alternative and down select one of the alternative solutions for S-CPAC measurement and execution condition configurations.

Proposal 2.A: RAN2 down select between following options for S-CPAC measurement and execution condition configurations.

- Alternative 1 :Separate Measurement-Config for SCPAC related measurements.

- Altennative 2: Delta configuration for measurement-config with support for incremental changes to conditional-configuration.

When the UE execute the retained configurations subsequently, using A4 event may lead unnecessary/unstable handovers as A4 event does not compare the serving cell against any neighbour cell. On the other hand, the MN cannot configure any A3/A5 events for any CPAC execution as the UE cannot identify whether the SpCell refers to PCell or PSCell (and by default the UE assumes that SpCell is PCell if it is configured by MN). As it was already agreed for SN initiated inter-SN S-CPAC where the candidate SNs determine the conditions for subsequent PSCell change, the same approach can be adopted here, i.e., in MN initiated inter-SN S-CPAC, the candidate SNs determine the execution conditions of subsequent PSCell change. Hence, the execution of an PSCell change based on A4 will be followed by A3 event which mitigates the unnecessary/unstable handovers.

Proposal 2.B: in MN initiated inter-SN S-CPAC, the candidate SNs determine the execution conditions of subsequent PSCell change.

In the case that the candidate SNs determine the execution conditions of subsequent PSCell change for both MN and SN initiated inter-SN S-CPACs, then the alternative 2 and 3 on measurement and execution condition configurations for subsequent PSCell change can be also considered here. Thereby, RAN2 should consider a common solution for both MN initiated and SN initiated inter-SN S-CPAC measurement and execution condition configurations for subsequent PSCell change.

Proposal 2.C: RAN2 should consider a common solution for both MN initiated and SN initiated inter-SN S-CPAC measurement and execution condition configurations for subsequent PSCell change.

The proposed RRC-Configuration message structure for Alternative-2 and Alternative 3 is illustrated below. The proposed RRC Reconfiguration changes for Alternative-3 is given below.

In alternative 3 the candidate configuration will include the updates to the Meas-config object for subsequent cell changes and provides as complete delta-configuration over reference configuration. OR it can only include Meas-Config changes in this case the RRC message size is minimised when these objects are updated for addition of new candidate cells to S-CPAC.

Proposal 2.D: The RRC Reconfiguration corresponds to the selected alternative solution indicated in the above Figure is considered as basis for TS38.331 changes for running CR.

## 2.3 	S-CPAC Reference Configuration Model

RAN 122 meeting had the following agreement:

(Agreement 1) The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN-initiated procedures.

RAN 121e meeting had the following agreements:

(Agreement 2) Assume for now that there is only one reference configuration.

The following may be included in the initial RRC reconfiguration message containing the Rel-18 CPC configurations:

- (Agreement 3) Reference SCG configuration (Optionality FFS). Assume as for LTM Reference configuration may be empty.

- FFS whether MCG configuration is included.

RAN 121 meeting had the following agreement:

(Agreement 4) For inter-SN CPC, MN should provide the reference configuration to all candidate T-SNs (in order to generate the T-SN candidate configuration).

The endorsed CR for TS 37.340 for selective activation has the following note captured:

NOTE 3a: In case that a MN initiated conditional reconfiguration (e.g. CHO, MN initiated inter-SN CPC or MN initiated inter-SN subsequent CPAC) is prepared, and if any execution of a prepared SN initiated intra-SN CPC or SN initiated intra-SN subsequent CPAC procedure or reconfiguration of the SCG, the SN notifies to the MN via the SN Modification Required message. In this case, the steps 2 and 3 are skipped.

Under the light of these agreements we propose how the reference configuration concept should work. Combining the agreement 2 and agreement 4. It is clear that the MN should be the anchor point for the reference configuration and it is the only anchor point to have a single reference configuration. This we discuss in section 2.XX..

As the MN is the anchor for the reference point we should discuss how the reference is propagated in the intra-SN selective activation configuration. This we discuss in section 2.XX.

Finally, the FFS (whether MCG configuration is included (in the reference config)) is crucial to decrease the interruption time related to selective activation execution. This we discuss in section 2.XX.

### 2.3.1	MN as anchor point for reference configuration

As the agreement 2 indicates that there is only one reference configuration, and the MN should provide it to target SNs. Furthermore, the TS 38.331 running CR captures a case where intra-SN S-CPAC is triggered after inter-SN S-CPAC is provided to the UE. This is not captured in previous agreements.

In case co-existence between different S-CPAC procedures such as MN initiated, SN initiated S-CPAC and intra-SN S-CPAC need to be supported at the same time there needs to be a synchronization between MN and SN for the reference configuration.

In order to fulfil those requirements, there should be an anchor point where each entity contacts the anchor to guarantee that there is never more than one reference configuration.

Observation 3: In case the co-existence between different S-CPAC procedures such as MN initiated, SN initiated S-CPAC and intra-SN S-CPAC, an anchor point for reference configuration needs to be defined to guarantee that UE is never configured with more than one reference configuration.

Before proceeding with the anchor decision it would be good to determine the co-existence of multiple procedures. To allow flexible extension to existing procedures, we think MN-initiated and SN-initiated inter-SN S-CPAC and intra-SN S-CPAC co-existence should be supported.

Proposal 3: Support the co-existence of MN-initiated S-CPAC, SN-initiated inter-SN S-CPAC, and intra-SN S-CPAC.

The procedure would be simplified if MN is selected as the anchor point for the reference configuration. MN is involved in the preparation for MN and SN-initiated inter-SN CPC preparations. For intra-SN CPC configuration this may require new interaction with MN to guarantee the existence of reference configuration at the SN at the time if intra-SN CPC preparation.

Proposal 4.A : MN is selected as the anchor point responsible to guarantee that only one reference configuration is provided to the UE.

It is important to note that the anchor point indicates the entity responsible to indicate the reference configuration. But the entity responsible for reference configuration generation has to be agreed upon separately. In order to generate the reference configuration, the entity needs to be aware of the source configuration and the candidate configurations.

As MN is selected as the anchor point for the reference configuration it makes sense that MN is responsible for generating and updating the reference configuration to avoid any race condition. For example, the MN may indicate the reference configuration to a candidate SN, if the source SN is responsible for generating and updating the reference configuration it may be updated while MN provided the reference configuration, which will create the race condition.

Proposal 4.B: MN is responsible for generating the reference configuration to avoid any race conditions.

Considering that MN generates the reference configuration and acts as the anchor for the reference configuration, considering intra-SN S-CPAC the source SN has to coordinate with MN to obtain the reference configuration.

Proposal 5: Source SN has to coordinate with MN to obtain the reference configuration for intra-SN S-CPAC.

The reference configuration in LTM mainly focuses on the MCG configuration. The reference configuration in S-CPAC is related to the SCG configuration. Some of the information elements of the RRC configuration may be beneficial to not include in the reference configuration if this is not expected to be modified by the target cell configuration such as DRB configuration or measurement configuration. Reference configuration covering a full configuration may lead to unnecessary signaling.

Proposal 6: RAN2 to discuss if reference configuration is a full configuration or some information elements can be taken out considering the S-CPAC configuration to be mainly focusing on SCG configuration.

## 2.4 	SN Security Counter List Management

The solution provided for security key change for S-CPAC [2] proposes that UE and SK-counter list for each involved SN is provisioned to UE as part of RRC Configuration preparing S-CPAC. UE make use of unused SN-counter value from this list for every Inter-SN change scenario. As the key change is only applicable for SN change scenarios, additional information is required at UE to identify the group of candidate cells belonging to the same SN. This can be achieved by assigning SK-change-group-ID to each candidate configuration. Based on this group-ID UE will know whether it changes across the group that requires SK-counter update.

Proposal 7A: SK-Update-Group-Id is assigned to each candidate configuration of S-CPAC for security-key management to identify the cell-change across SN

As indicated in the SA3 solution SK-counter list should be maintained for each group ID that can be referred to assign a new S-KeNB for the Inter-SN cell change scenario. As this list is for a group of candidate cells it has to be maintained as a common configuration for S-CPAC as similar to the reference configuration.

Proposal 7B: SK-Counter-List information is maintained as common parameter for S-CPAC similar to the reference configuration.

It is upto network implementation to provide a large list of SK-counter values for each involved SN in configuration or update the next set of counter values after the Inter-SN change scenario. As the counter list update via RRC configuration does not impact the ongoing CPAC evaluations and reconfigurations use of RRC configuration to update the counter list after cell change is not restricted.

Proposal 7C: Update of the SK-counter list using RRC Reconfiguration is allowed including the Inter-SN cell change scenario.

If Rel-17 CPAC is configured along with S-CPAC the MN should ensure that the SK-counter value assigned for CPAC is uniquely different from the SK-counter list assigned for S-CPAC purpose even though the target cell belongs to the same SN.

Proposal 8:  SK-Counter values assigned for Rel-17 CPAC is uniquely different from the SK-counter list assigned for S-CPAC.

The security solution proposed for S-CPAC requires the change in SK-counter information exchange between MN and SN related to S-CPAC preparation and execution. The LS from SA3 is not sent to RAN3. When RAN2 concludes on the key aspects of SK-counter management, RAN2 should inform RAN3 about the required interface changes for the new security key management solution.

Proposal 9:  RAN2 to send LS to RAN3 once RAN2 made agreements on SK counter handling for S-CPAC to start MN-SN interface changes required for S-CPAC procedures.

Whether S-CPAC configurations are to be maintained across PCell change for Intra-MN scenario is yet to be confirmed within RAN2.  If S-CPAC configurations are to be maintained across PCell-change and if the PCell-change also involve master-key update due to PDCP anchor point changes, whether SK counter list assigned prior to PCell change can be maintained or new configuration is needed requires further analysis from SA3.

Proposal 10:  If S-CPAC maintenance after PCell change is supported further discussion is needed with SA3 on the SK-Counter management in this scenario (For the case where master-key update is also needed as part of Pcell-change).

## 2.5	PDCP Re-establishment control in S-CPAC

During mobility scenarios of CPAC whether the PDCP needs to be re-established or not is decided based on whether PDCP anchor point changes as part of configuration. The PDCP anchor point changes happen for bearer-remapping scenarios where bearers are moved between MCG and SCG as part of reconfiguration. In other case, if the CPC involves inter-SN change or intra-SN change with CU-UP anchor point changes the PDCP re-establishment will be required.

This parameter is defined as action parameter and not stored in RRC configuration. The action depends on whether any of the above scenario occurs during cell change. For S-CPAC case the mobility to cell can happen from any of the candidate-cell. In this case configuring the PDCP Re-establishment based on the involved cells is required to avoid interruption due to unnecessary PDCP Re-establishments. The solutions for this problem are analysed in below sections.

### 2.5.1	Bearer Remapping Scenario

In case the bearers are moved from MN to SN (or vice-versa) the key in use for the moved bearers need to be changed, i.e., if a bearer is on MN, it will use MN key and if it is moved to SN, the key that will be used will be SN key after it is moved to SN. In case the keys for some bearers are changed the PDCP has to be re-established.

Thus, when UE is initially moving some bearers from MN to SN for a S-CPAC execution, the PDCP has to be setup from scratch per moved DRB. This is configured in the CG configuration with the parameter PDCP re-establishment flag being set to True per moved DRB.

However, in case of S-CPAC in the following scenarios the subsequent cell change is subject to unnecessary bearer remapping (PDCP re-establishment) for both MCG and SCG bearers which leads into unnecessary interruption.

- Inter-SN CPC (bearer mapping initially required) followed by intra-SN CPC (bearer mapping not required after initial inter-SN CPC).

- Inter-SN CPC (bearer mapping initially required) followed by inter-SN CPC (same bearer mapping is not required, different bearer mapping may be required).

- CPA (bearer mapping from MN to SN required) followed by intra-SN CPC (bearer mapping not required after inter-SN CPC)

- CPA (bearer mapping from MN to SN required) followed by inter-SN CPC (same bearer mapping is not required, different bearer mapping may be required).

- CPA to CPC conversion (CPA requires bearer mapping from MN to SN, however, for the CPC converted from CPA same bearer mapping is not required, different bearer mapping may be required).

We see two possible alternatives to avoid the unnecessary bearer re-mapping (PDCP re-establishment) the above cases.

Network can provide a S-CPAC-DRB bitmap along with the S-CPAC preparations for each DRB that associates the S-CPAC preparations and the bearer remapping. This CPAC and Bearer mapping table info of CG can be communicated in RRCReconfiguration message to UE along with the S-CPAC preparation configurations. Hence, the UE can avoid any unnecessary bearer remapping (and unnecessary interruption) if the bitmap indicates no bearer remapping among the subsequent PSCell changes.

Alternative-1: CPAC-DRB bitmap along with the S-CPAC preparation should be provided to the UE to indicate UE the mapping (PDCP re-establishment) of the bearer along with the subsequent PSCell change.

The unnecessary bearer remapping (PDCP re-establishment) can also be avoided using CG-bearer-config-table shared in RRCReconfiguration and Target-CG-bearer-Information as delta (based on CG-bearer-config-table ) for the corresponding Target Candidate PSCell.CG-Bearer-config-Table is included in the S-CPAC configuration that consists of all the CG bearer configurations including the bearers that are offloaded from CG during S-CPAC preparation. During S-CPAC execution, the UE generates the CG-Delta-configuration based on identification of bearers to be modified based on its current bearer configuration and CG-bearer-config-Table and the Target-CG-bearer-Information.

Alternative-2: CG-bearer-config-Table and Target-CG-bearer-Information (of corresponding Candidate PSCell) in RRCReconfiguration message as part of CPAC preparation to UE in order to derive the CG-Delta-configuration to be modified during PSCell change by using the foresaid tables and current bearer configuration.

Proposal 11: RAN2 should discuss alternative 1 and alternative 2 and down-select one of the solutions to avoid unnecessary re-establishment during bearer remapping in S-CPAC.

### 2.5.2 PDCP Anchor point changes without bearer remapping

Candidate cells of S-CPAC can consist of Intra SN and Inter-SN cells. Within Intra-SN cells also some of the candidate cells may belong to the same PDCP anchor point.  In this scenario of candidate cells belonging to different PDCP anchor points the PDCP re-establishment flag needs to be only set for cell change happening across cells belonging to different anchor point.

As the cell-change can happen from any candidate cell this parameter cannot be set statically in the candidate configuration during S-CPAC preparation.  Instead, new parameter in each candidate configuration to associate the candidate-cell to PDCP anchor point should be introduced and this parameter can be used to decide on PDCP-Reestablishment flag dynamically at the time of cell-change. LTM candidate configurations introduces such ID to identify whether L2-Reset is needed or not.

Proposal 12: Group-ID for PDCP-Re-establishment is introduced for each candidate configuration to identify PDCP anchor point change for S-CPAC.

## 2.6 	SCG Failure Handling

### 2.6.1	S-CPAC Recovery for S-RLF

In case of SCG failure (e.g. RACH failure), in Rel. 17, even if the MCG is not affected, the UE does not evaluate the rest of the configurations. However, if the MCG part of the configuration is common among the S-CPACs, in case of failure during the Conditional PSCell Addition, the UE could be enabled with a recovery mechanism, where after a failed SN addition attempt, other configurations with the same MCG part could be evaluated. The UE does not need to send the SCG Failure Information immediately and the MN informs the UE about the configurations that have a common MCG.

Proposal 13: In case the MCG part of the (S)CPAs is common among a subset of (S)CPAs, after a failed SN addition attempt, other configurations with the same MCG part could be evaluated.

### 2.6.2	Reconfiguration Failure Handling

After being configured with S-CPACs, if the execution condition of any of them is met, the UE will apply the corresponding configuration. However, in case of compliance error if the MCG part is affected, the UE is instructed to do a reestablishment and if the MCG part is not affected, the rest of the configurations will not be evaluated. The UE has to wait for further NW instructions, which might be a complete repreparation of all of the configurations.

In case the MCG part is not affected, the UE can be enabled a recovery mechanism, where after the compliance failure, the non-compliant configuration is discarded, but UE retains the rest of the configurations and continue to evaluate them. The UE can inform the NW about the failure and indication that the recovery is in progress. To avoid SN security issues, the UE will only use the SN security counter after the compliance check is successful.

Proposal 14: After a reconfiguration failure that is caused by applying one of the S-CPACs, the UE retains the rest of the S-CPACs and continues to evaluate them.

### 2.6.3	 SCG Failure Information Enhancement

If the UE is configured with S-CPACs via SN and without MN involvement, the SCG failure information message can be enhanced to inform the MN about the ongoing executions, prepared configurations, and/or latest measurement results. Based on this information the MN can react immediately by ordering the UE to execute immediate PSCell change to a prepared or non-prepared PSCell. With this enhanced SCG failure information the MN can also release the resources of the candidate PSCells that are not considered anymore.

Proposal 15: In case the UE is configured with S-CPACs via SN and without MN involvement, after an SCG failure, the UE sends SCG failure information with additional information about the candidate PSCells so that the MN orders the UE to perform PSCell change immediately.

## 2.7	MCG Failure Handling

In Rel-17 Conditional PSCell Change (CPC), a CPC-configured UE has to release the CPC configurations in case of a failure of the PCell. Thus the UE needs to be re-configured from scratch so as to perform CPC. Similarly, the same stands for subsequent CPC without prior CPC reconfiguration and re-initialization from the network.

Specifically, in case of Dual Connectivity (and the UE is configured with S-CPAC), if a Radio Link Failure (RLF) happens in the MCG, the UE will:

- Perform cell re-selection

- Perform RRC Re-establishment to the selected cell.

- Release all the available configurations (both CHO and S-CPAC) (irrespective of CHO recovery).

This means that:

- All the prepared cells will be released both in the UE and the network side

- UE needs to be provided with new configurations for the MN

- DC needs to be established from the beginning.

Observation 4: According to the current specification, in case of a failure in the MCG, the UE needs to re-establish DC from the beginning.

However, the above-described procedure is suboptimum, since the S-CPAC needs to be established from scratch practically, even though the configuration of the target cells may remain unchanged, with the only change that of the security key of the SN(s). It should be noted that in case of CHO recovery the UE does not have to release S-CPAC configurations.

Proposal 16: The UE can be configured to maintain the S-CPAC configurations in case of MCG failure recovery.

# 3	Conclusion

This document has made the following observations and proposals:

RRC Configuration Structure

Proposal 1:  New Information element is introduced that contains all the configurations related to S-CPAC in the running CR. The proposed RRC Configuration model is given in Fig 1.

Measurement configuration changes for S-CPAC

Observation 1: After the PSCell change, the measurements and reporting configurations for the conditional events (such as CIO ,TTT, Event thresholds) should be determined by the new serving cell  given that the radio propagations between the new serving PSCell and its neighbours are different from those corresponding to previous source PSCell.

Observation 2: Updating the CPC conditions by means of RRC reconfiguration after each cell change causes delayed evaluation and radio signalling overhead.  This option is possible using existing signalling procedures.

Proposal 2.A: RAN2 down select between following options for S-CPAC measurement and execution condition configurations.

- Alternative 1 : Separate Measurement-Config for SCPAC-related measurements.

- Alternative 2: Delta configuration for measurement-config with support for incremental changes to conditional configuration.

Proposal 2.B: In MN initiated inter-SN S-CPAC, the candidate SNs determine the execution conditions of subsequent PSCell change.

Proposal 2.C: RAN2 should consider a common solution for both MN-initiated and SN-initiated inter-SN S-CPAC measurement and execution condition configurations for subsequent PSCell change.

Proposal 2.D: The RRC Reconfiguration corresponds to the selected alternative solution indicated in the above Figure (Figure 2) is considered as basis for TS38.331 changes for running CR.

Reference configuration for S-CPAC

Observation 3: In case the co-existence between different S-CPAC procedures such as MN initiated, SN initiated S-CPAC and intra-SN S-CPAC, an anchor point for reference configuration needs to be defined to guarantee that UE is never configured with more than one reference configuration.

Proposal 3: Support the co-existence of MN-initiated S-CPAC, SN-initiated inter-SN S-CPAC, and intra-SN S-CPAC.

Proposal 4.A : MN is selected as the anchor point responsible to guarantee that only one reference configuration is provided to the UE.

Proposal 4.B: MN is responsible for generating the reference configuration to avoid any race conditions.

Proposal 5: Source SN has to coordinate with MN to obtain the reference configuration for intra-SN S-CPAC.

Proposal 6: RAN2 to discuss if reference configuration is a full configuration or some information elements can be taken out considering the S-CPAC configuration to be mainly focusing on SCG configuration.

Security Key Management for S-CPAC

Proposal 7A: SK-Update-Group-Id is assigned to each candidate configuration of S-CPAC for security-key management to identify the cell-change across SN.

Proposal 7B: SK-Counter-List information is maintained as common parameter for S-CPAC similar to the reference configuration.

Proposal 7C: Update of the SK-counter list using RRC Reconfiguration is allowed including the Inter-SN cell change scenario.

Proposal 8:  SK-Counter values assigned for Rel-17 CPAC is uniquely different from the SK-counter list assigned for S-CPAC.

Proposal 9:  RAN2 to send LS to RAN3 once RAN2 made agreements on SK counter handling for S-CPAC to start MN-SN interface changes required for S-CPAC procedures.

Proposal 10:  If S-CPAC maintenance after PCell change is supported further discussion is needed with SA3 on the SK-Counter management in this scenario (For the case where master-key update is also needed as part of Pcell-change).

PDCP Re-establishment control for S-CPAC

Proposal 11: RAN2 should discuss alternative 1 and alternative 2 and down-select one of the solutions to avoid unnecessary re-establishment during bearer remapping in S-CPAC.

Proposal 12: Group-ID for PDCP-Re-establishment is introduced for each candidate configuration to identify PDCP anchor point change for S-CPAC.

SCG Failure Handling

Proposal 13: In case the MCG part of the (S)CPAs is common among a subset of (S)CPAs, after a failed SN addition attempt, other configurations with the same MCG part could be evaluated.

Proposal 14: After a reconfiguration failure that is caused by applying one of the S-CPACs, the UE retains the rest of the S-CPACs and continues to evaluate them.

Proposal 15: In case the UE is configured with S-CPACs via SN and without MN involvement, after an SCG failure, the UE sends SCG failure information with additional information about the candidate PSCells so that the MN orders the UE to perform PSCell change immediately.

MCG Failure Handling

Observation 4: According to the current specification, in case of a failure in the MCG, the UE needs to re-establish DC from the beginning.

Proposal 16: The UE can be configured to maintain the S-CPAC configurations in case of MCG failure recovery.

# 4	Reference

[1] RAN2-122 Meeting Report,  Incheon May 2023

[2] S3-233200 LS on Security Solution for Selective SCG