3GPP TSG-RAN WG2 Meeting #123	R2-2307779

Toulouse, France, August 21-25, 2023

Agenda item:	7.11.2.1

Source:	Sharp

Title:	RRC Resume for Multicast in RRC_INACTIVE

WID/SID:	NR_MBS_enh-Core

Document for:	Discussion and Decision

1	Introduction

In this contribution, we discussed the issues related to UE in RRC_INACTIVE configured to receive multicast in RRC_INACTIVE resumes the RRC Connection for Multicast session receiving purpose.

2 Discussion

Currently, RRC Resume procedure can be triggered by: upper layers, RAN paging, RNA updates or data arrive for SDT DRB or SRB. Upon initiating the RRC Resume procedrue, the corresponding Access Catergory determined by the triggering events is used to perfrom the UAC to check whether UE is barred for the access attempt trigged by the events.

For UE in RRC_INACTIVE receiving multicast in RRC_INACTIVE may need to perform the RRC resume procedure for multicast receiving purpose. Currently, following three cases have been agreed to initiate RRC resume procedure for multicast reception:

Case I: if UE in RRC_INACTIVE reselectes to a new cell in which the PTM configuration of the session received in RRC_INACTIVE is not available, UE needs to initiate an RRC Resume procedure to acquire the PTM configuration [1].

Agreements in RAN2#119 Upon cell reselection to neighbour cells during active multicast session, if the configuration of the session is not available for the new cell for UEs in INACTIVE, then the UE is required to resume RRC connection to get the Multicast MRB configuration.

Case II: UE in RRC_INACTIVE may initiate the RRC Resume procedure for multicast reception purpose when the reception quality of the multicast data is below a configured threshold [2].

Agreements in RAN2#121-bis - Upon events like session activation/data transmission resumed, if PTM configuration is not available to UE, UE initiates RRC connection resumption. UE may trigger RRC connection resumption if the reception quality of the multicast data is below a configured threshold, FFS how to specify the threshold/reception quality.

Case III: the RRC resume procedure can also be triggered by enhanced group paging that used to indicate UE should receive multicast in RRC_CONNECTED [2].

Agreements in RAN2#121-bis - Rel-18 UE can stay in RRC_INACTIVE and start monitoring corresponding G-RNTI upon an enhanced group paging (e.g., upon session activation or data transmission resumed). Details FFS. - Upon events like session activation/data transmission resumed, if PTM configuration is not available to UE, UE initiates RRC connection resumption.

Observation 1: RRC Resume procedure for multicast reception can be triggered by:

Case I: UE reselectes to a new cell in which the PTM configuration of the session received in RRC_INACTIVE is not available;

Case II: The reception quality of the multicast data is below a configured threshold;

Case III: Enhanced group paging used to indicate UE should receive multicast in RRC_CONNECTED is received.

Same as other trigger events, UE should determine the Access Category for the RRC Resume procedure triggered for multicast reception purpose. Considering that the RRC Resume procedure triggered by enhanced group paging for multicast reception (i.e., Case III) is also a response to NG-RAN paging, the accss catergory can be set to 0.

For the RRC Resume procedure triggered by case I and II, although both of them are triggered by AS layer as RAN paging. But in current RRC resume procedure triggered by response to RAN paging and RNA updates, different Access Categories are used for access barring checking. In addition, RAN paging used to indicate UE switching to RRC_CONNECTED means NW is not congesting. But for the case that UE reselects to a new cell in which the PTM configuration is not available (Case I), and for the case that the reception quality belows a configured threshold (Case II), UE does not know the state of the NW. So the Access Category 0 for responding to RAN paging can not be reused. And Case I and Case II is similar to RNA updatem, so it is simple to reuse the Access Category 8 defined for RNA updates for case I and case II. Define a new Access Catergory for the event not available of the PTM configuration also works.

Proposal 1: Reuse the Access Catergory 8 or define a new Access Category for the RRC Resume procedure tiggered by UE reselectes to a new cell in which the PTM configuration is not available or triggered by the reception quality of multicast data is below a configured thershold.

It is beneficial to let NW knows the purpose of the RRC Resume request for multicast reception purpose, so that the NW can reply with proper response. For example, if the NW is still in congestion when receiving the RRC Resume for not available of PTM configuration, it can response an RRCRelease message which includes the latest PTM configuration. So, new resume cause should be defined for the RRC resume request triggered by not available of the PTM configuration.

Proposal 2: Define a new resume cause for RRC Resume request triggered for multicast reception in RRC_INACTIVE.

3 Summary

In this section, we summarized our proposals and observation as showed below:

Observation 1: RRC Resume procedure for multicast reception can be triggered by:

Case I: UE reselectes to a new cell in which the PTM configuration of the session received in RRC_INACTIVE is not available;

Case II: The reception quality of the multicast data is below a configured threshold;

Case III: Enhanced group paging used to indicate UE should receive multicast in RRC_CONNECTED is received.

Proposal 1: Reuse the Access Catergory 8 or define a new Access Category for the RRC Resume procedure tiggered by UE reselectes to a new cell in which the PTM configuration is not available or triggered by the reception quality of multicast data is below a configured thershold.

Proposal 2: Define a new resume cause for RRC Resume request triggered for multicast reception in RRC_INACTIVE.

4 References

[1] RAN2#119 chairman’s notes.

[2] RAN2#121bis chairman’s notes.