3GPP TSG-RAN WG2 Meeting #123 	R2-2307780

Toulouse, France, August 21-25, 2023

Agenda item:	7.17.2

Source:	Sharp

Title:	Indication of UE Capability Restriction for eMUSIM

WID/SID:	NR_DualTxRx_MUSIM-Core

Document for:	Discussion and Decision

1	Introduction

In RAN2#121bis meeting [1], RAN2 had following agreement:

Support “early indication” from UE to network during RRC connection setup/resume procedure.

FFS how to indicate this and in which message. The indication will tell network that UE capabilities are temporarily restricted.

FFS on details (i.e. when UE can indicate this, what does it indicate, how does it relate to UAI, etc.)

In this discussion paper, we resolve the FFS.

2 Discussion

The messages transmitted from UE to NW in the RRC connection setup or resume procedure, showed in fig 1, are RRCSetupRequest, RRCResumeRquest (or RRCResumeRquest1), RRCSetupComplete, or RRCResumeComplete:

Fig.1

There is still 1 spare bit in RRCSetupRequest and RRCResumeRquest (or RRCResumeRquest1) which can be used to indicate the capability restriction state of the UE. So, one option is to carry the indication in RRCSetupRequest and RRCResumeRquest (or  RRCResumeRquest1) using the spare bit.

The other option is to carry the indication in RRCSetupComplete and RRCResumeComplete message.

In current RRC setup procedure, configuration of SCG cannot be carried in RRCSetup message. If UE indicates the capability restriction in RRCSetupComplete message, no configuration incompatible problem may happen.

Observation 1 During RRC setup procedure, indicating the capability restriction in RRCSetupComplete message will not lead to configuration incompatible problem.

So, in RRC setup procedure, both RRCSetupRequest and RRCSetupComplete message can be used to indicate the capability restriction.

Proposal 1 In RRC setup procedure, both RRCSetupRequest and RRCSetupComplete message can be used to indicate the capability restriction.

But in current RRC resume procedure, configuration of SCG may be carried in RRCResume message.

If UE does not indicate the capability restriction in RRCResumeRequest or RRCResumeRequest1 message, configuration incompatible problem may happen if network configures SCG for the UE without knowing the UE is in capability restriction state.

Observation 2 During RRC resume procedure, indicating the capability restriction in RRCResumeComplete message may lead to configuration incompatible problem.

If RRCResumeComplete message is used to indicate the capability restriction state, the configuration incompatible issue needs to be resolved. So, it is better to use the RRCResumeRequest or RRCResumeRequest1 message to indicate the capability restriction state.

Proposal 2 In RRC resume procedure, RRCResumeRequest or RRCResumeRequest1 message is used to indicate the capability restriction state.

As RAN2 have already agreed to include the detail of capability restriction in UAI message. So, it is benefit to avoid repeat the capability restriction information in the RRCSetupRequest/RRCSetupCpmplete/RRCResumeReuqest/RRCResumeRequest1/RRCResumeComplete.

Proposal 3 RRCSetupRequest/RRCSetupCpmplete/RRCResumeReuqest/RRCResumeRequest1/RRCResumeComplete is only used to indicates the capability restriction state of the UE and the detail of capability restriction is included in UAI message.

3 Summary

In this section, we summarized our observations and proposals as showed below:

Observation 1 During RRC setup procedure, indicating the capability restriction in RRCSetupComplete message will not lead to configuration incompatible problem.

Observation 2 During RRC resume procedure, indicating the capability restriction in RRCResumeComplete message may lead to configuration incompatible problem.

Proposal 1 In RRC setup procedure, both RRCSetupRequest and RRCSetupComplete message can be used to indicate the capability restriction.

Proposal 2 In RRC resume procedure, RRCResumeRequest (or RRCResumeRequest1) message is used to indicate the capability restriction state.

Proposal 3 RRCSetupRequest/RRCSetupCpmplete/RRCResumeReuqest/RRCResumeRequest1/RRCResumeComplete is only used to indicates the capability restriction state of the UE and the detail of capability restriction is included in UAI message.

4 References

[1] RAN2#121bis chairman’s notes.