3GPP TSG-RAN WG2 Meeting #123	R2-2307782

# Toulouse, France, 21 – 25 August 2023

Agenda item:		7.23.2

Source:	Xiaomi

Title:	5GS network timing synchronization status and reporting

Document for:		Discussion and Decision

Introduction

In RAN2#122 meeting, following was agreed regarding 5GS network timing synchronization status and reporting:

Agreements

- Update of event ID is informed to UE by normal SI modification procedure.

- Confirm the AS layer of the UE determines if there a change of event ID and/or gNB ID. If there is a change, the AS layer notifies the change in the RAN timing synchronization status to NAS layer. For both IDLE and INACTIVE mode, NAS layer may requests the RRC layer to move to RRC_CONNECTED

- For RRC_CONNECTED mode UEs, the NW has the necessary information to determine whether to send detailed clock quality information to the UE, (i.e. it may choose to always send update or only when needed) and the details can be left to NW implementation

- Event ID is optional.  Under the same gNB, UE considers the change of timing synchronization if event ID field from SIB9 is different.   FFS if gNB always broadcasts event ID

- If the UE is in RRC_INACTIVE, the UE can acquire clock quality information using the SDT procedure, if it supports and/is configured with SDT procedure.

In this contribution, we discuss remaining issues of 5GS network timing synchronization status and reporting.

Discussion

UAC

In LS R2-2302106 [1], RAN2 asked CT1 “if we can use an existing access category for the access attempt to retrieve the latest available clock quality information, e.g., the one for MO signalling, or if it would require a new one since it should not block access triggered by other MO signalling.”

In LS R2-2304605 [2], CT1 replied that “Unless there is a service requirement to introduce a new access category for the access attempt to retrieve the latest available clock quality information, CT1 assumes that reusing Access Category 3 (MO_sig) is acceptable.” CT1 further asked SA1 “to inform if there is a service requirement to use a new access category for the access attempt described above.”

In LS R2-2307051 [3], SA1 informed that “there are no stage 1 requirements to have a separate access category for TRS.”

From the replies of CT1 and SA1, it can be seen that Access Category 3 (MO_sig) can be used when UE transits from RRC_IDLE/INACTIVE to RRC_CONNECTED to acquire the detailed clock quality information. In current RRC connection establishment and resume procedure, UE can use the Access Category provided by the upper layers. For example, for RRC connection establishment, following is specified in TS 38.331 clause 5.3.3.2:

Upon initiation of the procedure, the UE shall: 1> if the upper layers provide an Access Category and one or more Access Identities upon requesting establishment of an RRC connection: 2> perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers; 3> if the access attempt is barred, the procedure ends; …

And the setting of establishmentCause (which is related to Access Category) is specified in TS 38.331 clause 5.3.3.3,

The UE shall set the contents of RRCSetupRequest message as follows: ... 1> if the establishment of the RRC connection is the result of release with redirect with mpsPriorityIndication (either in NR or E-UTRAN): 2> set the establishmentCause to mps-PriorityAccess; 1> else: 2> set the establishmentCause in accordance with the information received from upper layers;

It can be seen that in RRC connection establishment procedure, as long as the Access Category information is provided by upper layers, there is no need to mention the exact value of Access Category / establishmentCause (e.g. mo-Signalling) in RRC specification. Similarly, for RRC connection resume procedure, according to TS 38.331 clause 5.3.13.2, as long as the Access Category information is provided by upper layers, there is no need to mention the detailed value of Access Category / resumeCause (e.g. mo-Signalling) in RRC specification.

Observation 1: In RRC connection establishment and resume procedure, as long as the Access Category information is provided by upper layers, there is no need to mention the detailed value of Access Category / establishmentCause / resumeCause (e.g. mo-Signalling) in RRC specification.

In RAN2#122 meeting, it was confirmed that “For both IDLE and INACTIVE mode, NAS layer may request the RRC layer to move to RRC_CONNECTED.” TS 38.300 clause 7.4 specifies that “For NAS triggered requests, NAS determines the Access Category and Access Identity(ies);” It is assumed that transition to RRC_CONNECTED to acquire clock quality information is triggered by NAS, and the Access Category is provided by NAS layer when UE performs RRC connection establishment or resume. From Observation 1, no RAN2 specification impact is foreseen when using Access Category 3 (MO_sig) to retrieve clock quality information.

Proposal 1: RAN2 confirms that existing Access Category 3 (MO_sig) is used to retrieve clock quality information, and the Access Category information is provided by NAS layer. No RAN2 specification impact is foreseen.

Presence/absence of event ID in SIB9

RAN2#122 meeting has the following agreement regarding event ID: “Event ID is optional. Under the same gNB, UE considers the change of timing synchronization if event ID field from SIB9 is different. FFS if gNB always broadcasts event ID”.

The above FFS is mainly related to whether the event ID should be always present in SIB9 for a gNB supporting the timing synchronization status feature. In SA2 TS 23.501 clause 5.27.1.12, following part is related to the reference report ID / event ID:

The gNB indicates the status change to the UEs via SIB information: - When the network timing synchronization status exceeds any of the pre-configured thresholds, the gNB includes in SIB information a reference report ID. When the network timing synchronization status meets the thresholds again (i.e. status improvement), the gNB broadcasts the reference report ID in SIB information. Either event serves as a notification for the UEs reading the SIB information that there is new TSS information available. … - If supported, the UE in the RRC_INACTIVE or RRC_IDLE state compares the reference report ID in SIB information with its locally stored reference report ID to determine whether it has the latest available clock quality information already or it needs to transit to the RRC_CONNECTED state to retrieve it. A reference report ID consists of a scope of the TSS and an Event ID. The Event ID is an integer indicating the clock quality information from a gNB. A scope of the TSS supports providing TSS information for a group of cells within a single gNB. Uniqueness of the event ID is ensured by combining with a gNB ID.

It can be seen that according to SA2, when the clock synchronization status changes, the reference report ID is signalled and the event ID is part of reference report ID. Given the RAN2 agreement “Update of event ID is informed to UE by normal SI modification procedure”, it is natural that once gNB broadcasts event ID, it should keep broadcasting the same event ID until the clock synchronization status changes, instead of omitting the event ID (which triggers SI modification with impact to UE power saving). This means that once gNB broadcasts event ID, it should always broadcast it until it stops provisioning reference time. One may argue that it is possible that when gNB initially provisions reference time, it may omit even ID and later on broadcasts event ID when clock synchronization status changes. However, this introduces additional complexity compared with always broadcasting event ID since additional specification efforts are needed to define UE behavior regarding the handling of presence of event ID to absence of event ID, and vice versa.

With above discussion, it is preferred that the gNB supporting clock synchronization status always broadcasts event ID in SIB9, which is a uniform approach and simplifies standardization efforts.

Proposal 2: The gNB supporting clock synchronization status always broadcasts event ID in SIB9.

Clock quality information signalling

In RAN2#121bis-e meeting, following was agreed “5G Clock Quality information is carried in DLInformationTransfer message.” Detailed signalling design is related to SA2 and RAN3 agreements / progress. In SA2 TS 23.501 clause 5.7.1.12, following is specified regarding the clock quality information:

Table 5.27.1.12-1: Information elements that gNB or UPF/NW-TT timing synchronization status information may contain (all optional) Information Name; Description. Synchronization state; Indicates the state of the node synchronization, represented by the values "Locked", "Holdover", or "Freerun" (NOTE 1).. Clock quality. >> Traceable to GNSS; Indicates whether the current time source is traceable to the GNSS and represented by values “Yes” or “No”.. >> Traceable to UTC; Indicates whether the current time source is traceable to the UTC and represented by values “Yes” or “No”.. >> Frequency stability; Describes the estimate of the variation of the local clock when it is not synchronized to another source (NOTE 2).. >> Clock Accuracy; Describes the mean in ns over an ensemble of measurements of the time between the clock under test and a reference clock (NOTE 3).. Parent time source; Describes the primary source the node is currently using, represented by the values "SyncE", "PTP", "GNSS", "atomic clock", "terrestrial radio", "serial time code", "NTP", "hand_set", "other".. NOTE 1: Clock is in the "Locked", "Holdover", or "Freerun" mode, as defined in ITU-T G.810 [164]. NOTE 2: Frequency stability is estimated in a similar manner as for offsetScaledLogVariance attribute defined in clause 7.6.3.5 of IEEE Std 1588 [126]. NOTE 3: Clock accuracy measurement considers accuracy up to gNB antenna and RAN internal process.. … For 5G access stratum time synchronization service, clock quality reporting control information manages the gNBs timing synchronization status reporting to the UE. When AMF provides the 5G access stratum time distribution indication and the Uu time synchronization error budget to gNB, the AMF also includes the clock quality reporting control information (CQRCI) provided by the TSCTSF or retrieved from UDM. CQRCI may be a part of Access and Mobility Subscription data at the UDM, or AF may include CQRCI in its request. CQRCI contains the following fields: - Clock quality detail level. It indicates whether and which clock quality information to provide to the UE and can take one of the following values: clock quality metrics or acceptable/not acceptable indication. - If the clock quality detail level equals "clock quality metrics", the NG-RAN provides clock quality metrics to the UE that reflect its current timing synchronization status. i.e. one or more of the following information elements: clock accuracy, traceability to UTC and to GNSS, frequency stability, parent time source, synchronization state as defined in Table 5.27.1.12-1. NG-RAN is locally configured which of the clock quality metrics supported by NG-RAN are provided to UE(s). - If the clock quality detail level equals "acceptable/not acceptable indication", clock quality acceptance criteria for the UE. The gNB provides an acceptable indication to the UE if the gNB's timing synchronization status matches the acceptance criteria received from the AMF; otherwise, the gNB indicates "not acceptable" to the UE. Clock quality acceptance criteria can be defined based on one or more information elements listed in Table 5.27.1.12-1. If AF includes clock quality acceptance criteria in its request towards TSCTSF, the AF shall be notified about the result once TSCTSF determines whether the clock quality acceptance criteria can be met or not. Based on the notification, the AF may decide to modify the service if preferred (e.g., disable the service upon degradation or enable it again upon recovery).

According to SA2 TS, gNB signals to the UE with either clock quality metrics or acceptable/not acceptable indication. In the former case, gNB provides clock accuracy, traceability to UTC and to GNSS, frequency stability, parent time source, synchronization state as defined in TS 23.501 Table 5.27.1.12-1. In the latter case, gNB simply signals “acceptable” / “not acceptable”.

From SA2 TS, it can be seen that the signalling for clock quality information in NGAP protocol between AMF and gNB (as specified in RAN3 TS 38.413) and the RRC signalling are almost the same. The main difference is that if the clock quality detail level equals "acceptable/not acceptable indication", clock quality acceptance indication (“acceptable” / “not acceptable”) is signalled in RRC signalling, while clock quality acceptance criteria is signalled in NGAP signalling.

Observation 2: The main difference between RRC signalling and NGAP signalling for clock quality information is that clock quality acceptance indication (“acceptable” / “not acceptable”) is signalled in RRC signalling, while clock quality acceptance criteria is signalled in NGAP signalling.

RAN3 has already started the related work, and has endorsed baseline CRs (e.g. R3-232551 for CR to TS 38.413) and further agreed TPs (e.g. R3-233477 for TP to TS 38.413). RAN2-related agreements from the CR/TP are as follows:

- 9.3.1.x1	Clock Quality Reporting Control Information

This IE indicates the clock quality reporting control information as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Quality Detail Level | M |  |  |  |
| >clock quality metrics |  |  |  |  |
| >>[FFS] |  |  |  |  |
| >acceptance indication |  |  |  |  |
| >>Clock Quality Acceptance Criteria | M |  | 9.3.1.x2 |  |

- 9.3.1.x2	Clock Quality Acceptance Criteria

This IE indicates the clock quality acceptance criteria as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| [FFS] |  |  |  |  |

- 9.3.1.x3	RAN Timing Synchronisation Status Information

This IE indicates the RAN timing synchronisation status information provided towards the AMF as defined in TS 23.501 [9].

Editor’s Note: The non-UE associated NGAP procedure(s) used to convey this IE towards the AMF is FFS.

Editor’s Note: This IE may be further refined based on SA2 and RAN3 progress.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | ENUMERATED (locked, holdover, freeRun, …) |  |
| Traceable to UTC | O |  | ENUMERATED (true, false, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, false, …) |  |
| Clock Frequency Stability | O |  | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [9]. |
| Clock Accuracy | O |  | 9.3.1.x5 |  |
| Parent Time Source | O |  | ENUMERATED (syncE, pTP, gNSS, atomicClock, terrestrialRadio, serialTimeCode, nTP, handSet, other, …) |  |

- 9.3.1.x5	Clock Accuracy

This IE indicates the clock accuracy as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Accuracy | M |  |  |  |
| >choice1 |  |  |  |  |
| >>[FFS] |  |  |  |  |

Editor’s Note: Encoding of the Clock Accuracy IE is to be decided by RAN3 and should allow for different RAN implementations (e.g., CHOICE structure). Details FFS.

The main open issue in RAN3 is the signalling of clock accuracy, as from the above Editor’s note. To avoid duplicated work in RAN2, RAN2 can wait for RAN3’s progress regarding signaling of clock accuracy.

Proposal 3: For RRC signalling of clock quality information in DLInformationTransfer message, a CHOICE structure is used to signal clock quality metrics and clock quality acceptance indication (which is signalled as “acceptable” / “not acceptable”).

Proposal 4: Clock quality metrics include the information of clock accuracy, traceability to UTC and to GNSS, frequency stability, parent time source, synchronization state, as defined by SA2 and RAN3. RAN2 can wait for RAN3’s progress regarding signaling of clock accuracy.

Conclusion

In this contribution, we discuss remaining issues of 5GS network timing synchronization status and reporting, and have the following observations:

Observation 1: In RRC connection establishment and resume procedure, as long as the Access Category information is provided by upper layers, there is no need to mention the detailed value of Access Category / establishmentCause / resumeCause (e.g. mo-Signalling) in RRC specification.

Observation 2: The main difference between RRC signalling and NGAP signalling for clock quality information is that clock quality acceptance indication (“acceptable” / “not acceptable”) is signalled in RRC signalling, while clock quality acceptance criteria is signalled in NGAP signalling.

We propose the following:

Proposal 1: RAN2 confirms that existing Access Category 3 (MO_sig) is used to retrieve clock quality information, and the Access Category information is provided by NAS layer. No RAN2 specification impact is foreseen.

Proposal 2: The gNB supporting clock synchronization status always broadcasts event ID in SIB9.

Proposal 3: For RRC signalling of clock quality information in DLInformationTransfer message, a CHOICE structure is used to signal clock quality metrics and clock quality acceptance indication (which is signalled as “acceptable” / “not acceptable”).

Proposal 4: Clock quality metrics include the information of clock accuracy, traceability to UTC and to GNSS, frequency stability, parent time source, synchronization state, as defined by SA2 and RAN3. RAN2 can wait for RAN3’s progress regarding signaling of clock accuracy.

- References

[1] R2-2302106, RAN2 LS, “Reply LS on Proposed method for Time Synchronization status reporting to UE(s)”

[2] R2-2304605, CT1 LS, “Response to Reply LS on Proposed method for Time Synchronization status reporting to UE(s)”

[3] R2-2307051, SA1 LS, “Response to Reply LS on Proposed method for Time Synchronization status reporting to UE(s)”