3GPP TSG-RAN WG2 Meeting #123	R2-2307785

Toulouse, France, Aug 21st – Aug 25th, 2023

Agenda item:	7.4.4

Source:	Qualcomm Incorporated

Title:	CHO with multiple candidate SCGs

WID/SID:	NR_Mob_enh2-Core – Release 18

Document for:	Discussion and Decision

# Introduction

RAN2 #122 agreements:

P3: The CHO execution conditions (for candidate PCells) and CPA/CPC execution conditions (for candidate PSCells) are provided based on the source MeasConfig.

P4: For CHO execution conditions, the source MN determines the execution conditions on candidate PCells, based on the source MCG MeasConfig.

P5: For CPA/CPC execution conditions, the candidate MN determines the parameters of the execution conditions for candidate PSCells (e.g., event A4 threshold).

P6: The candidate MN informs the source MN about the prepared candidate PSCells and parameters of the associated execution conditions (e.g., event A4 threshold). According to the received information from the candidate MN, the source MN generates the corresponding execution conditions based on the source MCG MeasConfig to the UE.

FFS how, if to support event A3/A5.

P8: For CHO with candidate SCGs for CPA/CPC, the RRCReconfiguration message in one CHO container includes one MCG configuration and one SCG configuration (i.e. similar to Rel-17 CHO with SCG configuration).

P9: The execution conditions associated with one CHO container includes both CHO execution condition(s) and CPA/CPC execution condition(s), i.e. triggering conditions on both candidate PCell and candidate PSCell.

P10: If there are multiple candidate PSCells associated with one candidate PCell, the NW can provide multiple CHO configurations for the same candidate PCell, i.e. each one contains one MCG configuration (for the same candidate PCell) and one SCG configuration (for different candidate PSCell).

P12: When the CPA/CPC execution condition is met but no CHO execution condition is met, the UE continues to evaluate both CHO and CPA/CPC execution conditions.

For CHO+CPC we only consider execution when BOTH conditions are met.

- (When the CHO execution condition is met but no CPC execution condition is met, if there is an available CHO-only or Rel-17 CHO with SCG configuration for which the CHO condition is met, the UE performs the CHO-only or Rel-17 CHO with SCG execution, and THUS the network can handle such situation by providing proper configurations).

In the last RAN2 meeting (RAN2 #122 [1]), there was significant progress on this topic, and many agreements were made on the preparation as well as the execution phases of the procedure when simultaneous evaluation of CHO and CPC/CPA is performed.

In this contribution, we discuss the following.

- Remaining Stage-2 aspects of the CHO with CPC procedure.

- The scenario where CPA/CPC is triggered by the candidate MN during a regular handover.

# Procedure for CHO with multiple candidate SCGs

We consider a UE in NR-DC operation. Figure 1 provides an example signalling flow of the procedure.

Steps 1, 2, 3: The source MN initiates the procedure and transmits Handover Request to a candidate MN. The Handover Request message includes:

- Source SN UE XnAP ID, which serves as a reference to the UE context in the SN,

- Target PCell ID,

- CHO indicator,

- UE measurement results,

- Source MCG and SCG configurations.

Following Rel-16 principles, source MN determines the execution conditions for the candidate target PCells.

Upon receiving the Handover Request, a candidate MN may decide to release the SCG, change the SCG, or keep the SCG. We consider the case where the candidate MN decides to keep the SCG or change the SCG and requests a set of candidate SNs to prepare PSCells. The candidate MN transmits SN Addition Request to each such candidate SN. In case the candidate MN decides to keep the SCG, the request in transmitted to the source SN.

Proposal 1. Candidate MN uses the UE measurement results information provided by source MN in Handover Request to determine the list of candidate PSCells to suggest to a candidate SN for preparation.

Proposal 2. RAN2 needs to discuss the maximum total number of candidate target cells (PCells and PSCells) that can be configured to a UE during the preparation procedure.

Based on this, source MN can indicate to a candidate MN the maximum number of candidate target PSCells that the candidate MN can suggest to the candidate SNs to prepare.

Proposal 3. Source MN indicates to a candidate MN the maximum number of candidate target PSCells that the candidate MN can suggest to the candidate SNs to prepare. Candidate MN indicates to a candidate SN in SN Addition Request the maximum number of candidate target PSCells that the candidate SN can prepare.

In SN Addition Request, candidate MN includes the following information:

- Suggested set of PSCells for the candidate SN to consider configuring,

- UE measurement results,

- CPC indicator,

- CHO indicator, indicating to the candidate SN that the procedure to configure multiple candidate PSCells is part of a CHO procedure,

- Source SN UE XnAP ID,

- Source SCG configuration.

Step 4: A candidate SN prepares a subset of the suggested set of PSCells indicated by the candidate MN, and responds with the SN Addition Request Acknowledge message containing:

- The prepared candidate target PSCells,

- Target SCG configuration associated with each candidate target PSCell.

Steps 5, 6, 7, 8:

A candidate MN transmits the Handover Request Acknowledge message to the source MN including the candidate target PCell IDs.

We now discuss the contents of the RRC reconfiguration message transmitted by the source MN to the UE.

Figure 1: Procedure for CHO including target MCG and target SCGs

The UE is configured with execution conditions for (candidate target PCell, candidate target PSCell) pairs. For the candidate target PCells, the execution conditions are generated by the source MN. For the candidate target PSCells, the candidate MN determines the execution conditions. The following are the agreements from the last RAN2 meeting (RAN2 #122 [1]) which help determine the contents of the RRC reconfiguration message.

- The CHO execution conditions (for candidate PCells) and CPA/CPC execution conditions (for candidate PSCells) are provided based on the source MeasConfig.

- For CHO execution conditions, the source MN determines the execution conditions on candidate PCells, based on the source MCG MeasConfig.

- For CPA/CPC execution conditions, the candidate MN determines the parameters of the execution conditions for candidate PSCells (e.g., event A4 threshold).

- The candidate MN informs the source MN about the prepared candidate PSCells and parameters of the associated execution conditions (e.g., event A4 threshold). According to the received information from the candidate MN, the source MN generates the corresponding execution conditions based on the source MCG MeasConfig to the UE.

- FFS how, if to support event A3/A5.

- For CHO with candidate SCGs for CPA/CPC, the RRCReconfiguration message in one CHO container includes one MCG configuration and one SCG configuration (i.e., similar to Rel-17 CHO with SCG configuration).

- The execution conditions associated with one CHO container includes both CHO execution condition(s) and CPA/CPC execution condition(s), i.e., triggering conditions on both candidate PCell and candidate PSCell.

- If there are multiple candidate PSCells associated with one candidate PCell, the NW can provide multiple CHO configurations for the same candidate PCell, i.e., each one contains one MCG configuration (for the same candidate PCell) and one SCG configuration (for different candidate PSCell).

Furthermore, it was agreed in the RAN2 #122 meeting [1] that the network provides suitable configurations to handle special situations that arise:

- When the CHO execution condition is met but no CPC execution condition is met, if there is an available CHO-only or Rel-17 CHO with SCG configuration for which the CHO condition is met, the UE performs the CHO-only or Rel-17 CHO with SCG execution, and THUS the network can handle such situation by providing proper configurations.

Steps 9, 10: In a previous RAN2 meeting (RAN2 #121 [3]), it was agreed to support simultaneous evaluation of CHO and CPC for the procedure.

RAN2 #121 agreement:

- RAN2 agrees to support the simultaneous evaluation of CHO and CPC in Rel-18.

In the previous two RAN2 meetings [1], [2], the following agreements were made which specify in some detail the execution phase of the procedure.

RAN2 #121-bis-e agreement [2]:

When both CHO and CPC conditions are met, both CHO and CPC cell change is executed.

RAN2 #122 agreements [1]:

- When the CPA/CPC execution condition is met but no CHO execution condition is met, the UE continues to evaluate both CHO and CPA/CPC execution conditions.

- For CHO+CPC we only consider execution when BOTH conditions are met.

(When the CHO execution condition is met but no CPC execution condition is met, if there is an available CHO-only or Rel-17 CHO with SCG configuration for which the CHO condition is met, the UE performs the CHO-only or Rel-17 CHO with SCG execution, and THUS the network can handle such situation by providing proper configurations).

We discuss proposals below which seek to clarify further details.

CHO gets triggered earlier than CPC

When the CHO execution condition is met but no CPC execution condition is met, UE should apply an available CHO-only configuration when a legacy mechanism for releasing the source SCG is triggered at the UE, e.g., UE receives an SCG release command from the network after transmitting an event A2 measurement report for the source PSCell. Otherwise, UE should apply a network provided Rel-17 CHO with SCG configuration.

Proposal 4. When the CHO execution condition is met but no CPC execution condition is met:

- UE applies a network provided CHO-only configuration when a legacy mechanism for releasing the source SCG is triggered at the UE, e.g., UE receives an SCG release command from the network after transmitting an event A2 measurement report for the source PSCell.

- Otherwise, UE applies a network provided Rel-17 CHO with SCG configuration.

# 3 Legacy handover with CPC/CPA

We consider the scenario where CPC/CPA is triggered by the candidate MN during a legacy handover. This scenario is a slight variation of the CHO with CPC/CPA scenario being considered in the WI and has similar advantages – the UE can choose among multiple configured PSCells upon handover. It is expected that the specification of this scenario does not take significant additional effort.

We discuss the CPC case below. Figure 2 provides a signalling flow of the procedure.

Slightly modified versions of Proposals 1, 2, and 3, for the CHO with CPC procedure in Section 2 hold in this case.

Proposal 5. Candidate MN uses the UE measurement results information provided by source MN in Handover Request to determine the list of candidate PSCells to suggest to a candidate SN for preparation.

Proposal 6. RAN2 needs to discuss the maximum total number of candidate target PSCells that can be configured to a UE.

Proposal 7. Candidate MN indicates to a candidate SN in SN Addition Request the maximum number of candidate target PSCells that the candidate SN can prepare.

Figure 2: Legacy handover with CPC

A different aspect compared to the CHO with CPC case is the following proposal.

Proposal 8. The candidate MN generates the CPC execution conditions, e.g., the event A4 thresholds, based on the target MCG MeasConfig, and includes the CPC configuration in the target cell configuration provided in Handover Req Ack. The CPC configuration is the same as legacy CPC and consists of (a) List of candidate target PSCells and their associated SCG configurations (and MCG configurations) (b) Execution conditions for the candidate target PSCells.

The RRCReconfiguration message with the handover command (Step 7 in Figure 2) contains the target cell configuration (which consists of the MCG and SCG parts) and which, in turn, includes the CPC configuration.

Upon receiving the RRCReconfiguration message containing the handover command, UE performs RACH on the candidate target cell, applies the target cell configuration, and transmits RRCReconfigurationComplete to the candidate MN.

After UE completes access of the candidate target cell, UE performs CPC evaluation as per the CPC configuration included in the target cell configuration. When CPC execution conditions are satisfied for a candidate target PSCell, UE accesses the candidate target PSCell.

# 4 Conclusions

Based on the above discussion, we recommend that RAN2 discuss the following observations and proposals.

Procedure for CHO with multiple candidate SCGs

Proposal 1. Candidate MN uses the UE measurement results information provided by source MN in Handover Request to determine the list of candidate PSCells to suggest to a candidate SN for preparation.

Proposal 2. RAN2 needs to discuss the maximum total number of candidate target cells (PCells and PSCells) that can be configured to a UE during the preparation procedure.

Proposal 3. Source MN indicates to a candidate MN the maximum number of candidate target PSCells that the candidate MN can suggest to the candidate SNs to prepare. Candidate MN indicates to a candidate SN in SN Addition Request the maximum number of candidate target PSCells that the candidate SN can prepare.

Proposal 4. When the CHO execution condition is met but no CPC execution condition is met:

- UE applies a network provided CHO-only configuration when a legacy mechanism for releasing the source SCG is triggered at the UE, e.g., UE receives an SCG release command from the network after transmitting an event A2 measurement report for the source PSCell.

- Otherwise, UE applies a network provided Rel-17 CHO with SCG configuration.

Legacy handover with CPC/CPA

Proposal 5. Candidate MN uses the UE measurement results information provided by source MN in Handover Request to determine the list of candidate PSCells to suggest to a candidate SN for preparation.

Proposal 6. RAN2 needs to discuss the maximum total number of candidate target PSCells that can be configured to a UE.

Proposal 7. Candidate MN indicates to a candidate SN in SN Addition Request the maximum number of candidate target PSCells that the candidate SN can prepare.

Proposal 8. The candidate MN generates the CPC execution conditions, e.g., the event A4 thresholds, based on the target MCG MeasConfig, and includes the CPC configuration in the target cell configuration provided in Handover Req Ack. The CPC configuration is the same as legacy CPC and consists of (a) List of candidate target PSCells and their associated SCG configurations (and MCG configurations) (b) Execution conditions for the candidate target PSCells.

# References

[1] Chair notes, RAN2 #122 meeting.

[2] Chair notes, RAN2 #121-bis-e meeting.

[3] Chair notes, RAN2 #121 meeting.