3GPP TSG-RAN WG2 Meeting #123	R2-2307787

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues on Cell DTX/DRX

WID/SID:	Netw_Energy_NR-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The following agreements have been made in the previous RAN2 meeting on Cell DTX/DRX:

Agreements RAN2 #121bis-e meeting

- A periodic cell DTX/DRX configuration is explicitly signalled to the UEs.

- A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling.

- The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration.

- As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released.

- From RAN2 point of view, majority companies see a benefit with L1 signalling for Cell DTX/DRX activation/deactivation, send a LS to RAN1 (email 308) with our preference and ask about feasibility and design details.   Ask about feasibility and reliability of using L1 signaling.  Clarify that the question is about activation/deactivation copy the agreement from last meeting that we are focusing on single configuration.  Extract a few key benefits of dynamic signaling from email discussion and online discussions

- As baseline, UE doesn’t monitor SPS occasions during Cell DTX non-active period. As baseline, gNB is assumed to be not transmitting PDSCH to that UE on such SPS occasions during the Cell DTX non-active period

- As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods

- As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic

- (for the SRs that will be dropped) If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH.  For the FFS case there may be some exceptions.

- The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.   UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.   FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

FFS how to deal with retransmissions

Agreements RAN2 #122 meeting

1	UE monitors PDCCH for RAR during Cell DTX non-active time. The ra-ResponseWindow could be started as legacy.

2	UE monitors PDCCH for msg4 during Cell DTX non-active time. The ra-ContentionResolutionTimer could be started as legacy.

3	Working assumption:  When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

4	Once gNB recognizes there is an emergency call or public safety related service (e.g. MPS/MCS), the NW should ensure there is no impact to the emergency call (e.g. may deactivate Cell DTX/DRX).  The behavior is captured in stage 2 spec

5	When an DG grant is received, by the gNB during cell DRX/DTX, the UE follows the grant assignment (i.e. like in legacy).  This includes DL HARQ feedback.

In this contribution, we discuss the remaining issues.

# 2	Discussion

## 2.1	Configuration of cell DTX/DRX

Two proposals from the email discussion [312] were left not treated due to time limit:

Proposal 6: An aligned UE C-DRX configuration with Cell DTX means that the on-duration of C-DRX falls within Cell DTX on-duration. FFS extension of Cell DTX active time beyond Cell DTX on-duration. (15/25) Proposal 7: The periodicity of UE C-DRX configurations in a cell should be the same or a multiple of the serving Cell’s DTX periodicity.

In our view, it should be possible to configure UE’s DRX cycles shorter than Cell DTX/DRX periodicity esp. if it intends to be used when the Cell DTX/DRX is not activated, but no reconfiguration is needed when Cell DTX/DRX is activated since the Cell DTX/DRX activation/deactivation can be done dynamically, while it does not make sense to require the NW to reconfigure the UE’s DRX configuration whenever Cell DTX/DRX activation/deactivation happens. Thus, the alignment should be up to NW implementation and the reconfiguration of UE’s DRX cycle is not necessarily needed.

Also note that, based on this agreement “10. The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.  UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.”, thus even if the Cell DTX and UE DRX is not fully aligned, there would be no impact to the UE power consumption as long as the NW ensures there is enough scheduling opportunities taking both NW’s active time and the UE’s active time into consideration, thus it should have no specification impact.

Proposal 1: alignment of UE’s C-DRX and NW cell DTX could be up to NW implementation.

At the RAN1#113 it was agreed that a cell DTX/DRX pattern can be activated/deactivated using a new group-common DCI. This means the pattern(s) can be dynamically activated compared to the time required to reconfigure the cell DTX/DRX and the UE’s DRX. Therefore, it is beneficial to define how a UE’s DRX On Duration can be dynamically changed in a similar way without requiring RRC reconfiguration of the DRX, for example if the UE has DRX On Duration aligned with cell DTX pattern A, but not with cell DTX pattern B. This is critical to ensure there is at least partial overlap between the cell DTX/DRX active period and the UE DRX On Duration.

In the scenario where the UE determines the DRX On Duration does not overlap with the activated cell DTX/DRX active period the UE could be triggered to apply a new start offset for the UE DRX On Duration. The new start offset should apply as long as the current cell DTX/DRX pattern is activated. The start offset could be computed based on the existing RRC configured UE DRX start offset or be part of the RRC configuration as a secondary offset.

Proposal 2:  If the UE DTX OnDuration does not overlap with active time of Cell DTX, the UE applies a new start offset for the UE DRX On Duration e.g. based on RRC configuration of an specific offset for Cell DTX activated case.

- Separate handling of cell DTX and cell DRX

The separate handling of cell DTX and cell DRX was already agreed in RAN2#121 meeting. In the post RAN2 meeting#122 discussion [307], the meaning of meaning of “separate” was questionable. In our view, the gNB can configure both Cell DTX and Cell DRX with different parameter values (e.g. different offset, on-duration, periodicity) and it is up to NW implementation to make them aligned or not.

Proposal 3: The alignment of cell DTX and cell DRX configuration is up to NW implementation.

- Support of multiple cell DTX/DRX patterns

The support of multiple cell DTX/DRX patterns was also raised as part of the post RAN2 meeting#122 discussion [307]. We clearly see the advantages to configure multiple cell DTX/DRX patterns and to enable the switching from one pattern to another based on the e.g., load, services requirements of the UEs etc. RAN1 agreed to define new DCI for activation/deactivation, it would not be a bottleneck to indicate one of the multiple cell DTX/DRX patterns to be activated

Proposal 4: Multiple patterns configuration of cell DTX/ DRX shall be supported.

- Cell DTX/DRX parameter value range

Cell DTX/DRX parameter consisting of the periodicity, start slot/offset, on duration were agreed in the previous RAN2 meeting but the value ranges is still under discussion as captured in the post RAN2 meeting#122 discussion [307]. We see that the UE DRX values can be supported as a starting point.

Proposal 5: UE’s DRX parameters’ value could be assumed as starting point for cell DTX/DRX.

## 2.2	SR and emergency call

Wrt. the FFS on SR, if agreed to allow SR during Cell DTX non-active time, it should be ensured that the UE will monitor PDCCH for SR responses after SR is sent. Otherwise, it would not make sense. It could be further discussed whether there could be some gap before the UE start monitoring to allow NW wake up the Tx if the cell is also in Cell DTX non-active period since typically there would be some alignment for DTX and DRX configuration and potential impact on SR prohibit timer.

Proposal 6: if to allow SR during the cell non-active period, the UE shall monitor responses for SR as well even if during non-active period.

While we decide not to allow SR to be sent during non-active period, the UE may trigger RACH upon initiation of emergency call during non-active time and indicate to the NW it is an emergency call so that the NW can deactivate the Cell DTX quickly, similar to the agreement made in the SDT session last meeting that the UE is allowed to do RA-SDT when the CG-SDT is too far away. UE NAS could indicate to AS if the service request is for emergency call which is current not indicated for connected mode.

- For both MO and MT-SDT, if the next CG-SDT resource is too far, then RACH resource can be selected first.   This is checked at the point of initial resource selection (e.g. CG SDT selection).   FFS what is too far and how this is configured.   Assumption is that we will continue this discussion in SDT session.  CONFIRM with main session [CB]

Proposal 7: UE NAS indicates to AS if the service request is for emergency call which is currently not indicated for connected mode.

Proposal 8: If SR is not allowed during non-active period, RACH is allowed for emergency call if the next active period is too far away.

## 2.3	Impact to SCell activation delay

SCell activation delay requirement in RAN4 is defined based on the assumption of there is periodic CSI-RS or TRS in MAC CE to be used for synchronization and measurements, while it has been agreed in RAN1 that periodic CSI-RS would not be transmitted when not in Cell DTX active period. If an SCell being activated is with Cell DTX activated, it might impact the SCell activation delay. As not all the UEs support cross carrier scheduling to deactivate it from other cells, it makes sense to define the behaviour of UE deactivates the Cell DTX for the SCell upon reception of the activation command.

Proposal 9: the UE deactivates the Cell DTX for the SCell (if activated) upon reception of the SCell activation command to ensure no impact on SCell activation delay requirement.

# 3	Conclusion

Further details on Cell DTX/DRX are discussed in this contribution with the following proposals proposed:

Proposal 1: alignment of UE’s C-DRX and NW cell DTX could be up to NW implementation.

Proposal 2:  If the UE DTX OnDuration does not overlap with active time of Cell DTX, the UE applies a new start offset for the UE DRX On Duration e.g. based on RRC configuration of an specific offset for Cell DTX activated case.

Proposal 3: The alignment of cell DTX and cell DRX configuration is up to NW implementation.

Proposal 4: Multiple patterns configuration of cell DTX/ DRX shall be support.

Proposal 5: UE’s DRX parameters’ value could be assumed as starting point for cell DTX/DRX.

Proposal 6: if to allow SR during the cell non-active period, the UE shall monitor responses for SR as well even if during non-active period.

Proposal 7: UE NAS indicates to AS if the service request is for emergency call which is currently not indicated for connected mode.

Proposal 8: If SR is not allowed during non-active period, RACH is allowed for emergency call if the next active period is too far away.

Proposal 9: the UE deactivates the Cell DTX for the SCell (if activated) upon reception of the SCell activation command to ensure no impact on SCell activation delay requirement.