3GPP TSG-RAN WG2 Meeting #123	R2-2307792

Toulouse, France, 21– 25 August 2023

Agenda item:	7.23.2

Source:	Nokia, Nokia Shanghai Bell

Title:	5GS network timing synchronization status and reporting

WID/SID:	TRS_URLLC-NR-core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The following agreements have been made in the previous RAN2 meeting wrt. the objective on 5GS network timing synchronization status and reporting:

RAN2#122 Agreements 1. Update of event ID is informed to UE by normal SI modification procedure. 2. Confirm the AS layer of the UE determines if there a change of event ID and/or gNB ID. If there is a change, the AS layer notifies the change in the RAN timing synchronization status to NAS layer. For both IDLE and INACTIVE mode, NAS layer may requests the RRC layer to move to RRC_CONNECTED 3. For RRC_CONNECTED mode UEs, the NW has the necessary information to determine whether to send detailed clock quality information to the UE, (i.e. it may choose to always send update or only when needed) and the details can be left to NW implementation 4. Event ID is optional. Under the same gNB, UE considers the change of timing synchronization if event ID field from SIB9 is different. FFS if gNB always broadcasts event ID 5. If the UE is in RRC_INACTIVE, the UE can acquire clock quality information using the SDT procedure, if it supports and/is configured with SDT procedure. RAN2#121 Agreements - RAN2 to confirm that there is no need to support “group of cells across gNBs” for scope of the report ID. The scenario, as per RAN3 will be supported - different cells within gNB that are served by different DUs and different clock quality is possible. - Confirm the following SA2’s conclusions regarding NG-RAN procedures on 5G clock quality information: - RRC_INACTIVE/IDLE UE to be informed of timing synchronization status via an event ID in SIB9 by comparison with the one maintained locally. - RRC_CONNECTED UE to be informed of 5G clock quality information via dedicated RRC message - Postpone the UAC until CT1’s feedback on UAC framework to randomize UE access to network in the time domain - 5G Clock Quality information is carried in DLInformationTransfer message. - FFS if update of event ID is informed to UE by SI modification procedure - For Idle/Inactive mode RAN2 assumes that the NAS layer triggers the RRC connection procedure based on inputs from AS. - The AS layer of the UE determines if there a change of event ID and it indicates a change in the RAN timing synchronization status to NAS layers. FFS to double check for connected mode and inactive

In this contribution, we further address the highlighted open issues:

- UE capabilities for supporting clock quality information methods

- Event ID broadcast in SIB9 based on the last SA2 progress

- Dedicated RRC signalling required for clock quality information reporting towards the UE

# 2	Discussion

## 2.1	UE capabilities for supporting clock quality information mechanism

To support of clock quality information towards the UE is notified via SIB9 to UEs in RRC_IDLE or RRC_INACTIVE state and the report is provisioned via dedicated RRC signalling to UEs in RRC_CONNECTED state. These signalling  methods do not require new capabilities in the UE to work. Therefore, from RAN2 perspective, it is our understanding that there is no need for new AS capabilities at the UE.

However, at NAS level there is a new capability introduced in Rel-18 to indicate the UE support for network reconnection due to RAN Timing Synchronization Status (TSS) change. This is a UE 5GMM Core Network Capability the UE can include during a registration request. The CN uses this indication to be aware of the expected behaviour of the UE when it is in RRC_IDLE/INACTIVE state and new clock quality information is available in the serving gNB. This is due to the CN needs to correlate the RAN TSS received from the gNBs and the UE locations to determine if a RAN TSS change impacts a UE or not (e.g., to reconfigure PTP service, to notify AF, etc).

Proposal 1: Confirm in RAN2 that no AS capability is needed for the UE to support clock quality information mechanisms.

## 2.2	Event ID broadcast in SIB9

For UEs in RRC_IDLE or RRC_INACTIVE, Event ID included in SIB9 works as a notification method to determine if the UE should reconnect to the network to receive the last clock quality information available at the gNB. When Event ID is broadcasted in SIB9, the UE compares the value broadcasted by the gNB with the locally stored values. This way the UE can determine if the clock quality information the gNB can provision to the UE is already stored or there is new information available the UE should retrieve. It is up to gNB implementation the allocation of Event IDs but uniqueness of the Event ID should be ensured by combining with a gNB ID.

Regarding the reporting of Event ID, in TS 23.501, the following text describes the gNB reporting reference report ID (that contains Event ID) in SIB “When the network timing synchronization status exceeds any of the pre-configured thresholds, the gNB includes in SIB information a reference report ID. When the network timing synchronization status meets the thresholds again (i.e. status improvement), the gNB broadcasts the reference report ID in SIB information. Either event serves as a notification for the UEs reading the SIB information that there is new TSS information available.” Compared to previous version of this text, SA2 has removed the start/stop condition for reference report ID in SIB information. From SA2 perspective, after SA2#157 the notification of Event ID was agreed to be always broadcasted in SIB (i.e., under normal operation or abnormal operation of the timing synchronization operation at the gNB). That way UEs moving to a new cell within the same gNB will be able to determine if new clock quality information available should trigger a RRC reconnection, or the UE has the current status stored.

Proposal 2: Confirm the gNB is always broadcasting Event ID in SIB9 if it supports the feature.

## 2.3	Clock quality information provisioning via dedicated RRC signalling

For UEs in RRC_CONNECTED, the gNB can provide the clock quality information as configured by the AMF with the Clock Quality Reporting Control Information IE that contains two fields:

- Clock Quality Detail Level: It indicates whether and which clock quality information to provide to the UE and can take one of the following values: clock quality metrics or acceptable/not acceptable indication.

- Clock Quality Acceptance Criteria: Condition for the gNB (or CN) to determine if the gNB’s timing synchronization status can satisfy the criteria requested or not for time synchronization as a service. This attribute is required if the clock quality detail level equals "acceptable/not acceptable indication”.

Focusing on the Clock Quality Detail Level IE, it defines two different sets of information to be reported to the UE via dedicated RRC signalling:

- If the Clock Quality Detail Level equals "clock quality metrics", the gNB needs to provide the supported RAN TSS attributes. The structure of the IE can follow the same semantics as currently being discussed in RAN3 and provided in Table 1 (R3-233576 F1AP baseline CR). Note Clock Accuracy is still under discussion in RAN3, therefore is listed as FFS.

- If the Clock Quality Detail Level equals "acceptable/not acceptable indication", the gNB needs to provide only the resultant indication, as provided in Table 2.

Table 1: Clock quality information towards the UE if Clock Quality Detail Level equals "clock quality metrics"

| IE/Group Name | Presence | IE type and reference | Semantics description |
| --- | --- | --- | --- |
| Synchronisation State | O | ENUMERATED (locked, holdover, freeRun, …) |  |
| Traceable to UTC | O | ENUMERATED (true, false, …) |  |
| Traceable to GNSS | O | ENUMERATED (true, false, …) |  |
| Clock Frequency Stability | O | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [9]. |
| Clock Accuracy | O | FFS |  |
| Parent Time Source | O | ENUMERATED (syncE, pTP, gNSS, atomicClock, terrestrialRadio, serialTimeCode, nTP, handSet, other, …) |  |

Table 2: Clock quality information towards the UE if Clock Quality Detail Level equals " acceptable/not acceptable indication "

| IE/Group Name | Presence | IE type and reference | Semantics description |
| --- | --- | --- | --- |
| Clock Quality Acceptance Criteria result | O | ENUMERATED (acceptable, not acceptable, …) |  |

Taking a look how RRC signalling may look to provide clock quality information to the UE, the RRC message should include the clock quality information the UE requires (i.e., metrics or acceptable/not acceptable indication) and the Event ID the clock quality information can be associated to. This way the UE can locally associate an Event ID with its own clock quality information. The same RRC messages that include ReferenceTimeInformation IE can be extended to include the new IEs, i.e., DLInformationTransfer message. As an example, ASN.1extensions for clock quality information reporting to the UE can be defined as follows:

DLInformationTransfer-v1800-IEs ::= SEQUENCE { clockQualityInfo-r18 ClockQualityInfo-r18 OPTIONAL } ClockQualityInfo-r18 ::= SEQUENCE { eventID-r18 INTEGER(0..X), clockQuality-Indication-r18 ENUMERATED {acceptable, notAcceptable} OPTIONAL, clockQuality-Metrics-r18 ClockQuality-Metrics-r18 OPTIONAL ClockQuality-Metrics-r18 ::= SEQUENCE { synchronisationState-r18 ENUMERATED {looked, holdover, freerun} OPTIONAL, traceableUTC-r18 ENUMERATED {true, false} OPTIONAL, traceableGNSS-r18 ENUMERATED {true, false} OPTIONAL, freqStability-r18 BIT STRING (SIZE (16)) OPTIONAL, clockAccuracy FFS OPTIONAL, parentTimeSource-r18 ENUMERATED {pTP, gNSS, atomicClock, terrestrialRadio, serialTimeCode, nTP, handSet, other, …) OPTIONAL }

Proposal 3: DLInformationTransfer message is extended to include clock quality information reporting towards the UE (i.e., event ID, clock quality metrics or clock quality indication).

# 3	Conclusion

Remaining issues on RAN timing synchronization status reporting and UE behaviour are discussed in this contribution with the following proposals made:

Proposal 1: Confirm in RAN2 that no AS capability is needed for the UE to support clock quality information mechanisms.

Proposal 2: Confirm the gNB is always broadcasting Event ID in SIB9 if it supports the feature.

Proposal 3: DLInformationTransfer message is extended to include clock quality information reporting towards the UE (i.e., event ID, clock quality metrics or clock quality indication).