3GPP TSG RAN WG2 #123	R2-2307794

Toulouse , FR, Aug 21-25, 2023

Agenda Item:	8.14.3

Source:	ZTE Corporation, Sanechips

Title:	Discussion on Rel-17 leftover issues for QoE

Document for:	Discussion and Decision

Introduction

RAN2 has reached below agreements in previous meetings:

RAN2#121 - Introduce the QoS flows ID information in the RVQoE reporting from the UE.

\RAN2#122 - - RAN2 confirms that buffer level threshold-based triggering of RVQoE reporting is triggered by application layer. - - RAN2 waits for RAN3’s conclusion on the buffer level threshold-based RVQoE reporting. - - RAN2 will align with RAN3’s agreement on support of radio-related event triggers for RVQoE reporting in Rel-18. - - Application layer measurement configuration and reporting for shared spectrum channel access is not supported in R18.

It can be observed that per-slice reporting and radio-related event triggers discussion in RAN2 has been closed. Then the remaining R17 leftover issues is the buffer-level threshold-based RVQoE reporting. This contribution intends to discuss  buffer-level threshold-based RVQoE reporting, with consideration on the new SA4 LS received in [3].

Discussion

An LS [2] was sent from SA4 to RAN2 indicating that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18 based on the corresponding QoE configuration received from the AS layer. However, in SA4#124 meeting, SA4 has further discussed this problem and came to an understanding that buffer level on its own is not an appropriate information for RAN to be able to assist the application due to a variety of reasons, and the other LS [3] was sent from SA4 to RAN2 as following:

- 1 Overall description SA4 replied to RAN2 on their LS on buffer level threshold-based RVQoE reporting in S4-230684, confirming RAN2 preference that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18 based on the corresponding QoE configuration received from the AS layer. However, in SA4#124 meeting, SA4 has further discussed this problem and came to an understanding that buffer level on its own is not an appropriate information for RAN to be able to assist the application due to a variety of reasons, e.g. - A higher buffer level may help reduce the probability of a playback stall, but may also increase the playback latency, so it’s a trade-off that needs to be balanced by applications. Different proprietary application mechanisms attempting to strike this balance may result in different application behavior for same buffer level or same content. - Also, different applications (e.g. low-latency live vs. on-demand video applications) have very different behaviour of maintaining the buffer level. Hence based on this, SA4 would like to update the reply sent in S4-230684, and does not confirm RAN2 preference that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18 based on the corresponding QoE configuration received from the AS layer. SA4 is considering appropriate alternatives for this purpose. SA4 requests RAN2 and RAN3 to take this information in account, and that SA4 will keep RAN2 and RAN3 updated on progress in this direction. - 2 Actions To RAN2, RAN3 ACTION: SA4 kindly requests RAN2 and RAN3 to take the above information in account. SA4 will keep RAN2 and RAN3 updated on their progress of work in this direction.

Based on the above LS, it can be seen that SA4 thinks buffer-level threshold alone cannot fulfill the QoE requirement and they are discussing alternatives in combination with buffer-level threshold-based RVQoE reporting in application layers. However it is uncertain if consensus can be reached shortly in SA4. Therefore it is difficult to confirm the feasibility to support the buffer-level threshold-based RVQoE in AL layer.

Observation 1: SA4 has reverted agreements that it is feasible to support buffer-level threshold-based RVQoE in AL layer, and it is uncertain if consensus can be reached shortly in SA4.

Considering current SA4 status,  there are two alternatives can be considered in RAN2, one is to postpone the support the buffer level threshold-based RVQoE to next release, another is to use AS layer to trigger the buffer-level threshold-based RVQoE reports. For AS layer-based solution, there is no coordination required between applciation layer and AS layer, therefore it is possible to continue the discussion without SA4 involvement.

On one hand, considering the remaining time budget, postpone to R19 can guarantee sufficient study is performed for this feature. On the other hand, since both RAN2/3 has spent a lot of time discussing the details of the buffer-level threshold-based RVQoE, a simple AS layer based solution may be sufficient for R18. Both alternatives are fine by us, and it is suggested that RAN2 decides which alternative is preferred for handling this topic.

Proposal 1:  RAN2 selects between below two options for handling buffer-level threshold-based RVQoE:

- Opt1: Postpone the discussion to R19

- Opt2: Using AS layer to trigger the buffer-level threshold-based RVQoE reporting.

Below discuss possible technical issues if option 2 of proposal 1 is supported with consideration on the progress made in RAN3.

The network can configure UE with two types of events for threshold based RVQoE reporting , the RVQoE reporting is triggered when the buffer level is below a threshold or above a threshold. Whether these two types of events for threshold based RVQoE reporting can be configured simultaneously needs to be further discussed.

Shown above are agreements achieved in previous RAN3 meeting. It can be observed that RAN3 has agreed two types of events for the buffer level threshold based RVQoE reporting. It is ffs whether the two types of thresholds can be used together or not.

Observation 2:  RAN3 has agreed two types of events for the buffer level threshold based RVQoE reporting. And it is ffs whether the two types of thresholds can be used together or not.

Considering the limited time budget, it is preferred to adopt a simple solution would be preferred in order to reduce the work load in RAN2, since anyway the gains to use both types of events for thresholds are not obvious. Therefore it is proposed that if Opt2 of P1 is supported, the two types of events  for threshold based RVQoE reporting cannot be configured simultaneously.

Proposal 2: If opt2 of P1 is supported, the two types of events for threshold based RVQoE reporting (i.e., the RVQoE reporting is triggered when the buffer level is below a threshold or above a threshold) cannot be configured simultaneously.

In addition, another issue that needs to be discussed is the details on entering and leaving conditions for trigger buffer level thresholds RVQoE reports in AS layer. In this case, AL layer still performs the RVQoE measurements periodically and send the RVQoE measurement results to AS layer periodically. It is noticed that currently each buffer level measurements AS layer received from AL layer will includes 8 buffer level results, and how to perform the event checking for the RVQoE measurements received will require additional thinking. Below are possible alternatives that can be considered:

- Alt1: For each buffer level measurements, UE checks whether it fulfills the event configured (e.g., higher than threshold), and only report the ones that fulfills the event.

- Alt2: For the list of buffer level measurements measured in the same period, UE treats them as a unity to check whether it fulfills the event configured, and either discard or report all the measurements within the list.

Observation 3: Below are possible alternatives to check buffer level threshold based  RVQoE measurements in AS layer:

- Alt1: For each buffer level measurements, UE checks whether it fulfills the event configured (e.g., higher than threshold), and only report the ones that fulfills the event.

- Alt2: For the list of buffer level measurements measured in the same period, UE treats them as a unity to check whether it fulfills the event configured, and either discard or report all the measurements within the list.

For alt1, the reported RVQoE results is incomplete which may loss some characteristics to correctly evaluate the QoE conditions therefore it is not preferred.

Observation 4: alt1 of observation3 leads to incomplete results collected within one measurement period which is not good for evaluate the QoE conditions.

Therefore it is proposed that for the list of buffer level measurements measured in the same period, UE treats them as a unity to check whether it fulfills the event configured, and either discard or report all the measurements.

Proposal 3: If opt2 of P1 is supported, UE treats the list of buffer level measurements measured in the same period as a unity to check whether it fulfills the event configured, and either discards or reports all the measurements within the list based on the outcome of evaluation.

Then RAN2 needs to discuss the details to decide whether the trigger condition is satisfied or not. Considering that there may be multiple buffer level of measurement results in the same list, the possible alternatives to check whether the trigger condition is satisfied are shown as below:

- Option 1: trigger reporting when  all buffer level measurements meet the thresholds requirement.

- Option 2: trigger reporting when at least one buffer level measurements meets the thresholds requirement.

- Option 3: trigger reporting when the average of all buffer level measurements meets the thresholds requirement.

- Option 4: trigger reporting, when the average value of all buffer level measurements excluding the maximum and minimum values meets the thresholds requirement.

- Option 5: trigger reporting, when the percentage of buffer level measurements meeting the thresholds requirement exceeds a certain value.

Observation 5: The possible alternatives for AS layer to decide the entering condition for buffer level threshold based event are shown as below:

- Option 1: trigger reporting when  all buffer level measurements meet the thresholds requirement.

- Option 2: trigger reporting when at least one buffer level measurements meets the thresholds requirement.

- Option 3: trigger reporting when the average of all buffer level measurements meets the thresholds requirement.

- Option 4: trigger reporting, when the average value of all buffer level measurements excluding the maximum and minimum values meets the thresholds requirement.

- Option 5: trigger reporting, when the percentage of buffer level measurements meeting the thresholds requirement exceeds a certain value.

For the above judgment methods, Option 1 may be too strict, which is not conducive to the network to obtain effective measurement results in time while option 2 is too loose which cannot fulfill the requirement to select the QoE of interests. As for option 3, the averaged results may be impacted by some random extreme buffer level values, which can that affects the overall judgment. Therefore option 4/5 is introduced to overcome the disadvantage of option 3  Based on above analyse, it is proposed that RAN2 considers between option 4 and option 5 for determine the entering condition for configured buffer level threshold based event.

Observation 6: Option 1/2 is either too strict or too loose for NW to obtain RVQoE measurements as intended, while option 3 is easily impacted by extreme buffer level measurements.

Proposal 4: If opt2 of P1 is agreed, RAN2 selects  between option4/5 of observation 5 to determine the entering condition for configured buffer level threshold based event.

- Option 4: UE triggers reporting, when the average value of all buffer level measurements excluding the maximum and minimum values meets the thresholds requirement.

- Option 5: UE triggers reporting, when the percentage of buffer level measurements meeting the thresholds requirement exceeds a certain value.

Conclusion

In previous sections, the following observations and proposals were made:

Observation 1: SA4 has reverted agreements that it is feasible to support buffer-level threshold-based RVQoE in AL layer, and it is uncertain if consensus can be reached shortly in SA4.

Observation 2:  RAN3 has agreed two types of events for the buffer level threshold based RVQoE reporting. And it is ffs whether the two types of thresholds can be used together or not.

Observation 3: Below are possible alternatives to check buffer level threshold based  RVQoE measurements in AS layer:

- Alt1: For each buffer level measurements, UE checks whether it fulfills the event configured (e.g., higher than threshold), and only report the ones that fulfills the event.

- Alt2: For the list of buffer level measurements measured in the same period, UE treats them as a unity to check whether it fulfills the event configured, and either discard or report all the measurements within the list.

Observation 4: alt1 of observation3 leads to incomplete results collected within one measurement period which is not good for evaluate the QoE conditions.

Observation 5: The possible alternatives for AS layer to decide the entering condition for buffer level threshold based event are shown as below:

- Option 1: trigger reporting when  all buffer level measurements meet the thresholds requirement.

- Option 2: trigger reporting when at least one buffer level measurements meets the thresholds requirement.

- Option 3: trigger reporting when the average of all buffer level measurements meets the thresholds requirement.

- Option 4: trigger reporting, when the average value of all buffer level measurements excluding the maximum and minimum values meets the thresholds requirement.

- Option 5: trigger reporting, when the percentage of buffer level measurements meeting the thresholds requirement exceeds a certain value.

Observation 6: Option 1/2 is either too strict or too loose for NW to obtain RVQoE measurements as intended, while option 3 is easily impacted by extreme buffer level measurements.

Proposal 1:  RAN2 selects between below two options for handling buffer-level threshold-based RVQoE:

- Opt1: Postpone the discussion to R19

- Opt2: Using AS layer to trigger the buffer-level threshold-based RVQoE reporting.

Proposal 2: If opt2 of P1 is supported, the two types of events for threshold based RVQoE reporting (i.e., the RVQoE reporting is triggered when the buffer level is below a threshold or above a threshold) cannot be configured simultaneously.

Proposal 3: If opt2 of P1 is supported, UE treats the list of buffer level measurements measured in the same period as a unity to check whether it fulfills the event configured, and either discards or reports all the measurements within the list based on the outcome of evaluation.

Proposal 4: If opt2 of P1 is agreed, RAN2 selects  between option4/5 of observation 5 to determine the entering condition for configured buffer level threshold based event.

- Option 4: UE triggers reporting, when the average value of all buffer level measurements excluding the maximum and minimum values meets the thresholds requirement.

- Option 5: UE triggers reporting, when the percentage of buffer level measurements meeting the thresholds requirement exceeds a certain value.

# References

- RP-221803, Enhancement on NR QoE management and optimizations for diverse services, June, 2022

- R2-2304492, Reply LS on buffer level threshold-based RVQoE reporting, April, 2023

- R3-233813, Reply LS on buffer level threshold-based RVQoE reporting, Aug, 2023