3GPP TSG RAN WG2 #123	R2-2307795

Toulouse , FR, Aug 21-25, 2023

Agenda Item:	7.14.4

Source:	ZTE Corporation, Sanechips

Title:	Discussion on QoE measurement for NR-DC

Document for:	Discussion and Decision

Introduction

Based on the discussion in RAN2#122 meeting, following agreements on QoE measurements forNR-DC have been achieved:

- 1: The network can use explicit indication per QoE config to indicate which SRB is used for the QoE reporting. Details can be discussed in Stage-3. - FFS how to handle the QoE report transmission if there is only one SRB and the QoE report is not (explicitly) configured for that SRB (to be checked if this is possible according to latest RAN3 agreements) - FFS how the above case works if SCG is deactivated or released. - RAN2 will follow RAN3 agreement on QoE config RRC IDs. - RAN2 will follow RAN3 agreements (for NR-DC when SRB5 is not configured) on forwarding the received encapsulated QoE reports to the correct recipient (i.e. MCE or SN)

In this contribution, we will firstly discuss solutions to the remaining open issues on QoE for NR-DC, which includes how to handle QoE reports, RRC configuration for QoE reporting and so on.

Discussion

QoE configuration over MN/SN

For RVQoE, RAN3 has agreed that the SN can send an RVQoE configuration directly to UE via SRB3 or in a transparent container to the MN, then MN can forward the received configuration to UE the last meeting.

Observation 1: RAN3 has agreed that SN can send an RVQoE configuration directly to UE via SRB3 or in a transparent container forwarded by MN via SRB1.

This behavior can already be supported by existing mechanism. That is, the RVQoE configuration can be provided as part of RRCReconfiguration message as generated by SN gNB. Then MN can include the received SN RRCReconfiguration message container in mrdc-SecondaryCellGroup of MN RRCReconfiguration message, and forward it to UE through SRB1. Therefore it is proposed to agree on below proposal to align with RAN3’s agreements.

Proposal 1: For RVQoE configuration, SN can configure the QoE configuration directly to UE through SRB3, or forwarded by MN via SRB1 as part of SN RRCReconfiguration container (e.g., in mrdc-SecondaryCellGroup) included in MN RRCReconfiguration message.

QoE measurement reporting over MN/SN

Last meeting RAN2 discussed about the case when a QoE configuration is originally configured with one SRB type (e.g., SRB5) and the SRB is released or suspended due to failure in the node or other causes. And in such case if there is available SRB over a different node that can be used for QoE reporting, whether UE can autonomous fallback to another SRB to report the QoE measurements.

Observation 2: RAN2 needs to further discuss whether a QoE originally configured with an SRB type (e.g., SRB5) can autonomous switch to another SRB type (e.g., SRB4) when the configured SRB is released/suspended while there are available SRB type for QoE reports.

For companies supporting autonomous fallback the motivation could be more efficient QoE report, since there are no additional latency caused due to reconfiguration. While for companies not supporting autonomous fallback, the reasoning could be that the receiving node may not have the required information for forwarding the QoE reports or the configuring doesn’t want the other node to read the RVQoE results.

Observation 3: Autonomous fallback to other SRB for reporting is efficient but may risk of the exposing RVQoE to other nodes or the receiving node may not knows the necessary information for forwarding the QoE report.

Whether such behavior is allowed is also relevant to RAN3’s decision on report leg switch and how MN/SN forwards the received reports, e.g.,  to MCE.

RAN3#117bis-e - Turn into an agreement the WA stating that, if QoE reports are received by the SN, the SN can forward the QoE reports to MCE directly. - If a node has configured the UE with QoE measurements, and the other node is receiving the QoE reports from the UE and forwarding them directly to the MCE, then: The node that has configured the UE with QoE measurements should indicate the QoE reference to the node that receives the reports and forwards them directly to MCE. RAN3#119bis-e - The leg switch for QoE reporting needs to be approved by both nodes serving the UE.

Based on RAN3’s agreements shown above it is obvious explicit NW configuration is needed to switch reporting leg regardless.

Observation 4: RAN3 has reached above agreements which indicates that the leg switch for QoE reporting needs to be approved by both nodes serving the UE.

However RAN3 also reached agreements for reporting which indicates that when a node (either MN/SN) receives the  QoE reports, the node can forward the reports to MCE directly as long as the receiving node receives the QoE reference of the QoE report from the configuring node. Which means if a QoE reference of QoE configuration is known to both MN/SN, then autonomous fallback to another SRB for reporting would be possible.

Observation 5: RAN3 has agreed that a node can forward the QoE reports to MCE directly if the QoE reference is known by the receiving node, which means if a QoE reference of QoE configuration is known to both MN/SN, then autonomous fallback to another SRB for reporting would be possible.

For RVQoE, since the RVQoE report is explicitly transmitted in RRC signalling, it's possible that the generating node doesn't want another node to view its RVQoE report, so it does not want UE to switch reporting leg automatically. But the downside of this approach is that RVQoE reports cannot even be uploaded. Another possibility is that RVQoE configuration is jointly decided by MN-SN with negotiation., in such case the generating node thinks that timely reporting is more conducive to optimization, and does not mind to allow UE to switch to another leg for reporting.

Observation 6: For RVQoE report, whether to allow autonomous switch report leg depends on different NW consideration on the time-requirement of RVQoE report and the willingness to share RVQoE report to other node.

Based on the above analysis, a compromised method that can balance between report efficiency and NW requirement on privacy as well as report forwarding, can be to introduce an indication per QoE configuration, where the indication is used to indicate whether autonomous switch is allowed when a QoE configuration is allowed to use another SRB for reporting when the originally configured SRB cannot be used. This indication also allows MN and SN to do negotiation before proving the information to UE, which guarantees that NW always takes full control on the report leg of a QoE configuration. It also complies with RAN3 agreed principle that the leg switch for QoE reporting needs to be approved by both nodes.

Observation 7: An indication to indicate whether autonomous switch report SRB type is allowed or not can be configured per QoE configuration, which balance between report efficiency and NW’s requirements on report forwarding  as well as privacy.

When a QoE Configuration does not receive such indication to allow automatic switching, UE suspends the current reporting when the report SRB is not longer available for reporting, and then waits for the reconfiguration.When a QoE configuration configured with indication to allow autonomous switch, UE switches to another SRB type for reporting if the originally configured SRB is released or suspended. Therefore, based on above analysis, below proposal is given:

Proposal 2: Introduce an indication per QoE configuration to indicate whether autonomous switch report SRB type is allowed or not.

Pause/resume of QoE measurement

Furthermore there are one ffs issue on whether paused QoE reports can be reported to SN (if SN is not overload). In our understanding RAN2 has agreed that legacy pause-resume mechanism is reused in R18, which means the pause -resume is performed in per QoE configuration basis.Then the remaining issue is whether separate pause-resume over MN and SN is needed.

In our understanding the reporting leg shall be controlled by the SRB type configuration, there is no need to mixed the configuration with pause-resume indication. In case both SRB4/5 is configured, if NW considers MN is overloaded and SN is not, then NW can reconfigure the SRB type from SRB4 to SRB5 to allow QoE report over SN or vice versa. Whereas pause-resume indication can be used when NW assumes both MN and SN is overloaded, and would like to pause the QoE report for all.

Observation 8: QoE Report leg is controlled by SRB type configuration associated to the QoE configuration while the pause-resume indication is used to pause or resume the reporting in both legs, there is no need to mix two things.

Therefore, based on above analysis no further enhancements on pause-resume mechanism is needed.

Proposal 3: No need to introduce further enhancement on pause-resume mechanism (e.g., pause-resume separately for MN/SN).

Conclusion

In previous sections, the following observations and proposals were made:

Observation 1: RAN3 has agreed that SN can send an RVQoE configuration directly to UE via SRB3 or in a transparent container forwarded by MN via SRB1.

Observation 2: RAN2 needs to further discuss whether a QoE originally configured with an SRB type (e.g., SRB5) can autonomous switch to another SRB type (e.g., SRB4) when the configured SRB is released/suspended while there are available SRB type for QoE reports.

Observation 3: Autonomous fallback to other SRB for reporting is efficient but may risk of the exposing RVQoE to other nodes or the receiving node may not knows the necessary information for forwarding the QoE report.

Observation 4: RAN3 has reached above agreements which indicates that the leg switch for QoE reporting needs to be approved by both nodes serving the UE.

Observation 5: RAN3 has agreed that a node can forward the QoE reports to MCE directly if the QoE reference is known by the receiving node, which means if a QoE reference of QoE configuration is known to both MN/SN, then autonomous fallback to another SRB for reporting would be possible.

Observation 6: For RVQoE report, whether to allow autonomous switch report leg depends on different NW consideration on the time-requirement of RVQoE report and the willingness to share RVQoE report to other node.

Observation 7: An indication to indicate whether autonomous switch report SRB type is allowed or not can be configured per QoE configuration, which balance between report efficiency and NW’s requirements on report forwarding  as well as privacy.

Observation 8: QoE Report leg is controlled by SRB type configuration associated to the QoE configuration while the pause-resume indication is used to pause or resume the reporting in both legs, there is no need to mix two things.

Proposal 1: For RVQoE configuration, SN can configure the QoE configuration directly to UE through SRB3, or forwarded by MN via SRB1 as part of SN RRCReconfiguration container (e.g., in mrdc-SecondaryCellGroup) included in MN RRCReconfiguration message.

Proposal 2: Introduce an indication per QoE configuration to indicate whether autonomous switch report SRB type is allowed or not.

Proposal 3: No need to introduce further enhancement on pause-resume mechanism (e.g., pause-resume separately for MN/SN).

# References

- RP-221803, Enhancement on NR QoE management and optimizations for diverse services, June, 2022