3GPP TSG-RAN2#123	R2-2307805

Toulouse, 21th Aug – 25th Aug, 2023

Agenda item:		7.20.2 (NR_MIMO_evo_DL_UL-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on TA maintenance in two TAs for multi-TRP

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 made an agreement as follows.

Configure one TAT per TAG to support two TAs for a serving cell, i.e., in this case 2 TAGs are configured for the serving cell.

The concept of PTAG/STAG was discussed, but there was no conclusion.

In this contribution, we discuss TAG modelling and TAG management in mTRP operation.

2.	Discussion

According to current specification, the TAG is defined as a group of serving cells using the same timing reference cell and the same TA value. A TAG containing the SpCell of a MAC entity is referred to as PTAG, and the STAG refers to other TAGs.

For mTRP operation in Rel-18 MIMO evo, the UE is configured with two CORESET pool index for a serving cell, and each CORESET pool index is associated with TRP. In addition, it is generally assumed that each TRP belongs to one TAG differently, i.e. two TAG per serving cell.

Considering that there are two TAGs for a SpCell, one issue in mTRP operation is whether two PTAGs need to be introduced for a MAC entity.

If two PTAGs are introduced, both of two TAGs for SpCell are defined as PTAG. In our view, introducing two PTAGs requires lots of further discussion. Firstly, whether the priority between two PTAGs should be defined. Depending on the decision, further discussion is needed as which PTAG between two PTAGs has priority. In addition, UE behaviour upon TAT expiry needs to be discussed on a case by case, e.g. when one TAT between primary/secondary PTAG expires and when both TAT of PTAG expires, etc.

On the other hands, if the concept of one PTAG is kept as in legacy, one of two TAGs for SpCell is defined as PTAG. In this case, how to define PTAG between two TAG of SpCell needs to be discussed. However, the legacy UE behaviour upon TAT of PTAG expiry can be reused

Comparing two PTAG and one PTAG, we don’t see any benefit of introducing two PTAGs. Therefore, we think it is the simplest way to define only one PTAG same as legacy even if SpCell has two TRPs.

Proposal 1. In mTRP operation, a MAC entity has only one PTAG, same as legacy.

If Proposal 1 is agreed, the next issue to be discussed is how the PTAG is defined in mTRP operation.

In legacy, the PTAG is defined as a TAG containing the SpCell of a MAC entity. Extending this concept to the mTRP operation, we think one of TRP in SpCell should be defined as SpTRP, and the PTAG is defined as a TAG containing the SpTRP of a MAC entity. The other TRPs in SpCell belongs to STAG and TRPs in SCell belongs to PTAG or STAG depending on its own TA.

Proposal 2. In mTRP operation, one of TRPs in SpCell is defined as SpTRP, and PTAG is defined as a TAG containing the SpTRP of a MAC entity. The other TRPs in SpCell belongs to STAG and TRPs in SCell belongs to PTAG or STAG depending on its own TA.

For Proposal 2 to work, the SpTRP should be defined.

In RAN1#121bis, RAN1 agreed that RAR is received from TRP associated with Type-1 CSS. Currently, Type-1 CSS is configured using ra-SearchSpace and ra-SearchSpace is configured to receive RAR, i.e. ra-SearchSpace is configured only for SpCell.

ra-SearchSpace ID of the Search space for random access procedure (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive RAR in this BWP. This field is mandatory present in the DL BWP(s) if the conditions described in TS 38.321 [3], clause 5.15 are met.

According to RRC specification [1], ra-SearchSpace is associated with one SerachSpaceId.

One SearchSpaceId is associated with one ControlResourceSetId and one ControlResourceSetId is associated with one coresetPoolIndex. Thus, we can see the association between ra-SearchSpace and coresetPoolIndex.

As mentioned above, each coresetPoolIndex is associated with TRP. Therefore, one TRP of SpCell would be associated with Type-1 CSS and RAR is received from the TRP of SpCell. We think this TRP can be defined as SpTRP.

Proposal 3. The TRP of SpCell which is associated with Type-1 CSS is defined as SpTRP.

In legacy, the uplink transmission of a serving cell is governed by the TAT. If TAT of PTAG is not running, uplink transmission of all serving cells is not allowed except RA procedure. On the other hand, if TAT of STAG is not running, uplink transmission of all serving cells belonging to the corresponding STAG is not allowed except RA procedure. We think this behaviour can still be applied to mTRP operation.

Proposal 4. If TAT of PTAG is not running, uplink transmission of all TRPs is not allowed except RA procedure.

Proposal 5. If TAT of STAG is not running, uplink transmission of all TRPs belonging to the corresponding STAG is not allowed except RA procedure.

When TAT of TAG is not running, TAC is obtained via UE-initiated RA procedure (CBRA) or PDCCH order based RA procedure (CFRA). Then, the TA is recovered and TAT is restarted for the corresponding TAG.

If TAT is not running and there is any uplink transmission, for PTAG, CBRA is initiated on SpCell to recover TA, whereas for STAG, CBRA is not initiated and relies on the network handling, i.e. CFRA. This is because SpCell is involved in the transmission related to control information, so SpCell needs to be handled specially and immediate recovery is needed via CBRA. We think that this principle should be kept in mTRP operation. For PTAG, if TAT is not running and there is any uplink transmission, CBRA is initiated for SpTRP.

Proposal 6. For PTAG, if TAT is not running and there is any uplink transmission, CBRA is initiated for SpTRP.

In addition, CFRA is also used to manage TA for both PTAG and STAG. We note that RAN1 agreed that PDCCH order can be sent from the other TRP, i.e. cross-TRP RA triggering. The PDCCH order would indicate for which TRP the CFRA should be initiated, and the UE initiates CFRA for the corresponding TRP.

Proposal 7. If PDCCH order indicates a TRP, CFRA is initiated for the corresponding TRP.

3.	Conclusion

In this document, we present our view on TA maintenance in two TAs for multiple TRP, and made proposals as follows.

Proposal 1. In mTRP operation, a MAC entity has only one PTAG, same as legacy.

Proposal 2. In mTRP operation, one of TRPs in SpCell is defined as SpTRP, and PTAG is defined as a TAG containing the SpTRP of a MAC entity. The other TRPs in SpCell belongs to STAG and TRPs in SCell belongs to PTAG or STAG depending on its own TA.

Proposal 3. The TRP of SpCell which is associated with Type-1 CSS is defined as SpTRP.

Proposal 4. If TAT of PTAG is not running, uplink transmission of all TRPs is not allowed except RA procedure.

Proposal 5. If TAT of STAG is not running, uplink transmission of all TRPs belonging to the corresponding STAG is not allowed except RA procedure.

Proposal 6. For PTAG, if TAT is not running and there is any uplink transmission, CBRA is initiated for SpTRP.

Proposal 7. If PDCCH order indicates a TRP, CFRA is initiated for the corresponding TRP.

4.	Reference

- 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification	V17.5.0