# 3GPP TSG RAN WG2 Meeting #123      	                           R2-2307808
Toulouse, France, Aug 21- Aug 25

Agenda item:	7.3.2

Source:	Apple

Title:	Discussion on key open issues of Cell DTX / DRX

WID/SID:	Netw_Energy_NR-Core– Release 18

Document for:	Discussion and Decision

# 1 Introduction

- Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3]

- Note: No change for SSB transmission due to cell DTX/DRX.

- Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

The Network Energy saving (NES) WID RP-223540 was agreed in RAN#98-e [1], the WI objective on Cell DTX / DRX is copied below:

In RAN2#121 [2], RAN2#121b-e [3] and RAN2#122 [4], some progress was made on Cell DTX / DRX. However, there are still some open issues. Meanwhile, there is one on-going post-meeting email discussion#307 [5], and some of issues are quite controversial (e.g. whether to specify NW alignment requirement).

[Post122][307][NES] DTX/DRX – alignment, single/multiple configurations, parameter values (Huawei)

In this contribution, we share our view on below key open issues:

- Key open issue on UE behavior

- Key open issue on signaling

- Alignment between Cell DTX and UE CDRX

- Measurement and reporting

# 2 Discussion

## 2.1 key open issues on UE behaviour

### 2.1.1 Retransmission in non-active duration

In RAN2#122 [4], below agreements were made. Among them, there is one WA on UE retransmission behaviour, as highlighted.

Agreements:

1	UE monitors PDCCH for RAR during Cell DTX non-active time. The ra-ResponseWindow could be started as legacy.

2	UE monitors PDCCH for msg4 during Cell DTX non-active time. The ra-ContentionResolutionTimer could be started as legacy.

3	Working assumption:  When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

4	Once gNB recognizes there is an emergency call or public safety related service (e.g. MPS/MCS), the NW should ensure there is no impact to the emergency call (e.g. may deactivate Cell DTX/DRX).  The behavior is captured in stage 2 spec

5	When an DG grant is received, by the gNB during cell DRX/DTX, the UE follows the grant assignment (i.e. like in legacy).  This includes DL HARQ feedback.

During the online discussion, this WA was agreed because it is majority view: although the UE's extra PDCCH monitoring behaviour (when retransmission timer is running) may impact NES gain, the impact is expected to be limited. This is because time of retransmission is generally only a small fraction of the whole transmission time. We share majority view that this WA can be confirmed.

Proposal 1: Confirm the WA: if both Cell DTX/DRX and UE C-DRX are configured, the UE is expected to monitor PDCCH when the retransmission timer is running, like in legacy.

However, as mentioned in the WA, this is only applicable if UE C-DRX is configured with Cell DTX/DRX. When only Cell DTX/DRX is configured (i.e. UE C-DRX is not configured), we prefer no exceptional PDCCH monitoring for retransmission (i.e. retransmission is not allowed in non-active duration). The reasonings include:

- MNOs should have flexibility to maximize NES gain by disallowing retransmission in non-active duration.

  - If MNOs prefer to consider more about QoS in deployment of Cell DTX/DRX, they can configure Cell DTX/DRX and UE C-DRX simultaneously.

- It is complex and unnecessary to introduce a new retransmission timer for Cell DTX/DRX.

  - If we mimic the WA solution to the case when UE C-DRX is not configured, it will bring complex and unnecessary specification changes, which may include:

      - Whether the new retransmission timer is UE specific or Cell specific?

      - If the new timer is cell specific, how to align the timer value among all the UEs in the cell?

      - If both Cell DTX/DRX and UE C-DRX are configured, which retransmission timer to use?

Observation 1: No need to allow retransmission in non-active duration of Cell DTX/DRX because:

- MNOs should have flexibility to maximize NES gain by disallowing retransmission.

- It is complex and unnecessary to introduce a new retransmission timer for Cell DTX/DRX.

Thus, we propose:

Proposal 2: If only Cell DTX/DRX is configured (i.e. UE C-DRX is not configured), retransmission is not allowed during non-active duration.

Furthermore, please note that RAN2#121 [2] left a FFS on whether to introduce DTX UE specific inactivity timer.

- Pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell.   FFS whether we have DTX UE specific inactivity timer .  FFS on configuration signaling and stage 3.

Following the same logic of Observation 1, we prefer not to introduce any new timer for Cell DTX/DRX, including new RTT / retransmission / inactivity timer.

Proposal 3: Not introduce any new timer (including new RTT / retransmission / inactivity timer) for purpose of extending active time of Cell DTX / DRX pattern due to one particular UE's pending traffic.

### 2.1.2 Handling of CG timer

In RAN2#121b-e [3], it was agreed that the UE doesn't transmit on CG occasion during non-active duration.

- As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods

From section 5.4.1 of TS 38.321

......

For each Serving Cell and each configured uplink grant, if configured and activated, the MAC entity shall:

1>	if the MAC entity is configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response or with the PUSCH duration of an uplink grant addressed to Temporary C-RNTI or the PUSCH duration of a MSGA payload for this Serving Cell; or

1>	if the MAC entity is not configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response or the PUSCH duration of a MSGA payload for this Serving Cell:

2>	set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration;

2>	if, for the corresponding HARQ process, the configuredGrantTimer is not running and cg-RetransmissionTimer is not configured (i.e. new transmission):

3>	consider the NDI bit for the corresponding HARQ process to have been toggled;

3>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

......

To implement this agreement, we think the UE behaviour on configuredGrantTimer needs clarification. According to TS 38.321 [6], when configuredGrantTimer expires, the UE assumes positive acknowledge for the corresponding HARQ process.

Observation 2: According to TS 38.321, when configuredGrantTimer expires, the UE assumes positive acknowledge for the corresponding HARQ process.

Upon entering non-active periods of Cell DRX, it is possible that the UE has pending HARQ process(es) with the corresponding configuredGrantTimer running. Then, because the UE does not transmit on CG occasions and doesn't monitor PDCCH, all the running configuredGrantTimer should be stopped. Otherwise, the above CG timer mechanism will make UE to mistakenly drop pending UL traffic in buffer. It will cause UL data loss and not sync between UE and NW.

Proposal 4: Upon entering non-active periods of Cell DRX, the UE stops all the running configuredGrantTimer.

With regarding to cg-RetransmissionTimer, we think RAN2 may deprioritize its discussion because it is only target for NR-U.

## 2.2 Key open issues on signalling

### 2.2.1 L1/L2 group common signalling

In RAN1#113 [7], it was agreed to support UE group common L1 signaling for Cell DTX/DRX activation / deactivation. RAN1 have sent reply LS to RAN2 in [8], and ask RAN2 to consider the additional support of a MAC-CE indication.

With respect to questions asked by RAN2, RAN1 has made the following agreements: - Support of L1 signaling at least for activation/deactivation of a cell DTX and/or DRX configuration is feasible (e.g., in terms of enabling/disabling the feature) from RAN1 perspective. - RAN1 supports the group common L1 signaling using PDCCH for cell DTX/DRX activation and deactivation without HARQ feedback. - subject to UE capability - RAN1 asks RAN2 to consider the additional support of a MAC CE based indication. RAN1 is further working on the details of the group common L1 signaling using PDCCH and will inform RAN2 as further details are agreed and made available.

First, we would like RAN2 to confirm the support of RAN1 introduced UE group common signaling. Furthermore, we think it is necessary to clarify both implicit activation / deactivation by RRC signaling (as agreed in RAN2#121b-e [3]) and explicit activation / deactivation by UE group common DCI are supported. And NW indicates the UE which way to go in RRC similar to CG type1 / type2.

Agreements of RAN2#121b-e

3.  As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released.

Proposal 5: RAN2 confirm that both implicit (by RRC) and explicit (by UE group common DCI) activation / deactivation are supported. And NW indicates the UE which way to go in RRC similar to CG type1 / type2.

Then, regarding to additional support of a MAC CE based indication, we don't prefer to introduce it. Its intention is duplicated with UE group common DCI but don't enjoy the benefit of signaling overhead reduction. One may argue that MAC CE based indication can improve the robust performance over UE group common DCI. However, we think it is an RAN1 issue. Since RAN1 didn't raise any concern on its robust performance in the reply LS to RAN2 [8], we see no reason for RAN2 to consider such enhancement.

Proposal 6: Not introduce additional support of a MAC CE based indication for Cell DTX/DRX activation / deactivation.

### 2.2.2 Alignment between Cell DTX and Cell DRX

In post-meeting email discussion#307 [5], one controversial issue is whether Cell DTX and Cell DRX with different patterns (e.g. slot offset) can be activated simultaneously for the same serving cell, i.e. whether Case a) in Figure.1 is allowed.

Figure.1 Illustration of different understandings of joint Cell DTX and cell DRX operation

This issue was controversial in [5]:

- View 1: some companies think that only Case b) is allowed. Their justifications mainly include:

  - The NES gain of case a) over case b) is not clear because the power consumption of DL is much higher than UL according to TR38.864 [9].

  - DL and UL are sometimes tightly coupled (e.g. DL transmission and its UL HARQ feedback). Allowing case a) will significantly complicate UE behaviors and introduce extra standard work.

- View 2: some companies think that Case a) is also allowed and alignment between Cell DTX and Cell DRX configuration are left to NW implementation.

We prefer View 1, i.e. only Case b) is allowed. For View 2, it is not clear to us how NW implementation can work without introducing new UE behaviors. At least in the 4 scenarios illustrated in Figure. 2, new UE behavior needs to be specified:

- Case a): UE sent SR in active duration of Cell DRX, but can’t monitor followed PDCCH in non-active duration of Cell DTX. Possible solutions include:

  - Alt-1: introduce new exceptional UE behavior to keep monitoring PDCCH after sending SR.

  - Atl-2: allow the UE to postpone SR transmission to next active duration of Cell DRX when a configured monitoring window is non-active duration of Cell DTX.

- Case b): UE sent CG in active duration of Cell DRX, but can’t monitor its HARQ feedback in non-active duration of Cell DTX. Possible solutions include:

  - Alt-1: introduce new UE behavior to stop CG timer in active duration of Cell DRX if a configured monitoring window is within non-active duration of Cell DRX.

- Case c): NW sent SPS in active duration of Cell DTX, but the UE can’t send HARQ feedback in non-active duration of Cell DRX. Possible solutions include:

  - Alt-1: introduce new exceptional UE behavior to send HARQ A/N in non-active duration of Cell DTX when reception of SPS.

  - Alt-2: introduce a new UL HARQ timeline which can be postponed to active-duration of Cell DRX.

- Case d): NW sent DCI to trigger aperiodic CSI in active duration of Cell DTX, but the UE can’t send CSI in non-active duration of Cell DRX. Possible solutions include:

  - Alt-1: introduce new exceptional UE behavior to send aperiodic CSI in non-active duration of Cell DRX when reception of DCI triggering.

  - Alt-2: introduce a new aperiodic CSI reporting timeline to postpone to active-duration of Cell DRX.

Figure.2 Illustration of 4 scenarios where new UE behavior needs to be specified if Cell DTX and Cell DRX with different patterns are activated in one serving cell

Please note that NW implementation (e.g. dynamically change Cell DTX pattern and/or Cell DRX pattern) can't work in above 4 scenarios because Cell DTX/DRX pattern is common to all Rel-18 UEs in the cell (i.e. DCI to activate / deactivate Cell DTX / DRX is common to a group of UEs).

Observation 3: If Cell DTX and Cell DRX have different patterns, new UE behaviors have to be introduced to handle special cases. NW implementation (e.g. dynamically change Cell DTX pattern and/or Cell DRX pattern) can't work because Cell DTX/DRX pattern is common to all Rel-18 UEs in the cell.

Thus, we propose to confirm Case a) is not allowed.

Proposal 7: When Cell DTX and Cell DRX are jointly operated, only single pattern (i.e. one set of periodicity, offset and on duration) can be activated for one serving cell.

If Proposal 7 can't be agreed, we think it is necessary to discuss how to specify UE behavior in the above 4 cases.

Proposal 8: If Proposal 7 is not agreed, RAN2 discuss how to specify UE behaviors in below 4 cases:

- Case a): UE sent SR in active duration of Cell DRX, but can’t monitor followed PDCCH in non-active duration of Cell DTX.

- Case b): UE sent CG in active duration of Cell DRX, but can’t monitor its HARQ feedback in non-active duration of Cell DTX.

- Case c): NW sent SPS in active duration of Cell DTX, but the UE can’t send HARQ feedback in non-active duration of Cell DRX.

- Case d): NW sent DCI to trigger aperiodic CSI in active duration of Cell DTX, but the UE can’t send CSI in non-active duration of Cell DRX.

## 2.3 Alignment between Cell DTX and UE CDRX

In post-meeting email discussion#307 [5], another controversial issue is whether to specify NW alignment requirement between Cell DTX and UE C-DRX.

We think it is not necessary to specify alignment requirement for NW. If such requirement is agreed, one followed issue is that the NW may be forced to immediately reconfigure multiple UE CDRX upon Cell DTX is activated, to ensure each UE's CDRX on-duration within on-duration of Cell DTX. Such reconfiguration of UE CDRX requires the NW to send multiple RRC messages immediately, which is unnecessarily complex and incurs high signalling overhead. In our understanding, such strong requirement is unnecessary. As long as the UE behaviour in all possible overlapping duration is clear, we can leave it to NW implementation to choose appropriate configurations of UE CDRX and Cell DTX.

Observation 4: If the requirement for NW is agreed, one followed issue is that the NW may be forced to immediately reconfigure multiple UE CDRX upon Cell DTX is activated, to ensure each UE's CDRX on-duration within on-duration of Cell DTX. It is unnecessarily complex and incurs high signalling overhead.

Thus, we propose:

Proposal 9: As long as UE behavior in all possible overlapping duration is clear, it is unnecessary to introduce NW alignment requirement between Cell DTX and UE CDRX, i.e. it is up to NW implementation to choose appropriate configurations of UE CDRX and Cell DTX.

Figure. 3 illustrates 3 possible overlapping durations between Cell DTX and UE CDRX. In our understanding, UE behaviour in T2 is already covered by below agreement made in RAN2#121b-e [3] (i.e. the UE doesn't monitor PDCCH).

- The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.   UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.   FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

FFS how to deal with retransmissions

However, the UE/gNB behaviour during T1 and T3 in Figure.2 is not clear. Thus, further discussion is needed.

- T1 (Part of OFF duration of UE DRX is overlapped with active duration of Cell DTX)

- T3 (OFF duration of UE DRX is overlapped with non-active duration of Cell DTX)

Figure.3 Illustration of issue scenarios of Cell DTX and UE CDRX alignment

Our analysis for the two cases are:

- For T1, we think the reasonable UE behaviour is to follow UE CDRX behaviour (i.e. stop monitoring PDCCH) by assuming that gNB will not transmit DCI for it (i.e. gNB may transmit DCI for other UEs).

- For T3, although final agreement of UE behaviour in Cell DTX is not made yet, it seems that the common understanding is that Cell DTX has more restricted UE behaviours than UE CDRX (e.g. all SPS / CG are allowed in OFF duration of UE CDRX). Thus, we think the reasonable UE behaviour is to follow Cell DTX.

Proposal 10: RAN2 agree below UE/gNB behavior in T1 and T3 durations:

- T1 (OFF duration of UE CDRX is overlapped with active duration of Cell DTX): the UE follows UE CDRX behaviour (i.e. stop monitoring PDCCH), assuming no DCI for itself.

- T3 (OFF duration of UE CDRX is overlapped with non-active duration of Cell DTX): the UE follows behaviour specified for Cell DTX.

## 2.4 Measurement and reporting

For RRC_CONNECTED UE, it may perform the following measurements and reporting:

- SSB based L1/L3 measurements and reporting

- CSI-RS based L1/L3 measurement and reporting (including TRS)

- PRS based measurement and reporting

For SSB based measurement / reporting, according to below note in WID objective, SSB transmission is not impacted by Cell DTX / DRX.

- Note: No change for SSB transmission due to cell DTX/DRX.

However, it is not clear whether the UE can stop or relax measurement requirement during non-active duration of Cell DTX/DRX, considering gNB is sleeping. This issue should be first discussed in RAN4 and RAN1.

For CSI-RS/PRS based measurement / reporting, it is not clear whether gNB will stop transmitting CSI-RS/PRS during non-active duration of Cell DTX/DRX to save power consumption. In addition, if it is allowed, it is not clear whether the UE can stop or relax CSI-RS based measurement requirement during non-active duration of Cell DTX/DRX, considering gNB is sleeping. This issue should be first discussed in RAN1 and RAN4.

Observation 5: Although SSB transmission is not impacted by Cell DTX / DRX, whether the UE can stop or relax measurement during non-active duration of Cell DTX/DRX need to be first discussed in RAN4/RAN1.

Observation 6: For CSI-RS/TRS/PRS, whether they can be turned off and their measurement requirement during non-active duration need to be first discussed in RAN4/RAN1.

Measurement and reporting requirements are RAN4 expertise, but they may have RAN2 impact (e.g. whether/how the UE performs RRM/RLM/BFR and its signaling). Before starting work on RRM/RLM/BFR, RAN2 need to know whether any change on measurement requirements. Thus, we propose RAN2 send LS to RAN1/RAN4 on requirements of measurement and reporting.

Proposal 11: In order to start RRM/RLM/BFR work, RAN2 send LS to RAN1/RAN4 on whether any change on requirements of measurement and reporting based on SSB/CSI-RS/TRS/PRS.

# 3 Conclusion

In this contribution, we further discuss key open issues of Cell DTX / DRX. Our observations are:

Observation 1: No need to allow retransmission in non-active duration of Cell DTX/DRX because:

- MNOs should have flexibility to maximize NES gain by disallowing retransmission.

- It is complex and unnecessary to introduce a new retransmission timer for Cell DTX/DRX.

Observation 2: According to TS 38.321, when configuredGrantTimer expires, the UE assumes positive acknowledge for the corresponding HARQ process.

Observation 3: If Cell DTX and Cell DRX have different patterns, new UE behaviors have to be introduced to handle special cases. NW implementation (e.g. dynamically change Cell DTX pattern and/or Cell DRX pattern) can't work because Cell DTX/DRX pattern is common to all Rel-18 UEs in the cell.

Observation 4: If the requirement for NW is agreed, one followed issue is that the NW may be forced to immediately reconfigure multiple UE CDRX upon Cell DTX is activated, to ensure each UE's CDRX on-duration within on-duration of Cell DTX. It is unnecessarily complex and incurs high signalling overhead.

Observation 5: Although SSB transmission is not impacted by Cell DTX / DRX, whether the UE can stop or relax measurement during non-active duration of Cell DTX/DRX need to be first discussed in RAN4/RAN1.

Observation 6: For CSI-RS/TRS/PRS, whether they can be turned off and their measurement requirement during non-active duration need to be first discussed in RAN4/RAN1.

Based on observations, our proposals can be found below.

Key open issues on UE behavior

Proposal 1: Confirm the WA: if both Cell DTX/DRX and UE C-DRX are configured, the UE is expected to monitor PDCCH when the retransmission timer is running, like in legacy.

Proposal 2: If only Cell DTX/DRX is configured (i.e. UE C-DRX is not configured), retransmission is not allowed during non-active duration.

Proposal 3: Not introduce any new timer (including new RTT / retransmission / inactivity timer) for purpose of extending active time of Cell DTX / DRX pattern due to one particular UE's pending traffic.

Proposal 4: Upon entering non-active periods of Cell DRX, the UE stops all the running configuredGrantTimer.

Key open issues on signaling

Proposal 5: RAN2 confirm that both implicit (by RRC) and explicit (by UE group common DCI) activation / deactivation are supported. And NW indicates the UE which way to go in RRC similar to CG type1 / type2.

Proposal 6: Not introduce additional support of a MAC CE based indication for Cell DTX/DRX activation / deactivation.

Proposal 7: When Cell DTX and Cell DRX are jointly operated, only single pattern (i.e. one set of periodicity, offset and on duration) can be activated for one serving cell.

Proposal 8: If Proposal 7 is not agreed, RAN2 discuss how to specify UE behaviors in below 4 cases:

- Case a): UE sent SR in active duration of Cell DRX, but can’t monitor followed PDCCH in non-active duration of Cell DTX.

- Case b): UE sent CG in active duration of Cell DRX, but can’t monitor its HARQ feedback in non-active duration of Cell DTX.

- Case c): NW sent SPS in active duration of Cell DTX, but the UE can’t send HARQ feedback in non-active duration of Cell DRX.

- Case d): NW sent DCI to trigger aperiodic CSI in active duration of Cell DTX, but the UE can’t send CSI in non-active duration of Cell DRX.

Alignment between UE CDRX and Cell DTX/DRX

Proposal 9: As long as UE behavior in all possible overlapping duration is clear, it is unnecessary to introduce NW alignment requirement between Cell DTX and UE CDRX, i.e. it is up to NW implementation to choose appropriate configurations of UE CDRX and Cell DTX.

Proposal 10: RAN2 agree below UE/gNB behavior in T1 and T3 durations:

- T1 (OFF duration of UE CDRX is overlapped with active duration of Cell DTX): the UE follows UE CDRX behaviour (i.e. stop monitoring PDCCH), assuming no DCI for itself.

- T3 (OFF duration of UE CDRX is overlapped with non-active duration of Cell DTX): the UE follows behaviour specified for Cell DTX.

Measurement and reporting

Proposal 11: In order to start RRM/RLM/BFR work, RAN2 send LS to RAN1/RAN4 on whether any change on requirements of measurement and reporting based on SSB/CSI-RS/TRS/PRS.

# 4 References

[1] RP-223540, New WID: Network energy savings for NR, Huawei

[2] RAN2#121, Chair Notes

[3] RAN2#121b-e, Chair Notes

[4] RAN2#122, Chair Note

[5] R2-230xxxx, Outcome of [Post122][307][NES] DTX/DRX – alignment, single/multiple configurations, parameter values (Huawei), Huawei.

[6] TS 38.321-v17.1.0, Medium Access Control (MAC) protocol specification, 2022-6.

[7] RAN1#113, Chair Notes

[8] R1-2306169, Draft Reply LS on Cell DTX/DRX activation/deactivation.

[9] TR 38.864-v1.0.0, Study on Network energy savings for NR, 2022-12.