# 3GPP TSG RAN WG2 Meeting #123      	                           R2-2307813
Toulouse, France, Aug 21- Aug 25

Agenda item:	7.16.2.1

Source:	Apple

Title:	Remaining issues on model ID and AI/ML architecture

WID/SID:	FS_NR_AIML_air – Release 18

Document for:	Discussion and Decision

# 1 Introduction

SID of AI/ML for NR air interface (RP-213599) was agreed in RAN#94e [1]. After several rounds of discussion, RAN2 scope mainly include AI/ML model identification, signaling of AI/ML model transfer / delivery, and procedure of LCM and data collection.

Up to now, AI/ML model identification made some progress in RAN2. In RAN2#121b-e [2], below high level agreements were made:

Model ID can be used to identify model or models for the following LCM purposes:

- model selection/activation/deactivation/switching (or identification, if that will be supported as a separate step).

- (e.g. for so called “model ID based LCM”)

If model transfer/delivery is supported, model ID can be used for model transfer/delivery LCM purpose.

How to achieve globality of the Model ID is FFS.

- Initial discussion in RAN2: the following global unique model ID definition directions can be considered as a starting point:

- Direction1: Pre-defined/hard-coded global unique model ID

- Direction3: Assigned global unique model ID via specific ID management node.

- Note: Other global unique model ID definition is not precluded.

- Model ID structure, if any, is FFS

R2 assumes that Information such as FFS:vendor info, applicable conditions, model performance indicators, etc. may be required for model management and control, and should, as a starting point, be part of meta information.

Meanwhile, there is an on-going post-meeting email discussion on mapping of functions to physical entities:

[Post122][060][AIML] Mapping of functions to physical entities (CMCC)

Scope: Starting from relevant contents in R2-2305613, attempt to produce an agreeable description of Mapping of functions to physical entities. UP to rapp to structure

Intended outcome: Report

Deadline: Long

There were some controversial discussions during the email discussion [3]. We would like to also share our views on these issues.

In this contribution, we continue to discuss remaining issues on Model ID and AI/ML architecture.

- Model ID and meta information

- Remaining issues on physical entity mapping

# 2 Discussion

## 2.1 Model ID and meta info

### 2.1.1 Use case of model ID

In RAN2#121b-e [2], RAN2 confirmed below use cases of model ID, i.e. model identification, model transfer/delivery and LCM.

Model ID can be used to identify model or models for the following LCM purposes:

- model selection/activation/deactivation/switching (or identification, if that will be supported as a separate step).

- (e.g. for so called “model ID based LCM”)

If model transfer/delivery is supported, model ID can be used for model transfer/delivery LCM purpose.

First, there seems some understanding ambiguity on model identification. So, we provide our understanding. In our understanding, AI model identification is used during product development phase as part of feature alignment. Specifically, based on "globally" unique model ID, AI model identification can be done between vendors/operators during product development phase as part of feature alignment. The procedure is not needed to be specified, i.e. up to implementation.

Proposal 1: Model identification is performed between vendors/operators during product development phase as part of feature alignment. This procedure is not needed to be specified.

Then, we think one use case of model ID is missed: model pairing in two-sided model (e.g. training collaboration type 2 and training collaboration type 3 without model transfer):

- For training collaboration type 2 with joint training of the two-sided model at network side and UE side jointly, the NW and UE will align the model ID following 3GPP specified model ID format. Gradient exchanges are between partial models with the same model ID.

- For training collaboration type 3 with separate training at NW side and UE side, entity who trained first will select the model ID based on 3GPP specificized model ID format. For example, for UE-first training, UE will collect data, perform initial training, select a model ID, and generate the training dataset for NW training. The dataset is transmitted together with the model ID label, so NW side can perform separate training using the received dataset and knows how to pair the UW side model with UE side model.

Observation 1: Model ID is used for model pairing between UE and NW in two-sided model at least for training collaboration type 2 and type 3.

Thus, we propose:

Proposal 2: Besides model identification, LCM and model delivery, AI/ML model ID can also be used for model pairing between UE and NW in two-sided model without model transfer (e.g. training collaboration type 2 and type 3).

### 2.1.2 Model ID generation

In RAN2#121 [4] and RAN2#121b-3 [2], it was agreed that model ID is global unique and two directions on how to achieve globality are identified.

RAN2 assumes that Model ID is unique “globally”, e.g. in order to manage test certification each retrained version need to be identified.

How to achieve globality of the Model ID is FFS.

- Initial discussion in RAN2: the following global unique model ID definition directions can be considered as a starting point:

- Direction1: Pre-defined/hard-coded global unique model ID

- Direction3: Assigned global unique model ID via specific ID management node.

- Note: Other global unique model ID definition is not precluded.

- Model ID structure, if any, is FFS

For the FFS on how to achieve globality of the Model ID, RAN1#113 also identified similar two directions:

Agreement For model identification of UE-side or UE-part of two-sided models, categorize model identification types as follows, and further study relevant aspects, necessity, and specification impact (if any). - Type A: Model is identified to NW (if applicable) and UE (if applicable) without over-the-air signaling - The model may be assigned with a model ID during the model identification, which may be referred/used in over-the-air signaling after model identification. - FFS: Spec impact to other WGs - Type B: Model is identified via over-the-air signaling, - Type B1: - Model identification initiated by the UE, and NW assists the remaining steps (if any) of the model identification - the model may be assigned with a model ID during the model identification - FFS: details of steps - Type B2: - Model identification initiated by the NW, and UE responds (if applicable) for the remaining steps (if any) of the model identification - the model may be assigned with a model ID during the model identification - FFS: details of steps - Note: The support and applicability of each model identification Type is a separate discussion. This study does not imply that model identification is necessary.

We think the evaluation for Direction 3 in Study Item phase is difficulty because whether to introduce a new NF is SA2 expertise. But SA2 is not involved in Rel-18 SI.

Observation 2: The evaluation for Direction 3 in Study Item phase is difficulty because whether to introduce a new NF is SA2 expertise. But SA2 is not involved in Rel-18 SI.

Based on above consideration, we propose that RAN2 to conclude both Directions are feasible, and they can be further studied in normative phase.

Proposal 3: On how to achieve globality of the Model ID, RAN2 capture in TR that both Direction 1 and Direction 3 are feasible, and they can be further studied in normative phase.

With regarding to model ID structure, we think 3GPP can consider putting multiple essential fields together. Specifically, AI/ML model ID should at least include UE/NW vendor ID, PLMN ID, use case, version number etc., so that the global model ID is unique among different operators and UE/NW vendors. Its details need further discussion.

Proposal 4: The global model ID is specified by putting multiple essential fields together. At least use case, UE/NW vendor ID, PLMN ID, and version number are included. FFS other fields.

Finally, for model ID in LCM purpose (model selection/activation/deactivation/switching), since LCM is between UE and gNB after model identification is aligned, it is possible to use a local model ID which is configured in RRC, to reduce overhead of global model ID.

Observation 3: For model ID in LCM purpose (model selection/activation/deactivation/switching), since LCM is between UE and gNB after model identification is aligned, it is possible to use a local model ID which is configured by RRC, to reduce overhead of global model ID.

Meanwhile, a local ID can also alleviate some security and privacy concern on possible exposure of information related to the model, e.g. vendor information of the model.

Observation 4: A local model ID can also alleviate some security and privacy concern on possible exposure of information related to the model, e.g. vendor information of the model.

Thus, we propose:

Proposal 5: For LCM purpose (model selection/activation/deactivation/switching/monitor), model ID can be a local model ID which is configured in RRC.

### 2.1.3 Meta information

RAN2 agreed to introduce meta info of AI/ML model from Management or Control point of view in RAN2#119b-e and some further agreements were made in RAN2#121b-e [2].

R2 assumes that from Management or Control point of view mainly some meta info about a model may need to be known, details FFS.

R2 assumes that Information such as FFS:vendor info, applicable conditions, model performance indicators, etc. may be required for model management and control, and should, as a starting point, be part of meta information.

In our understanding, the meta info means some important AI/ML model description information which are not essential to be included in model ID. For example, the meta info can include model input type/size, model output type/size, model file type/size, etc. Similar to global model ID, its details need further discussion. Model ID is mandatory but meta info should be optional.

Proposal 6: Meta info of an AI/ML model is optional information which includes important model description information except the fields of model ID (i.e. supplement to model ID).

Proposal 7: Meta info includes at least model input type/size, model output type/size, model file type/size/ compression status.

We disagree to include below information in meta info:

- Vendor info:

  - We think the global unique model ID can already identify the vendor. So, the further vendor information is not needed.

  - We think further vendor info is sensitive and there are user privacy issues.

  - We don't understand why 3GPP need to specify vendor information.

- Model version info

  - Based on RAN2 agreement " Model ID is unique “globally”, e.g. in order to manage test certification each retrained version need to be identified ", we think model ID can identify model version.

- Model performance indicators

  - We think it is dynamic information, which seems not suitable to include in meta info.

Proposal 8: Meta info does not include vendor information, model version info and model performance indicators.

### 2.1.4 Format of model representation file

Finally, it is worth discussing the format of model representation file, which is the main payload which represent the trained model itself. In our understanding, we can have below possible formats:

- Binary image.

  - To compile the model to a run-time binary image, device hardware specific information is needed. Binary image can only be used for model delivery between vendor server to the same vendor devices.

- Existing model representation formats in industrial

  - The number of popular formats is actually quite limited which are illustrated in below Table 1.

- Public format (e.g. ONNX)

- 3GPP specify a new model representation format.

Among them, we think 1) and 4) should be precluded to be endorsed in 3GPP due to below reasons:

- 1) needs device hardware specific information to compile the model to a run-time binary image. Thus, binary image can only be used for model delivery between vendor server of the same vendor. It doesn't make sense for 3GPP to endorse device hardware specific format.

- 4) is conflicted with below note in SID objective:

Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved.

Observation 5: Binary image needs device hardware specific information to compile the model to a run-time binary image. Thus, it can only be used for model delivery between vendor server of the same vendor.

Observation 6: The number of model representation formats in industrial is quite limited.

For existing AI/ML model representation formats illustrated in 2) and 3), we think some of them can be endorsed as 3GPP defined format. RAN2 can further study which mode formats are necessary. Meanwhile, if more than 1 model representation formats are endorsed, maybe some model format coordination procedure between UE and NW needs to be specified when UE and NW support different model formats. We think it can also be further studied.

Proposal 9: Endorse some existing AI/ML model representation formats (e.g., h5, ONNX) as 3GPP defined format, and RAN2 do not specify new model format for model delivery. FFS which existing model representation format(s) are endorsed. FFS whether / how to specify model format coordination procedure between UE and NW.

Table 1: Existing popular model representation formats

## 2.2 Remaining issues on physical entity mapping

We think there are 3 key open issues, and would like to share our view in this section.

Remaining issue 1: whether CN entity is considered for model training

We prefer not to include CN entity in this release of AI/ML SI/WI because below considerations:

- CN is not suitable to train models for PHY procedures.

It is basic principle in 3GPP that radio resource configurations should not be exposed to CN. As we know, CSI compression / prediction and beam management are pure PHY procedure between UE and gNB. For positioning, maybe LMF can train models but it needs SA2 confirmation. Other CN entities are not suitable for model training.

- It is conflicted with SID objective

In existing specification, no NF in CN is specified to support AI/ML modeling training yet. If they are allowed, it will be conflicted with below Note captured in SID of AI/ML for NR air interface (RP-213599).

Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

Observation 7: Currently no NF in CN is specified to support AI/ML modeling training. If offline training is performed in one new NF in CN, it is conflicted with Note in SID " The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced".

Thus, we propose to exclude CN entity for model training.

Proposal 10: Exclude CN entity (except LMF) for model training in Rel-18 AI/ML for air interface.

Remaining issue 2: whether the UE may make decision on Model/functionality control

In the table of CSI compression with two-sided model, only gNB is captured to make decision on Model/functionality control in row e). Whether the UE can also make decision is FFS.

Table 1: The mapping of functions to physical entities for CSI compression with two-sided model AL/ML functions (if applicable); Mapped entities. a); Model training(offline training); gNB, OAM, OTT server, UE, [FFS: CN]. b); Model transfer/delivery; For training Type 1: gNB->UE, or OAM->gNB&UE, or OTT server->gNB&UE, or UE->gNB, [FFS: CN->gNB&UE] For training Type 3: - For UE part of two-sided model: OTT server->UE, [FFS: CN->UE]; - For NW part of two-sided model: OAM->gNB, [FFS: CN->gNB];. c); Inference; NW part of two-sided model: gNB UE part of two-sided model: UE. d); Model/functionality monitoring; NW-side: NW monitors the performance UE-side: UE monitors the performance and may report to NW. e); Model/functionality control (selection, (de)activation, switching, updating, fallback); gNB, [FFS: UE]. Note 1: For a), only data collection part may be further discussed, how to perform the model training is up to implementation. Note 2: For b), no model transfer/delivery is expected if the entity for model training and model inference is the same one. Note 3: Whether/how OAM is to be involved may need to consult RAN3, SA5. Note 4: Whether/how CN is to be involved may need to consult SA2.

Please note that UE autonomous model switch was agreed in RAN1#112 [5], as highlighted below:

Agreement

For UE-side models and UE-part of two-sided models:

- In functionality-based LCM

  - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI).

  - Models may not be identified at the Network, and UE may perform model-level LCM.

    - Study whether and how much awareness/interaction NW should have about model-level LCM

- In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

Meanwhile, in RAN1 discussion on CSI compression, one solution is: UE can choose the proper model/CSI output based on rank. gNB configures some restrictions. UE let NW know the final selection via CSI report.

Thus, we don't see any reason that RAN2 can preclude one RAN1 agreed direction. So, UE should also be captured to make decision on Model / functionality control.

Proposal 11: In the table of CSI compression with two-sided model, capture that UE may make decision on Model / functionality control.

Remaining issue 3: whether to capture details of Model/functionality monitoring

We take example of the table of CSI compression with two-sided model. As highlighted, details of RAN1 agreements on monitoring are captured.

| d) | Model/functionality monitoring | NW-side: gNB monitors the performance UE-side: UE monitors the performance and reports to NW |
| --- | --- | --- |

We think this table is just intended to focus on possible function entries and no need to discuss / capture details of RAN1 agreement on monitoring. Because RAN1 related agreement is already captured in TS 38.843, it is not clear why RAN2 need to capture duplicated details in this table. And RAN1 may add new monitoring solution, which may be conflicted with this table.

Observation 8: RAN1 agreements on model / functionality monitoring were already captured in TS 38.843. It is not clear why RAN2 need to capture duplicated details in the function mapping table.

Thus, we propose to only capture function entity (e.g. UE, gNB, LMF) in the row of "Model/functionality monitoring", and the details of RAN1 agreements should be removed. The same issue is in the row of " Model/functionality control (selection, (de)activation, switching, updating, fallback)". Thus, we propose to simplify the description of both row d) and e) of all tables.

Proposal 12: Simplify the description in row d) and e) of all tables, i.e., only capture function entity (e.g. UE, gNB, LMF) and remove the details related to RAN1 agreements.

# 3 Conclusion

In this contribution, we further discuss RAN2 general aspects of Rel-18 AI/ML for air interface. Our observations are:

Observation 1: Model ID is used for model pairing between UE and NW in two-sided model at least for training collaboration type 2 and type 3.

Observation 2: The evaluation for Direction 3 in Study Item phase is difficulty because whether to introduce a new NF is SA2 expertise. But SA2 is not involved in Rel-18 SI.

Observation 3: For model ID in LCM purpose (model selection/activation/deactivation/switching), since LCM is between UE and gNB after model identification is aligned, it is possible to use a local model ID which is configured by RRC, to reduce overhead of global model ID.

Observation 4: A local model ID can also alleviate some security and privacy concern on possible exposure of information related to the model, e.g. vendor information of the model.

Observation 5: Binary image needs device hardware specific information to compile the model to a run-time binary image. Thus, it can only be used for model delivery between vendor server of the same vendor.

Observation 6: The number of model representation formats in industrial is quite limited.

Observation 7: Currently no NF in CN is specified to support AI/ML modeling training. If offline training is performed in one new NF in CN, it is conflicted with Note in SID " The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced".

Observation 8: RAN1 agreements on model/functionality monitoring were already captured in TS 38.843. It is not clear why RAN2 need to capture duplicated details in the function mapping table.

Based on observations, our proposals are:

Model ID and meta info

Proposal 1: Model identification is performed between vendors/operators during product development phase as part of feature alignment. This procedure is not needed to be specified.

Proposal 2: Besides model identification, LCM and model delivery, AI/ML model ID can also be used for model pairing between UE and NW in two-sided model without model transfer (e.g. training collaboration type 2 and type 3).

Proposal 3: On how to achieve globality of the Model ID, RAN2 capture in TR that both Direction 1 and Direction 3 are feasible, and they can be further studied in normative phase.

Proposal 4: The global model ID is specified by putting multiple essential fields together. At least use case, UE/NW vendor ID, PLMN ID, and version number are included. FFS other fields.

Proposal 5: For LCM purpose (model selection/activation/deactivation/switching/monitor), model ID can be a local model ID which is configured in RRC.

Proposal 6: Meta info of an AI/ML model is optional information which includes important model description information except the fields of model ID (i.e. supplement to model ID).

Proposal 7: Meta info includes at least model input type/size, model output type/size, model file type/size/ compression status.

Proposal 8: Meta info does not include vendor information, model version info and model performance indicators.

Proposal 9: Endorse some existing AI/ML model representation formats (e.g., h5, ONNX) as 3GPP defined format, and RAN2 do not specify new model format for model delivery. FFS which existing model representation format(s) are endorsed. FFS whether / how to specify model format coordination procedure between UE and NW.

Remaining issues on physical entity mapping

Proposal 10: Exclude CN entity (except LMF) for model training in Rel-18 AI/ML for air interface.

Proposal 11: In the table of CSI compression with two-sided model, capture that UE may make decision on Model / functionality control.

Proposal 12: Simplify the description in row d) and e) of all tables, i.e., only capture function entity (e.g. UE, gNB, LMF) and remove the details related to RAN1 agreements.

# 4 References

[1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, 3GPP TSG RAN meeting #94, Electronic meeting, Dec 6-17, 2022

[2] RAN2#121b-e, Chair Notes

[3] R2-23xxxx, Report of [Post122][060][AIML] Mapping of functions to physical entities (CMCC), CMCC.

[4] RAN2#121b-e, Chair Notes

[5] RAN1#112, Chair Notes.