# 3GPP TSG RAN WG2 Meeting #123      	                           R2-2307816
Toulouse, France, Aug 21- Aug 25

Agenda item:	7.15.2

Source:	Apple

Title:	Remaining issues on SL consistent LBT failure recovery

WID/SID:	NR_SL_enh2 – Release 18

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#122-e [1], consistent LBT failure recovery was discussed, and below agreements were made:

Agreements on SL C-LBT failure recovery (mode 2, RRC ide/inactive UE)

1: 	Exclusion of RB set(s) that SL C-LBT failure was detected in candidate resource selection + resource pool (re)selection

2:	The UE performs resource pool (re)selection

-  When SL C-LBT failure was detected for all RB-sets within a selected resource pool or;

-  Up to UE implementation although the above condition is not met

3a:	MAC informs L1 of the RB set information where SL C-LBT failure was detected.

3b:	L1 performs the resource exclusion for the RB set that SL C-LBT failure was detected.

3c:	RAN2 will send a LS to RAN1 to ask to take it into consideration in their job.

4:	It is up to UE implementation to select a resource pool out of resource pools that has at least one RB-set that SL C-LBT failure was not detected.

Agreements on SL C-LBT failure recovery (mode 1)

1: 	Leave it to gNB implementation after UE reporting SL C-LBT failure indication. No spec change.

Agreements on SL C-LBT failure recovery (mode 2, RRC connected UE)

1: 	RAN2 confirms that SL C-LBT failure indication is reported to the gNB also for mode 2, RRC connected UE.

Agreements on SL C-LBT failure and S-SSB

1: 	Counting LBT failure indication regardless of whether LBT failure was provided because of S-SSB transmission or data transmission when RB set for S-SSB transmission belongs to the selected TX resource pool.

Agreements on SL C-LBT failure and PSFCH

1: 	Counting LBT failure indication regardless of whether LBT failure was provided because of PSFCH transmission or not when RB set for PSFCH transmission belongs to the selected TX resource pool. FFS when multiple PSFCH occasions are configured.

Agreements on SL C-LBT cancellation

1: 	For mode 1, SL C-LBT is cancelled upon SL C-LBT failure MAC CE transmission

In this contribution, we address the remaining issues of consistent LBT failure recovery.

# 2 Discussion

## 2.1 Counter/timer handling

In RAN2#119b-e [2], RAN2 agreed to reuse counter/timer mechanism of NR-U:

Agreements on timer/counter mechanism in RAN2#119b:

1:	As the general principle, reuse the consistent LBT failure detection procedure in NR-U as the baseline for SL-specific consistent LBT failure detection in SL-U.

2:	As in NR-U, introduce the following parameters and variables for the SL-specific consistent LBT failure detection in SL-U as the baseline:

- An SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER);

- An SL-specific maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount);

- An SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer).

3:	Reuse the following MAC behaviors on TIMER/COUNTER handling in NR-U for SL-specific consistent LBT failure detection procedure in SL-U as the baseline:

- As in NR-U, if an SL-specific LBT failure indication is received from the lower layer, the SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is incremented by one.

- As in NR-U, if an SL-specific LBT failure indication is received from the lower layer, start or restart the SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer)

- As in NR-U, if the SL-specific LBT failure indication counter value is equal to or larger than the SL-specific maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount), consistent LBT failure is triggered/declared by the MAC entity.

- As in NR-U, if the SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer) expires, the SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is reset to 0.

- As in NR-U, if the maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount) or SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer) is reconfigured, SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is reset to 0.

However, the above agreement is just a baseline. During RAN2#119b discussion, one issue without consensus is the granularity of timer/counter (e.g. whether it is per BWP or per resource pool). In NR-U, Consistent LBT failure is detected per UL BWP but the timer/counter mechanism is running per serving cell as highlighted in below copy of TS 38.321 [3]. In other word, the LBT_COUNTER continues if gNB indicates BWP switch for one serving cell. The intention behind is that only one BWP can be activated at one time for each serving cell.

Observation 1: In NR-U, consistent LBT failure is detected per UL BWP but the timer/counter mechanism is running per serving cell because only one BWP can be activated for one serving cell at one time. In other word, the LBT_COUNTER continues if gNB indicates BWP switch.

For SL consistent LBT failure, RAN2 has agreed the granularity of LBT failure indication is per RB set. Therefore, the granularity of timer/counter can be two alternatives:

- Alt-1: per RB set

- Atl-2: per BWP/carrier, i.e. the counter continues when RB set is changed.

Between them, we prefer Alt-2 because of below two considerations:

- It is simple to maintain just one set of counter/timer per BWP/carrier rather than multiple sets

- In one SL carrier, only one resource / resource pool can be used by the TX UE at one time point, which is similar to NR-U that only one BWP can be activated at one time for each serving cell.

The following UE variable is used for the consistent LBT failure detection procedure:

-	LBT_COUNTER (per Serving Cell): counter for LBT failure indication which is initially set to 0.

For each activated Serving Cell configured with lbt-FailureRecoveryConfig, the MAC entity shall:

1>	if LBT failure indication has been received from lower layers:

2>	start or restart the lbt-FailureDetectionTimer;

2>	increment LBT_COUNTER by 1;

2>	if LBT_COUNTER >= lbt-FailureInstanceMaxCount:

3>	trigger consistent LBT failure for the active UL BWP in this Serving Cell;

3>	if this Serving Cell is the SpCell:

4>	if consistent LBT failure has been triggered in all UL BWPs configured with PRACH occasions on same carrier in this Serving Cell:

5>	indicate consistent LBT failure to upper layers.

4>	else:

5>	stop any ongoing Random Access procedure in this Serving Cell;

5>	switch the active UL BWP to an UL BWP, on same carrier in this Serving Cell, configured with PRACH occasion and for which consistent LBT failure has not been triggered;

5>	initiate a Random Access Procedure (as specified in clause 5.1.1).

1>	if all triggered consistent LBT failures are cancelled in this Serving Cell; or

1>	if the lbt-FailureDetectionTimer expires; or

1>	if lbt-FailureDetectionTimer or lbt-FailureInstanceMaxCount is reconfigured by upper layers:

2>	set LBT_COUNTER to 0.

Thus, we propose to adopt Alt-2:

Proposal 1: For SL consistent LBT failure detection, the timer/counter mechanism is running per BWP/carrier, i.e. SL_LBT_COUNTER continues when RB set is changed (e.g. due to resource / resource pool reselection).

## 2.2 Remaining issues on recovery mechanism

In RAN2#122 [1], it was agreed that MAC layer counts LBT failure indication regardless of whether LBT failure was due to transmission of S-SSB and PSFCH. But there is one remaining FFS when multiple PSFCH occasions are configured.

Agreements on SL C-LBT failure and S-SSB

1: 	Counting LBT failure indication regardless of whether LBT failure was provided because of S-SSB transmission or data transmission when RB set for S-SSB transmission belongs to the selected TX resource pool.

Agreements on SL C-LBT failure and PSFCH

1: 	Counting LBT failure indication regardless of whether LBT failure was provided because of PSFCH transmission or not when RB set for PSFCH transmission belongs to the selected TX resource pool. FFS when multiple PSFCH occasions are configured.

We address this FFS in this section. First, we don't think MAC layer is visible to single vs multiple PSFCH occasions. Thus, MAC layer should just count LBT failure indication of PFSCH transmission from L1, irrespective of whether single or multiple PSFCH occasions. Then, for the followed behavior, please note that RAN2 has agreed that MAC layer informs L1 of RB set information where SL C-LBT failure was detected, and L1 will perform resource exclusion for the concerned RB set.

3a:	MAC informs L1 of the RB set information where SL C-LBT failure was detected.

3b:	L1 performs the resource exclusion for the RB set that SL C-LBT failure was detected.

Upon reception of MAC notification on issued RB set, we think L1 can identify whether multiple PSFCH occasions are configured, and can take corresponding action (e.g. switch to another PSFCH occasion). The details of action can be left to L1.

Proposal 2: MAC layer counts LBT failure indication of PFSCH transmission, irrespective of whether single or multiple PSFCH occasions being configured.

Proposal 3: For a SL C-LBT failure declared due to PFSCH transmission, MAC layer just informs L1 of the concerned RB set information and leaves L1 to take reaction.

## 2.3 Remaining issues on cancellation conditions

In RAN2#122 [1], SL C-LBT failure cancellation conditions were discussed. The discussion was controversial, and only mode-1 UE behavior was agreed.

Agreements on SL C-LBT cancellation

1: 	For mode 1, SL C-LBT is cancelled upon SL C-LBT failure MAC CE transmission

We discuss other valid cancellation conditions in this section. As highlighted in copy of existing TS 38.321 [3], the triggered UL consistent LBT failure is cancelled in below 3 cases:

- Upon reporting the LBT failure MAC-CE to gNB

- Upon RACH procedure successfully completed in SpCell

- Upon reconfiguration of Uu consistent LBT failure by RRC

The MAC entity shall:

...

1>	if a MAC PDU is transmitted and LBT failure indication is not received from lower layers and this PDU includes the LBT failure MAC CE:

2>	cancel all the triggered consistent LBT failure(s) in SCell(s) for which consistent LBT failure was indicated in the transmitted LBT failure MAC CE.

1>	if consistent LBT failure is triggered and not cancelled in the SpCell; and

1>	if the Random Access procedure is considered successfully completed (see clause 5.1) in the SpCell:

2>	cancel all the triggered consistent LBT failure(s) in the SpCell.

1>	if lbt-FailureRecoveryConfig is reconfigured by upper layers for a Serving Cell:

2>	cancel all the triggered consistent LBT failure(s) in this Serving Cell.

Observation 2: In NR-U, the triggered UL consistent LBT failure is cancelled in below 3 cases:

- Upon reporting the LBT failure MAC-CE to gNB

- Upon RACH procedure successfully completed in SpCell

- Upon reconfiguration of Uu consistent LBT failure by RRC (e.g. timer/counter)

We tend to think the intention of 1) and 2) is the assumption that gNB may recover the triggered consistent LBT failure after notified. And 3) is general handling of MAC reset upon RRC reconfiguration.

In RAN2#122 [1], 1) has been agreed for mode-1 UE, which is quite similar to above condition 1). Then, we think other conditions include:

- Upon reconfiguration of SL consistent LBT failure by RRC (e.g. timer/counter).

    - It is similar to cancellation case 3) of NR-U.

    - It is applicable to RRC_CONNECTED UE of both mode-1 and mode-2 UE.

- Upon reconfiguration of resource / resource pool by RRC.

    - It is similar to cancellation case 3) of NR-U.

    - The reconfiguration means the autonomous resource change needs reset.

    - It is applicable to RRC_CONNECTED UE of both mode-1 and mode-2 UE.

- Upon successful completion of autonomous resource or resource pool selection (i.e. the UE successfully transmits packet after switching resource / resource pool)

    - It is similar to intention of cancellation case 1) and case 2) of NR-U.

    - It is applicable to mode-2 UE.

- Upon a timer expiry

    - The timer starts when SL C-LBT is detected in one RB set.

    - The timer stops when SL C-LBT is cancelled via other conditions.

    - It is applicable to mode-2 UE.

We think above cancellation conditions are straight forward, and propose RAN2 to agree them.

Proposal 4: Besides mode-1 UE reporting failure MAC-CE to NW, RAN2 agree below 4 other SL C-LBT failure cancellation conditions:

- Proposal 3a: Upon reconfiguration of SL consistent LBT failure by RRC (e.g. timer/counter). It is applicable to RRC_CONNECTED UE of both mode-1 and mode-2 UE.

- Proposal 3b: Upon reconfiguration of resource / resource pool by RRC. It is applicable to RRC_CONNECTED UE of both mode-1 and mode-2 UE.

- Proposal 3c: Upon successful completion of autonomous resource or resource pool selection (i.e. the UE successfully transmits packet after switching resource / resource pool). It is applicable to mode-2 UE.

- Proposal 3d: Upon a new timer expiry, where the timer starts when SL C-LBT is detected in one RB set, and the timer stops when SL C-LBT is cancelled via other conditions. It is applicable to mode-2 UE.

## 2.4 Remaining issues on signalling

Besides the confirmed WAs in RAN2#121b-e [4], we think one missed issue is the priority of SL consistent LBT failure MAC-CE in LCP procedure. According to TS 38.321 [3], priority of UL LBT failure MAC-CE is between MAC CE for Sidelink Configured Grant Confirmation and MAC CE for Timing Advance Report.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	MAC CE for C-RNTI, or data from UL-CCCH;

-	MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation;

-	MAC CE for Sidelink Configured Grant Confirmation;

-	MAC CE for LBT failure;

-	MAC CE for Timing Advance Report;

-	MAC CE for SL-BSR prioritized according to clause 5.22.1.6;

-	MAC CE for (Extended) BSR, with exception of BSR included for padding;

-	MAC CE for (Enhanced) Single Entry PHR, or MAC CE for (Enhanced) Multiple Entry PHR;

-	MAC CE for Positioning Measurement Gap Activation/Deactivation Request;

-	MAC CE for the number of Desired Guard Symbols;

-	MAC CE for Case-6 Timing Request;

-	MAC CE for (Extended) Pre-emptive BSR;

-	MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding;

-	MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC CE for Recommended bit rate query;

-	MAC CE for BSR included for padding;

-	MAC CE for SL-BSR included for padding.

NOTE 2:	Prioritization among MAC CEs of same priority is up to UE implementation.

The MAC entity shall prioritize any MAC CE listed in a higher order than 'data from any Logical Channel, except data from UL-CCCH' over transmission of NR sidelink communication.

Observation 3: According to TS 38.321, priority of UL LBT failure MAC-CE is between MAC CE for Sidelink Configured Grant Confirmation and MAC CE for Timing Advance Report

We think SL consistent LBT failure MAC-CE is right after UL consistent LBT failure MAC-CE.

Proposal 5: During LCP procedure, the priority of SL consistent LBT failure MAC-CE is right after UL consistent LBT failure MAC-CE

# 3 Conclusion

In this contribution, we discuss remaining issues on consistent LBT failure. Our observations are:

Observation 1: In NR-U, consistent LBT failure is detected per UL BWP but the timer/counter mechanism is running per serving cell because only one BWP can be activated for one serving cell at one time. In other word, the LBT_COUNTER continues if gNB indicates BWP switch.

Observation 2: In NR-U, the triggered UL consistent LBT failure is cancelled in below 3 cases:

- Upon reporting the LBT failure MAC-CE to gNB

- Upon RACH procedure successfully completed in SpCell

- Upon reconfiguration of Uu consistent LBT failure by RRC (e.g. timer/counter)

Observation 3: According to TS 38.321, priority of UL LBT failure MAC-CE is between MAC CE for Sidelink Configured Grant Confirmation and MAC CE for Timing Advance Report

Based on observations, our proposals are:

Proposal 1: For SL consistent LBT failure detection, the timer/counter mechanism is running per BWP/carrier, i.e. SL_LBT_COUNTER continues when RB set is changed (e.g. due to resource / resource pool reselection).

Proposal 2: MAC layer counts LBT failure indication of PFSCH transmission, irrespective of whether single or multiple PSFCH occasions being configured.

Proposal 3: For a SL C-LBT failure declared due to PFSCH transmission, MAC layer just informs L1 of the concerned RB set information and leaves L1 to take reaction.

Proposal 4: Besides mode-1 UE reporting failure MAC-CE to NW, RAN2 agree below 4 other SL C-LBT failure cancellation conditions:

- Proposal 3a: Upon reconfiguration of SL consistent LBT failure by RRC (e.g. timer/counter). It is applicable to RRC_CONNECTED UE of both mode-1 and mode-2 UE.

- Proposal 3b: Upon reconfiguration of resource / resource pool by RRC. It is applicable to RRC_CONNECTED UE of both mode-1 and mode-2 UE.

- Proposal 3c: Upon successful completion of autonomous resource or resource pool selection (i.e. the UE successfully transmits packet after switching resource / resource pool). It is applicable to mode-2 UE.

- Proposal 3d: Upon a timer expiry, where the timer starts when SL C-LBT is detected in one RB set, and the timer stops when SL C-LBT is cancelled via other conditions. It is applicable to mode-2 UE.

Proposal 5: During LCP procedure, the priority of SL consistent LBT failure MAC-CE is right after UL consistent LBT failure MAC-CE

# 4 References

[1] RAN2#122, Chair Notes

[2] RAN2#119b-e, Chair Notes

[3] TS 38.321-v17.1.0, Medium Access Control (MAC) protocol specification, 2022-6.

[4] RAN2#121b-e, Chair Notes