# 3GPP TSG RAN WG2 Meeting #123      	                            R2-237817
Toulouse, France, Aug 21- Aug 25

Agenda item:	7.15.3

Source:	Apple

Title:	Remaining issues on LCP and resource (re)selection in SL-U

WID/SID:	NR_SL_enh2 – Release 18

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#121b-e [1], RAN2 discussed LCP impact due to COT sharing. Below agreements were made:

Agreement:

If the resource to be used is within a shared COT, and if PDU not generated before COT arrival, and there is data in buffer satisfying COT requirement, at least enhanced LCP should be allowed. FFS on the condition for UE to use enhanced LCP. FFS on spec impact.

If a UE decides to use the resource in a shared COT, and when enhanced LCP is decided to be used, for destination selection step in enhanced LCP, at least further restrict the destinations to be the candidates allowed by the COT (as defined by RAN1).

RAN2 will wait for more conclusion from RAN1 on the assistance information for COT sharing.

Then, in RAN2#122 [2], a WA assumption on LCH restriction on CAPC was agreed.

Agreements on SL enhanced LCP

1: 	Working assumption: For shared COT, CAPC restriction is applicable to enhanced LCP according RAN1 agreement on CAPC requirement.

Meanwhile in RAN2#121 [3], RAN2 discussed type 1 LBT blocking issue. Below agreements were made:

Agreements on SL resource (re)selection

1: 	RAN2 understands UE triggers a resource (re)selection when PSSCH transmission was not performed due to an LBT failure indication from L1. FFS on MCST case. Send LS to RAN1 to check if there is any concern.

2a:	RAN2 understands L1 handles LBT impact to/from other UEs’ reserved resources in SL candidate resource selection (inter-UE case).

2b:	RAN2 will study how MAC performs resource (re)selection with the consideration of LBT impact to its own candidate resource (intra-UE case).

3:	Will send LS to RAN1 to check if there is any concern.

The agreement 2b requires RAN2 to further discuss MAC impacts of resource (re)selection for intra-UE case.

In this contribution, we share our views on LCP impacts and resource (re)selection.

- LCP impacts due to COT sharing

- Resource (re)selection in COT sharing

- Type 1 LBT blocking issue (intra-UE)

- LBT failure triggered resource (re)selection

# 2 Discussion

## 2.1 LCP impacts due to COT sharing

According to below agreements made in RAN2#121b-e [1], our understanding is that at least a new destination selection procedure based on destination information in received COT sharing information will be specified. And RAN2 need to further discuss the condition when the new destination selection procedure is used and when the legacy priority based procedure is used.

Agreement:

If the resource to be used is within a shared COT, and if PDU not generated before COT arrival, and there is data in buffer satisfying COT requirement, at least enhanced LCP should be allowed. FFS on the condition for UE to use enhanced LCP. FFS on spec impact.

If a UE decides to use the resource in a shared COT, and when enhanced LCP is decided to be used, for destination selection step in enhanced LCP, at least further restrict the destinations to be the candidates allowed by the COT (as defined by RAN1).

So, basically it requires a different LCP procedure like: 1) There is a COT sharing (channel resource )  2) Select destination to be initiating UE  3) Multiplex LCH/MAC CEs which are of lower CPAC priority to be in MAC PDU  4) find resource (mode 2 RA or mode 1 grant).

Observation 1: A different LCP procedure is required: 1) There is a COT sharing (channel resource )  2) Select destination to be initiating UE  3) Multiplex LCH/MAC CEs which are of lower CPAC priority to be in MAC PDU  4) find resource (mode 2 RA or mode 1 grant).

Based on above analysis, we think above new LCP procedure requires below detailed specification changes:

- New Step 1) requires PHY layer of the responding UE to send the decoded COT sharing information (at least including initialling UE ID, remaining COT duration and CAPC value to acquire the resource) to its MAC layer.

- New Step 2) requires the MAC layer of the responding UE to select destination based on the COT sharing information (i.e. selecting initiating UE as destination) rather than based on logical channel priority as in legacy.

- New Step 3) requires the MAC layer of the responding UE to add a new LCP restriction for CAPC value.

- New Step 4) requires the Resource selection window to be restricted by remaining COT duration.

We think above 2)/3)/4) require MAC specification changes, and would like to discuss in detail one by one.

### 2.1.1 Enhanced destination selection and its condition

In existing TS 38.321 [4], the destination is selected based on priority of LCH and MAC-CE, as highlighted below:

###### 5.22.1.4.1.2	Selection of logical channels

The MAC entity shall for each SCI corresponding to a new transmission:

1>	select a Destination associated to one of unicast, groupcast and broadcast, having at least one of the MAC CE and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, for the SL grant associated to the SCI:

2> SL data is available for transmission; and

2> SBj > 0, in case there is any logical channel having SBj > 0; and

2> sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and

2> sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

2> sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI.

NOTE 1: If multiple Destinations have the logical channels satisfying all conditions above with the same highest priority or if multiple Destinations have either the MAC CE and/or the logical channels satisfying all conditions above with the same priority as the MAC CE, which Destination is selected among them is up to UE implementation.

Observation 2: In existing TS 38.321, the destination is selected based on priority of LCH and MAC-CE.

According to below agreement of RAN2#121b-e [1]:

If a UE decides to use the resource in a shared COT, and when enhanced LCP is decided to be used, for destination selection step in enhanced LCP, at least further restrict the destinations to be the candidates allowed by the COT (as defined by RAN1).

We think the enhanced destination selection means that the responding UE is allowed to select initiating UE as destination based on candidate destination information from COT sharing info (rather than priority).

Proposal 1: Enhanced destination selection procedure is that the responding UE is allowed to select initiating UE as destination based on candidate destination information from COT sharing info (rather than priority).

Then, regarding when to use the above enhanced destination selection procedure, we think it needs to at least satisfy below conditions:

- SL data or MAC-CE towards initiating UE is available for transmission;

- Remaining COT duration has available resource to complete the data transmission;

  - Especially in case that multiple MAC PDUs are required to send data for initiating UE, or multi-TTI scheduling is applied.

- SL data towards initiating UE has at least one LCH's CAPC value ≤ CAPC value indicated in COT information

Thus, we propose:

Proposal 2: The enhanced destination selection procedure is allowed to apply when all below conditions are met:

- SL data or MAC-CE towards initiating UE is available for transmission;

- Remaining COT duration has available resource to complete the data transmission;

- SL data towards initiating UE has at least one LCH's CAPC value ≤ CAPC value indicated in COT information.

Otherwise, the legacy destination selection procedure based on priority is applied

What's more, even if the responding UE satisfies all the above 3 conditions, it may also have SL data / MAC-CE towards other UEs with higher priority. This issue was also raised in RAN2#121 [3], but not concluded. Basically, there are two solutions:

- Solution 1: It is up to UE implementation whether to select initiating UE or the UE with highest priority SL data/MAC-CE

- Solution 2: Initiating UE is selected if highest priority of other UEs' data/MAC-CE is smaller than a threshold, similar to existing prioritization rule between UL and SL transmission

We suggest RAN2 to also discuss how to handle this case.

Proposal 3: RAN2 discuss how MAC layer of the responding UE handles the case that it has SL data / MAC-CE towards other UEs with higher priority than the SL data / MAC-CE towards initiating UE:

- Solution 1: It is up to UE implementation whether to select initiating UE or the UE with highest priority SL data/MAC-CE

- Solution 2: Initiating UE is selected if highest priority of other UEs' data/MAC-CE is smaller than a threshold, similar to existing prioritization rule between UL and SL transmission

### 2.1.2 LCP restriction

In existing TS 38.321 [4], the SL LCP restriction includes:

- Whether CG type 1 is allowed

- Allowed CG index associated to the SL grant

- HARQ feedback mode

Note that RAN2 made below WA on LCP restriction with CAPC value in RAN2#122 [2].

Agreements on SL enhanced LCP

1: 	Working assumption: For shared COT, CAPC restriction is applicable to enhanced LCP according RAN1 agreement on CAPC requirement.

Given the enhanced destination selection procedure, we think it is straight forward to confirm this WA. Specifically, the new SL LCP will restrict multiplexing any MAC SDU(s) of LCH(s) with CAPC value higher than the CAPC value indicated in the COT sharing information.

Proposal 4: Confirm the WA: For shared COT, if the initiating UE is selected as the destination, introduce a new SL LCP restriction that the responding UE shall not include any MAC SDU(s) of LCH(s) having CAPC value higher than the CAPC value indicated in the COT sharing information.

### 2.1.3 Impacts to resource selection window

According to existing TS 38.321 [4], the resource selection window is restricted by PDB of the pending SL data available in the LCH(s):

From Clause 5.22.1.1 of TS 38.321:

....

3>	if transmission based on random selection is configured by upper layers:

4>	randomly select the time and frequency resources for one transmission opportunity from the resources pool, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier.

3>	else:

4>	randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier.

....

Observation 3: According to existing TS 38.321, the resource selection window is restricted by PDB of the pending SL data available in the LCH(s).

If the initiating UE is selected as the destination, we think it is straight forward that the resource selection window needs to be further restricted by remaining COT duration.

Proposal 5: If the initiating UE is selected as the destination, MAC layer of the responding UE needs to further restrict the resource selection window within remaining COT duration besides PDB of the pending SL data available in the LCH(s).

## 2.2 Resource (re)selection in COT sharing

In legacy resource (re)selection specified in TS 38.321 [4], PHY layer provides candidate resource set S_A to MAC layer, and MAC layer performs a random selection within the set S_A. However, followed by our proposal 5, we think the legacy resource (re)selection may not work well because:

- The randomly selected resource within set S_A may be out of COT duration.

- Even if MAC layer can filter the candidate set S_A according to our Proposal 5, another issue is whether there are available resources within the restricted window (i.e. minimal between COT duration and PDB). If there is no available resource, we think the responding UE has to ignore the reserved COT. Such UE behavior should be clearly specified.

Observation 4: If there is no available resource within the restricted window (i.e. minimal between COT duration and PDB), the responding UE has to ignore the reserved COT.

To resolve above 2 issues, a simple solution is:

- PHY layer provides the S_A candidate set to MAC layer.

- MAC layer uses the remaining COT duration to further filter S_A.

- If the number of resources in this filtered subset is larger than a threshold, then MAC layer randomly selects resources within this subset.

- If the number of resources in this filtered subset is smaller than a threshold, then MAC layer performs legacy resource selection (i.e. ignore COT sharing and randomly select resources within S_A).

  - Note that the size of threshold is intended to handle whether retransmission resource can be reserved and multi-channel / MCSt transmission.

Proposal 6: Introduce below resource selection enhancement for COT sharing:

- MAC layer uses the remaining COT duration to further filter S_A reported from PHY layer.

- If the number of resources in this filtered subset is larger than a threshold, then MAC layer randomly selects resources within this subset.

- If the number of resources in this filtered subset is smaller than a threshold, then MAC layer performs legacy resource selection (i.e., ignore COT sharing and randomly select resources within S_A).

## 2.3 Type 1 LBT blocking issue (intra-UE)

We address the below highlighted agreement in this section.

Agreements on SL resource (re)selection

1: 	RAN2 understands UE triggers a resource (re)selection when PSSCH transmission was not performed due to an LBT failure indication from L1. FFS on MCST case. Send LS to RAN1 to check if there is any concern.

2a:	RAN2 understands L1 handles LBT impact to/from other UEs’ reserved resources in SL candidate resource selection (inter-UE case).

2b:	RAN2 will study how MAC performs resource (re)selection with the consideration of LBT impact to its own candidate resource (intra-UE case).

3:	Will send LS to RAN1 to check if there is any concern.

According to RAN2#121 discussion [3], we understand that the intention is to ensure sufficient time gap between selected resource and the resource reserved by the same TX UE (i.e. intra-UE case). Otherwise, the candidate resource may be unavailable if LBT failure happens.

However, we noticed RAN1 also discussed similar issue, and identified 7 candidate solutions in RAN1#112b-e [5]. And in RAN1#113 [6], it was agreed to support Option 2 and Option 1 with different capabilities.

Working assumption

For Type 1 LBT block issue (inter-UE case), the following option 2 and option 1 are supported separately based on UE capability

- Option 2: If transmission in slot(s) before a reserved resource is able to share its initiated COT to the reservation [with high L1 SL priority], UE may prioritize/select resource(s) in the slot(s) for transmission.

  - FFS: details of applying this prioritization, which layer to perform above prioritization behaviour, and if the reserved resource belongs to a MCSt, the COT initiating UE should be able to share the COT to cover the whole MCSt

  - (pre)configuring enabling/disabling option 2 is supported

- Option 1:

  - UE may avoid selection of N consecutive resource(s) before a reserved resource with high L1 SL priority.

    - The value of N can be selected from {0, 1, 2}

    - The selection of the value of N is up to UE implementation

      - FFS: unless (pre-)configured or indicated by UE reserved resource in SCI

  - UE may avoid selection of M consecutive resource(s) after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource.

    - M is determined based on UE implementation (at least including 0)

  - FFS: Which layer to perform above behaviour

  - FFS: any restriction of M

  - (pre)configuring enabling/disabling option 1 is supported

- FFS: Whether the above high priority is determined according to a (pre)configured threshold

- Note: both option1 and option2 are optional UE features

In our understanding, although RAN2 sent LS to notify RAN2 will study intra-UE case, it seems RAN1 don't intend to differentiate intra-UE and inter-UE case. Meanwhile, if RAN1 agree Option 2 is performed in MAC layer, it seems straight forward to apply it for both intra-UE and inter-UE case.

Observation 5: Although RAN2 sent LS to notify RAN2 will study intra-UE case, it seems RAN1 don't intend to differentiate intra-UE and inter-UE case. Meanwhile, if RAN1 agree Option 2 is performed in MAC layer, it seems straight forward to apply it for both intra-UE and inter-UE case.

Thus, although RAN2 agreed to study intra-UE case, we think it is better to wait for RAN1 conclusion, to avoid the duplicated discussion in different WGs.

Proposal 7: RAN2 wait for RAN1 conclusion on type 1 LBT blocking issue before studying intra-UE case.

## 2.4 LBT failure triggered resource (re)selection

Finally, one remaining issue is whether LBT failure of MCSt triggers resource (re)selection or not. In RAN2#120 [6], RAN2 agreed LBT failure triggers a resource (re)selection for non-MCSt case and MCSt case is FFS.

RAN2 understands UE triggers a resource (re)selection when PSSCH transmission was not performed due to an LBT failure indication from L1. FFS on MCST case. Send LS to RAN1 to check if there is any concern.

MCSt (FFS whether SL LBT failure triggers resource (re)selection or not)

- Yes (P3:4806/P4-5:5686)

  - Proposal 4: For MCST case, reselect consecutive resources for dropped PSSCH transmission due to an LBT failure indication from L1

  - Proposal 5: For MCST case, the resource (re)selection is triggered when LBT succeed and there has dropped PSSCH transmission due to an LBT failure indication from L1

- No (P3:4793)

  - Proposal 3. When it comes to the MCSt, RAN2 does not pursue the UE behavior that UE triggers resource (re-)selection when the PSSCH transmission was not performed due to the LBT failure.

- To be concluded next meeting.

RAN2 further discussed MCSt case in RAN2#122 [2], but no conclusion was made.

For this issue, we tend to think the intention of MCSt and LBT failure triggered resource reselection are same, i.e. increase channel time diversity. Thus, the LBT failure of MCSt triggered resource reselection is a kind of over-design. We prefer that LBT failure of MCSt doesn't trigger resource reselection because the intention of MCSt.

Proposal 8: In MCSt case, LBT failure of PSSCH transmission don't trigger resource reselection.

# 3 Conclusion

In this contribution, we discuss LCP impacts and resource (re)selection in SL-U. Our observations are:

Observation 1: A different LCP procedure is required: 1) There is a COT sharing (channel resource )  2) Select destination to be initiating UE  3) Multiplex LCH/MAC CEs which are of lower CPAC priority to be in MAC PDU  4) find resource (mode 2 RA or mode 1 grant).

Observation 2: In existing TS 38.321, the destination is selected based on priority of LCH and MAC-CE.

Observation 3: According to existing TS 38.321, the resource selection window is restricted by PDB of the pending SL data available in the LCH(s).

Observation 4: If there is no available resource within the restricted window (i.e. minimal between COT duration and PDB), the responding UE has to ignore the reserved COT.

Observation 5: Although RAN2 sent LS to notify RAN2 will study intra-UE case, it seems RAN1 don't intend to differentiate intra-UE and inter-UE case. Meanwhile, if RAN1 agree Option 2 is performed in MAC layer, it seems straight forward to apply it for both intra-UE and inter-UE case.

Based on observations, our proposals are:

Proposal 1: Enhanced destination selection procedure is that the responding UE is allowed to select initiating UE as destination based on candidate destination information from COT sharing info (rather than priority).

Proposal 2: The enhanced destination selection procedure is allowed to apply when all below conditions are met:

- SL data or MAC-CE towards initiating UE is available for transmission;

- Remaining COT duration has available resource to complete the data transmission;

- SL data towards initiating UE has at least one LCH's CAPC value ≤ CAPC value indicated in COT information.

Otherwise, the legacy destination selection procedure based on priority is applied

Proposal 3: RAN2 discuss how MAC layer of the responding UE handles the case that it has SL data / MAC-CE towards other UEs with higher priority than the SL data / MAC-CE towards initiating UE:

- Solution 1: It is up to UE implementation whether to select initiating UE or the UE with highest priority SL data/MAC-CE

- Solution 2: Initiating UE is selected if highest priority of other UEs' data/MAC-CE is smaller than a threshold, similar to existing prioritization rule between UL and SL transmission

Proposal 4: Confirm the WA: For shared COT, if the initiating UE is selected as the destination, introduce a new SL LCP restriction that the responding UE shall not include any MAC SDU(s) of LCH(s) having CAPC value higher than the CAPC value indicated in the COT sharing information.

Proposal 5: If the initiating UE is selected as the destination, MAC layer of the responding UE needs to further restrict the resource selection window within remaining COT duration besides PDB of the pending SL data available in the LCH(s).

Proposal 6: Introduce below resource selection enhancement for COT sharing:

- MAC layer uses the remaining COT duration to further filter S_A reported from PHY layer.

- If the number of resources in this filtered subset is larger than a threshold, then MAC layer randomly selects resources within this subset.

- If the number of resources in this filtered subset is smaller than a threshold, then MAC layer performs legacy resource selection (i.e., ignore COT sharing and randomly select resources within S_A).

Proposal 7: RAN2 wait for RAN1 conclusion on type 1 LBT blocking issue before studying intra-UE case.

Proposal 8: In MCSt case, LBT failure of PSSCH transmission don't trigger resource reselection.

# 4 References

[1] RAN2#121b-e, Chair Notes

[2] RAN2#122, Chair Notes

[3] RAN2#121, Chair Notes

[4] TS 38.321-v17.1.0, Medium Access Control (MAC) protocol specification, 2022-6.

[5] RAN1#112b-e, Chair Notes

[6] RAN1#120, Chair Notes