# 3GPP TSG RAN WG2 Meeting #123      	                           R2-2307820
Toulouse, France, Aug 21- Aug 25

Agenda item:	7.12.2.1

Source:	Apple

Title:	CONNECTED mobility enhancement in mobile IAB

WID/SID:	NR_mobile_IAB-Core – Release 18

Document for:	Discussion and Decision

# 1 Introduction

WID of mobile IAB (RP-213601) was agreed in RAN#94e [1]. The related WID objectives on mobility enhancement are summarized below.

- Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2]

The following principles should be respected:

- Mobile IAB-nodes should be able to serve legacy UEs.

- Solutions providing optimization for Mobile IAB may entail Rel-18 UE enhancements, provided that such enhancements are backwards compatible

In RAN2#121b-e [2] and RAN2#122 [3], RAN2 made some progress on RACH-less HO and CHO enhancement. But FFSs are left:

RACH-less HO:

RAN2 think that to have a fast handover from UE point of view for legacy UEs it is important that the target cell is known to the UE (detected and measured).

For RACH-less, if supported, there would need to be a beam indication (in RRC HO command), which seems feasible in this release from R2 perspective. R2 assumes that the network can know/select the beam, either from network impl specific knowledge or from UE measurement report (legacy report).

for the UL grant and HO completion in RACH-less HO:

- 1. Both type-1 configured grant and dynamic grant are supported

- 2. FFS handling of supervision timer and when HO is considered successfully complete (expect to align with other WI).

Send LS to RAN3 to check whether there are issues / feasibility concerns

CHO enhancement:

FFS: May support CHO with CondT1 if it is “for free”, i.e. if TS impact is just to slightly modify the description to make it also applicable to TN.

In this contribution, we share our view on how to address the remaining issue on RACH-less HO and CHO enhancement in mobile IAB.

# 2 Discussion

## 2.1 RACH-less HO

In NR Rel-18, there are following two RACH-less HO procedures are being specified:

- NTN solution

- It is similar to LTE RACH-less HO which was specified in LTE Rel-14

- It supports both intra-DU and inter-DU HO

- Early-TA solution

- It is being specified in Rel-18 WI of NR further mobility enhancements as part of LTM.

- It only supports intra-DU HO

Between them, we prefer 1) because:

- Inter-DU HO is target HO scenario of mobile IAB because dual-DU modeling was agreed in RAN2#119b-e [4].

- Early-TA solution is strongly coupled with LTM. However, we think it doesn't make much sense for a mobile IAB UE to support LTM (e.g. for fast HO via L2 signaling). Meanwhile, the RACH-less solution in NTN is only target for the requirement of same TA between source and target cell. We think it is more suitable for mobile IAB UEs.

Observation 1: RACH-less HO in NTN is more suitable for mobile IAB because early-TA doesn't support inter-DU HO but it is target HO scenario of mobile IAB because dual-DU modeling was agreed in RAN2#119b-e.

Thus, we propose:

Proposal 1: RAN2 confirm the RACH-less HO procedure agreed in NTN WI can be reused in mobile IAB.

To facilitate discussion, agreements on RACH-less HO in NTN were list below:

Agreements via email – from offline 109:

- NTN RACH-less HO is supported for Intra-satellite handover with the same feeder link. i.e., with same gateway/gNB;

- NTN RACH-less HO can be supported for intra-satellite handover with different feeder links, i.e., with gateway/gNB switch, inter-satellite handover with gateway/gNB switch, and inter-satellite handover with same gateway/gNB.

- RAN2 confirms the general UE procedure for NTN RACH-less HO

1.	receive a RACH-less HO command which can include pre-allocated grant optionally. FFS N_TA is optional. (RRC)

2.	start timer T304 for the target cell (RRC)

3.	perform DL and UL synchronization, and start timer T430. FFS how to perform RACH-less UL synchronization to NTN target cell. (RRC, MAC)

4.	start time alignment timer (MAC)

5.	monitor target cell PDCCH for dynamic grant if pre-allocated grant is not configured in RACH-less HO command (MAC, PHY)

6.	send initial UL transmission including RRCReconfigurationComplete message using the available UL grant (RRC, MAC, PHY)

7.	consider RACH-less HO is completed upon receiving NW confirmation. FFS how to confirm RACH-less HO is successfully completed. (RRC, MAC)

8.	stop timer T304 for the target cell. (RRC)

FFS whether to release UL grant if pre-allocated after RACH-less HO completion

FFS RACH-less HO failure handling, e.g. whether UE fallback to RACH-based HO to the target cell

FFS procedure for RACH-less HO combined with PCI unchanged or CHO if supported

4.	The pre-allocated grant is provided as type-1 CG

5.	Send an LS to RAN1 informing RAN2 agreements on NTN RACH-less HO and check RAN1 views on the following aspects:

1. whether the pre-allocated grant is provided with association to SSBs; if so, whether a RSRP threshold is configured for SSB selection.

2. to monitor target cell PDCCH for dynamic grant for initial UL transmission, whether beam indication can be provided in RACH-less HO command.

3. power control for initial UL transmission

Agreements online:

- At least for pre-allocated grant, for the confirmation of RACH-less HO completion we reuse of LTE approach, i.e., UE Contention Resolution Identity MAC CE is used but UE ignores the content of this field. FFS if anything else is needed for dynamic grant

- Consider to support combining RACH-less HO with time-based CHO for NTN, taking into account the 1) validity of pre-allocated grant and potential waste of reserved resource; 2) when/how to provide dynamic grant in PDCCH.

Agreements (in RAN2#122):

- From RAN2 perspective synchronization among source and target cells is not an issue in NTN RACH-less HO

- Release pre-allocated UL grant after RACH-less HO completion

- LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant. FFS any enhancement to the confirmation of RACH-less HO completion, e.g. the NW does not send the UE Contention Resolution Identity MAC CE, and sends PDCCH/PDSCH addressed to C-RNTI

- Remove “FFS how to perform RACH-less UL synchronization to NTN target cell”, RAN2 assumes the UL sync handling in the target cell is the same in RACH-based HO and RACH-less HO, except how to acquire NTA (FFS on the spec impact , if any)

Based on above highlighted part, NTN has agreed to consider below two beam handling to check with RAN1:

- HO command provides Type-1 CG with associated SSB and RSRP threshold as pre-allocated grant.

  - The UE behavior is same as beam selection in CG based SDT.

- HO command provides beam indication for monitoring PDCCH of initial UL transmission in target cell, if pre-allocated grant is not provided.

  - The UE behavior is to monitor PDCCH with assumption that its transmit beam is same as the indicated TCI.

Observation 2: In RAN2#121b-e, NTN WI has agreed below two basic beam handling mechanisms with LS to check RAN1:

- HO command provides Type-1 CG with associated SSB and RSRP threshold as pre-allocated grant. The UE behavior is same as beam selection in CG based SDT.

- HO command provides beam indication (TCI) to monitor PDCCH of initial UL transmission in target cell, if pre-allocated grant is not provided. The UE monitors PDCCH with the beam indicated by TCI.

They are aligned with below RAN2#122 agreement made in mobile IAB WI [3].

for the UL grant and HO completion in RACH-less HO:

- 1. Both type-1 configured grant and dynamic grant are supported

Thus, we think mobile IAB WI can confirm it.

Proposal 2: RAN2 confirm below two beam handling mechanisms agreed in NTN WI are reused in mobile IAB:

- HO command provides Type-1 CG with associated SSB and RSRP threshold as pre-allocated grant. The UE behavior is same as beam selection in CG based SDT.

- HO command provides beam indication (TCI) to monitor PDCCH of initial UL transmission in target cell, if pre-allocated grant is not provided. The UE monitors PDCCH with the beam indicated by TCI.

Then, we address below FFS:

- 2. FFS handling of supervision timer and when HO is considered successfully complete (expect to align with other WI).

Based on highlighted part of NTN RACH-less HO agreements, we understand that at least legacy LTE RACH-less HO solution is reused for handling of supervision timer and when HO is considered successfully complete. It is sufficient for RACH-less in mobile IAB.

Observation 3: In RACH-less HO of NTN, it was agreed that at least legacy LTE RACH-less HO solution is reused for handling of supervision timer and when HO is considered successfully complete.

Thus, we propose:

Proposal 3: On handling of supervision timer and when HO is considered successfully complete, reuse the same solution of RACH-less HO of NTN (i.e. UE Contention Resolution Identity MAC CE is used but UE ignores the content of this field.)

## 2.2 CHO enhancement

In RAN2#121b-e [3], CHO enhancement was discussed, and it is FFS whether to support CHO with CondT1 for mobile IAB.

R2-2303852	On the need for connected mode mobility enhancements for mIAB	LG Electronics France	discussion	NR_mobile_IAB-Core

P4

-	AT&T think this is aligned with the AT&T problem scenario.

-	CATT think the enhancement cmp to legacy HO is small. .

-	AT&T think that also the timing is important, and CHO has less dependency to real time restrictions.

-	Ericsson think that once CHO can be configured then also legacy HO can be done.

-	Chair: there is some support and some opposition.

-	Apple wonder if the UE need to support GNSS. LG think T1 is based on absolute time.

FFS: May support CHO with CondT1 if it is “for free”, i.e. if TS impact is just to slightly modify the description to make it also applicable to TN.

Because the FFS requires analysis of the TS impact, we provide a brief overview of existing CondT1. According to TS 38.300 [4], TS 38.331 [5] and TS 38.306 [6], the key points of CondT1 can be summarized below:

- CondT1 is only applicable to NTN, as specified in TS 38.306 [6].

| timeBasedCondHandover-r17 Indicates whether the UE supports time based conditional handover, i.e., CondEvent T1 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN bands and the support of nonTerrestrialNetwork-r17. UE shall set the capability value consistently for all FDD-FR1 NTN bands. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

- CondT1 is based on absolute UTC time (i.e. Threshold of T1 is UTC time) as specified in TS 38.331 [5].

  - The absolute UTC time (rather than relative timer) is used because it is not easy for gNB to pre-calculate RTT time of NTN. And NTN UE is always equipped with GNSS.

condEventT1-r17                  SEQUENCE {

t1-Threshold-r17                 INTEGER (0..549755813887),

duration-r17                     INTEGER (1..6000)

}

duration This field is used for defining the leaving condition T1-2 for conditional HO event condEventT1. Each step represents 100ms.

t1-Threshold The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900).

- CondT1 is always configured together with one of the measurement-based trigger conditions Ax (i.e. CHO events A3/A4/A5). Only when both CondT1 and CondAx are fulfilled at the same time, UE will consider the target candidate cell as a triggered cell (i.e. AND between CondT1 and Cond Ax).

  - AND (rather than OR) is specified because the time in CondT1 is to indicate the arriving time of the target cell due to fixed satellite moving pattern. Thus, the UE is allowed to execute HO only when both conditions are fulfilled.

From Clause 5.3.5.13.4 of TS 38.331:

....

2> if event(s) associated to all measId(s) within condTriggerConfig for a target candidate cell within the stored condRRCReconfig are fulfilled:

3> consider the target candidate cell within the stored condRRCReconfig, associated to that condReconfigId, as a triggered cell;

3> initiate the conditional reconfiguration execution, as specified in 5.3.5.13.5;

Observation 4: The key points of existing CondT1 can be summarized below:

- CondT1 is only applicable to NTN

- CondT1 is based on absolute UTC time (i.e. Threshold of T1 is UTC time)

- CondT1 is always configured together with one of the measurement-based trigger conditions (i.e. CHO Ax events A3/A4/A5). Only when both CondT1 and CondAx are fulfilled at the same time, UE will consider the target candidate cell as a triggered cell (i.e. AND between CondT1 and Cond Ax).

Thus, if CondT1 is supported in TN, we think these 3 aspects all need specification changes. Specifically, the changes should include:

- Specification change in TS 38.306 to allow non-NTN UE to apply CondT1.

- Specification change in TS 38.331 to allow non-NTN UE without equipping GNSS to apply CondT1.

  - For example, CondT1 can be revised to use time of source DU for non-NTN UEs.

  - As another example, add procedure text that after configured with CondT1, non-NTN UEs need to obtain SIB9 to acquire UTC time.

- Specification change in TS 38.331 to use "OR" (rather than "AND") between CondT1 and CondAx to determine whether the target candidate cell as a triggered cell.

  - Please note that different from NTN, surrounding UEs served by mobile IAB DU may move to cell boundary before DU migration is triggered (i.e. before CondT1 is fulfilled). In this case, these UE should execute HO immediately.

Observation 5: To allow non-NTN UE without equipping GNSS to apply CondT1, CondT1 can be revised to use time of source DU (i.e. not UTC time) for non-NTN UEs. Or a new procedure text can be added that after configured with CondT1, non-NTN UEs need to obtain SIB9 to acquire UTC time.

Observation 6: Different from NTN, surrounding UEs served by mobile IAB DU may move to cell boundary before DU migration is triggered (i.e. before CondT1 is fulfilled). In this case, these UE should execute HO immediately.

We are not sure whether it is worth pursuing these specification changes to support CondT1 in mobile IAB. Thus, we suggest RAN2 to discuss the necessity.

Proposal 4: RAN2 confirm below specification changes are needed, if CondT1 is supported in mobile IAB.

- Specification change in TS 38.306 to allow non-NTN UE to apply CondT1.

- Specification change in TS 38.331 to allow non-NTN UE without equipping GNSS to apply CondT1.

- Specification change in TS 38.331 to use "OR" (rather than "AND") between CondT1 and CondAx to determine whether the target candidate cell as a triggered cell.

# 3 Conclusion

In this contribution, we discuss remaining issues of RACH-less handover and CHO enhancement for Rel-18 mobile IAB.

RACH-less HO

Our observations are:

Observation 1: RACH-less HO in NTN is more suitable for mobile IAB because early-TA doesn't support inter-DU HO but it is target HO scenario of mobile IAB because dual-DU modeling was agreed in RAN2#119b-e.

Observation 2: In RAN2#121b-e, NTN WI has agreed below two basic beam handling mechanisms with LS to check RAN1:

- HO command provides Type-1 CG with associated SSB and RSRP threshold as pre-allocated grant. The UE behavior is same as beam selection in CG based SDT.

- HO command provides beam indication (TCI) to monitor PDCCH of initial UL transmission in target cell, if pre-allocated grant is not provided. The UE monitors PDCCH with the beam indicated by TCI.

Observation 3: In RACH-less HO of NTN, it was agreed that at least legacy LTE RACH-less HO solution is reused for handling of supervision timer and when HO is considered successfully complete.

Based on observations, our proposals are:

Proposal 1: RAN2 confirm the RACH-less HO procedure agreed in NTN WI can be reused in mobile IAB.

Proposal 2: RAN2 confirm below two beam handling mechanisms agreed in NTN WI are reused in mobile IAB:

- HO command provides Type-1 CG with associated SSB and RSRP threshold as pre-allocated grant. The UE behavior is same as beam selection in CG based SDT.

- HO command provides beam indication (TCI) to monitor PDCCH of initial UL transmission in target cell, if pre-allocated grant is not provided. The UE monitors PDCCH with the beam indicated by TCI.

Proposal 3: On handling of supervision timer and when HO is considered successfully complete, reuse the same solution of RACH-less HO of NTN (i.e. UE Contention Resolution Identity MAC CE is used but UE ignores the content of this field.)

CHO enhancement

Our observations are:

Observation 4: The key points of existing CondT1 can be summarized below:

- CondT1 is only applicable to NTN

- CondT1 is based on absolute UTC time (i.e. Threshold of T1 is UTC time)

- CondT1 is always configured together with one of the measurement-based trigger conditions (i.e. CHO Ax events A3/A4/A5). Only when both CondT1 and CondAx are fulfilled at the same time, UE will consider the target candidate cell as a triggered cell (i.e. AND between CondT1 and Cond Ax).

Observation 5: To allow non-NTN UE without equipping GNSS to apply CondT1, CondT1 can be revised to use time of source DU (i.e. not UTC time) for non-NTN UEs. Or a new procedure text can be added that after configured with CondT1, non-NTN UEs need to obtain SIB9 to acquire UTC time.

Observation 6: Different from NTN, surrounding UEs served by mobile IAB DU may move to cell boundary before DU migration is triggered (i.e. before CondT1 is fulfilled). In this case, these UE should execute HO immediately.

Based on observations, our proposals are:

Proposal 4: RAN2 confirm below specification changes are needed, if CondT1 is supported in mobile IAB.

- Specification change in TS 38.306 to allow non-NTN UE to apply CondT1.

- Specification change in TS 38.331 to allow non-NTN UE without equipping GNSS to apply CondT1.

- Specification change in TS 38.331 to use "OR" (rather than "AND") between CondT1 and CondAx to determine whether the target candidate cell as a triggered cell.

# 4 References

[1] RP-213601, New WID on mobile IAB.

[2] RAN2#121, Chair Notes.

[3] RAN2#122, Chair Notes.

[4] TS 38.300-v17.4.0, Overall description, stage 2, 2023-3.

[5] TS 38.331-v17.4.0, Radio Resource Control (RRC) protocol specification, 2023-3.

[6] TS 38.306-v17.4.0, User Equipment (UE) radio access capabilities, 2023-3.