3GPP TSG-RAN WG2 Meeting #123		R2-2307825

Toulouse, France, August 21-25, 2023

Agenda item:	7.13.6

Source:	Apple

Title:	RACH enhancements for slicing

Document for:	Discussion and Decision

# 1	Introduction

In RAN#121bis-e, SON enhancements for RACH have been discussed based on the summary document [6]. No agreements have been reached, but the following FFS’ were captured in the meeting minutes:

1	FFS: Include the actual number of msg3 repetitions in RA report.

2	FFS: Include NSAG priority in RA report.

3b	FFS: UE reports NSAG IDs which are associated with the S-NSSAI(s) that triggered the random access attempt or NSAG IDs which associated with the S-NSSAI(s) triggering the access attempt and that are included in SIB1.

3c	FFS: Include S-NSSAI(s) in RA report.

These issues have been discussed at RAN2#122, based on [7], but due to lack of time the issues related to slicing have not been resolved. In the present paper we provide our view on these issues.

# 2   	Discussion

### 2.2	NSAG priority

With regards to NSAG priority, its usage (according to TS 23.501 [4]) is as follows: “If the signalling transaction triggering the access attempt is related to more than one network slice, and the S-NSSAIs of these network slices are associated with more than one NSAG for Random Access, the NSAG with the highest priority is selected.”. As we can see, NSAG priority isn’t used by NG-RAN and in fact isn’t even signaled to NG-RAN. According to TS 48.413 [5], NSAG information (TAI NSAG Support List IE), contains only the NSAG ID and the S-NSSAI (which are signalled by NG-RAN top AMF in NG Setup and RAN Configuration Update messages.

The same actually applies to all “other” NSAG-related information, since as we have shown above, no “NSAG-related information” (besides NSAG ID and S-NSSAI) is ever signalled on the NG/N2 interface and therefore is not known by NG-RAN.

Observation 1: NSAG priority (and other “NSAG information”) is not used by NG-RAN and in fact isn’t even known by NG-RAN, as it is never signalled over the NG/N2 interface.

It is reasonable to assume that SON signalling of the information which is only known to UE and AMF (but not to NG-RAN) is not needed.

Proposal 1: signalling of the NSAG priority (or “NSAG information” besides NSAG ID) which is only known to UE and AMF (but not to NG-RAN) is not needed.

### 2.2	S-NSSAI(s)

Some companies proposed to report S-NSSAI, in addition to (already agreed) NSAG ID. The argument is that the NSAG “resolution” is too coarse and it may be better for the network to optimize slice partitioning taking into account how slices within a group are used.

There may be some technical merits in this argument. Having said so, if RAN2 is to agree that S-NSSAI is to be included in the RA report, the (previously agreed) NSAG ID becomes irrelevant.

Observation 2: if RAN2 is to agree that S-NSSAI is to be included in the RA report, the (previously agreed) NSAG ID.

Therefore, we are OK to discuss the inclusion of S-NSSAI, provided there is no redundancy. In other words, we select either NSAG ID or S-NSSAI, but not both.

Proposal 2: to discuss the inclusion of S-NSSAI, provided there is no redundancy - we select either NSAG ID or S-NSSAI, but not both.

### 2.1	NSAG ID

RAN2#120 have agreed that “For RACH report for RACH partitioning, RAN2 to agree to include NSAG ID when the applicable feature is slicing”. However, in RAN2#122 it became obvious that different companies have different interpretation of that agreement, specifically [7]:

- Understanding 1: UE reports all NSAG ID(s) which are associated with the S-NSSAI(s) triggering the random access attempt irrespective if it is included in SIB1 or not.

- Understanding 2: UE includes the NSAG IDs are associated with the S-NSSAI(s) triggering the RACH attempts and are included in SIB1

It is hard to make a proper pain-vs-gain analysis here, as both (“pain” and “gain”) are minor. Storing and reporting all the NSAG IDs (as opposed to only those signaled in SIB1) requires a bit of extra memory, while storing only those which are signaled in SIB1 requires an additional filtering – both have some UE impacts, neither impact is big.

We have a preference for “understanding 2”, but can also accept “understanding 1”.

Proposal 3: UE includes the NSAG IDs are associated with the S-NSSAI(s) triggering the RACH attempts and are included in SIB1.

# 3	Conclusions and Proposals

Observation 1: NSAG priority (and other “NSAG information”) is not used by NG-RAN and in fact isn’t even known by NG-RAN, as it is never signalled over the NG/N2 interface.

Proposal 1: signalling of the NSAG priority (or “NSAG information” besides NSAG ID) which is only known to UE and AMF (but not to NG-RAN) is not needed.

Observation 2: if RAN2 is to agree that S-NSSAI is to be included in the RA report, the (previously agreed) NSAG ID.

Proposal 2: to discuss the inclusion of S-NSSAI, provided there is no redundancy - we select either NSAG ID or S-NSSAI, but not both.

Proposal 3: UE includes the NSAG IDs are associated with the S-NSSAI(s) triggering the RACH attempts and are included in SIB1.

# 6	References

[1]	TS 38.331,		Radio Resource Control (RRC) protocol specification

[2]	R2-2212225, 	Report of [Post119bis-e][877][R18 SON/MDT] RACH enhancement (Huawei)

[3]	TS 38.214,		Physical layer procedures for data

[4]	TS 23.501,		System architecture for the 5G System (5GS); Stage 2

[5]	TS 38.413,		NG Application Protocol (NGAP)

[6]	R2-2304196,	Summary of 7.13.6 RACH enhancement	(Apple)

[7]	R2-2306760,	Summary of 7.13.6 RACH enhancement (ZTE)