3GPP TSG-RAN WG2 Meeting #123	R2-2307830

Toulouse, France, 21– 25 August 2023

Agenda item:	7.5.4.1

Source:	Apple

Title:	Buffer Delay Reporting and BSR Enhancements for XR

WID/SID:	NR_XR_enh-Core

Document for:	Discussion / Decision

# Introduction

According to the revised WID for NR XR [1], RAN2 should address the following objectives to specify the enhancements for capacity improvement:

- Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2); - Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2); - Delay reporting of buffered data in uplink (RAN2); - Discard operation of PDU Sets for DL and UL (RAN2, RAN3);

This paper aims to provide some of our views on delay information reporting and BSR enhancements.

# Discussions

## Buffer Delay Reporting

We have made the following agreements for buffer delay reporting in RAN2 #122:

1: UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this. When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this.

In this section, we would like to present our views of the overall buffer delay reporting procedure, the MAC CE format design, and the considerations of intra-UE prioritization.

### Detailed Design of Buffer Delay Reporting

Based on the agreements, the UE should report the remaining time till the PDCP discarding timer expiry as the buffer delay information. Also, the remaining time should take the timing of the first transmission of such report as the reference time. The overall buffer delay reporting procedure can be illustrated in the timing diagram shown in Figure 1:

Figure 1 A timing diagram of buffer delay reporting

Since the delay information evolves dynamically with time, reporting based on MAC CE would be the most feasible approach. However, it is still questionable whether RAN2 should introduce a new type of MAC CE that is solely used for delay information reporting (which could be dubbed as “delay status report” or DSR), or we can simply extend the BSR (i.e. by having new BSR formats) for such purposes.

It is quite clear RAN2 has agreed in SI phase that delay information should be coupled with data volume information. Hence, from our point of view it makes more sense to report delay information by extending the BSR, as the gNB can directly become aware of which buffered data that this delay information is referring to, by decoding the single BSR MAC CE. If the delay information is reported via a separate new MAC CE, some additional overhead is needed to “link” the delay information with the concerned buffer status of specific LCH/LCG, which increases both signaling overhead and implementation complexity. Thus, we think the delay information should be reported by extending the BSR, i.e. introducing a new BSR format that has fields for buffer delay information, rather than relying on a new type of MAC CE.

Proposal 1: For reporting of delay information, RAN2 should introduce a new BSR format that includes buffer delay information field in the BSR MAC CE.

Furthermore, the buffer delay information is more important for traffic flows with stringent latency requirements, and therefore triggering of delay information reporting should be applied only to specific LCGs/LCHs corresponding to these delay-sensitive traffic flows (e.g. QoS flows with shorter PDB). Otherwise, if we allow the delay information reporting to be triggered for any LCH/LCG, the UE operation may become much more complicated due to unnecessary triggering of delay information reporting. Hence, we think the gNB can pre-configure which LCH/LCG can trigger delay information reporting.

Proposal 2: The gNB can pre-configure which LCH/LCG can trigger delay information reporting.

According to TS 38.321, essentially the UE would trigger BSR when the data becomes available in the buffer. For the delay information, this may not be necessary for the UE to trigger BSR with delay information reporting immediately after the data arrives at the buffer. Instead, we think the UE should trigger BSR with delay information when the remaining time till the delivery deadline of a buffered packet is smaller than a threshold. It is worth noting that, regular BSR, periodic BSR and padding BSR for this LCG could be triggered either before or after the time point where the remaining time till the delivery deadline reaches the threshold. Thus, we think it boils down to a set of BSR format selection rule that takes buffer delay into account:

- Before the remaining time till the delivery deadline of a buffered packet in a LCG reaches the threshold, the BSR triggered for the LCG can be reported using a BSR format (e.g. a legacy BSR format) without delay information field.

- When/After the remaining time till the delivery deadline of a buffered packet in a LCG reaches the threshold, the BSR triggered for the LCG should be reported using a BSR format with delay information field.

Proposal 3: The UE can choose the BSR format with or without delay information field for the BSR triggered for a LCG, based on whether the remaining time till the delivery deadline of a buffered packet in the LCG has reached a threshold.

It is worth noting that, there can be multiple packets or PDU Sets in the buffer of a LCG/LCH that are associated to different delay values (remaining time). It has been argued that the UE should only report the most urgent delay information, i.e. only the delay information of the data portion that triggers such reporting is included in the MAC CE. At the first glance, it seems to be a good approach from signalling overhead point of view. However, the triggering conditions for the other data portions may also be satisfied shortly, and the UE may need to trigger delay reporting again, which is a heavy burden for the UE as it may need to prepare/transmit buffer delay information quite frequently. Besides, we think it is more useful if a more complete picture of the buffer is reported to the gNB at once, rather than only providing a fraction of information when such report is triggered. With this in mind, we think the buffer delay reporting should include multiple buffer delay values corresponding to different data portions in the buffer (e.g. different PDUs of a PDU Set), or corresponding to different LCHs of a LCG.

Having said that, it is true that reporting multiple delay values in one MAC CE can result in higher signaling overhead. To alleviate this issue, we think some differential reporting mechanism can be considered. For instance, assuming multiple delay reports correspond to different PDUs in one PDU Set, the delay information of the first PDU can serve as the reference, and the delay information of the second PDU can be reported as the difference with respect to the reference (i.e. the delay information of the first PDU). Thus, two buffer delay tables can be introduced:

- A nominal buffer delay table for reference delay information

- A differential buffer delay table with a smaller number of code points than the nominal buffer delay table.

The reference report can be encoded using the nominal buffer delay table, while the other reports can be encoded using the differential buffer delay table, which in turn reduces the overall number of bits that is needed to indicate multiple delay values in one MAC CE.

Proposal 4: The buffer delay report should include multiple delay values associating with multiple different buffered data portions, and differential reporting can be used to reduce the signaling overhead.

### Impacts of Intra-UE Prioritization to Buffer Delay Reporting

It has been agreed that the reference time for the buffer delay information reporting corresponds to the first transmission of such report. The intention is to make sure that the buffer delay information received by the gNB is up to date, as required by the agreement made in RAN2 #120:

RAN2 will introduce data volume information associated with delay information (e.g. remaining time) in a MAC CE. FFS if this is extension of BSR or new format. FFS how to do that (e.g. what exactly is reported) and how to ensure this information is up-to-date e.g. considering UL scheduling delay.

Nevertheless, we may not be able to guarantee that such information is up to date if the UE is also configured with intra-UE prioritization. In particular, there may be some issues when the UE intends to include delay information MAC CE into an uplink resource of a configured grant configuration with autonomousTX enabled. According to TS 38.321, if a generated MAC PDU of a CG is de-prioritized in the middle of its PUSCH transmission, it can be transmitted “again” autonomously on a subsequent CG resource with the same HARQ PID, if the CG is configured with autonomousTX. Since the autonomous transmission is modeled as a new transmission from gNB perspective, any delay information conveyed in this MAC PDU may be misleading, because the delay information is already outdated when it is received by the gNB due to incorrect interpretation of the reference time. The problematic scenario is depicted in Figure 3. We also want to stress that, autonomousTX is a feature that works in conjunction with intra-UE prioritization, and hence we think it is reasonable to foresee autonomousTX in XR use cases, as intra-UE prioritization could be employed in XR to facilitate more rapid transmission of some delay-sensitive traffics.

Figure 2 An illustration of potential problem of transmitting delay information in CG configured with autonomousTX.

To solve this problem, we think we can simply avoid the problem by mandating the UE not to use CG configured with autonomousTX for buffer delay reporting. That is, when buffer delay reporting is triggered, the UE should refrain from selecting a UL resource from a CG configuration with autonomousTX enabled. Although this may not be an optimal approach, we think such a simple solution is sufficient to handle such cases in Rel-18 at least.

Proposal 5: To handle the impacts of intra-UE prioritization, the UE should refrain from using CG resources configured with autonomousTX to transmit MAC CE for buffer delay information reporting.

## BSR Enhancements with New BS Table

### Design of New Static BS Table

RAN2 has agreed to introduce a new static 8-bits BS table, which can complement the legacy BS table by offering finer granularity and hence reducing the quantization error. In addition, the new BS table should cover a narrower range than the legacy table. With all these considerations in mind, we think the new BS table should aim to cover a range of higher BS values, where the quantization error with the legacy table is more pronounced.

The new BS table can be defined by three components, namely the maximum BS value (B_max), the minimum BS value (B_min), and the step size. Since we have already agreed that the new BS table has 256 entries (8-bits) and should be based on uniform distribution, the step size is straightforward as soon as both B_max and B_min are determined. That is, the step size is simply (B_max – B_min)/255. Thus, we focus on how B_max and B_min should be selected.

For the sake of simplicity, we think we can align the B_max with the legacy BS table, because if the buffer sizes near the B_max of the legacy table can be expected by the traditional use cases, needless to say that we can expect the same in XR use cases. On the other hand, the value of B_min can be selected as the point where the maximum quantization error starts to become intolerable with the legacy table.

A simple illustration of our views on buffer size range of new BS table(s) is shown in Figure 1:

Figure 3 Illustration of the Buffer Size range to be covered by the new BS Table (comparing to the legacy BS Table)

Proposal 6: For the new BS table, the B_max can be aligned with the legacy BS table, while the B_min can be selected as the point where the maximum quantization error with the legacy BS table starts to become intolerable.

### BS Table Selection

There are also instances where the XR buffer size is not within the narrow range covered by the new BS table. In light of this, RAN2 has made the following agreement:

6. Network can configure which BSR table(s) an LCG is eligible to use. UE determines which BSR table (i.e. legacy or something else) the LCG should use. FFS details of this determination (e.g. based on buffer size) and how network knows which BSR table each LCG uses.

In our views, a sensible implementation reflecting this agreement would be: a LCG for XR traffic can be configured to switch between the legacy BS table and the new BS table. Since the new BS table aims to the cover the higher BS value range, the LCG for XR traffic should be able to select the BS table depending on its buffer size. To be specific, the new BS table should be used for this LCG, when the buffer size is in the range covered by the new BS table; otherwise, if the buffer size is not within the range covered by the new BS table, the UE can select the legacy BS table instead. If the new BS table is designed in accordance with Proposal 6, then the selection criteria is simply whether the BS value is larger or smaller than the B_min of the new BS table.

There are some proposals that mandate the UE to select the BS table that provides the lower quantization error with respect to the instantaneous buffer size of the LCG. From our perspective, it potentially increases UE complexity as the UE may need to “try out” every BS table in order to make the selection. Thus, we think it is much simpler if the UE can just map the instantaneous buffer size to a table without checking the resultant quantization error.

On the other hand, for the LCG corresponding to non-XR traffics, the legacy BS table may be sufficient, and therefore this is not necessary to configure such BS table switching behavior for the non-XR LCG. However, we understand this is up to network implementation on whether a LCG should be configured to have such behavior or not. A simple illustration of mapping between LCGs and BS tables is depicted in Figure 2:

Figure 4 An illustration of BS table selection for different LCG

Proposal 7: A LCG can be configured to select between the legacy BS table and the new BS table, based on whether its buffer size is in the range of the new BS table.

### BS Table Indication

Following the discussions above, it is clear that the gNB needs to know which BS table is chosen by the UE for the LCG corresponding to XR traffics, in order to decode the BSR correctly. Furthermore, we may have the cases where different BS tables are selected for multiple LCGs with data available in the buffer. To report BSR associating to different BS tables for different LCGs, we have considered the following options to handle such situations:

- Option 1: The BS table is indicated by the LCID of the MAC CE, and only the LCGs selecting the same BS table can be reported in the same BSR MAC CE. The BSR for LCGs selecting different BS table should be transmitted using a separate BSR MAC CE with a different LCID.

- Option 2: The BS table is indicated by the LCID of the MAC CE, and the LCGs to be reported in the same BSR MAC CE are enforced to select the BS table corresponding to the LCID, regardless of the configured BS table selection rule.

- Option 3: Introduce a new field in the MAC CE along with identification of LCG, which indicates the BS table selected by each LCG.

With Option 1, when different BS tables are selected for multiple LCGs, the UE needs to send two or more separate BSR MAC CEs (one for each BS Table), depending on how many BS tables in total are used by these LCGs. This is indeed very inefficient in terms of overhead and latency, especially we can only have up to one BSR MAC CE in one MAC PDU according to TS 38.321.

With Option 2, one or more LCGs may be enforced to use an inappropriate BS table in spite of the potential large quantization error. For example, a LCG may have to use the new BS table in order to send its BSR together with other LCGs in the same BSR MAC CE, even if the buffer size of the LCG is actually out of the range of new BS table. The advantages of having a new BS table with lower quantization error may be diminished eventually if this option is adopted.

With Option 3, we need a new BSR MAC CE structure comprising BS table indication associating to each LCG ID. This provides a clear picture to the gNB about which BS table is selected for a LCG, and so the BSR can be decoded correctly. As compared to Option 1 and Option 2, we think Option 3 is the most sensible approach.

Proposal 8: To report the BS of multiple LCGs selecting different BS tables in one single BSR MAC CE, RAN2 should introduce a new BSR MAC CE structure comprising indication of BS table selection along with identification of LCGs.

## Handling of Pending SR/BSR Upon Packet Discarding

According to TS 38.321, the triggered BSR may be cancelled if all pending data in the buffer can be transmitted on a MAC PDU:

TS 38.321: All triggered BSRs may be cancelled when the UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC CE plus its subheader.

Essentially, when the UL grant is able to accommodate all the data, the pending BSR is no longer needed as the buffer becomes empty. With the similar train of thought, we think there could be a scenario where a BSR is triggered based on UL data availability in a LCH, but then the buffer becomes empty due to packet discarding, even before the triggered BSR is transmitted. This could be a common situation especially when there is network congestion and PSI-based discarding mechanism is activated. In such cases, we think the already-triggered BSR can be cancelled upon packet discarding, as the triggering event of this BSR (i.e. UL data availability) may no longer be valid when packets are discarded.

Proposal 9: RAN2 can consider cancellation of previously triggered-BSR upon packet discarding that results in empty buffer.

In addition, according to TS 38.321 the UE can stop an ongoing Random Access procedure for a SR if all the data in the buffer can already be accommodated by a UL grant or when the BSR can be transmitted in a MAC PDU:

TS 38.321: The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for BSR, which was initiated by the MAC entity prior to the MAC PDU assembly and which has no valid PUCCH resources configured, if: - a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU includes a BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR (see clause 5.4.5) prior to the MAC PDU assembly; or - the UL grant(s) can accommodate all pending data available for transmission.

When all data in the buffer are discarded, there is no need to transmit the pending BSR/SR that has led to a random access procedure. Hence, we think the UE can also stop the ongoing random access procedure in this case.

Proposal 10: An ongoing Random Access procedure due to a pending SR for BSR can be stopped, if all pending data in the buffer of LCH/LCG that triggers the pending SR/BSR are discarded.

# Conclusions

For buffer delay reporting, we propose the following:

Proposal 1: For reporting of delay information, RAN2 should introduce a new BSR format that includes buffer delay information field in the BSR MAC CE.

Proposal 2: The gNB can pre-configure which LCH/LCG can trigger delay information reporting.

Proposal 3: The UE can choose the BSR format with or without delay information field for the BSR triggered for a LCG, based on whether the remaining time till the delivery deadline of a buffered packet in the LCG has reached a threshold.

Proposal 4: The buffer delay report should include multiple delay values associating with multiple different buffered data portions, and differential reporting can be used to reduce the signaling overhead.

Proposal 5: To handle the impacts of intra-UE prioritization, the UE should refrain from using CG resources configured with autonomousTX to transmit MAC CE for buffer delay information reporting.

For the new BS table, we propose the following:

Proposal 6: For the new BS table, the B_max can be aligned with the legacy BS table, while the B_min can be selected as the point where the maximum quantization error with the legacy BS table starts to become intolerable.

Proposal 7: A LCG can be configured to select between the legacy BS table and the new BS table, based on whether its buffer size is in the range of the new BS table.

Proposal 8: To report the BS of multiple LCGs selecting different BS tables in one single BSR MAC CE, RAN2 should introduce a new BSR MAC CE structure comprising indication of BS table selection along with identification of LCGs.

Finally, we have discussed some potential UE behaviors relating to triggered SR/BSR upon packet discarding:

Proposal 9: RAN2 can consider cancellation of previously triggered-BSR upon packet discarding that results in empty buffer.

Proposal 10: An ongoing Random Access procedure due to a pending SR for BSR can be stopped, if all pending data in the buffer of LCH/LCG that triggers the pending SR/BSR are discarded.

# References

- RP-230786, Updated WID on XR Enhancements for NR, Nokia, Qualcomm, 3GPP TSG RAN #99, March 2023, Rotterdam, Netherlands.