3GPP TSG-RAN WG2 Meeting #123	R2-2307834

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.14.2

Source:	Apple

Title:	QoE Measurements Discarding in IDLE/INACTIVE States

WID/SID:	NR_QoE_enh-Core

Document for:	Discussion / Decision

# Introduction

During RAN2 #122, several agreements have been made for QoE measurements in RRC-IDLE/INACTIVE states:

- RAN2 #122 Agreements for QoE Measurements in RRC-IDLE/INACTIVE: 4: Do not support delta configuration of the QoE configuration applied in RRC IDLE when the UE moves to RRC CONNECTED state unless it causes issues for QoE AL continuity in state transition. 5: UE is allowed to release stored reports and configuration after 48h (similar to logged MDT). No timer is configured by the network. 1: As working assumption, RAN2 will use explicit indicator in AS-layer on whether a QoE configuration is also applicable in RRC-IDLE/INACTIVE states. Can be revisited if RAN3 decides to introduce a service type. 1. Do not introduce SIB1 indicator on whether UE is allowed to indicate presence of QoE measurements. UE always indicates if it has stored QoE report(s), and it’s up to network whether/when to retrieve them. Introduce AS layer minimum memory requirement for storing Rel-18 QoE reports measured in RRC_IDLE/RRC_INACTIVE. Could have larger values than in Rel-17. FFS what is the minimum size requirement capability. FFS what is the value range of the capability.

Furthermore, RAN2 has sent a LS to SA4/SA5 with some questions relating to area scope (R2-2306569). However, since no response is received so far, we think RAN2 can keep this issue pending for now.

In this paper, we aim to discuss how the UE should handle the situations when the AS buffer for QoE measurements becomes full while it is still in RRC-IDLE/INACTIVE state.

# Discussions

For QoE measurements collected in RRC-IDLE/INACTIVE states, by default the UE does not proactively return to RRC-CONNECTED state for QoE reporting, according to the following agreement:

1: UE can be configured to do QoE measurements for MBS broadcast in all RRC states. As a baseline, UE does not tigger RRC Resume – RRC Setup just for the sake of reporting QoE. FFS whether there are cases where we deviate from this baseline.

Therefore, the AS should continuously store the QoE measurements received from the APP layer, and eventually the AS buffer can become full. RAN2 has concluded that, in such cases, the older QoE measurements in the AS buffer can be over-written by the newer QoE measurements, meaning the UE can discard some stored QoE measurements to free up some buffer memory. Moreover, during RAN2#121bis-e we have further discussed how the UE should decide which QoE report to be discarded when the buffer is full. The detailed agreements are listed below:

4: As a default behavior, when the UE’s buffer for storing QoE reports is full and a new report arrives, the UE should discard older report(s) to make room for the new one. 5: FFS whether it is possible to provide information (e.g. priority, service type, etc.) to UE about buffering for the UE to decide which reports to discard in case the UE’s QoE buffer becomes full.

From our perspective, there are three possible approaches for the UE to decide the QoE report to be discarded:

- Option 1: Based on the “age” of collected QoE measurements

- Option 2: Based on network configuration (e.g. with QoE configuration priority).

- Option 3: Based on UE implementation

Here we discuss each of these options below.

Option 1: Based on the “age” of collected QoE measurements

In Option 1, the UE can simply drop the “oldest” QoE report in its buffer, in order to make rooms for new QoE measurements.

Previously, SA5 has provided the following information in the reply LS (R2-2302461):

Question 3: Is there a time after which the QoE reports collected by the UE are no longer useful for the OAM? SA5: There is no time limit after which the QoE reports are no longer useful for the consumer, e.g. OAM, assurance or analytics functions. When a consumer has enough data, the QoE session shall be completed. Question 4: In case of limited storage space for QoE reports at the UE, is there any preference from the OAM side on which QoE reports should be reported and which should be discarded, e.g. is there a principle that newer or older reports are more useful for the network? SA5: From SA5’s point of view, some selection policies from consumers could be possible, but there are no use cases for it yet.But as a consistent behavior is wanted from all UEs, a default behavior should be to prioritize new data.

Based on this LS, it seems there is no time-sensitivity for QoE reports, and therefore QoE measurement collected at any time may still be deemed to be valuable from the OAM perspective. Having said that, in the same LS, SA5 has also mentioned that, as a default behavior, the UE should prioritize the new reports in cases the AS buffer becomes full. Thus, Option 1 seems to a simple approach that is acceptable.

Option 2: Based on network configuration (e.g. with QoE configuration priority)

In Option 2, the UE can receive some “guidelines” from the gNB about which QoE report should be dropped when the AS buffer becomes full, in order to make rooms for new QoE measurements.

We have noted that RAN3 is now considering if any assistance information can be provided to RAN to handle QoE reporting upon RAN overload, according to the LS from RAN3 (R2-2302425):

In case assistance information for handling of QoE reporting upon RAN overload is sent to the RAN, it is sent together with QoE measurement configuration. RAN3 to further discuss what the assistance information is. From RAN3 perspective, there is no need to send assistance information to UE.

In our understanding, although it is not fully confirmed (RAN3 is still waiting for reply from SA5), the assistance information could be some type of priority level associating to each QoE configuration. Thus, the gNB is able to use such information to decide which QoE configuration should pause/resume its reporting when the RAN is overloaded. If such assistance information can be made available at RAN, it seems to serve as a good guideline for the UE to make decisions on QoE report discarding in RRC-IDLE/INACTIVE states as well. In particular, when the buffer becomes full, the UE can first drop the QoE measurements for QoE configurations with lowest priority. Thus, Option 2 is also sensible in cases RAN is able to get such assistance information.

Option 3: Based on UE implementation

From specification impact perspective, Option 3 is the simplest, as we do not have to specify any new configuration or UE behavior. We think Option 3 is also acceptable as it gives more leeway for the UE in terms of reducing implementation complexity.

We think RAN2 should continue to discuss the desired UE behavior to handle this issue based on these three options, and conclude which of the approaches is the preferred way forward.

Proposal: RAN2 should discuss the following three options on how to discard QoE measurements when the AS buffer becomes full in the RRC-IDLE/INACTIVE states:

- Option 1: Based on the “age” of collected QoE measurements

- Option 2: Based on network configuration (e.g. with QoE configuration priority)

- Option 3: Based on UE implementation

# Conclusions

In this contribution, we have discussed how the UE should decide which QoE report to discard when the AS buffer becomes full in RRC-IDLE/INACTIVE states, and proposed the following:

Proposal: RAN2 should discuss the following three options on how to discard QoE measurements when the AS buffer becomes full in the RRC-IDLE/INACTIVE states:

- Option 1: Based on the “age” of collected QoE measurements

- Option 2: Based on network configuration (e.g. with QoE configuration priority)

- Option 3: Based on UE implementation