3GPP TSG-RAN WG2 Meeting #123	R2-2307836

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.14.4

Source:	Apple

Title:	Discussions on QoE Reporting for NR-DC

WID/SID:	NR_QoE_enh-Core

Document for:	Discussion / Decision

# Introduction

This paper aims to provide some our views on the WI objective relating to QoE in NR-DC [1]. We discuss the following:

- The issues relating to reporting of SN-associated QoE reports via SRB4, and

- The issues relating to cases where the SRB indicated in a QoE configuration is actually not available.

# Discussions

## Forwarding of SN-Associated QoE Report by MN

During RAN2 #121bis-e, we have discussed whether the UE can send SN-associated QoE reports to the MN when SRB5 is not configured. The following agreement is reached:

9: For NR-DC, if SRB5 is not configured (FFS on the SCG deactivation case), UE can transmit the SN-associated QoE reports via SRB4. FFS whether there are some ambiguities how MN knows where to forward this.

Furthermore, in RAN2#122 we have agreed that RAN2 can just follow RAN3 on how the encapsulated QoE received by MN can be forwarded correctly:

2: RAN2 will follow RAN3 agreements (for NR-DC when SRB5 is not configured) on forwarding the received encapsulated QoE reports to the correct recipient (i.e. MCE or SN)

Based on the progress made in RAN3, it seems MN and SN can coordinate to make sure the MN (or SN) knows how to deliver the QoE reports to the MCE properly. In addition, the MN can also forward the RVQoE report to the SN when SRB5 is not configured. Moreover, the SN is able to indicate to MN whether it prefers to receive QoE/RVQoE via SRB4 or SRB5. Some of the agreements made in RAN3#119bis-e and RAN3#120 are shown below:

RAN3 #119bis-e Agreements: When SN indicates its interest in configuring m-based QoE a measurement to a UE: The SN can indicate to the MN that the reports are to be sent via the SRB5. The SN can request the use of the SRB4 for reporting, which the MN can confirm or reject. FFS whether the indication is explicit or implicit. … If the SN is asked by the MN to forward to the MCE the QoE reports pertaining to a measurement configured by the MN, the MN should indicate to the SN the QoE Reference, the MCE IP Address and the RRC ID. If the MN is asked by the SN to forward to the MCE the QoE reports pertaining to a measurement configured by the SN, the SN should indicate to the MN the QoE Reference and the MCE IP Address. … If the SRB5 is not configured, the RVQoE reports can be sent on the SRB4 from the UE via the MN to the SN. RAN3 #120 Agreements: Agreements: When a management-based QoE configuration is received directly by the SN from the OAM, the SN can explicitly indicate to the MN whether it is going to receive the QoE/RVQoE reports via the SRB5 or SRB4, whether it is transparently via SRB4 needs further discussion.

Thus, we think RAN2 can confirm that MN is able to forward the SN-associated QoE reports correctly (to MCE or SN) as long as MN and SN are properly coordinated.

Proposal 1: RAN2 can confirm that MN knows where the forward the SN-associated QoE reports, since MN and SN can coordinate based on RAN3 agreements.

Based on the RAN3 agreement, the SN is able to receive QoE/RVQoE via either SRB4 (from MN) or SRB5 (from UE directly), and whether it should be transmitted transparently over SRB4 should be discussed further. It is our understanding that, how the SN-associated QoE is reported via SRB4 (e.g. transparent or not) falls into RAN2’s scope. In our view, since the SN-associated QoE report is intended to be received by the SN, there is no need for the MN to know what is transmitted. Thus, it can be transmitted as a transparent container.

To enable this, we believe we can reuse the procedure based on ULInformationTransferMRDC which is typically used to transfer MR-DC dedicated information from the UE to the network. The information such as RRC Measurement Report, UE Assistance Information, and Failure Information can be included as transparent containers in the ULInformationTransferMRDC message. Currently, the ULInformationTransferMRDC message is sent on either SRB1 or SRB3, depending on whether the UE intends to transfer MCG Failure Information. Therefore, when the UE intends to provide SN-associated QoE measurements via SRB4, the ULInformationTransferMRDC message can be extended by allowing it to carry application layer measurement information for SN. Moreover, when the UE intends to transfer application layer measurement information with ULInformationTransferMRDC message, such message should be transmitted over SRB4 (which is a MCG bearer). At the network side, there is no ambiguity as the MN will naturally forward the transparent containers of ULInformationTransferMRDC message to the SN.

Proposal 2: The ULInformationTransferMRDC message can be extended to transmit SN-associated QoE reports transparently via SRB4.

## Issues Relating to “Unavailable” SRB

In RAN2#122, we have agreed that the reporting SRB can be explicated indicated in the QoE configuration, along with a few open issues that need to be further studied:

1: The network can use explicit indication per QoE config to indicate which SRB is used for the QoE reporting. Details can be discussed in Stage-3. FFS how to handle the QoE report transmission if there is only one SRB and the QoE report is not (explicitly) configured for that SRB (to be checked if this is possible according to latest RAN3 agreements) FFS how the above case works if SCG is deactivated or released.

Both of the FFS listed above are relating to the cases where the SRB indicated in a QoE configuration is actually “unavailable”. For instance, the SRB is not configured, or the SCG for that SRB is deactivated/released. We are not sure how likely such situations will happen in practice, as typically the network should make sure this is properly configured. However, if anything is needed, we think this is also reasonable to define a UE behavior where the UE can send the QoE report to another SRB (which is available) when the default SRB as indicated in the QoE configuration cannot be used for whatever reasons. As discussed earlier, according to RAN3 agreements it is anyway possible for the MN to forward SN-associated QoE reports to SN, we think at least the UE should be able to use SRB4 in cases where the UE cannot use SRB5 for QoE reporting.

Proposal 3: When SRB5 cannot be used (e.g. when the SRB5 is not configured, or when there is no active cells in the corresponding SCG), the UE can use SRB4 to report for the QoE configuration that has been indicated to use SRB5.

On the other hand, we have noted there is another open issue about pause/resume identified in RAN2 #121bis-e that needs further discussion:

14: As a baseline, Rel-17 pause/resume procedure is reused to pause/resume reporting of one or multiple QoE measurement configurations in a UE for NR-DC. Details are FFS, e.g. whether paused QoE reports can be reported to SN (if SN is not overload).

In Rel-17, if the network instructs the UE to pause QoE reporting for a QoE measurement configuration (e.g. the network is overloaded), the UE should stop transmitting the corresponding encapsulated QoE measurements, but it should continue to transmit the RVQoE. This is because RVQoE measurements may still be useful for RAN to perform resource optimization in order to handle the overload situations. With this in mind, for NR-DC, when the QoE configuration with an indicated SRB is paused, it means the gNB (e.g. SN) corresponding to the indicated SRB is experiencing some sort of overloading. Since we have confirmed that the UE is able to report SN-associated QoE reports to MN based on RAN3 agreements, we think the following UE behaviors are possible and beneficial when the QoE measurement configuration indicated with SRB5 is paused:

- The UE may stop reporting of both encapsulated QoE and RVQoE for the QoE configuration on SRB5, but continue to report RVQoE (only) for the same QoE configuration on SRB4, or;

- The UE may stop reporting of both encapsulated QoE and RVQoE for the QoE configuration on SRB5, but continue to report both encapsulated QoE and RVQoE for the same QoE configuration on SRB4.

This is of course only allowed when the MN is not overloaded. In general, we think such UE behavior is helpful for the SN to alleviate overload situation, while being able to continue to receive RVQoE (and possibly encapsulated QoE) from the UE via MN.

Proposal 4: When a QoE configuration for reporting on SRB5 is paused, the UE may continue to send RVQoE (and possibly encapsulated QoE) of this QoE configuration on SRB4.

# Conclusions

In this contribution, we have discussed some of our opinions about QoE reporting in NR-DC scenarios, and proposed the following:

Proposal 1: RAN2 can confirm that MN knows where the forward the SN-associated QoE reports, since MN and SN can coordinate based on RAN3 agreements.

Proposal 2: The ULInformationTransferMRDC message can be extended to transmit SN-associated QoE reports transparently via SRB4.

Proposal 3: When the SRB indicated in a QoE configuration cannot be used (e.g. when the SRB is not configured, or when there is no active cells in the corresponding SCG), the UE can use another SRB that is available to report for this QoE configuration.

Proposal 4: When a QoE configuration for reporting on SRB5 is paused, the UE may continue to send RVQoE (and possibly encapsulated QoE) of this QoE configuration on SRB4.

# References

[1] RP-223488, WID update for Enhancement on NR QoE, China Unicom, RAN #98, December 2022.