3GPP TSG-RAN WG2 Meeting #123	R2-2307873

Toulouse, France, August 21-25, 2023

Agenda Item:	[7.17.3] Allowed MUSIM temporary capability restriction

Source: 	Apple Inc

Title:  	Allowed MUSIM temporary capability restriction for Band Conflict Mitigation

Document for:	Discussion and Decision

# Introduction

In the last RAN2#121-bis-e meeting, the following agreements were made with regards to the allowed parameters for temporary UE capability restriction for MUSIM purpose.

1: For Rel-18 MUSIM dual active operation, the maximum MIMO layer may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B.

3: For Rel-18 MUSIM dual active operation, the measurement gap requirement may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B.

4: For Rel-18 MUSIM dual active operation, the measurement gap requirement change is reported for each serving cells, and for target bands or all supported NR bands depending on whether target bands are configured by the NW. FFS on whether the reporting can reuse the current needForGapInfoNR in RRC reconfiguration complete or extend the similar function in UAI.  FFS if this is only for NW A or also NW B.

8: The maximum UL power may be changed due to Rel-18 MUSIM dual active operation, but there is no need to introduce any new UE behavior for reporting this change.

To discuss: P6

6: UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. FFS how/whether this works for the proactive case.

9: RAN2 should avoid duplicating all the capabilities that UE reports via the UECapabilityInformation in the UAI for R18 MUSIM purpose.

In the last RAN2#121 meeting, the following agreements were made with regards to the allowed parameters for temporary UE capability restriction for MUSIM purpose.

RAN2 confirms that the band conflict scenarios will be covered by the temporary UE capability restrictions. FFS on signalling details.

In this contribution, we would like to provide our views on the band conflict aspect for temporary capability restriction.

# Discussion

As agreed in RAN2#121 meeting [2], the band conflict scenarios will also be handled via temporary UE capability restriction. At a very minimum, UE should have the flexibility to report either

- The list of conflicting bands and the band combinations that includes the conflicting bands OR

- The list of nonconflicting bands and the band combinations that includes some nonconflicting bands

It is obvious that a preferred solution of a Vs b, would depend on whichever results in the least amount of signalling information over the air, which is exchanged as part of UE – NW RRC signalling.

Observation 1: Band conflict mitigation can be solved using temporary UE capability restriction with the UE indicating a subset of bands and/or band combinations to be enabled or disabled.

In our view, UE indicating the list of conflicting bands via UAI signalling and allowing the NW to derive the list of band combinations which includes the list of conflicting bands could be one possible way forward. This is also in line with the agreement reached in RAN2#121-bis-e [1] meeting wherein it was agreed that the entire UE capability (as originally carried in the UECapabilityInformation message) should not be duplicated in the UE ASSISTANCE INFORMATION message.

Observation 2: UE indicating just a list of conflicting bands, followed by both NW and UE deriving all the conflicting band combinations could result in a smaller amount of signalling load, without duplicating all the contents of the previously indicated UECapabilityInformation message.

Our understanding is that such restriction on band and/or band combination is obviously temporary in nature, and as soon as the requirement for such band conflicts are alleviated on the UE side, UE shall request for removal of such temporary UE capability restriction.

Observation 3: Any request for temporary UE capability restriction should be followed by UE requesting to remove such restriction once the conditions for band conflicts are alleviated on the UE side.

Proposal 1: UE can signal the set of conflicting bands via UE ASSISTANCE INFORMATION (UAI) framework, using which both UE and NW can derive the set of conflicting band combinations to help in band conflict mitigation.

Proposal 2: UE can request via the same UE ASSISTANCE INFORMATION (UAI) framework to remove such temporary UE capability restrictions when conditions that alleviate the band conflict scenario are met.

# Conclusion

In summary, we have the following observations with respect to parameters for temporary capability restriction signalling issue for MUSIM UEs from Band conflict mitigation perspective.

Observation 1: Band conflict mitigation can be solved using temporary UE capability restriction with the UE indicating a subset of bands and/or band combinations to be enabled or disabled.

Observation 2: UE indicating just a list of conflicting bands, followed by both NW and UE deriving all the conflicting band combinations could result in a smaller amount of signalling load, without duplicating all the contents of the previously indicated UECapabilityInformation message.

Observation 3: Any request for temporary UE capability restriction should be followed by UE requesting to remove such restriction once the conditions for band conflicts are alleviated on the UE side.

Proposal 1: UE can signal the set of conflicting bands via UE ASSISTANCE INFORMATION (UAI) framework, using which both UE and NW can derive the set of conflicting band combinations to help in band conflict mitigation.

Proposal 2: UE can request via the same UE ASSISTANCE INFORMATION (UAI) framework to remove such temporary UE capability restrictions when conditions that alleviate the band conflict scenario are met.

# Reference

[1]  R2-2304202	 Report from session on LTE legacy, XR, QoE and Multi-SIM	Vice Chairman (Nokia)

[2]  R2-2301902 Report from session on LTE legacy, XR, QoE and Multi-SIM	Vice Chairman (Nokia)