3GPP TSG-RAN WG2 Meeting #123	R2-2307876

Toulouse, France, 21-25 August 2023

Source: 			Apple Inc, Qualcomm Incorporated, Ericsson

Title: 	Building on agg BW signaling for FR2 R2-R12 BW classes

Document for: 	Decision

Agenda Item: 	6.1.3.2

# Introduction

Based on the below way forward on the topic of aggregated BW discussion related to inter-band CA, it was also agreed to discuss and progress the topic of using aggregated BW signalling for FR2, which would address the LSes[2][3] from RAN4 on this particular topic

R2-2304856 Maximum aggregated bandwidth for FR1 inter-band CA Qualcomm Incorporated, Apple, Ericsson discussion Rel-17 NR_BCS4-Core DISCUSSION - Samsung wonder if this is an optimization. Chair think yes. - HW wonder about legacy BCS0. - Nokia has some sympathy for the Huawei concern. Think that BCS5-only optimization can be ok. - Apple think P6 should be applicable to FR2 as well. - HW wonder if this applies to only-bcs5 or if it also includes legacy BCS. QC think this is up to R4 and a UE may include a legacy BCS if the UE is “nice”. - MTK think BCS5 was intended to include all max BWs, including also those covered by legacy BCS. - Nokia think BCS5 is not a requirement from network point of view. - QC think it is clear that with BCS5 the UE need to support more combinations. QC think there are cases in the future where only BCS4 and BCS5 are supported, thus this is clearly useful. - Nokia think this can be supported it BW compatibility is ensured. - HW think that if BCS0 is indicated the network may overestimate the UE cap. QC think that BCS0 can be indicated. Chair think that BCS0 indication may need to indicate lower capability than the BCS5 indication in order to work/make sense. - Working Assumption that we go with P1-P6 in R2-2304856, under the condition that BW compatibility is supported. Next meeting describe how BW compatibility can work and confirmation of WA (or the opposite if serious issues are found)

In this document, we provide some more refresher on the need for agg BW signaling for FR2 BCs with the intent to have the CRs (which were built on top of agg BW for inter-band FR1 CA CRs) agreed.

# Discussion

  - The need for agg BW signalling for FR2

The virtue of the CA BW classes and the associated fallback rule lies in that UE only needs to signal the highest order CA BW class to indicate its capability for supporting intra-band contiguous CA and all the lower order CA BW classes are expected to be supported without additional signaling, in a way to minimize the signaling overhead. The CA BW class contains two key parameters, number of component carriers (CCs) and aggregated BW range. The number of CCs is associated with UE’s capability on processing the number of carriers simultaneously. The aggregated BW upper limit is normally determined by the number of CCs multiplying the maximum supported BW per carrier assuming all carriers have the same maximum supported BW. Nevertheless, UE’s capability on maximum aggregated BW support can be less than its supported number of CCs multiplying the maximum supported BW per carrier, such as limited by its overall digital processing capacity. In that case, for the traditional CA BW classes where all the cells in the same fallback group have the same maximum BW capability, UE may just signal the next lower order CA BW class to align with its maximum aggregated BW capability. However, for the newly introduced FR2 CA BW classes in FBG5 as shown in Table 2-1, where the aggregated BW can be combined with a mixture of 100MHz cells and 200MHz cells, the signaling of UE’s maximum aggregated BW capability which is less than the CA BW class upper limit to the network could be rather complicated as UE is assumed to have no prior knowledge on how the CA would be configured by the network and UE may have to signal multiple FeatureSets in order to cover all the possible network configurations. Such signaling complexity is further exemplified below.

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4,5 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| D | 200 MHz < BWChannel_CA ≤ 400 MHz | 2 | 2 |
| E | 400 MHz < BWChannel_CA ≤ 600 MHz | 3 |  |
| F | 600 MHz < BWChannel_CA ≤ 800 MHz | 4 |  |
| R | 800 MHz < BWChannel_CA ≤ 1000 MHz | 5 |  |
| S | 1000 MHz < BWChannel_CA ≤ 1200 MHz | 6 |  |
| T | 1200 MHz < BWChannel_CA ≤ 1400 MHz | 7 |  |
| U | 1400 MHz < BWChannel_CA ≤ 1600 MHz | 8 |  |
| G | 100 MHz < BWChannel_CA ≤ 200 MHz | 2 | 3 |
| H | 200 MHz < BWChannel_CA ≤ 300 MHz | 3 |  |
| I | 300 MHz < BWChannel_CA ≤ 400 MHz | 4 |  |
| J | 400 MHz < BWChannel_CA ≤ 500 MHz | 5 |  |
| K | 500 MHz < BWChannel_CA ≤ 600 MHz | 6 |  |
| L | 600 MHz < BWChannel_CA ≤ 700 MHz | 7 |  |
| M | 700 MHz < BWChannel_CA ≤ 800 MHz | 8 |  |
| O | 100 MHz ≤ BWChannel_CA ≤ 200 MHz | 2 | 4 |
| P | 150 MHz ≤ BWChannel_CA ≤ 300 MHz | 3 |  |
| Q | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 4 |  |
| R2 | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 2 | 5 |
| R3 | 300 MHz ≤ BWChannel_CA ≤ 600 MHz | 3 |  |
| R4 | 400 MHz ≤ BWChannel_CA ≤ 800 MHz | 4 |  |
| R5 | 500 MHz ≤ BWChannel_CA ≤ 1000 MHz | 5 |  |
| R6 | 600 MHz ≤ BWChannel_CA ≤ 1200 MHz | 6 |  |
| R7 | 700 MHz ≤ BWChannel_CA ≤ 1400 MHz | 7 |  |
| R8 | 800 MHz ≤ BWChannel_CA ≤ 1600 MHz | 8 |  |
| R9 | 900 MHz ≤ BWChannel_CA ≤ 1800 MHz | 9 |  |
| R10 | 1000 MHz ≤ BWChannel_CA ≤ 2000 MHz | 10 |  |
| R11 | 1100 MHz ≤ BWChannel_CA ≤ 2200 MHz | 11 |  |
| R12 | 1200 MHz ≤ BWChannel_CA ≤ 2400 MHz | 12 |  |
| NOTE 1: Maximum supported component carrier bandwidths for fallback groups 1, 2, 3, 4 and 5 are 400 MHz, 200 MHz, 100 MHz, 100 MHz and 200 MHz respectively except for CA bandwidth class A. For CA bandwidth classes of fallback group 5, requirements apply for non-interlaced 100 MHz and 200 MHz channel bandwidths (each CA bandwidth class consisting of up to two contiguous sub-blocks each with component carriers of a single channel bandwidth). NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group. NOTE 3: In this release of the specification, the minimum requirements for intra-band contiguous CA configurations apply for aggregated channel bandwidths up to 1600 MHz (this note is not relevant for UE capability parsing by the network). |  |  |  |

Table 2-1 The newly introduced FR2 CA BW classes R2 – R12 in FBG5 as highlighted

For example, if a UE can support 12 CCs and each CC’s maximum BW is 200 MHz, whereas its maximum aggregated BW is only 1200 MHz, how would the UE report its capability to the network based on the current signaling design?

One possibility is to only signal BW class “R6” as its BW upper range aligns with UE’s maximum aggregated BW capability. But this limits the number of carriers the UE/NW can use! In other words, 1200MHz could not be configured to the UE which only signals “R6” despite UE does have the capability to support all other configurations.

To achieve the goal of UE reporting that it CAN support upto 1200 MHz, and upto 12 carriers, the below configurations (which the UE would support) would need to be signalled separately – repeating the band combination definition multiple times, with different BW classes (R7-R12) AND for each of the band combination, the associated overhead of repeating the featureSet/featureSetPerCC (for both UL and DL!).

- 2x100MHz + 5x200MHz (7 CCs)  - BW class R7

- 4x100MHz + 4x200MHz (8 CCs)  - BW class R8

- 6x100MHz + 3x200MHz (9 CCs)  - BW class R9

- 8x100MHz + 2x200MHz (10 CCs)  - BW class R10

- 10x100MHz + 1x200MHz (11 CCs)  - BW class R11

- 12x100MHz (12 CCs)  - BW class R12

Observation 1: FR2 R2-R12 (and future R*) BW class definition is such that the UE can support an aggregated BW with a min and a max range, as well as multiple CCs which can support the earlier mentioned aggregated carrier BW. But we can UE implementations that support an intermediate BW (between the min and max BW of the R* class, but supporting the same number of carriers

Observation 2: To allows signalling of such capabilities, the UE would have to “repeat” the same bandcombination multiple times using the closely linked R* BW class to denote the number of carriers that can be aggregated to support the UE BW. Each BC repeated will need to include the associated overhead (featureSetCombination, featureSet(DL/UL)PerBand, featureSet(DL/UL)PerCC etc..).

Fortunately, this issue can be simply by using the ‘agg BW for inter-band CA for FR1’ platform that is described in [4].

Observation 3: The agg BW for inter-band CA for F1 can be built upon to solve the RAN4 FR2 LSes trying to addres the R* problem.

  - Using Agg BW for FR2

The critical logic here is that the UE reports it’s maximum aggregate BW it supports for a particular FR2 R* BW class. This provides the NW with the maximum number of carriers (from the BW class) and the max BW (from the new signalling) the UE supports – across the carriers of the signalled FR2 R* BW class. And the UE only needs to signal “ONE” band combination.

Since the R* BW class carriers are all either 100 MHz or 200 MHz, the NW only needs to know the (UL and DL) featureSetDown(Up)linkPerCC for the 100 and 200 MHz parts, and the NW can use these two featureSetPerCC capabilities to derive the UE capability for “ALL” carriers within that R* BW class, as long as the configured BW is within the newly signalled agg BW from the UE.

Proposal 1: A new IE is signalled by the UE per each FR2 band that has R* class, and this new IE provides the maximum aggregated BW that UE support across all the carriers in that R* class. This is only applicable to bands in BC which FR2 with R* BW class.

    - Backward compatiability and other details

Since the R2-R12 BW classes are very recently introduced and there is still discussion/specification work happening in RAN4 related to this, we do not see the need to introduce additional UE capability filter request and response logic.

Observation 4: R2-R12 BW classes are introduced recently and no UE or NW vendor has yet deployed this feature, so there shoud be no back-ward compatiability issue, as long as the CR captures that UEs and NWs implementing R2-R12 FR2 BW classes need to implement this CR.

Also as stated in the previous section, the UE can just include atleast two (in DL and in UL) featureSetPerCC references pointing to capabilities for 100 and 200 MHz carriers, for the NW to derive the capability for the all the carriers across which the NW can configure the BW upto agg BW that the UE signalled.

We understand that the UE can point a featureSetPerCC with a BW higher than 200 MHz and this can be assumed to be used for 200 or 100 MHz. So we think no new requirement is needed at the UE on how the UE should signal as long as the UE reports atleast (separately for UL and DL) a featureSetPerCC references pointing to capabilities for atleast 200 MHz carrier (the NW can use this for 100 MHz carrier as well).

Proposal 2: If the UE signals the agg BW for a R* BW class for FR2, the UE should include atleast one featureSetPerCC (for DL and for UL if applicable) with atleast 200 MHz BW capability and this is used by the NW for deriving the associated capabilities for all carriers as long as the BW across these carriers is not more than the UE signalled agg BW.

CRs [7][8] based on the inter-band FR1 CA agg BW CRs [5][6] are provided for agreement.

Proposal 3: Agree CRs from [7][8]

# Conclusion

Observation 1: FR2 R2-R12 (and future R*) BW class definition is such that the UE can support an aggregated BW with a min and a max range, as well as multiple CCs which can support the earlier mentioned aggregated carrier BW. But we can UE implementations that support an intermediate BW (between the min and max BW of the R* class, but supporting the same number of carriers

Observation 2: To allows signalling of such capabilities, the UE would have to “repeat” the same bandcombination multiple times using the closely linked R* BW class to denote the number of carriers that can be aggregated to support the UE BW. Each BC repeated will need to include the associated overhead (featureSetCombination, featureSet(DL/UL)PerBand, featureSet(DL/UL)PerCC etc..).

Observation 3: The agg BW for inter-band CA for F1 can be built upon to solve the RAN4 FR2 LSes trying to addres the R* problem.

Observation 4: R2-R12 BW classes are introduced recently and no UE or NW vendor has yet deployed this feature, so there shoud be no back-ward compatiability issue, as long as the CR captures that UEs and NWs implementing R2-R12 FR2 BW classes need to implement this CR.

Proposal 1: A new IE is signalled by the UE per each FR2 band that has R* class, and this new IE provides the maximum aggregated BW that UE support across all the carriers in that R* class. This is only applicable to bands in BC which FR2 with R* BW class.

Proposal 2: If the UE signals the agg BW for a R* BW class for FR2, the UE should include atleast one featureSetPerCC (for DL and for UL if applicable) with atleast 200 MHz BW capability and this is used by the NW for deriving the associated capabilities for all carriers as long as the BW across these carriers is not more than the UE signalled agg BW.

Proposal 3: Agree CRs from [7][8]

# References

[1]	   R2-2303398	On servicing RAN4 request on aggregate BW signaling for FBG5 CA BW classes

[2]     R2-2302440 - LS on signaling for FR2 FBG5 CA BW classes

[3]		R2-2209347 - LS on new contiguous BW classes for legacy networks

[4]    R2-230xxxx     Backward compatibility analysis on new UE capability signalling of new maximum aggregated bandwidth for FR1 inter-band CA

[5]  R2-2307203  Introduction of maximum aggregated bandwidth for FR1 inter-band CA 38.331

[6]  R2-2307204  Introduction of maximum aggregated bandwidth for FR1 inter-band CA 38.306

[7]  R2-2307874  Introduction of maximum aggregated bandwidth for FR1 inter-band CA and for FR2 intra-band CA 38.331

[8]  R2-2307875  Introduction of maximum aggregated bandwidth for FR1 inter-band CA and for FR2 intra-band CA 38.306