3GPP TSG-RAN WG2 Meeting #123		R2-2307888

Toulouse, France, August 21-25, 2023

Agenda Item:	7.4.2.1

Source: 	ITRI

Title:  	Discussion the remaining issues for LTM early TA acquisition

Document for:	Discussion

Introduction

In RAN1 LS [1], two options were presented for the PDCCH ordered RACH procedure to enable early TA acquisition on the candidate cell, one involves utilizing RAR (Random Access Response) from either the serving or candidate cell, while the other option excludes the use of RAR.

After last RAN2 meeting discussion, it was agreed that “RAN2 doesn’t see a need for a solution with RAR in for Rel-18.” As a result, RAN2 has determined that the appropriate approach for the PDCCH ordered RACH procedure to achieve early TA acquisition is the "without RAR" option.

This contribution focuses on addressing the failure handling regarding the LTM early TA acquisition procedure.

Discussion

Early TA acquisition failure detection

The early TA acquisition failure may happen due to:

- Preamble transmission failure

- RRCReconfigurationComplete message transmission failure

For the RRCReconfigurationComplete message transmission failure case, the agreed LTM supervisor timer will be used to handle this failure case. The agreed behaviors of LTM supervisor timer is as following:

- 1: The UE starts the LTM supervisor timer, upon reception of the LTM cell switch MAC CE;

- 2: The UE stops the LTM supervisor timer, upon successful completion of LTM cell switch;

- 3: If the LTM supervisor timer for MCG expires, as baseline, the UE considers LTM failure and initiates RRC re-establishment. (SCG switch case FFS)

However, the LTM supervisor timer may not be able to handle the preamble transmission failure case. This is because, when the preamble transmission failure occurs, the cell switch procedure will not be triggered. As a result, the UE will not receive the LTM cell switch MAC CE, and the LTM supervisor timer will not be started.

To address the preamble transmission failure problem, we considered reusing the legacy ra-ResponseWindow. Similar to the legacy approach, the UE should initiate the ra-ResponseWindow when it transmits the preamble for LTM early TA acquisition. However, in this scenario, instead of receiving the RAR (Random Access Response), the UE receives the cell switch MAC CE (Control Element) after transmitting the preamble for LTM early TA acquisition. There may be two cases for the received cell switch MAC CE:

- Case 1: the cell switch MAC CE contains the TA value. In this case, it means that the preamble is successfully received by the target-DU. Therefore, the ra-ResponseWindow should stop.

- Case 2: the cell switch MAC CE doesn’t contain the TA value. In this case, it means that the source-DU may want to trigger another cell switch procedure without early TA acquisition. Therefore, the ra-ResponseWindow should also stop.

Based on the two cases mentioned above, the UE should halt the ra-ResponseWindow upon receiving the cell switch MAC CE, irrespective of whether the MAC CE contains the Timing Advance (TA) value or not.

Proposal 1: The UE start the ra-ResponseWindow when the UE transmits the preamble for LTM early TA acquisition.

Proposal 2: The UE should stop the ra-ResponseWindow timer after receiving the cell switch MAC CE.

In the legacy RACH procedure, if the ra-ResponseWindow timer expires and the PREAMBLE_TRANSMISSION_COUNTER does not exceed the preambleTransMax, the UE will initiate the RACH procedure again. However, if the PREAMBLE_TRANSMISSION_COUNTER exceeds the preambleTransMax, the UE will consider the RACH procedure as unsuccessful.

In RAN1, it was agreed that ”For PDCCH ordered-RACH, if reception of RAR is not configured, UE autonomous re-transmission of PRACH is not allowed, regardless of the configuration of PreambleTransMax.” As a result, in the case of the early TA RACH without RAR solution, the UE won’t be able to autonomously re-attempt the RACH procedure when the ra-ResponseWindow timer expires. Consequently, UE should indicate the LTM early TA acquisition has failed.

Proposal 3: For the early TA acquisition, the UE should declare the LTM early TA acquisition is failure when the ra-ResponseWindow timer expires.

As the RAN2 agreement, the RRC re-establishment could be utilized to address LTM failure when the LTM supervisor timer expires. However, in the context of preamble transmission failure, triggering the RRC re-establishment might not be appropriate.

The reason for this is that, in the event of preamble transmission failure occurs and the non-receipt of the cell switch MAC CE, the UE won’t initiate the LTM procedure. Thus, this situation cannot be classified as an LTM failure.

Furthermore, when the preamble transmission failure occurs, it should be a radio link problem between target-DU and UE, rather than between the serving cell and the UE. Hence, activating the RRC re-establishment procedure is unsuitable in such a scenario.

A simpler approach to manage preamble transmission failure during LTM early TA acquisition is to report this failure information to the serving cell. Subsequently, the serving cell can opt for an alternative candidate target-DU for the UE.

Proposal 4: For the early TA acquisition, when the ra-ResponseWindow timer expires, UE should report the failure information to the serving cell.

Conclusion

In this paper, we discussion the RAN2 issues for the remaining early TA acquisition, and we have the following proposals.

Proposal 1: The UE start the ra-ResponseWindow when the UE transmits the preamble for LTM early TA acquisition.

Proposal 2: The UE should stop the ra-ResponseWindow timer after receiving the cell switch MAC CE.

Proposal 3: For the early TA acquisition, the UE should declare the LTM early TA acquisition is failure when the ra-ResponseWindow timer expires.

Proposal 4: For the early TA acquisition, when the ra-ResponseWindow timer expires, UE should report the failure information to the serving cell.

Reference

- R1-2212948 	LS on RAN1 agreements for L1/L2-based inter-cell mobility, RAN1

- R2-2212865 	Discussion on security issue in cell switch, NTT DOCOMO INC.