3GPP TSG-RAN WG2 Meeting #123	R2-2307899

Toulouse, France, Aug 21st – Aug 25th, 2023

Agenda Item:	7.20.2

Source:	FGI

Title:	Discussion on two TAs for multi-DCI multi-TRP

Document for:	Discussion

Introduction

In RAN#98-e meeting, the Rel-18 WID on NR MIMO evolution for downlink and uplink was approved [1], which includes the following objective corresponding to the TA enhancement for multi-DCI multi-TRP:

- Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

In the previous meetings, some agreements were achieved to make progress for TA enhancement of the UL multi-DCI multi-TRP transmission by RAN1 and RAN2. In this contribution, we further discuss the RAN2 impact on 2 TAs for multi-DCI based multi-TRP based UL transmission based on RAN1 input.

Discussion

TA acquisition

During the past RAN1 meetings for Rel-18 two TA for multi-DCI multi-TRP, RAN1 agreed that UE can acquire two TAs associated with 2 TRPs associated with different CORESETPoolIndex values respectively by the CFRA triggered by PDCCH order. However, one issue similar to the issue discussed in multi-carrier enhancement needs to be discussed for 2 TA enhancement for multi-DCI multi-TRP, i.e., whether to support performing RA procedures corresponding to different TRPs associated with different CORESETPoolIndex values at the same time. In the current specification, the UE only performs one RA procedure at one time. It is up to UE to decide which RA procedure is to be performed if the ongoing RA procedure collides with the new RA procedure [2].

NOTE 1: If a new Random Access procedure is triggered while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure (e.g. for SI request).

To avoid increasing the UE complexities and specification impact, it is better not to support parallel RA procedures on different TRPs in Rel-18 two TAs for multi-DCI multi-TRP.

Proposal 1: In Rel-18 two TAs for multi-DCI multi-TRP, parallel RA procedures on different TRPs associated with different CORESETPoolIndex values is not supported.

TAG

In RAN1#113 meeting, the following agreement was made in [3]:

Agreement - Proposed answer to Question Q1a in RAN2 LS R1-2304326: - Apart from the agreements RAN1 has sent in LS R1-2302226 to RAN2 before, RAN1 has not agreed to any further restrictions on the association of serving cells and/or TRPs to the TAGs at this point. If RAN1 agrees to such restrictions, RAN1 will inform RAN2. - Proposed answer to Question Q1b in RAN2 LS R1-2304326: - RAN1 has not reached consensus to increase the current number of TAGs per cell group. - Proposed answer to Question Q2 in RAN2 LS (R1-2304326): - RAN1 confirms that when the TA timer associated to one TRP expires for a TAG associated with a TCI state, UL or DL operation associated to the another TRP is not impacted. This further depends on PTAG/STAG definition, which is up to RAN2 to decide. - Which UL or DL operation is impacted have not been discussed in RAN1.

Currently, the maximum number of TAGs is determined based on the UE capability. In addition, the maximum TAG number supported by the UE is 4. In Rel-17, even though the UE can be configured with 32 serving cells, there are only 4 TAGs (i.e., maximum TAG number) used to maintain the TAs corresponding to different serving cells. From our perspective, it is not necessary to increase the maximum TAG number to further increase UE complexities.

Proposal 2: In Rel-18 two TAs for multi-DCI multi-TRP, the maximum TAG number is still 4 to avoid further increasing UE complexities.

In Rel-18 two TAs for multi-DCI multi-TRP, one serving cell can be associated with two TAGs. According to the current specification, i.e., a timing advance group containing the SpCell of a MAC entity is referred to as a Primary Timing Advance Group (PTAG), these two TAGs are both PTAGs. According to the current MAC specification, if the TAT associated with the PTAG is expired, UL transmissions corresponding to all serving cells will be terminated/released. But, in the latest RAN1 agreement, RAN1 confirms that when the TAT associated with one TRP expires, the UL or DL operation associated with another TRP is not impacted. To achieve RAN1’s confirmation, some changes in MAC specification are needed. To have less specification impact, we can simply change the condition that all UL synchronization statuses corresponding to all serving cells are out-of-sync, e.g., UL transmissions corresponding to all serving cells will be terminated/released when both TATs associated with PTAGs are expired.

Proposal 3: If there are two TATs associated with two TAGs belonging to SpCell configured to the UE, all UL synchronization statuses corresponding to UL transmissions associated with all serving cells are out-of-sync.

Conclusions

In this contribution, we have the following proposals:

Proposal 1: In Rel-18 two TAs for multi-DCI multi-TRP, parallel RA procedure on different TRPs associated with different CORESETPoolIndex values is not supported.

Proposal 2: In Rel-18 two TAs for multi-DCI multi-TRP, the maximum TAG number is still 4 to avoid further increasing UE complexities.

Proposal 3: If there are two TATs associated with two TAGs belonging to SpCell configured to the UE, all UL synchronization statuses corresponding to UL transmissions associated with all serving cells are out-of-sync.

References

- [1] RP-223276, WID Update: MIMO Evolution for Downlink and Uplink, Samsung

- [2] 3GPP TS 38.321, V17.5.0

- [3] Chair’s notes RAN1#113, 3GPP TSG RAN WG1 #113, May 22nd – 26th, 2023