3GPP TSG-RAN WG2 Meeting #123	                                    R2-2307900

Toulouse, France, Aug. 21st – Aug. 25th, 2023

Agenda item:		7.4.4

Source:			FGI

Title:		Discussion on CHO including target MCG and candidate SCGs

Document for:	 	Discussion and decision

- Introduction

In Rel-18 WIDs, the CHO including target MCG and candidate SCGs for CPAC is one of the objectives for further enhancement NR mobility [1].

- For CHO including target MCG and target SCG in NR-DC [RAN3]: - to specify data forwarding optimizations; and - to specify, if needed, a solution to avoid unnecessary signaling exchange between source MN and target SN. - To specify CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC [RAN3, RAN2] CHO including target MCG and target SCG is used as the baseline

In RAN2#119, agreements and assumptions have been made as follows [2].

Observation: Current RAN2 Stage-3 specifications can support CHO including target MCG and target SCG in Rel-17. CHO configuration referring to or including CPC/CPA configuration (intended to be applicable together) can be supported. FFS: When triggering CHO, UE perform CPC/CPA configuration to start CPC/CPA evaluation, FFS if CHO evaluation and CPC/CPA evaluation is concurrent or sequential

In RAN2#120, the following agreement has been made [3].

Execution order: the UE doesn’t execute CPC/CPA unless CHO condition is fulfilled (regardless parallel or sequential evaluation)

In RAN2#121, the following agreements have been made [4]

RAN2 agrees to support the simultaneous evaluation of CHO and CPC in Rel-18 The UE should not need to unpack any of the nested conditional configuration containers in order to measure, acc to agreement above

In RAN2#121-bis, the following agreements have been made [5]

- For the CHO+CPC case: When both CHO and CPC conditions are met, both CHO and CPC cell change is executed. Baseline: The UE waits until both CHO and CPC conditions are met (always). (furthermore, it is assumed that if needed, the network can provide a complementary CHO-only configuration, to avoid failure in deployments where failure would otherwise be likely to happen). Alternative: FFS if when CHO condition is met, but CPC condition is not met, CHO execution is triggered (and somehow source SCG can be released). If allowed in the new configuration the UE may continue evaluation of CPC/CPAC conditions.

In RAN2#122, the following agreements have been made [6].

The CHO execution conditions (for candidate PCells) and CPA/CPC execution conditions (for candidate PSCells) are provided based on the source MeasConfig. For CHO execution conditions, the source MN determines the execution conditions on candidate PCells, based on the source MCG MeasConfig. For CPA/CPC execution conditions, the candidate MN determines the parameters of the execution conditions for candidate PSCells (e.g. event A4 threshold). The candidate MN informs the source MN about the prepared candidate PSCells and parameters of the associated execution conditions (e.g. event A4 threshold). According to the received information from the candidate MN, the source MN generates the corresponding execution conditions based on the source MCG MeasConfig to the UE. - FFS how, if to support event A3/A5. For CHO with candidate SCGs for CPA/CPC, the RRCReconfigurtaion message in one CHO container includes one MCG configuration and one SCG configuration (i.e. similar to Rel-17 CHO with SCG configuration). The execution conditions associated with one CHO container includes both CHO execution condition(s) and CPA/CPC execution condition(s), i.e. triggering conditions on both candidate PCell and candidate PSCell. If there are multiple candidate PSCells associated with one candidate PCell, the NW can provide multiple CHO configurations for the same candidate PCell, i.e. each one contains one MCG configuration (for the same candidate PCell) and one SCG configuration (for different candidate PSCell). When the CPA/CPC execution condition is met but no CHO execution condition is met, the UE continues to evaluate both CHO and CPA/CPC execution conditions. For CHO+CPC we only consider execution when BOTH conditions are met. - When the CHO execution condition is met but no CPC execution condition is met, if there is an available CHO-only or Rel-17 CHO with SCG configuration for which the CHO condition is met, the UE performs the CHO-only or Rel-17 CHO with SCG execution, and THUS the network can handle such situation by providing proper configurations.

Throughout the meetings, plenty of major issues are discussed and determined. As a result, in this contribution, we would share our views on the remaining issues and some possible enhancements for the CHO with target MCG and candidate SCG.

- Discussion

Configuration of CHO with target MCG and candidate SCGs

As reference and delta configurations are adopted in LTM and in selective activation of SCGs, it could be beneficial for the CHO with target MCG and candidate SCGs to have such features. We identified three options for the RRC signalling model. In the remaining parts of this section, the term “reference configuration” refers to a reference configuration for CHO with target MCG and candidate SCGs, and the term “delta configuration” refers to a delta configuration for CHO with target MCG and candidate SCGs.

Observation 1: Reference and delta configurations are feasible for CHO with target MCG and candidate SCGs.

The first option is that a single reference configuration is common to all the delta configurations. In this option, a single reference configuration is configured, so there is no need for the network to explicitly indicate the association between the reference configuration and the delta configurations. However, if the conditional configurations cover multiple PCells or multiple SNs, the decreased commonality could degrade the advantage of the reference configuration.

The second option is that multiple reference configurations are configured and each of them is common to delta configurations which are associated with the same PCell. In this option, multiple reference configurations are configured, and each of them is common to delta configurations which are associated with the same PCell. As a result, the network needs to provide the association between a reference configuration and its corresponding delta configurations. The association can be configured either explicitly (e.g., for each reference configuration, a list of conditional configuration IDs of the delta configurations which are associated with the reference configuration) or implicitly (e.g., for each PCell ID, a list of conditional configuration IDs of the delta configurations  which are associated with the PCell ID).

The third option is that multiple reference configurations are configured and each of them is common to delta configurations which are associated with the same candidate target SN. In this option, multiple reference configurations are configured, and each of them is common to delta configurations which are associated with the same candidate target SN. Hence, the network needs to provide the association between a reference configuration and its corresponding delta configurations. Since the UE does not have the information of the SNs, the network needs to explicitly configure the association (i.e., for each reference configuration, a list of conditional configuration IDs of the delta configurations which are associated with the reference configuration).

Among the options, we think the third option is the most suitable. From the perspective of commonality, the first option is limited to the cases where all the conditional configurations are associated with the same SN, and the second one is limited to the cases where all the conditional configurations for a PCell are associated with the same SN. To be applied in general cases, the third option is the most suitable although it requires more signalling for the associations.

Proposal 1: RAN2 to support the reference and delta configurations for CHO with target MCG and candidate SCGs.

Proposal 2: If reference and delta configurations are supported for CHO with target MCG and candidate SCGs, the reference configurations are configured on a per-SN basis.

Selection upon multiple satisfied conditional configurations

With the conditional configurations, it is possible that multiple conditional configurations associated with the same PCell are satisfied at the same time. The conditional configuration includes the CHO-only configurations, the CHO with SCG configurations, and the CHO with target MCG and candidate SCG configurations. To address such cases, a UE needs to prioritize the conditional configurations for execution.

The prioritization can be based on conditional configuration types (i.e., a type of conditional configurations is always prioritized over others), based on the measurement results (e.g., the signal quality of the cell, the holding duration for the entering condition, etc.), or based on some specific indication indicated by the network (e.g., whether the candidate target PCell and PSCell support some features). In addition, the combination of the above prioritization methods can also be considered.

In our view, a three-stage selection method is feasible and reasonable for the case. In the first stage, the specific indication by the network is considered to filter the satisfied conditional configurations. If more than one conditional configurations remain, in the second stage, the conditional configuration type is considered to filter them. If there are still more than one conditional configurations remain, in the third stage, the measurement results are considered to determine the selection. During the filtering of the conditional configurations, the criteria (e.g., which feature is prioritized, which type is prioritized, which measurement result is prioritized, etc.) can be configured by the network, or up to UE’s implementation.

Observation 2: It is possible that multiple conditional configurations associated with the same PCell are satisfied simultaneously, so the prioritization of conditional configurations is necessary.

Proposal 3: RAN2 to consider the prioritization criteria for the case that multiple conditional configurations are satisfied.

Conclusion

Based on the above, we have the following observations and proposals.

Observation 1: Reference and delta configurations are feasible for CHO with target MCG and candidate SCGs.

Observation 2: It is possible that multiple conditional configurations associated with the same PCell are satisfied simultaneously, so the prioritization of conditional configurations is necessary.

Proposal 1: RAN2 to support the reference and delta configurations for CHO with target MCG and candidate SCGs, and, if supported, the reference configurations are configured on a per-SN basis.

Proposal 2: If reference and delta configurations are supported for CHO with target MCG and candidate SCGs, the reference configurations are configured on a per-SN basis.

Proposal 3: RAN2 to consider the prioritization criteria for the case that multiple conditional configurations are satisfied.

Reference

- RP-223520 Revised WID on Further NR mobility enhancements 3GPP TSG RAN Meeting #98-e Electronic Meeting, December 12-16, 2022

- R2-2209301 Report of 3GPP TSG RAN WG2 meeting#119-e, August 17-26, 2022

- R2-2200002 Report of 3GPP TSG RAN WG2 meeting#120, November 14-18, 2022

- R2-2302401 Report of 3GPP TSG RAN WG2 meeting #121, February 27 - March 3, 2023

- R2-2306553 Report of 3GPP TSG RAN WG2 meeting #121bis-e, April 17-26, 2023

- RAN2#122 Chairman Notes, May 22-26, 2023