3GPP TSG-RAN WG2 #123	R2-2307907

Toulouse, France, August 21 – 25, 2023

Agenda Item:	7.15.6

Source:	Ericsson

Title:	Aspects of SL CA

Document for:	Discussion, Decision

# 1	Introduction

In the WID for NR_SL_enh2 RP-230077, the objective for CA is defined as follows:

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4]

- Support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, power control for simultaneous sidelink TX, packet duplication)

- The work is limited to intra-band CA for the ITS band in FR1 (Band n47).

- No specific enhancements of Rel-17 sidelink features with sidelink CA support.

- This feature is backwards compatible in the following regards

  - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI)

- Only Mode 2 operation

- Same subcarrier spacing (SCS) among CA carriers to avoid resource selection enhancements and AGC issues

  - Time resources for PSFCH are aligned among the carriers for CA

- No enhancement related to SCI transmissions on PSCCH/PSSCH, PSFCH transmission, RSRP feedback, CSI feedback and congestion control compared to Rel-16 (i.e., per-carrier operation)

  - SL resource indication remains to be per-resource pool and per-carrier basis (no cross-carrier scheduling in SCI)

  - UE transmits SL HARQ feedback on the same carrier on which it receives the associated PSSCH

- No consideration for limited transmission and reception capability

- No primary/secondary carrier differentiation

- Reuse the LTE sidelink CA design for the following aspects:

  - Sidelink carrier (re-)selection, synchronization of aggregated carriers, Tx power split for simultaneous sidelink transmissions, packet duplication

- The CA band combination work in RAN4 is limited to intra-band contiguous CA in Rel-18.

- Note: The SL CA work in Rel-18 mainly targets some V2X use cases

This contribution describes our initial view on the RAN2 scope for this objective.

# 2	Discussion

A SL UE is configured with SL CA towards a peer UE, means that the UE is configured with multiple SL carriers for its SL transmissions or receptions. So the UE can aggregate these SL carriers together for its SL transmissions or receptions. The UE would be able to perform transmissions or receptions according to at least one of the following modes

- The UE may only use one of the SL carriers to perform SL transmission or reception

- The UE may use multiple SL carriers of the configured SL carriers simultaneously to perform SL transmission or reception. A transmission or reception on a SL carrier may be fully or partially overlapping in time domain with another transmission or reception on another SL carrier.

## 2.1 Carrier management

How to map the services to frequencies were discussed in RAN2#112bis. RAN2 has made the below agreements for unicast.

Proposal 1: Based on observation that section 6.1.2.12 of TS 24.587-v18.0.0 has captured V2X layer can be provisioned with service to frequency mapping for unicast. RAN2 assume it is applicable to PC5 unicast SL CA after link has been established. RAN2 notify SA2 this assumption and ask their input on identified questions.

There is a remaining issue on whether the UE can map initial PC5 signaling to carriers.

In clause 5.1.2.1 of TS 23.287, the Destination L2 IDs for V2X communications in BC and GC over PC5 reference point is provisioned to the UE:

6)	Policy/parameters when NR PC5 is selected:

-	The mapping of V2X service types to V2X frequencies with Geographical Area(s).

-	The mapping of V2X service types to the default mode of communication (i.e. broadcast mode, groupcast mode or unicast mode).

-	The mapping of V2X service types to Destination Layer-2 ID(s) for broadcast.

-	The mapping of V2X service types to Destination Layer-2 ID(s) for groupcast mode communication.

-	The mapping of V2X service types to default Destination Layer-2 ID(s) for initial signalling to establish unicast connection.

Based on the above texts ”the mapping of V2X service types to Destination L2 ID(s)” for broadcast or groupcast is provisioned to UE, means that there may be both 1 to 1 mapping and M to 1 mapping between service types to Destination L2 ID.

The existing SA2/CT1 specs allow both 1 to 1 and M to 1 mapping between service types to Destination L2 IDs.

Since the service type is invisible in AS layer, companies have the majority view that services to carriers mapping would be achieved/implemented in AS layer via services to L2 IDs in RAN2#112bis.

Since the service type is invisible in AS layer, companies have the majority view that services to carriers mapping would be achieved/implemented in AS layer via mapping from services to L2 IDs.

Further clarification/update would be needed. The V2X layer needs to provision further information in order to let the AS layer to derive the mapping between services and frequencies. RAN2 has sent the LS (S2-2306317/R2-2304236) to SA2 asking about relevant questions. SA2 has provided answers to the RAN2 LS (S2-2306317/R2-2304236) in S2-2307794.

- Question 1: According to TS 24.588, V2X layer is only provisioned with a mapping between service identifier and initial L2 address used for unicast. But service identifier is invisible to AS-layer, and the initial L2 ID will only be used in DCR and be replaced by a self-chosen L2 ID in PC5-S link establishment procedure. Then, after L2 ID changes, whether/how can UE's AS layer obtain the mapping between L2 ID and frequencies?

- Answer 1 with question for clarification:

According to discussion in SA2, most companies' understanding is Question 1 is about how the AS layer can know frequency information associated with the PC5-S messages sent during the PC5 unicast link establishment procedure. Anyhow some companies think that Question 1 is related to the scenario after the PC5 unicast link is established. Therefore, SA2 would like to ask RAN2 about what the scenario Question 1 is related to.

SA2 also discussed how to let the AS layer know frequency information associated with the PC5-S messages sent for PC5 unicast link establishment, by providing frequency information together with the PC5-S message to the AS layer from the V2X layer, however there was no corresponding CR approved yet in SA2.

- Question 2: According to TS 24.587, PC5 unicast allows UEs to add/modify/remove V2X services/PC5 QoS flows to the same L2 ID pair. Then, given service info is invisible to AS layer, how can the UE ensure the modified V2X services to be transmitted only on the corresponding frequencies mapped in the V2X layer?

- Answer 2: Based on the following operations as described in clauses 5.4.1.1.3 and 6.3.3 of TS 23.287, it is considered that the UE can ensure the modified V2X services to be transmitted only on the corresponding frequencies mapped in the V2X layer.

i) The AS layer maintains the PC5 unicast link related context for the lifetime of the PC5 unicast link based on the various information provided by the V2X layer, e.g. PC5 Link Identifier, source L2 ID, destination L2 ID, etc.

ii) The V2X layer updates the AS layer about PC5 QoS Flow addition/modification/removal for the established PC5 unicast link. For PC5 QoS Flow addition, the V2X layer provides one or more radio frequencies associated with the added PC5 QoS Flow to the AS layer. The V2X layer can determine the radio frequencies based on the mapping of V2X service type(s) associated with the PC5 QoS Flow to V2X frequencies by using the related configuration as specified in clause 5.1.2.1 of TS 23.287 (i.e. "The mapping of V2X service types to V2X frequencies with Geographical Area(s)").

iii) The V2X layer ensures that V2X service types associated with different radio frequencies are classified into distinct PC5 QoS Flows.

Based on the SA2 response to Q1, RAN2 shall answer to SA2 that the Question 1 is related to the scenario after the PC5 unicast link is established

RAN2 answers to to SA2 that the Question 1 in RAN2 LS (S2-2306317/R2-2304236) is related to the scenario after the PC5 unicast link is established.

In addition, the answers to Q1 can be inferred based on the SA2 answers to Q2.

For PC5 QoS Flow addition, the V2X layer provides one or more radio frequencies associated with the added PC5 QoS Flow to the AS layer. The V2X layer can determine the radio frequencies based on the mapping of V2X service type(s) associated with the PC5 QoS Flow to V2X frequencies by using the related configuration as specified in clause 5.1.2.1 of TS 23.287 (i.e. "The mapping of V2X service types to V2X frequencies with Geographical Area(s)").

Since the V2X layer will map services (associated with different carriers/frequencies) to different QoS flows wherein each QoS flow is associated with a QoS profile

SL-QoS-Profile-r16 ::=        SEQUENCE {

sl-PQI-r16                    SL-PQI-r16                                                  OPTIONAL,   -- Need R

sl-GFBR-r16                   INTEGER (0..4000000000)                                     OPTIONAL,   -- Need R

sl-MFBR-r16                   INTEGER (0..4000000000)                                     OPTIONAL,   -- Need R

sl-Range-r16                  INTEGER (1..1000)                                           OPTIONAL,   -- Need R

...

}

It would be feasible for the V2X layer to provide the AS layer with flow QoS profile information (e.g., PQI) associated with each frequencies, based on which, the AS layer can derive the mapping between services and frequencies. This solution is applicable to all cast types.

In addition, this solution is also aligned with SA2 understanding/preferred solution based on the SA2’s answers to Q2. Therefore, in order to speed up discussion progress in RAN2 given there are few meetings left for RAN2 to address the issue, RAN2 shall agree with this solution without waiting for further response from SA2. RAN2 can modify the agreement in the future meeting if SA2 gives further response.

Without waiting for further response from SA2, RAN2 to agree that the V2X layer provides the AS layer with the mapping between flow QoS profiles (e.g., PQIs) and frequencies based on that the AS layer can derive the mapping between services and frequencies.

In the response, SA2 also discussed how to let the AS layer know frequency information associated with the PC5-S messages sent for PC5 unicast link establishment, by providing frequency information together with the PC5-S message to the AS layer from the V2X layer.

We think the above solution is fine to all PC5-S signalling and discovery message.

For any PC5-S signalling or discovery message provided by the V2X layer, the V2X layer also provides frequency information to the AS layer.

For PC5-RRC signaling, it would be beneficial to define a mapping between PC5-RRC signaling and frequencies, since PC5-RRC signaling is generated by the AS layer. Therefore, we make the below proposal

Introduce mapping between PC5-RRC signalling and frequencies.

## 2.2 Backward compatibility

The compatibility issue has been discussed in RAN2#112bis. RAN2 made the following agreement

Agreement:

Proposal 3:	For groupcast/broadcast, as in LTE SL CA, the carrier(s) that can be used for transmitting data are configured by V2X layer for the L2 destination. FFS on backwards compatibility issue.

For unicast, Rel-16/17 UE can communicate with ReL-18 UE via exchange of UE capabilities to address the issue.

During RAN2#122, to address the backward compatibility issue for groupcast/broadcast, two options were discussed

Option 1: based on services to carriers mapping to address the issue

Option 2: a TX profile is introduced to define the mapping between service types and/or L2 IDs and indicators on whether SL CA is supported or not.

In our view, it is beneficial to adopt Option 2 due to below reasons

- TX profile based option has been adopted to address backward compatibility issue due to introduction of SL DRX in Rel-17, it is reasonable to adopt the same option to address backward compatibility issue due to SL CA.

- With option 1, for a service that needs to reach legacy V2X UEs (i.e., V2X UE in Rel-16 and Rel-17), a Rel-18 TX UE has to always use the single carrier (i.e., the carrier used by legacy UEs) to transmit the data of the service, and not able to exploit other carriers to increase communication range for the service (e.g., enabling PDCP duplication). This would limit the QoS performance of the Rel-18 UE.

For SL unicast, Rel-16/Rel-17 UEs can communicate with Rel-18 UE via exchange of UE capabilities to determine whether UE supports SL CA or not.

For SL groupcast and broadcast, TX profiles are defined to show the mapping between service types or L2 IDs and indicators of whether SL CA is supported or not.

## 2.3 Potential SL unicast specific enhancement

### 2.3.1 CSI report

The Sidelink CSI Reporting MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.4-1. The priority of the Sidelink CSI Reporting MAC CE is fixed to '1'. The Sidelink CSI Reporting MAC CE is defined as follows (Figure 6.1.3.35-1):

-	RI: This field indicates the derived value of the Rank Indicator for sidelink CSI reporting as specified in clause 8.5 of TS 38.214 [7]. The length of the field is 1 bit;

-	CQI: This field indicates the derived value of the Channel Quality Indicator for sidelink CSI reporting as specified in clause 8.5 of TS 38.214 [7]. The length of the field is 4 bit;

-	R: Reserved bit, set to 0.

SL CSI measurement and reporting is per SL carrier.

For CA, at least for CSI reporting, it is feasible for UE to report CSI for multiple carriers in the same MAC CE. In this way, a CSI reporting MAC CE is not needed to be limited to only transmit on the carrier where the MAC CE is generated. For SL unicast

Define a new CSI report MAC CE to comprise CSI measurement results across multiple carriers.

A CSI report MAC CE can be transmitted on any carrier. It is up to UE implementation to choose which carrier to transmit the MAC CE if there are grants available on multiple carriers.

### 2.3.2 PDCP duplication

### RAN2 has made the below agreements

Agreement on criterion for packet duplication

1:	SLRB configures whether PDCP duplication is used or not

Agreement on PDCP duplication/SL CA for SL SRB

1:	Working assumption: SL CA/PDCP duplication is applied to PC5-RRC after SL link is established. FFS on exact time when it can be started.

it is reasonable to confirm the below WA

Working assumption: SL CA/PDCP duplication is applied to PC5-RRC after SL link is established. FFS on exact time when it can be started.

Conform the WA that SL CA/PDCP duplication is applied to PC5-RRC after SL link is established.

For both DRB and SRB, a remaining issue is whether to support duplication activation/deactivation MAC CE.

Given limited time in this release, it is sufficient to support RRC signalling to activate or deactivate PDCP duplication.

For both DRBs and SRBs, PDCP duplication activation and deactivation is only indicated via PC5-RRC signalling.

In addition, for TX UE in RRC CONNECTED, PDCP duplication activation and deactivation shall be configured by the gNB.

For TX UE in RRC CONNECTED, PDCP duplication activation and deactivation shall be configured by the gNB via RRC signalling.

For the FFS regarding SRB PDCP duplication, i.e., FFS on exact time when it can be started

The TX UE can start PDCP duplication for SRB after UE capability has been exchanged with the peer UE.

TX UE indicates PDCP duplication activation/start to the peer UE via PC5-RRC after TX UE has exchanged UE capabilities with the peer UE.

### 2.3.3 RLF handling

For SL RLF in SL CA, RAN2 has made the below agreements

Agreement on DTX based SL RLF in SL CA

1:	The counting is calculated per carrier.

2:	Legacy SL RLF is not declared when the counting is reached to sl-MaxnumConsecutiveDTX) for carrier(s) and the UE has other available SL carrier(s) for SL CA.

The following issues need to be further discussed.

Issue 1: whether sl-maxNumConsecutiveDTX needs to be configured per carrier?

Issue 2: how does the UE handle a carrier on which the number of consecutive DTX on PSFCH reception occasions has reached sl-maxNumConsecutiveDTX?

For issue 1, it is not necessary to configure sl-maxNumConsecutiveDTX per carrier in this release, given all configured SL carriers belong to the same band i.e., ITS band, meaning that radio channel conditions are similar among carriers in typical cases. Carriers may have different congestion status in terms of CBR, however, CBR measurements can be considered in the carrier selection/reselection procedure.

sl-MaxnumConsecutiveDTX is configured per PC5-RRC connection and common to all carriers.

Since RAN2 has agreed that the counting is calculated per carrier, this means that the UE variable numConsecutiveDTX is maintained per carrier.

The UE variable numConsecutiveDTX is maintained per carrier.

For issue 2, for a carrier on which the number of consecutive DTX on PSFCH reception occasions has reached sl-maxNumConsecutiveDTX, the UE shall perform the below actions

- Inform the peer UE of the failure event

- Inform the gNB of the failure event if the UE is in RRC CONNECTED

- Not consider the carrier as a candidate in TX carrier (re)selection procedure

Therefore, we make the below proposal.

UE takes the below actions to handle a carrier on which the number of consecutive DTX on PSFCH reception occasions has reached sl-maxNumConsecutiveDTX

  - Inform the peer UE of the failure event

  - Inform the gNB of the failure event if the UE is in RRC CONNECTED

  - Not consider the carrier as a candidate in TX carrier (re)selection procedure

### 2.3.3 DRX

SL DRX is supported for SL in NR Rel-17. For a UE performing CA operation, it may be necessary to study if the existing SL DRX mechanism needs to be enhanced for CA operation, e.g., introduce a secondary DRX configuration as in Uu DRX.

Therefore, we make the below proposal:

A secondary SL DRX configuration is introduced for SL CA.

## 2.6 Synchronization of aggregated carriers

A UE capable of V2X sidelink communication and SLSS (sidelink synchronization signal)/PSBCH transmission shall transmit sidelink synchronization information (i.e. SLSS and sidelink MIB) on the frequency used for V2X sidelink communication when the following conditions are met:

- Condition1:

- List of candidate synchronisation carrier frequencies (syncFreqList) are not (pre)configured

Or

- The list (syncFreqList) is (pre)configured, but it does not include any frequency selected by the UE for its sidelink transmission, or the concerned frequency is selected as the synchronisation carrier frequency

Or

- The list (syncFreqList) is (pre)configured and includes the concerned frequency, and the UE has selected a frequency other than the concerned frequency as the synchronisation carrier frequency, and the UE is able to transmit SLSS/PSBCH on the concerned frequency

- Condition2:

- The UE is in RRC connected, and configured by the NW that synchronization information shall be transmitted.

Or

- The UE is in coverage, and the measured Uu RSRP is lower than a configured threshold.

Or

- The UE is out of coverage, and selects GNSS as the synchronization reference source.

Or

- The UE is out of coverage and not directly synchronized to GNSS, and has no selected synchronization reference (SyncRef) UE or the measured SL-RSRP to the selected SyncRef UE is below a preconfigured threshold.

In the case of limited transmission capabilities on multiple carrier frequencies, when the UE is configured with syncFreqList, whether to transmit SLSS/PSBCH on a frequency selected for V2X sidelink transmission which is other than the synchronisation carrier frequency, is up to UE implementation.

There are several issues the LTE mechanism in order to directly reuse for SL CA in NR:

- It could happen that the UE transmits to multiple destinations over SL, while different SL carriers are configured/selected for transmission towards different destinations, and syncFreqList only includes carriers configured/selected for transmission towards some of the destinations. In this case sidelink synchronization information will not be transmitted in the configured/selected carriers for transmission towards the other  destinations which are not overlapped with carriers included in syncFreqList, and the Rx UEs only associated to the other destinations may not be able to receive the transmissions due to no sidelink synchronization information from the Tx UE.

- When syncFreqList is not (pre)configured or it is (pre)configured but does not include any frequency selected by the UE for its sidelink transmission, the UE has to transmit sidelink synchronization information on all the frequencies used for sidelink transmission which is not efficient and consumes more power.

- NR supports unicast where UE can exchange signaling over SL, it is not optimal to apply the LTE mechanism designed only for broadcast

Study how to support synchronization of aggregated carriers in CA.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	The existing SA2/CT1 specs allow both 1 to 1 and M to 1 mapping between service types to Destination L2 IDs.

Observation 2	Since the service type is invisible in AS layer, companies have the majority view that services to carriers mapping would be achieved/implemented in AS layer via mapping from services to L2 IDs.

Observation 3	SL CSI measurement and reporting is per SL carrier.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 answers to to SA2 that the Question 1 in RAN2 LS (S2-2306317/R2-2304236) is related to the scenario after the PC5 unicast link is established.

Proposal 2	Without waiting for further response from SA2, RAN2 to agree that the V2X layer provides the AS layer with the mapping between flow QoS profiles (e.g., PQIs) and frequencies based on that the AS layer can derive the mapping between services and frequencies.

Proposal 3	For any PC5-S signalling or discovery message provided by the V2X layer, the V2X layer also provides frequency information to the AS layer.

Proposal 4	Introduce mapping between PC5-RRC signalling and frequencies.

Proposal 5	For SL unicast, Rel-16/Rel-17 UEs can communicate with Rel-18 UE via exchange of UE capabilities to determine whether UE supports SL CA or not.

Proposal 6	For SL groupcast and broadcast, TX profiles are defined to show the mapping between service types or L2 IDs and indicators of whether SL CA is supported or not.

Proposal 7	Define a new CSI report MAC CE to comprise CSI measurement results across multiple carriers.

Proposal 8	A CSI report MAC CE can be transmitted on any carrier. It is up to UE implementation to choose which carrier to transmit the MAC CE if there are grants available on multiple carriers.

Proposal 9	Conform the WA that SL CA/PDCP duplication is applied to PC5-RRC after SL link is established.

Proposal 10	For both DRBs and SRBs, PDCP duplication activation and deactivation is only indicated via PC5-RRC signalling.

Proposal 11	For TX UE in RRC CONNECTED, PDCP duplication activation and deactivation shall be configured by the gNB via RRC signalling.

Proposal 12	TX UE indicates PDCP duplication activation/start to the peer UE via PC5-RRC after TX UE has exchanged UE capabilities with the peer UE.

Proposal 13	sl-MaxnumConsecutiveDTX is configured per PC5-RRC connection and common to all carriers.

Proposal 14	The UE variable numConsecutiveDTX is maintained per carrier.

Proposal 15	UE takes the below actions to handle a carrier on which the number of consecutive DTX on PSFCH reception occasions has reached sl-maxNumConsecutiveDTX

a.	Inform the peer UE of the failure event

b.	Inform the gNB of the failure event if the UE is in RRC CONNECTED

c.	Not consider the carrier as a candidate in TX carrier (re)selection procedure

Proposal 16	A secondary SL DRX configuration is introduced for SL CA.

Proposal 17	Study how to support synchronization of aggregated carriers in CA.

# 4 References

- RP-230077, WID revision: NR sidelink evolution, RAN #98-e, December 12-16, 2022.