3GPP TSG-RAN WG2 Meeting #123	 R2-2307920

# Touluse, France, 21th-25th Aug, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NR sidelink relay enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_enh-Core |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Sidelink UE-to-UE relay, UE-to-Network service continuity enhancement and multi-path relay are introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Sidelink relay for UE-to-UE, service continuity, and multi-path operation are not supported in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 16.12, 16.12.1, 16.12.2.x, 16.12.3, 16.12.4, 16.12.x, 16.12.6.1, 16.12.6.2, 16.12.6.x, 16.x, 16.x.1, 16.x.2, 16.x.3, 16.x.3.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR TS38.331, TS38.321, TS38.351 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

3	Abbreviations and Definitions

3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AGC	Automatic Gain Control

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCCH	Broadcast Control Channel

BCH	Broadcast Channel

BFD	Beam Failure Detection

BH	Backhaul

BL	Bandwidth reduced Low complexity

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CAG	Closed Access Group

CAPC	Channel Access Priority Class

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFR	Common Frequency Resource

CFRA	Contention Free Random Access

CG	Configured Grant

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CLI	Cross Link interference

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

CP	Cyclic Prefix

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

DAG	Directed Acyclic Graph

DAPS	Dual Active Protocol Stack

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DL-AoD	Downlink Angle-of-Departure

DL-SCH	Downlink Shared Channel

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell-ID (positioning method)

EHC	Ethernet Header Compression

ePWS	enhancements of Public Warning System

ETWS	Earthquake and Tsunami Warning System

FS	Feature Set

FSA ID	Frequency Selection Area Identity

G-CS-RNTI	Group Configured Scheduling RNTI

G-RNTI	Group RNTI

GFBR	Guaranteed Flow Bit Rate

GIN	Group ID for Network selection

GNSS	Global Navigation Satellite System

GSO	Geosynchronous Orbit

H-SFN	Hyper System Frame Number

HAPS	High Altitude Platform Station

HRNN	Human-Readable Network Name

IAB	Integrated Access and Backhaul

IFRI	Intra Frequency Reselection Indication

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

L2	Layer-2

L3	Layer-3

LDPC	Low Density Parity Check

LEO	Low Earth Orbit

MBS	Multicast/Broadcast Services

MCE	Measurement Collection Entity

MCCH	MBS Control Channel

MDBV	Maximum Data Burst Volume

MEO	Medium Earth Orbit

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MP	Multi-Path

MPE	Maximum Permissible Exposure

MRB	MBS Radio Bearer

MT	Mobile Termination

MTCH	MBS Traffic Channel

MTSI	Multimedia Telephony Service for IMS

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

MUSIM	Multi-Universal Subscriber Identity Module

NB-IoT	Narrow Band Internet of Things

NCD-SSB	Non Cell Defining SSB

NCGI	NR Cell Global Identifier

NCL	Neighbour Cell List

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NGSO	Non-Geosynchronous Orbit

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

NSAG	Network Slice AS Group

NTN	Non-Terrestrial Network

P-MPR	Power Management Maximum Power Reduction

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDC	Propagation Delay Compensation

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PEI	Paging Early Indication

PH	Paging Hyperframe

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated NPN

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PRS	Positioning Reference Signal

PS-RNTI	Power Saving RNTI

PSS	Primary Synchronisation Signal

PTM	Point to Multipoint

PTP	Point to Point

PTW	Paging Time Window

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QMC	QoE Measurement Collection

QoE	Quality of Experience

QPSK	Quadrature Phase Shift Keying

RA	Random Access

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RIM	Remote Interference Management

RLM	Radio Link Monitoring

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

RTT	Round Trip Time

SCS	SubCarrier Spacing

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SDT	Small Data Transmission

SD-RSRP	Sidelink Discovery RSRP

SFI-RNTI	Slot Format Indication RNTI

SHR	Successful Handover Report

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SL-RSRP	Sidelink RSRP

SMC	Security Mode Command

SMF	Session Management Function

SMTC	SS/PBCH block Measurement Timing Configuration

S-NSSAI	Single Network Slice Selection Assistance Information

SNPN	Stand-alone Non-Public Network

SNPN ID	Stand-alone Non-Public Network Identity

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRAP	Sidelink Relay Adaptation Protocol

SRS	Sounding Reference Signal

SRVCC	Single Radio Voice Call Continuity

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SSSG	Search Space Set Group

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TB	Transport Block

TCE	Trace Collection Entity

TNL	Transport Network Layer

TPC	Transmit Power Control

TRP	Transmit/Receive Point

TRS	Tracking Reference Signal

U2N	UE-to-Network

U2U	UE-to-UE

UCI	Uplink Control Information

UDC	Uplink Data Compression

UE-Slice-MBR	UE Slice Maximum Bit Rate

UL-AoA	Uplink Angles of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

VR	Virtual Reality

V2X	Vehicle-to-Everything

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

BH RLC channel: an RLC channel between two nodes, which is used to transport backhaul packets.

Boundary IAB-node: as defined in TS 38.401 [4].

Broadcast MRB: A radio bearer configured for MBS broadcast delivery.

CAG Cell: a PLMN cell broadcasting at least one Closed Access Group identity.

CAG Member Cell: for a UE, a CAG cell broadcasting the identity of the selected PLMN, registered PLMN or equivalent PLMN, and for that PLMN, a CAG identifier belonging to the Allowed CAG list of the UE for that PLMN.

CAG-only cell: a CAG cell that is only available for normal service for CAG UEs.

Cell-Defining SSB: an SSB with an RMSI associated.

Child node: IAB-DU's and IAB-donor-DU's next hop neighbour node; the child node is also an IAB-node.

Conditional Handover (CHO): a handover procedure that is executed only when execution condition(s) are met.

CORESET#0: the control resource set for at least SIB1 scheduling, can be configured either via MIB or via dedicated RRC signalling.

DAPS Handover: a handover procedure that maintains the source gNB connection after reception of RRC message for handover and until releasing the source cell after successful random access to the target gNB.

Direct Path: a type of UE-to-Network transmission path, where data is transmitted between a UE and the network without sidelink relaying.

Downstream: direction toward child node or UE in IAB-topology.

Early Data Forwarding: data forwarding that is initiated before the UE executes the handover.

Earth-centered, earth-fixed: a global geodetic reference system for the Earth intended for practical applications of mapping, charting, geopositioning and navigation, as specified in NIMA TR 8350.2 [51].

Feeder link: wireless link between the NTN Gateway and the NTN payload.

Geosynchronous Orbit: earth-centered orbit at approximately 35786 kilometres above Earth's surface and synchronised with Earth's rotation. A geostationary orbit is a non-inclined geosynchronous orbit, i.e. in the Earth's equator plane.

Group ID for Network Selection: an identifier used during SNPN selection to enhance the likelihood of selecting a preferred SNPN that supports a Default Credentials Server or a Credentials Holder, as specified in TS 23.501 [3].

gNB: node providing NR user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

High Altitude Platform Station: airborne vehicle embarking the NTN payload placed at an altitude between 8 and 50 km.

IAB-donor: gNB that provides network access to UEs via a network of backhaul and access links.

IAB-donor-CU: as defined in TS 38.401 [4].

IAB-donor-DU: as defined in TS 38.401 [4].

IAB-DU: gNB-DU functionality supported by the IAB-node to terminate the NR access interface to UEs and next-hop IAB-nodes, and to terminate the F1 protocol to the gNB-CU functionality, as defined in TS 38.401 [4], on the IAB-donor.

IAB-MT: IAB-node function that terminates the Uu interface to the parent node using the procedures and behaviours specified for UEs unless stated otherwise. IAB-MT function used in 38-series of 3GPP Specifications corresponds to IAB-UE function defined in TS 23.501 [3].

IAB-node: RAN node that supports NR access links to UEs and NR backhaul links to parent nodes and child nodes. The IAB-node does not support backhauling via LTE.

IAB topology: the unison of all IAB-nodes and IAB-donor-DUs whose F1 and/or RRC connections are terminated at the same IAB-donor-CU.

Indirect Path: a type of UE-to-Network transmission path, where data is forwarded via a U2N Relay UE between a U2N Remote UE and the network.

Inter-donor partial migration: migration of an IAB-MT to a parent node underneath a different IAB-donor-CU while the collocated IAB-DU and its descendant IAB-node(s), if any, are terminated at the initial IAB-donor-CU. The procedure renders the said IAB-node as a boundary IAB-node.

Intra-system Handover: handover that does not involve a CN change (EPC or 5GC).

Inter-system Handover: handover that involves a CN change (EPC or 5GC).

Late Data Forwarding: data forwarding that is initiated after the source NG-RAN node knows that the UE has successfully accessed a target NG-RAN node.

Mapped Cell ID: in NTN, it corresponds to a fixed geographical area.

MBS Radio Bearer: A radio bearer configured for MBS delivery.

MSG1: preamble transmission of the random access procedure for 4-step random access (RA) type.

MSG3: first scheduled transmission of the random access procedure.

MSGA: preamble and payload transmissions of the random access procedure for 2-step RA type.

MSGB: response to MSGA in the 2-step random access procedure. MSGB may consist of response(s) for contention resolution, fallback indication(s), and backoff indication.

Multicast/Broadcast Service: A point-to-multipoint service as defined in TS 23.247 [45].

Multicast MRB: A radio bearer configured for MBS multicast delivery.

Multi-hop backhauling: using a chain of NR backhaul links between an IAB-node and an IAB-donor.

ng-eNB: node providing E-UTRA user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

NG-C: control plane interface between NG-RAN and 5GC.

NG-U: user plane interface between NG-RAN and 5GC.

NG-RAN node: either a gNB or an ng-eNB.

Non-CAG Cell: a PLMN cell which does not broadcast any Closed Access Group identity.

Non-Geosynchronous orbit: earth-centered orbit with an orbital period that does not match Earth's rotation on its axis. This includes Low and Medium Earth Orbit (LEO and MEO). LEO operates at altitudes between 300 km and 1500 km and MEO at altitudes between 7000 km and 25000 km, approximately.

Non-terrestrial network: an NG-RAN consisting of gNBs, which provide non-terrestrial NR access to UEs by means of an NTN payload embarked on an airborne or space-borne NTN vehicle and an NTN Gateway.

NR backhaul link: NR link used for backhauling between an IAB-node and an IAB-donor, and between IAB-nodes in case of a multi-hop backhauling.

NR sidelink communication: AS functionality enabling at least V2X communication as defined in TS 23.287 [40] and the ProSe communication (including ProSe non-Relay and UE-to-Network Relay communication) as defined in TS 23.304 [48], between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink discovery: AS functionality enabling ProSe non-Relay Discovery and ProSe UE-to-Network Relay discovery for Proximity based Services as defined in TS 23.304 [48] between two or more nearby UEs, using NR technology but not traversing any network node.

NTN Gateway: an earth station located at the surface of the earth, providing connectivity to the NTN payload using the feeder link. An NTN Gateway is a TNL node.

NTN payload: a network node, embarked on board a satellite or high altitude platform station, providing connectivity functions, between the service link and the feeder link. In the current version of this specification, the NTN payload is a TNL node.

Numerology: corresponds to one subcarrier spacing in the frequency domain. By scaling a reference subcarrier spacing by an integer N, different numerologies can be defined.

Parent node: IAB-MT's next hop neighbour node; the parent node can be IAB-node or IAB-donor-DU

PC5 Relay RLC channel: an RLC channel between L2 U2N Remote UE and L2 U2N Relay UE, or between L2 U2U Remote UE and L2 U2U Relay UE, which is used to transport packets over PC5 for L2 UE-to-Network/UE-to-UE Relay.

PC5 Relay RLC channel: an RLC channel between L2 U2N Remote UE and L2 U2N Relay UE, which is used to transport packets over PC5 for L2 UE-to-Network Relay.

PLMN Cell: a cell of the PLMN.

RedCap UE: a UE with reduced capabilities as specified in clause 4.2.21.1 in TS 38.306 [11].

Relay discovery: AS functionality enabling 5G ProSe UE-to-Network Relay Discovery as defined in TS 23.304 [48], using NR technology but not traversing any network node.

Satellite: a space-borne vehicle orbiting the Earth embarking the NTN payload.

Service link: wireless link between the NTN payload and UE.

Sidelink Discovery RSRP: RSRP measurements on PC5 link related to NR sidelink discovery.

Sidelink RSRP: RSRP measurements on PC5 link related to NR sidelink communication.

SNPN Access Mode: mode of operation whereby a UE only accesses SNPNs.

SNPN-only cell: a cell that is only available for normal service for SNPN subscribers.

SNPN Identity: the identity of Stand-alone NPN defined by the pair (PLMN ID, NID).

Transmit/Receive Point: part of the gNB transmitting and receiving radio signals to/from UE according to physical layer properties and parameters inherent to that element.

U2N Relay UE: a UE that provides functionality to support connectivity to the network for U2N Remote UE(s).

U2N Remote UE: a UE that communicates with the network via a U2N Relay UE. If the UE is configured with multi-path relay, the UE can also communicate with the network.

U2U Relay UE: a UE that provides functionality to support connectivity between U2U Remote UE(s).

U2U Remote UE: a UE that communicates with other UE via a U2U Relay UE.

MP Relay UE: a UE that provides functionality to support connectivity to the network for MP Remote UE(s).

MP Remote UE: a UE that communicates with the network via a MP Relay UE. If the UE is configured with multi-path relay, the UE can also communicate with the network.

Upstream: direction toward parent node in IAB-topology.

Uu Relay RLC channel: an RLC channel between L2 U2N Relay UE and gNB, which is used to transport packets over Uu for L2 UE-to-Network Relay.

V2X sidelink communication: AS functionality enabling V2X communication as defined in TS 23.285 [41], between nearby UEs, using E-UTRA technology but not traversing any network node.

Xn: network interface between NG-RAN nodes.

Next change

16.12	Sidelink Relay

16.12.1	General

Sidelink relay is introduced to support 5G ProSe UE-to-Network Relay (U2N Relay) function (specified in TS 23.304 [48]) to provide connectivity to the network for U2N Remote UE(s). Both L2 and L3 U2N Relay architectures are supported. The L3 U2N Relay architecture is transparent to the serving NG-RAN of the U2N Relay UE, except for controlling sidelink resources. The detailed architecture and procedures for L3 U2N Relay can be found in TS 23.304 [48].

A U2N Relay UE shall be in RRC_CONNECTED to perform relaying of unicast data.

For L2 U2N Relay operation, the following RRC state combinations are supported:

-	Both L2 U2N Relay UE and L2 U2N Remote UE shall be in RRC_CONNECTED to perform transmission/reception of relayed unicast data; and

-	The L2 U2N Relay UE can be in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED as long as all the L2 U2N Remote UE(s) that are connected to the L2 U2N Relay UE are either in RRC_INACTIVE or in RRC_IDLE.

A single unicast link is established between one L2 U2N Relay UE and one L2 U2N Remote UE. The traffic to the NG-RAN of L2 U2N Remote UE via a given L2 U2N Relay UE and the traffic of the L2 U2N Relay UE shall be separated in different Uu RLC channels.

For L2 U2N Relay, the L2 U2N Remote UE can only be configured to use resource allocation mode 2 (as specified in 5.7.2 and 16.9.3.1) for data to be relayed.

A sidelink relay UE is to support UE-to-UE relay (U2U Relay) function as specified in TS 23.304 to provide coverage extension of the sidelink transmissions between two sidelink UEs. For the coverage extension, the U2U Remote UE can communicate with the peer U2U remote UE which could not have been reached within the sidelink coverage.

The U2U Relay UE and U2U Remote UE can be in any RRC state. The U2U Relay UE and the U2U Remote UEs can be in the coverage of different cells. For U2U Relay, NR PC5 sidelink is supported between U2U Relay UE and U2U Remote UEs. Only unicast is supported between U2U Relay UE and U2U Remote UEs.

Next change

16.12.2.x	L2 UE-to-UE Relay

For L2 UE-to-UE Relay architecture, the protocol stacks are other than the fact that the termination points are two Remote UEs. The protocol stacks for the user plane and the control plane of the L2 U2N Relay architecture are illustrated in Figure 16.12.2.x-1 and Figure 16.12.2.x-2. The SRAP sublayer is placed above the RLC sublayer for both CP and UP at both PC5 interfaces. The sidelink SDAP, PDCP, and RRC are terminated between two L2 U2U Remote UEs, while SRAP, RLC, MAC, and PHY are terminated in each PC5 link wherein each link is between L2 U2U Remote UE and the L2 U2U Relay UE.

Figure 16.12.2.x-1: User plane protocol stack for L2 UE-to-UE Relay

Figure 16.12.2.x-2: Control plane protocol stack for L2 UE-to-UE Relay

For L2 UE-to-UE Relay:

-	The SRAP sublayer existing in L2 U2U Remote UE and L2 U2U Relay UE performs bearer mapping to map end-to-end SL bearer (SRB, DRB) of L2 U2U Remote UE into PC5 U2U Relay RLC Channel. The PC5 SRAP sublayer existing in L2 U2U Relay UE and L2 U2U Remote UE supports L2 U2U Remote UE identification for the sidelink traffic from the L2 U2U Remote UE to the peer L2 U2U Remote UE. . [FFS how to identify the traffic at SRAP layer]

-  The identification of L2 U2U Remote UE end-to-end bearer and a Remote UE ID included in the SRAP header support:

-	to correlate the received packets with the RLC Channel of the L2 U2U Relay UE;

-  to correlate the transmit packets for the PDCP entity associated with the end-to-end radio bearer and the peer L2 U2U Remote UE ID;

-  to correlate the received packets for the PDCP entity associated with the end-to-end radio bearer and the L2 U2U Remote UE ID

-  The PC5 SRAP sublayer at L2 U2U Remote UE supports identification of the peer L2 U2U Remote UE. An ID being able to be mapped to the peer L2 U2U Remote UE is included in the SRAP header of the traffic from the L2 U2U Remote UE to the L2 U2U Relay. The PC5 SRAP sublayer at L2 U2U Relay UE supports identification of the L2 U2U Remote UEs. An ID being able to be mapped to the L2 U2U Remote UE is included in the SRAP header of the traffic from L2 U2U Relay UE to the peer L2 U2U Remote UE.

Editor’s Notes: FFS on the details related to whether to use L2 ID/shorten ID or whether to include both or one ID in the SRAP header.

-  The SRAP sublayer at L2 U2U Relay UE can perform data multiplexing from multiple RLC channels of the different L2 U2U Remote UE to the same RLC channel of the peer L2 U2U Remote UE for the different end-to-end bearers. The SRAP sublayer at L2 U2U Relay UE can perform data demultiplexing from the same RLC channel of the L2 U2U Remote UE to the multiple RLC channel of the different peer L2 U2U Remote UE for the different end-to-end bearers.

16.12.3	Relay Discovery

Model A and Model B discovery models as defined in TS 23.304 [48] are supported for U2N Relay discovery. The protocol stack used for discovery is illustrated in Figure 16.12.3-1.

Figure 16.12.3.1: Protocol Stack of Discovery Message for UE-to-Network/UE-to-UE Relay

The U2N Remote UE can perform Relay discovery message (i.e., as specified in TS 23.304 [48]) transmission and may monitor the sidelink for Relay discovery message while in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED. The network may broadcast or configure via dedicated RRC signalling a Uu RSRP threshold, which is used by the U2N Remote UE to determine if it can transmit Relay discovery messages to U2N Relay UE(s).

The U2N Relay UE can perform Relay discovery message (i.e., as specified in TS 23.304 [48]) transmission and may monitor the sidelink for Relay discovery message while in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED. The network may broadcast or configure via dedicated RRC signalling a maximum Uu RSRP threshold, a minimum Uu RSRP threshold, or both, which are used by the U2N Relay UE to determine if it can transmit Relay discovery messages to U2N Remote UE(s).

The U2U Remote UE and U2U Relay UE can perform Relay discovery message transmission and may monitor for Relay discovery message while in RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED. The network will not broadcast or configure a Uu RSRP threshold, which is used by the U2U Remote UE or the U2U Relay UE to determine if it can transmit Relay discovery messages to the other U2U Remote UE or U2U Relay UE.

The network may provide the Relay discovery configuration using broadcast or dedicated signalling. In addition, the U2N Remote UE and L3 U2N Relay UE may use pre-configuration for Relay discovery.

The resource pool(s) used for NR sidelink communication can be used for Relay discovery or the network may configure resource pool(s) dedicated for Relay discovery. Resource pool(s) dedicated for Relay discovery can be configured simultaneously with resource pool(s) for NR sidelink communication in system information, dedicated signalling and/or pre-configuration. Whether dedicated resource pool(s) for Relay discovery are configured is based on network implementation. If resource pool(s) dedicated for Relay discovery are configured, only those resource pool(s) dedicated for Relay discovery shall be used for Relay discovery. If only resource pool(s) for NR sidelink communication are configured, all the configured resource pool(s) can be used for Relay discovery and NR sidelink communication.

For U2N Remote UE (including both in-coverage and out of coverage cases) that has been connected to the network via a U2N Relay UE, only resource allocation mode 2 is used for Relay discovery message transmission.

For in-coverage U2N Relay UE, and for both in-coverage and out of coverage U2N Remote UEs, NR sidelink resource allocation principles are applied for Relay discovery message transmission.

For both in-coverage and out-of-coverage U2U Remote UEs, or U2U Relay UE, NR sidelink resource allocation principles, both mode-1 and mode-2, are applied for Relay discovery message transmission.

The sidelink power control for the transmission of Relay discovery messages is same as for NR sidelink communication.

No ciphering or integrity protection in PDCP layer is applied for the Relay discovery messages.

The U2N/U2U Remote UE and U2N/U2U Relay UE can determine from SIB12 whether the gNB supports Relay discovery, or Non-Relay discovery, or both.

Editor’s Notes: The detailed description for forwarding discovery messages based on PC5 RSRP threshold depending on the discovery model (model A/B, integrated-discovery) will be added after further discussion.

16.12.4	Relay Selection/Reselection

The U2N Remote UE performs radio measurements at PC5 interface and uses them for U2N Relay selection and reselection along with higher layer criteria, as specified in TS 23.304 [48]. When there is no unicast PC5 connection between the U2N Relay UE and the U2N Remote UE, the U2N Remote UE uses SD-RSRP measurements to evaluate whether PC5 link quality towards a U2N Relay UE satisfies relay selection criterion.

For relay reselection, U2N Remote UE uses SL-RSRP measurements towards the serving U2N Relay UE for relay reselection trigger evaluation when there is data transmission from U2N Relay UE to U2N Remote UE, and it is left to UE implementation whether to use SL-RSRP or SD-RSRP for relay reselection trigger evaluation in case of no data transmission from U2N Relay UE to U2N Remote UE.

A U2N Relay UE is considered suitable by a U2N Remote UE in terms of radio criteria if the PC5 link quality measured by U2N Remote UE towards the U2N Relay UE exceeds configured threshold (pre-configured or provided by gNB). The U2N Remote UE searches for suitable U2N Relay UE candidates that meet all AS layer and higher layer criteria (see TS 23.304 [48]). If there are multiple such suitable U2N Relay UEs, it is up to U2N Remote UE implementation to choose one U2N Relay UE among them. For L2 U2N Relay (re)selection, the PLMN ID and cell ID can be used as additional AS criteria.

The U2N Remote UE triggers U2N Relay selection in following cases:

-	Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold;

-	Indicated by upper layer of the U2N Remote UE.

The U2N Remote UE may trigger U2N Relay reselection in following cases:

-	PC5 signal strength of current U2N Relay UE is below a (pre)configured signal strength threshold;

-	Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by U2N Relay UE via PC5-RRC signalling;

-	When U2N Remote UE receives a PC5-S link release message from U2N Relay UE;

-	When U2N Remote UE detects PC5 RLF;

-	Indicated by upper layer.

For L2 U2N Remote UEs in RRC_IDLE or RRC_INACTIVE and L3 U2N Remote UEs, the cell (re)selection procedure and relay (re)selection procedure run independently. If both suitable cells and suitable U2N Relay UEs are available, it is up to the U2N Remote UE implementation to select either a cell or a U2N Relay UE. A L3 U2N Remote UE may select a cell and a L3 U2N Relay UE simultaneously and this is up to implementation of L3 U2N Remote UE.

For both L2 and L3 U2N Relay UEs in RRC_IDLE or RRC_INACTIVE, the PC5-RRC message(s) are used to inform their connected U2N Remote UE(s) when U2N Relay UEs select a new cell. The PC5-RRC message(s) are also used to inform their connected L2 or L3 U2N Remote UE(s) when L2 or L3 U2N Relay UE performs handover, detects Uu RLF, or its Uu RRC connection establishment/resume fails. Upon reception of the PC5 RRC message for notification, it is up to U2N Remote UE implementation whether to release or keep the unicast PC5 link. If U2N Remote UE decides to release the unicast PC5 link, it triggers the PC5 release procedure and may perform cell or relay reselection.

The U2U Remote UE performs radio measurements(i.e., SD-RSRP and SL-RSRP) at PC5 interface and uses them for U2U Relay selection and reselection along with higher layer criteria, as specified in TS 23.304 [48].

For relay selection, U2U Remote UE uses SL-RSRP measurements towards the peer U2U Remote UE for relay selection trigger evaluation when there is data transmission from U2U Remote UE to the peer U2U Remote UE. For the relay reselection, U2U Remote UE uses SL-RSRP measurement towards the U2U Relay UE for relay reselection trigger evaluation when there is data transmission from U2U Remote UE to U2U relay UE. It is left to U2U Remote UE implementation whether to use SL-RSRP or SD-RSRP for relay selection or reselection trigger evaluation in case of no data transmission from the U2U Remote UE to the peer U2U Remote UE or U2U Relay UE. The thresholds for SD-RSRP and SL-RSRP are configured separately for the trigger of U2U relay selection or reselection.

A U2U Relay UE is considered suitable by a U2U Remote UE in terms of radio criteria if the PC5 link quality measured by the U2U Remote UE towards the U2U Relay UE exceeds a configured threshold.

The U2U Remote UE triggers U2U Relay selection in the following cases:

- The PC5 signal strength between U2U Remote UEs is below a (pre)configured signal strength threshold;

- Indicated by the upper layer of the U2U Remote UE.

The U2U Remote UE triggers U2U Relay reselection in the following cases:

- The PC5 signal strength of the current U2U Relay UE is below a (pre)configured signal strength threshold;

- When U2U Remote UE detects PC5 RLF;

- When U2U Remote UE receives the PC5 RLF indication from the U2U Relay UE;

- When U2U Remote UE receives a PC5-S link release message from U2U Relay UE;

- Indicated by the upper layer of the U2U Remote UE.

The relay selection/reselection can be triggered by each U2U Remote UE based on the current hop quality.

For the discovery model A, the U2U Relay UE should only announce the neighbour U2U Remote UE(s) for which the SD-RSRP/SL-RSRP between the U2U Relay and the neighbour U2U Remote UE is above a configured threshold in a discovery announce message. Upon discovery message reception, U2U Remote UE considers a U2U Relay UE as a candidate U2U Relay UE if the SD-RSRP towards the U2U Relay UE is above a configured threshold.

For the discovery model B, upon discovery response message reception, the U2U remote UE considers a U2U Relay UE as a candidate U2U Relay UE if the SD-RSRP towards the U2U Relay UE is above a configured threshold.

For the integrated-discovery, when receiveing DCR message from one or multiple U2U Relay UEs, the U2U Remote UE should consider candidate U2U Relay UEs towards which the SL-RSRP is above a configured threshold to respond and that satisfy upper-layer criteria, and select a U2U Relay UE from among them.

Next change

16.12.x	Control plane procedures for L2 U2U Relay

Editor’s Notes: The detail description will be added after further discussion.

Next change

16.12.6	Service Continuity for L2 U2N relay

16.12.6.0	General

The service continuity procedure is applicable only for the mobility cases of path switch from indirect to direct path, and from direct to indirect path when the L2 U2N Remote UE and L2 U2N Relay UE belong to the same gNB or different gNB. This procedure is also applicable for the mobility cases of path switch from indirect to indirect path when the two L2 U2N Relay UEs belong to the same gNB or different gNBs. For inter-gNB path switching, the source gNB decides to trigger path switching and the path switch type, i.e. direct or indirect path.

16.12.6.1	Switching from indirect to direct path

For service continuity of L2 U2N Relay, the following procedure is used, in case of L2 U2N Remote UE switching to from indirect to direct path under the same gNB:

Figure 16.12.6.1-1: Procedure for L2 U2N Remote UE switching intra-gNB indirect to direct path switching -

1.	The Uu measurement configuration and measurement report signalling procedures are performed to evaluate both relay link measurement and Uu link measurement. The measurement results from L2 U2N Remote UE are reported when configured measurement reporting criteria are met. The sidelink relay measurement report shall include at least L2 U2N Relay UE's source L2 ID, serving cell ID (i.e., NCGI/NCI), and sidelink measurement quantity result. The sidelink measurement quantity can be SL-RSRP of the serving L2 U2N Relay UE, and if SL-RSRP is not available, SD-RSRP is used.

2.	The gNB decides to switch the L2 U2N Remote UE onto direct Uu path.

3.	The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The L2 U2N Remote UE stops User Plane and Control Plane transmission via the L2 U2N Relay UE after reception of the RRCReconfiguration message with the path switch configuration.

4.	The L2 U2N Remote UE synchronizes with the gNB and performs Random Access.

5.	The UE (i.e., L2 U2N Remote UE in previous steps) sends the RRCReconfigurationComplete message to the gNB via the direct path, using the configuration provided in the RRCReconfiguration message. From this step, the UE (i.e., L2 U2N Remote UE in previous steps) uses the RRC connection via the direct path to the gNB.

6.	The gNB sends the RRCReconfiguration message to the L2 U2N Relay UE to reconfigure the connection between the L2 U2N Relay UE and the gNB. The RRCReconfiguration message to the L2 U2N Relay UE can be sent any time after step 3 based on gNB implementation (e.g., to release Uu Relay RLC Channel and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

7.	Either L2 U2N Relay UE or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation.

8.	The data path is switched from indirect path to direct path between the UE (i.e., previous L2 U2N Remote UE) and the gNB. The PDCP re-establishment or PDCP data recovery in uplink is performed by the UE (i.e., previous L2 U2N Remote UE) for lossless delivery during path switch if gNB configures it.

NOTE:	Step 8 can be executed any time after step 4. Step 8 is independent of step 6 and step 7.

For service continuity of L2 U2N Relay, the following procedure is used, in case of L2 U2N Remote UE switching from indirect path to direct path under another gNB:

Editor’s Notes: The detail will be added based on the RAN3 input.

16.12.6.2	Switching from direct to indirect path

The gNB can select a L2 U2N Relay UE in any RRC state i.e., RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED, as a target L2 U2N Relay UE for direct to indirect path switch.

For service continuity of L2 U2N Remote UE, the following procedure is used, in case of the L2 U2N Remote UE switching from direct to indirect path under the same gNB via a L2 U2N Relay UE in RRC_CONNECTED:

Figure 16.12.6.2-1: Procedure for L2 U2N Remote UE intra-gNB switching from direct to indirect path via a L2 U2N Relay UE in RRC_CONNECTED

1.	The L2 U2N Remote UE reports one or multiple candidate L2 U2N Relay UE(s) and Uu measurements, after it measures/discovers the candidate L2 U2N Relay UE(s):

-	The L2 U2N Remote UE filters the appropriate L2 U2N Relay UE(s) according to relay selection criteria before reporting. The L2 U2N Remote UE shall report only the L2 U2N Relay UE candidate(s) that fulfil the higher layer criteria;

-	The reporting includes at least a L2 U2N Relay UE ID, a L2 U2N Relay UE' s serving cell ID, and a sidelink measurement quantity information. SD-RSRP is used as sidelink measurement quantity.

2.	The gNB decides to switch the L2 U2N Remote UE to a target L2 U2N Relay UE. Then the gNB sends an RRCReconfiguration message to the target L2 U2N Relay UE, which includes at least the L2 U2N Remote UE's local ID and L2 ID, Uu Relay RLC channel and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration.

3.	The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the L2 U2N Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end Uu radio bearer(s). The L2 U2N Remote UE stops User Plane and Control Plane transmission over the direct path after reception of the RRCReconfiguration message from the gNB.

4.	The L2 U2N Remote UE establishes PC5 RRC connection with target L2 U2N Relay UE.

5.	The L2 U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the L2 U2N Relay UE.

6.	The data path is switched from direct path to indirect path between the L2 U2N Remote UE and the gNB.

In case the selected L2 U2N Relay UE for direct to indirect path switch is in RRC_IDLE or RRC_INACTIVE, after receiving the path switch command, the L2 U2N Remote UE establishes a PC5 link with the L2 U2N Relay UE and sends the RRCReconfigurationComplete message via the L2 U2N Relay UE, which triggers the L2 U2N Relay UE to enter RRC_CONNECTED state. The procedure for L2 U2N Remote UE switching to indirect path in Figure 16.12.6.2-1 can be also applied for the case that the selected L2 U2N Relay UE for direct to indirect path switch is in RRC_IDLE or RRC_INACTIVE with the exception that the RRCReconfiguration message is sent from the gNB to the L2 U2N Relay UE after the L2 U2N Relay UE enters RRC_CONNECTED state, which happens during step 5.

For service continuity of L2 U2N Remote UE, the following procedure is used, in case of the L2 U2N Remote UE switching from direct path to indirect path under another gNB via a L2 U2N Relay UE in RRC_CONNECTED:

Editor’s Notes: The detail will be added based on the RAN3 input.

16.12.6.x	Switching from indirect to indirect path

The gNB can select an L2 U2N Relay UE in any RRC state i.e., RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED, as a target L2 U2N Relay UE for indirect to indirect path switch.

For service continuity of L2 U2N Remote UE, the following procedure is used, in case of the L2 U2N Remote UE switching from indirect path via L2 U2N Relay UE to indirect path via a target L2 U2N Relay UE in RRC_CONNECTED:

Figure 16.12.6.x-1: Procedure for L2 U2N Remote UE intra-gNB switching from indirect to indirect path

- The L2 U2N Remote UE reports one or multiple candidate L2 U2N Relay UE(s) and the measurement between L2 U2N Remote UE and the L2 U2N relay UE to evaluate the sidelnk with candidate L2 U2N Relay UE(s) and the sidelink with L2 U2N Relay UE. The detailed reporting components can be referred to the cases for switching from direct to indirect path (see clause 16.12.6.2). The L2 U2N Remote UE can provide information to the gNB according to the measurement configuration on which the gNB can decide to direct or indirect path switching.

2.	The gNB decides to switch the L2 U2N Remote UE to a target L2 U2N Relay UE.

3.	The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the target L2 U2N Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic, and the associated end-to-end radio bearer(s). The L2 U2N Remote UE stops UP and CP transmission over the indirect path after the reception of the RRCReconfiguration message from the gNB.

4.	The gNB sends an RRCReconfiguration message to the target L2 U2N Relay UE, which includes at least the L2 U2N Remote UE's local ID and L2 ID, Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration.

5.	The L2 U2N Remote UE establishes PC5 RRC connection with the target L2 U2N Relay UE.

6.	The L2 U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the target L2 U2N Relay UE.

7.	The gNB sends the RRCReconfiguration message to the L2 U2N Relay UE to reconfigure the connection between the L2 U2N Relay UE and the gNB. The RRCReconfiguration message to the L2 U2N Relay UE can be sent any time after step 3 based on gNB implementation (e.g., to release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

8.	Either L2 U2N Relay UE or L2 U2N Remote UE's AS layer can release the PC5-RRC connection and indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation.

In case the selected L2 U2N Relay UE for indirect to indirect path switch is in RRC_IDLE or RRC_INACTIVE, after receiving the path switch command, the L2 U2N Remote UE establishes a PC5 link with the target L2 U2N Relay UE and sends the RRCReconfigurationComplete message via the target L2 U2N Relay UE, which triggers the target L2 U2N Relay UE to enter RRC_CONNECTED state. The procedure for L2 U2N Remote UE switching for indirect to indirect path in Figure 16.12.6.x-1 can be also applied for the case that the selected target L2 U2N Relay UE for indirect to indirect path switch is in RRC_IDLE or RRC_INACTIVE with the exception that the RRCReconfiguration message is sent from the gNB to the target L2 U2N Relay UE after the target L2 U2N Relay UE enters RRC_CONNECTED state, which happens between step 5 and step 6.

For service continuity of L2 U2N Remote UE between inter-gNBs, the following procedure is used, in case of the L2 U2N Remote UE, which is connected to indirect path, switching to inter-gNB indirect path via a target L2 U2N Relay UE in RRC_CONNECTED:

Editor’s Notes: The detail will be added based on the RAN3 input.

Next change

16.X	Multi-path Relay

16.X.1	General

Multi-path relay operation is introduced to support the connectivity of one direct path and one indirect path for MP Remote UE. Multi-path relay operation can enhance user data throughput and reliability compared to the single path. It can provide efficient path switching between direct path and indirect path or vice versa. The gNB can offload the direct connection of the MP Remote UE in congestion to the indirect connection via the MP Relay UE. MP Relay UE and MP Remote UE can be at different intra/inter-frequency cells.

The direct path and indirect path are connected to the same gNB. Both L2 and L3 MP Relay architectures are supported. The L3 MP Relay architecture is transparent to the serving gNB of the MP Relay UE, except for controlling sidelink resources.

The indirect path for supporting multi-path relay has either of the following links between MP Relay UE and MP Remote UE.

- 	The indirect path is connected via MP Relay UE using sidelink.

-  The indirect path is connected via MP Relay UE using non-3GPP link. The relation between MP Remote UE and MP Relay UE is pre-configured or static, and how the relation is pre-configured or static is out of 3GPP specification.

Multi-path relay is applicable when MP Remote UE is in RRC_CONNECTED state. The PCell is only on the direct path.

Multi-path relay is supported in the following cell deployment scenarios:

-  The MP Relay UE and MP Remote UE are served by the same cell.

-  The MP Relay UE and MP Remote UE are served by different intra-frequency cells of the same gNB.

-  The MP Relay UE and MP Remote UE are served by different inter-frequency cells of the same gNB.

Multi-path relay is supported in the following sidelink scenarios:

-  Sidelink TX/RX and Uu link share the same carrier at the MP Remote UE.

-  Sidelink TX/RX and Uu link use different carriers at the MP Remote UE.

-  Sidelink TX/RX and Uu link share the same carrier at the MP Relay UE.

-  Sidelink TX/RX and Uu link use different carriers at the MP Relay UE.

16.X.2	Protocol Architecture

For the multi-path relay operation using sidelink, the protocol stacks for the user plane and control plane of L2 MP Relay architecture are illustrated in Figure 16.x.2-1 and Figure 16.x.2-2. L2 MP Relay UE is supported by a direct bearer which is mapped to the direct path on the Uu link, an indirect bearer which is mapped to the indirect path via L2 MP Relay UE, and an MP split bearer which is mapped to both paths. The split bearer framework is used the same as specified in TS 37.340 [XX].

In the multi-path relay using sidelink, the SRAP sublayer is placed above the RLC sublayer for both CP and UP at gNB, MP Relay UE, and MP Remote UE in the indirect path. The Uu SDAP, PDCP, and RRC are terminated at gNB and MP Remote UE, while SRAP, RLC, MAC, and PHY are terminated in each hop (i.e., the link between L2 MP Remote UE and L2 MP Relay UE and L2 MP Relay UE and gNB) in the indirect path. And Uu RLC, MAC, and PHY are terminated at L2 MP Remote UE and gNB (i.e., the link between L2 MP Remote UE and gNB) in the direct path. For example, for the UL traffic, a PDCP entity in the L2 MP Remote UE delivers the UL traffic to a Uu RLC entity and a PC5 RLC entity having an SRAP entity over it. For the DL traffic, a PDCP entity in the L2 MP Remote UE receives the DL traffic from a Uu RLC entity and a PC5 RLC entity having an SRAP entity over it. If the split bearer is configured and the PDCP DRB duplication is activated on the PDCP entity, the duplicated PDCP PDU is delivered via both direct path and indirect path.

Figure 16.x.2-1: User plane protocol stack for L2 Multi-path Relay using sidelink

Figure 16.x.2-2: Control plane protocol stack for L2 Multi-path Relay using sidelink

For the multi-path relay usgin non-3GPP link, the protocol stacks for the user plane and control plane of L2 MP Relay architecture are illustrated in Figure 16.x.2-3 and Figure 16.x.2-4. The L2 MP Relay UE and the L2 MP Remote UE are connected via a non-3GPP link.

In the multi-path relay using non-3GPP link, the SRAP sublayer is placed above the Uu RLC sublayer for both CP and UP at gNB and L2 MP Relay UE to support the indirect path. The Uu SDAP, PDCP, and RRC are terminated at gNB and L2 MP Remote UE, while, the Uu SRAP is terminated between L2 MP Relay UE and gNB, and the Uu RLC, MAC, and PHY are terminated in both between L2 MP Remote UE and gNB and between L2 MP Relay UE and gNB. Without the SRAP entity between L2 MP Remote UE and L2 MP Relay UE, a PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support for more than one RB over the Uu link by configuring 1:1 bearer mapping. The Uu RLC channels for the PDU delivery of the L2 MP Relay UE’s local traffic and relay traffic are configured differently. Bearer identification except LCID is not needed in L2 PDU over the Uu link. Only 1:1 bearer mapping is supported over the Uu link for the indirect path. If the split bearer is configured and the PDCP DRB duplication is activated on the PDCP entity, the duplicated PDCP PDU is delivered via both direct path and indirect path.

Figure 16.x.2-3: User plane protocol stack for L2 Multi-path Relay using non-3GPP link

Figure 16.x.2-4: Control plane protocol stack for L2 Multi-path Relay using non-3GPP link

16.X.3	Control plane procedure for multi-path relaying

#### 16.x.3.x RRC Connection Management

The L2 MP Remote UE needs to establish both a direct path and an indirect path. The L2 MP Remote UE using sidelink adds the indirect path on the only direct path under the same gNB. And also, the L2 MP Remote UE using sidelink adds the direct path on the only indirect path under the same gNB.

Meanwhile, the L2 MP Remote UE using non-3GPP link adds the indirect path on the only direct path under the same gNB. But it is not allowed that the L2 MP Remote UE using non-3GPP link adds the direct path on the indirect path. The MP Relay UE using non-3GPP is restricted to serve only one MP Remote UE. And the non-split SRB1/2 is allowed to be configured only on the direct path.

For L2 MP Remote UE, the primary path of split SRB1 and SRB2 is always configured on direct path. In the L2 MP Remote UE using non-3GPP link, non-split SRB1/2 is allowed to be configured only on direct path.

Editor’s Notes: FFS whether non-split SRB1/2 is allowed to be configured only on direct path in the L2 MP Remote UE using sidelink.

Figure 16.x.3.x-1 describes the procedures for the indirect path addition on the direct path for the L2 MP relaying.

Figure 16.x.3.x-1: Procedure for indirect path addition on top of direct path

1. 	L2 MP Remote UE performs measurement reporting based on the measurement configuration.

If the L2 MP Remote UE is connected with L2 MP Relay UE using sidelink, the L2 MP Remote UE reports at least the list of the candidate L2 MP Relay UE ID and the cell IDs of the candidate L2 MP Relay UEs.

Otherwise, the MP Remote UE is connected with L2 MP Relay UE using non-3GPP link, the L2 MP Remote UE reports at least C-RNTI and the cell ID of the serving L2 MP Relay UE.

2.	The gNB decides to add the indirect path for the L2 MP Remote UE. The cell serving the direct path and the cell serving the L2 MP Relay UE on the indirect path belong to the same gNB but can be same or different.

3.	The gNB sends an RRCReconfiguration message to the L2 MP Relay UE to configure the indirect path of the L2 MP Remote UE, if the L2 MP Relay UE is in RRC_CONNECTED.

4.	The gNB sends the RRCReconfiguration message to the L2 MP Remote UE. The RRCReconfiguration message may include at least the target L2 MP Relay UE ID, indirect path addition configuration, RLC channel configuration, bearer mapping and the associated end-to-end radio bearer(s).

5.	If the L2 MP Remote UE using sidelink has no PC5 unicast link with the target L2 MP Relay UE, the L2 MP Remote UE establishes a PC5 unicast link with the target L2 MP Relay UE. If the L2 MP Remote UE is using non-3GPP link, this procedure can be omitted.

6a.	The remote UE sends the RRCReconfigurationComplete message to the gNB at least via the direct path in order to complete the indirect path addition procedure.

6b. If a split SRB1 with duplication is configured, the remote UE also sends the RRCReconfigurationComplete message to the gNB via the indirect path served by the relay UE.

In the case the selected L2 MP Relay UE for indirect path addition is in RRC_IDLE or RRC_INACTIVE, after receiving the path addition command, the L2 MP Remote UE has to trigger the L2 MP Relay UE in RRC_IDLE or RRC_INACTIVE to be in RRC_CONNECTED. If the non-split SRB1 is configured only on the direct path, the L2 MP Remote UE sends the RRCReconfigurationComplete message only on the direct path. The L2 MP Remote UE using sidelink sends PC5-RRC message to the L2 MP Relay UE. The L2 MP Relay UE initiates a RRC connection establishment upon receiving the PC5-RRC message. Otherwise, if the L2 MP Remote UE is using non-3GPP link, it is an implementation of L2 MP Remote UE how the L2 MP Remote UE triggers the L2 MP Relay UE to initiate RRC connection establishment procedure. If the split SIB1 with duplication is configured, the L2 MP Relay UE receiving RRCReconfigurationComplete message from the L2 MP Remote UE initiates a RRC connection establishment for sending the RRCReoncfigurationComplete message to the gNB.

Figure 16.x.3.x-2 describes the procedures for the direct path addition on the indirect path for the L2 MP relay operation. This procedure can apply only the L2 MP Remote UE using sidelink.

Figure 16.x.3.x-2: Procedure for direct path addition on top of indirect path

1. 	The L2 MP Remote UE and the L2 MP Relay UE perform measurements based on measurement configuration.

2.	The gNB decides to add the direct path for the L2 MP Remote UE. The cell serving the direct path and the cell serving the L2 MP Relay UE on the indirect path belong to the same gNB but can be same or different.

3.	The gNB sends an RRCReconfiguration message to the L2 MP Relay UE to update the indirect path configuration, if necessary.

4.	The gNB sends the RRCReconfiguration message to the L2 MP Remote UE via the L2 MP Relay UE. The contents in the RRCReconfiguration message includes at least a target cell, direct path addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s).

5.	The L2 MP Remote UE synchronizes to DL of the target cell serving the direct path and performs random access procedure towards the cell serving the direct path. The L2 MP Remote UE configures the target cell as PCell.

6a.	The L2 MP Remote UE sends the RRCReconfigurationComplete message to the gNB at least via the direct path in order to complete the direct path addition procedure.

6b. If a split SRB1 with duplication is configured, the L2 MP Remote UE also sends the RRCReconfigurationComplete message to the gNB via the indirect path served by the L2 MP Relay UE.

Editor’s Notes: The detail about direct/indirect path change will be added after further discussion in RAN2

#### 16.x.3.x Radio Link Failure Report

The L2 MP Remote UE in RRC_CONNECTED performs Uu RLM (as described in clause 9.2.7).

When the L2 MP Remote UE detects Uu Radio Link Failure (RLF) on the direct path, the L2 MP Remote UE is triggered path failure reporting through the indirect path via a RRC message if split SRB1 is configured and the indirect path is not suspended. Otherwise, RRC connection re-establishment is initiated. While, the L2 MP Remote UE detects PC5 Radio Link Failure (RLF) on the indirect path, the L2 MP Remote UE is triggered path failure reporting through the direct path via a RRC message if split SRB1 or SRB1 is configured and the direct path is not suspended. Upon detecting both Uu and PC5 RLF, the L2 MP Remote UE triggers RRC connection re-establishment. The PC5 RLF detection includes either PC5 link failure or notification of Uu link failure on the indirect path.

#### 16.x.3.x RRC Connection Re-establishment

When L2 MP Remote UE using sidelink is configured with multi-path and initiates the RRC connection re-establishment procedure, the L2 MP Remote UE does not perform RRC connection re-establishment directly into a multi-path configuration.

Editor’s Notes: FFS how to handle in scenario 2. The current agreement is about scenario 1

#### 16.x.3.x RRC Connection Resume

Even though the L2 MP Remote UE is storing indirect path configuration, resuming directly into multi-path configuration is not supported.

#### 16.x.3.x System Information

The L2 MP Remote UE can acquire any necessary SIB(s) over Uu interface or indirect path. If common search space for system information is configured with the active BWP on PCell, the L2 MP Remote UE can perform direct system information acquisition on PCell. The L2 MP Remote UE can also receive system information at lease PBCH/MIB on the direct path and directly acquires SFN from MIB on the direct path. Besides, the L2 MP Remote UE can receive the system information via dedicated RRC signalling via SRB1, if SRB1 is configured on direct and/or indirect path as specified in TS 38.331 [12].

#### 16.x.3.x Paging

The RRC state of the L2 MP Remote UE is RRC_CONECTED for MP Relay operation. If CSS for Paging is configured within the active BWP on the direct path on PCell, the L2 MP Remote UE in RRC_CONNECTED monitors paging on PCell for the updated system information or ETWS/CMAS indication as specified in TS 38.331 [12]. The gNB can also provide updated system information or warning message(s) to the MP Remote UE on SRB1 as specified in TS 38.331 [12].

End of change