3GPP TSG-RAN WG2 Meeting #123	R2-2307932

Touluse, France, Aug, 2023

Agenda Item	: 7.9.2

Source	: LG Electronics Inc.

Title	: Control plane procedure for U2U relay

Document for	: Discussion and Decision

1.	Introduction

The work item on NR sidelink relay enhancements was approved for Rel-18 and the recent WID [1] is shown in RP-221262 including the following objectives:

- Specify mechanisms to support single-hop Layer-2 and Layer-3 UE-to-UE relay (i.e., source UE -> relay UE -> destination UE) for unicast [RAN2, RAN3, RAN4]. - Common part for Layer-2 and Layer-3 relay to be prioritized until RAN#98 - Relay discovery and (re)selection [RAN2, RAN4] - Signalling support for Relay and remote UE authorization if SA2 concludes it is needed [RAN3] - Layer-2 relay specific part - UE-to-UE relay adaptation layer design [RAN2] - Control plane procedures [RAN2] - QoS handling if needed, subject to SA2 progress [RAN2] Note 1A: This work should take into account the forward compatibility for supporting more than one hop in a later release. Note 1B: A remote UE is connected to only a single relay UE at a given time for a given destination UE.

In this contribution, we will share our view about the control plane procedure and measurement configuration for the U2U relay.

2.	Discussion

2.1 Discovery message forwarding in relay UE

Depending on the discovery model (e.g., discovery model A/B, integrated-discovery model), the following agreement was concluded in the RAN2 meeting. The agreements are considering the measurement value on the side of remote UE. However, we think the measurement value on the side of relay UE needs to be considered for deciding whether the discovery message should be responded to or forwarded.

RN2 #121b agreements:

For Model-B discovery, after receiving a discovery message from a relay UE, a target remote UE transmits the discovery response message only if the PC5 RSRP between the target remote UE and the relay UE is above a configured threshold.

For the integrated-discovery case, the relay UE forwards the discovery message for DCR message with integrated Discovery case only if the PC5 RSRP between the relay UE and the source remote UE is above a threshold.

RN2 #122 agreements:

For Model A discovery, the relay UE should only announce the neighbour UEs for which the SD-RSRP/SL-RSRP between the relay UE and the neighbour UE is above a configured threshold in a discovery announcement message. LS is sent to SA2.

For Model A discovery, upon discovery message reception, remote UE considers a relay UE as a candidate relay UE if the SD-RSRP towards the relay UE is above a configured threshold.

For Model B discovery, upon discovery response messages reception, the source remote UE considers a relay UE as a candidate relay UE if the SD-RSRP towards the relay UE is above a configured threshold.

For integrated-discovery, when receiving DCR message from one or multiple relay UEs, the target remote UE should consider candidate relay UEs towards which the SL-RSRP is above a configured threshold to respond and that satisfy upper-layer criteria, and select a relay UE from among them.

For non-integrated U2U relay discovery model B, when relay (re)selection is triggered at the remote UE, the discovery transmission may be triggered at the same time to search for candidate relay UEs.

According to the discovery model B in SA2 spec (TR 23.700-33) for the U2U relay, the source remote UE sends a solicitation message to the candidate relay UEs. After receiving the solicitation message, each of the candidate relay UEs sends the response message to the source remote UE if the candidate relay UEs discovered the target remote UE. The candidate relay UEs should send the response message only if the SD-RSRP toward the target remote UE is above a configured threshold and the SD-RSRP toward the source remote UE is above a configured threshold.

As the same principle, for the integrated-discovery model, the relay UE forwards the discovery message for DCA message with integrated Discovery case only if the PC5-RCRP between the relay UE and the target remote UE is above a threshold.

<TR 23.700-33> Figure 6.33.2.1.2-1: UE-to-UE Relay discovery with Model B 1. The UE-to-UE Relay may have discovered other UEs in proximity via the previous direct discovery or direct communication procedures. The UE-to-UE Relay obtains the User Info ID, Relay Service Code (RSC) and Layer-2 ID of other UEs in proximity. 2. The Source UE sends a Solicitation message. The Solicitation message may include the Type of Discovery Message, User Info ID of Source UE, RSC, and User Info ID of Target UE. The Source Layer-2 ID of the Announcement message is self-assigned by the Source UE, and the Destination Layer-2 ID is selected based on the ProSe policy. 3. The UE-to-UE Relay may perform direct discovery procedure to discover the Target UE if it has not been discovered in step 1. 4. The UE-to-UE Relay that match the values of the User Info ID of Target UE and RSC contained in the Solicitation message respond to the Source UE with a Response message. The Response message contains the Type of Discovery Message, User Info ID of UE-to-UE Relay, RSC, and User Info ID and Layer-2 ID of Target UE. The Source Layer-2 ID of the Response message is self-assigned by the UE-to-UE Relay, and the Destination Layer-2 ID is set to the Source Layer-2 ID of the received Solicitation message.

Proposal 1: For model B discovery, Relay UE transmits discovery a response message when the following both conditions are satisfied.

- When the SD-RSRP toward the source remote UE is above a configured threshold.

- When the SD-RSRP between relay UE and the target remote UE is above a configured threshold.

Proposal 2: As the same principle, for the integrated-discovery model, the relay UE forwards the discovery message for DCA message with integrated Discovery case only if the PC5-RCRP between the relay UE and the target remote UE is above a threshold.

2.2 Control plane procedure for U2U relay

2.2.1 U2U relay connection procedure

For U2U relay operation, the connection between source remote UE and relay UE and between relay UE and target remote UE is needed. On the side of the source remote UE, the connection completion means the SL connection to the target remote UE. So, we propose the following connection establishment procedure in Figure 1. Source remote UE sends an RRCReconfigurationSidelink message to the relay UE. This message contains SL RLC channel configuration for the 1st-hop, end-to-end bearer configuration for the target remote UE, and the mapping configuration between end-to-end bearer and SL RLC channel in the 1st-hop. The end-to-end bearer configuration is not applied on the relay UE side. The configuration is for the target remote UE.

When the relay UE receives the RRCReconfigurationSielink message from the source remote UE, the relay UE is triggered to send RRCReconfigurationSidelink message to the target remote UE. For the triggering of sending RRCReconfigurationSidelink message in relay UE, the initial RRCReconfigurationSidelink message from the source remote UE may include the source remote UE ID and the target remote UE ID. The source remote UE ID may configure a short ID which identifies each of the source remote UE and the target remote UE to the relay UE. The contents of RRCReconfigurationSdielink message from relay UE to target remote UE are SL RLC channel configuration for the 2nd-hop, end-to-end bearer received from the source remote UE, and the mapping configuration between end-to-end bearer and SL RLC channel in the 2nd-hop.

The target remote UE receiving the RRCReconfigurationSidelink message from the relay UE sends RRCReconfigurationCompleteSidelink message to the relay UE if the configuration is acceptable. And when the relay UE receives the RRCReconfigurationCompleteSidelink message from the target remote UE, the relay UE is triggered sending the RRCReconfigurationCompleteSidelink message to the source remote UE.

The T400 timer for using the SL connection establishment may be applied a different value compared to the legacy T400 timer or a different name of the timer. The source remote UE has to wait a longer time to receive the RRCReconfigurationCompleteSidelink message after sending the RRCReconfigurationSidelink message. So, the T400 timer used in source remote UE may be a different timer value or a different name of the timer of the T400 timer used in relay UE.

Figure 1 connection establishment for U2U relay operation

Proposal 3: The following SL connection procedure is needed sequentially:

-  Source remote UE send RRCReconfigurationSidelink message to the relay UE.

- The relay UE receiving RRCReconfigurationSidelink from the source remote UE is triggered sending RRCReconfigurationSideliknk message to the target remote UE.

- The target remote UE receiving RRCReconfigurationSidelink from the relay UE can send RRCReconfigurationCompleteSidelink message to the relay UE, if the configuration included in the RRCReconfigurationSidelink is applicable.

- The relay UE receiving RRCReconfigurationCompleteSidelink from the target remote UE is triggered sending RRCReconfigurationCompleteSidelink to the source remote UE.

Proposal 4: The RRCReconfigurationSidelink message from source remote UE to relay UE includes at least the following configurations:

- SL RLC channel configuration for the 1st-hop (e.g., between source remote UE and relay UE).

- End-to-end bearer configuration from source remote UE and target remote UE.

- The mapping configuration between SL RLC channel in the 1st-hop and end-to-end bearer.

Proposal 5: The RRCReconfigurationSidelink message from relay UE to target remote UE includes at least the following configurations:

- SL RLC channel configuration for the 2nd-hop (e.g., between relay UE and target remote UE).

- End-to-end bearer configuration from source remote UE and target remote UE. This configuration is forwarded from the source remote UE.

- The mapping configuration between SL RLC channel in the 2nd-hop and end-to-end bearer.

Proposal 6: The mapping configuration between ingress RLC channel ID and egress RLC channel ID is configured by relay UE itself based on the end-to-end bearer.

Proposal 7: The initial RRCReconfigurationSidelink message from source remote UE may include a single short ID configuration that identifies source remote UE and target remote UE.

Proposal 8: The T400-like timer which has a longer timer value than the legacy T400 timer may be required for the SL connection between source remote UE and target remote UE.

2.2.2 Whether to apply short ID (/local ID)

In the last RAN2 meeting, the following agreement was concluded based on discussion.

RN2 #122 agreements:

For the possible use of a short ID in U2U relay, RAN2 will downselect between the following options for identifying the source and destination remote UEs at the SRAP layer:

a)	Single ID, identifying the source and destination remote UEs

b)	Source ID and Destination ID

For the possible use of a short ID in U2U relay, the U2U relay UE performs the ID assignment. FFS if this ID should be assigned hop-by-hop or globally.

These agreements do not imply agreement to use a short ID.

There are still open issues such as which ID (e.g., full ID or short ID) will be applied for the L2 U2U relaying, or which identification methods will be used if short ID is applied.

If using the full L2 ID of both source remote UE (i.e., SRC L2 ID) and target remote UE (i.e., DST L2 ID) are applied for U2U relay operation, the SRAP header has overhead. Delivering every packet from the source remote UE to the target remote UE via relay UE, 48 bits full ID (i.e., each L2 ID is 24 bits) should be included in the SRAP header. It’s not an efficient way to compose the SRAP header. Also, if the full ID is used, it doesn’t seem to have any advantage compared to the layer-3 U2U relay operation. So, we need to think about applying the short ID in the SRAP header.

The reason for using full ID or short ID in the SRAP header is for identifying the packet in the target remote UE. When the target remote UE receives the packet originating from the source remote UE, the target remote UE can identify it by using the full ID or short ID. If SL connection is established between remote UE and relay UE or between relay UE and other relay UE to reach the target remote UE, each relay UE already knows that the receiving packet has to be delivered for which next relay UE or which target remote UE. When the SL connection is established from the source remote UE to the target remote UE, the ingress RLC channel and egress RLC channel mapping configuration is already accomplished for each relay UE based on the end-to-end bearer.

So, even if the single short ID is duplicated in the relay UE, there is no problem. Because the SRC/DST L2 ID in the MAC layer is different if the packet is delivered by a different previous relay UE or source remote UE. On the relay UE side, even if the single short ID is the same value, the packet can be identified based on the L2 ID from the MAC layer.  If the relay UE stores the SRAP configuration based on the L2 ID from the MAC layer, the relay UE can identify when receiving a packet with the same single short ID in the SRAP header.

Regardless of using a hop-by-hop short ID or global short ID, the U2U relay can be operated well. A single short ID is not used to route the packet in relay UE after the SL connection is established from the source remote UE and target remote UE. Someone has a concern that using global ID can make a problem when RLF happens in a multi-hop U2U relay operation. But, even if the RLF happens in a multi-hop U2U relay, there is no problem in the case of using the global short ID. If RLF happens in any one hop, the relay UE detecting the RLF may recover the link toward the same target remote UE. After recovering the broken one hop, the relay UE which was recovered the broken link can use the same global short ID, if the global ID identifies the same pair of the source remote UE and target remote UE. The global short ID is assigned only by source remote UE, while the hop-by-hop short ID is assigned at each relay UE. In terms of local ID assignment, using the global ID looks simpler than using the hop-by-hop local ID.

Observation 1: Using the full ID of both L2 SRC ID for the source remote UE and L2 DST ID for the target remote UE has an overhead problem when it should be included in every single SRAP header. And it doesn’t seem to have any advantage compared to the layer-3 U2U relay operation.

Observation 2: Using a single short ID in the SRAP header is an efficient way to deliver a packet.

Observation 3: After SL connection is established from the source remote UE to the target remote UE, the RLC channel mapping configuration (e.g., the mapping between ingress RLC channel and egress RLC channel) is accomplished based on the end-to-end bearer.

Observation 4: A short ID in the SRAP header does not decide the packet routing. The relay UE only follows the RLC channel mapping configuration for delivering the received packet.

Observation 5: The reason for using a single short ID is to identify at the target remote UE side that the packet originated from which source remote UE.

Observation 6: Even if a single short ID or end-to-end bearer is duplicated in one relay UE, there is no problem. Because the relay UE identifies it based on the L2 ID on the MAC layer. The relay UE needs to store the SL configuration based on the L2 ID on the MAC layer pair.

Observation 7: The global short ID can be assigned by source remote UE.

Observation 8: Using global short ID looks like a simple solution compared to using hop-by-hop short ID. Because the assignment is happened only source remote UE.

Observation 9: If RLF happens in a multi-hop relay operation, using a short ID, e.g., hop-by-hop or globally, is no problem. Relay UE detecting RLF may recover the link. After the recovery the RLF link, the relay UE can assign hop-by-hop short ID or global short ID used before RLF to the new next relay UE. If the global short ID can identify the same pair of source remote UE and target remote UE, the global short ID keeps being used after recovery.

Proposal 9: Using a single short ID is an efficient way in terms of SRAP header overhead.

Proposal 10: After SL unicast is established from the source remote UE to the target remote UE, the direction of delivering the packet is decided based on ingress and egress RLC channel configuration.

Proposal 11: Local ID is just for identifying at the target remote UE side that the packet originated from which source remote UE.

Proposal 12: Even if the single short ID is duplicated at a relay UE, the relay UE can identify the receiving packet based on L2 ID of the MAC layer.

Proposal 13: In terms of local ID assignment, using the global ID looks simpler than using the hop-by-hop local ID.

Proposal 14: Using a global short ID, identifying the pair of the source remote UE and the target remote UE, is preferable.

2.2.3 QoS split and configure procedure

In the last RAN2 meeting, it was agreed that AS layer is responsible for the QoS split in the L2 U2U relay, and the QoS split is performed in the L2 U2U relay was agreed. According to the current TS 38.331 spec, gNB configures the SL-RLC-ChannelConfig by dedicated RRC message to the relay or remote UE. In the SL-RLC-ChannelConfig element, QoS split is performed based on RLC channel ID and PDB granularity. A similar QoS split configuration can be performed for the U2U relay operation. For the U2U relay operation, the QoS split can be performed only for the pack delay budget.

SL-ConfigDedicatedNR information element

SL-RLC-ChannelConfig

Proposal 15: QoS split is performed only for PDB such as L2 U2N.

Proposal 16: The result of QoS split can be configured by PDB value per RLC channel using RRCReconfigurationSidelink (e.g., SL-RLC-ChannelConfig).

We need to consider the compatibility of the future release, for example, multi-hop U2U Relay. So, we will think QoS split procedure based on the multi-hop U2U relay operation.

(Option 1 – relay UE consider only the measurement value measured by itself)

The QoS split procedure needs to be combined with SL connection procedure. If a separated PC5-RRC message is used for QoS split, there could be unnecessary signalling-handshaking. Source remote UE can give QoS-related information to the relay UE1 via RRCReconfigurationSidelink message, which includes RLC channel configuration for the 1st-hop and the required PDB value from the source remote UE to target remote UE. The relay UE1 receiving the RRCReconfigurationSidelink included required PDB per RLC channel performs QoS split. Then the split QoS value sends to the source remote UE via RRCReconfigurationCompleteSidelink message which includes split QoS per RLC channel ID. The relay UE1 can consider only the measurement value for the 1st-hop and the 2nd-hop. The relay UE1 cannot consider the measurement value of the 3rd-hop.

After the Relay UE1 performs the QoS split for the 1st-hop, the relay UE1 sends RRCReconfigurationSidelink message to the relay UE2. The RRCReconfigurationSidelink message from the relay UE1 includes RLC channel configuration for the 2nd-hop and the remaining QoS-related information per RLC channel ID. The relay UE2 receiving the message from the relay UE1 performs QoS split and sends RRCReconfigurationCompleteSidelink message to the relay UE1. The message includes split QoS value per each RLC channel ID. The relay UE2 only considers the measurement value for the 2nd-hop and the 3rd-hop. Similar to the relay UE1, the relay UE2 sends RRCReconfigurationSidelink message which is including the remaining QoS per RLC channel ID to the target remote UE, and the target remote UE sends RRCReconfigurationCompleteSidelink message to the relay UE2.

Figure 2 QoS split procedure combined with SL connection establishment procedure (option 1)

(Option 2 - relay UE can consider the measurement value of the whole U2U link)

Without the measurement value of the whole link, it’s hard the relay UEs performs the QoS split fairly. To perform QoS split fairly, the following QoS split procedure can be considered. Source remote UE sends RRCReconfigurationSidelink message, which includes RLC channel configuration for the 1st-hop and QoS related information per RLC channel ID, to the relay UE1. When the relay UE1 receives the message from the source remote UE, the relay UE1 is triggered to send RRCReconfigurationSidelink message, which includes RLC channel configuration for the 2nd-hop, QoS related information per RLC channel ID, and measurement result of 1st-hop, to the relay UE2. When the relay UE2 receives the message from the relay UE1, the relay UE2 is triggered to send RRCReconfigurationSidelink message, which includes RLC channel configuration for the 3rd-hop, QoS related information per RLC channel ID, and measurement result of both 1st-hop and 2nd-hop, to the target remote UE. When target remote UE receives the RRCReconfigurationSidelink message from the relay UE2, the target remote UE knows the whole link measurement quality. Then the target remote UE performs QoS split for the 3rd-hop considering the whole link quality. The split QoS value per RLC channel ID and remaining PDB value are delivered via RRCReconfigurationCompleteSidelink message to the relay UE2. After the relay UE2 receives RRCReconfigurationCompleteSidelink message, the relay UE2 performs QoS split within the remaining PDB value.  And the split QoS value and the remaining PDB deliver to the relay UE1 via RRCReconfigurationCompleteSidelink message.  If the Relay UE1 receives the RRCReconfigurationCompleteSidelink message from relay UE1, the remaining PDB value is delivered to the source remote UE.

Figure 3 QoS split procedure combined with SL connection establishment procedure (option 2)

Observation 10: QoS split procedure needs to be combined the SL connection establishment procedure.

Observation 11: To combine QoS split procedure and SL connection establishment procedure, RRCReconfigurationSidelink message includes remaining QoS information per RLC channel and the RRCReconfigurationCompleteSidelink message includes split-QoS value per RLC channel.

Observation 12: The QoS split procedure can be different whether the whole link measurement result is considered or not.

Proposal 17: QoS split procedure needs to combine with the SL connection establishment procedure to reduce signalling handshaking.

Proposal 18: For the U2U relay operation, the RRCReconfigurationSidelink message from the UE source remote UE to relay UE or from relay UE to the target remote UE includes at least the followings:

- SL RLC channel configuration

- The required (/remaining) PDB information per RLC channel ID

Proposal 19: For the U2U relay operation, the RRCReconfigurationCompleteSidelink message from the target remote UE to relay UE or from the relay UE to the source remote UE includes at least the followings:

- The split PDB per RLC channel ID

If the source remote UE or relay UE is in mode-1 resource allocation, the serving gNB of remote UE or relay UE may need to know the configured split PDB information from another relay UE. In the mode-1 operation, the gNB assigns resource grants for the SL transmission. When the serving gNB of the remote UE or relay UE knows the split PDB value configured by another relay UE, the gNB can assign the SL grant properly.

Proposal 20: If the remote UE or relay UE is in mode-1 resource allocation, the remote UE or relay UE should report the split PDB value configured by another relay UE.

2.3 SRAP header structure

The U2U SRAP layer can be based on the existing SRAP specification i.e., the U2N SRAP layer can be reused for U2U SRAP layer. After the initial SL connection establishment between source remote UE and target remote UE via relay UE, the U2U SRAP layer can be configured. After the U2U SARP layer configuration, the message for the source remote UE and the target remote UE transmits with the SRAP header. The SRAP header may include a single short ID and end-to-end bearer ID similar to the legacy rel-17 U2N SRAP layer.

Proposal 21: If a single short ID is applied, the SRAP header includes the end-to-end bearer ID and a single short ID similar to the legacy rel-17 U2N.

2.4 Path switching from indirect to direct

Regardless of lossless delivery, the direct path between remote UE and the target remote UE can be changed to the indirect path via relay UE. Contrary the indirect path between remote UE and the target remote UE via relay UE can be changed to the direct path. The condition of triggering relay selection/reselection has been discussed in the RAN2 meeting. But it’s not clear in which condition the indirect path can be triggered to the direct path in U2U. For example, if both PC5-RSRP in the 1st-hop and the PC5-RSRP in the 2nd-hop are above the pre-defined threshold, each end remote UE can trigger path switching from the indirect path to the direct path. And this case, release and add SLRB via the RRCReconfigurationSidelink can be configured to the target remote UE.

Proposal 22. Even though not considering the lossless path switching, the remote UE performing U2U relay can switch from the indirect path to the direct path by release and add SLRB via RRCReconfigurationSidelink message.

Proposal 23: If both SL-RSRP, one is for the SL-RSRP between remote UE and relay UE in the 1st-hop and the other is for the SL-RSRP between relay UE and the target remote UE in the 2nd-hop, are above a pre-defined threshold, the remote UE can trigger path switching from indirect path to the direct path.

2.5 Measurement configuration for Path switching

While the source remote UE transmits data to the target remote UE via a U2U relay UE, the source remote UE needs to know whether path switching to a direct path is possible. If the source remote UE knows whether the direct connection is possible, the source remote UE may try to a direct connection to the target remote UE. If the PC5 RSRP of each hop is above a threshold, the source remote UE can try path switching to the direct path. But the source remote UE doesn’t know the PC5 RSRP of the 2nd-hop. One of the methods for the source remote UE to know the 2nd-hop PC5 RSRP is to configure measurement reporting to the relay UE or target remote UE. For example, if the source remote UE configures measurement report to the relay UE, the relay UE reports the PC5 RSRP between the relay UE and the target remote UE. Or if the source remote UE configures a measurement report to the target remote UE via the indirect path, the target remote UE reports the measurement between the target remote UE and the relay UE. We think these measurement reporting mechanisms can be utilized by using the current SL measurement framework. If there are no these kinds of measurement configurations, the source remote UE may send the discovery message to the target remote UE at any time to know the possibility of the direct connection.

Proposal 24: While the source remote UE communicates with the target remote UE via a relay UE, the source remote UE should know whether direct path communication is possible.

Proposal 25: There could be three following methods for the source remote UE to know whether direct path communication is possible.

- The source remote UE sends a discovery message to the target remote UE at any time (by UE implementation).

- The source remote UE configures measurement report to the relay UE to receive 2nd-hop PC5 RSRP using the current SL measurement framework.

- The source remote UE configures measurement report to the target remote UE to receive 2nd-hop PC5 RSRP using the current SL measurement framework.

3.	Conclusion

Discovery message forwarding in relay UE

Proposal 1: For model B discovery, Relay UE transmits discovery a response message when the following both conditions are satisfied.

- When the SD-RSRP toward the source remote UE is above a configured threshold.

- When the SD-RSRP between relay UE and the target remote UE is above a configured threshold.

Proposal 2: As the same principle, for the integrated-discovery model, the relay UE forwards the discovery message for DCA message with integrated Discovery case only if the PC5-RCRP between the relay UE and the target remote UE is above a threshold.

U2U relay connection procedure

Proposal 3: The following SL connection procedure is needed sequentially:

-  Source remote UE send RRCReconfigurationSidelink message to the relay UE.

- The relay UE receiving RRCReconfigurationSidelink from the source remote UE is triggered sending RRCReconfigurationSideliknk message to the target remote UE.

- The target remote UE receiving RRCReconfigurationSidelink from the relay UE can send RRCReconfigurationCompleteSidelink message to the relay UE, if the configuration included in the RRCReconfigurationSidelink is applicable.

- The relay UE receiving RRCReconfigurationCompleteSidelink from the target remote UE is triggered sending RRCReconfigurationCompleteSidelink to the source remote UE.

Proposal 4: The RRCReconfigurationSidelink message from source remote UE to relay UE includes at least the following configurations:

- SL RLC channel configuration for the 1st-hop (e.g., between source remote UE and relay UE).

- End-to-end bearer configuration from source remote UE and target remote UE.

- The mapping configuration between SL RLC channel in the 1st-hop and end-to-end bearer.

Proposal 5: The RRCReconfigurationSidelink message from relay UE to target remote UE includes at least the following configurations:

- SL RLC channel configuration for the 2nd-hop (e.g., between relay UE and target remote UE).

- End-to-end bearer configuration from source remote UE and target remote UE. This configuration is forwarded from the source remote UE.

- The mapping configuration between SL RLC channel in the 2nd-hop and end-to-end bearer.

Proposal 6: The mapping configuration between ingress RLC channel ID and egress RLC channel ID is configured by relay UE itself based on the end-to-end bearer.

Proposal 7: The initial RRCReconfigurationSidelink message from source remote UE may include a single short ID configuration that identifies source remote UE and target remote UE.

Proposal 8: The T400-like timer which has a longer timer value than the legacy T400 timer may be required for the SL connection between source remote UE and target remote UE.

Whether to apply short ID (/local ID)

Observation 1: Using the full ID of both L2 SRC ID for the source remote UE and L2 DST ID for the target remote UE has an overhead problem when it should be included in every single SRAP header. And it doesn’t seem to have any advantage compared to the layer-3 U2U relay operation.

Observation 2: Using a single short ID in the SRAP header is an efficient way to deliver a packet.

Observation 3: After SL connection is established from the source remote UE to the target remote UE, the RLC channel mapping configuration (e.g., the mapping between ingress RLC channel and egress RLC channel) is accomplished based on the end-to-end bearer.

Observation 4: A short ID in the SRAP header does not decide the packet routing. The relay UE only follows the RLC channel mapping configuration for delivering the received packet.

Observation 5: The reason for using a single short ID is to identify at the target remote UE side that the packet originated from which source remote UE.

Observation 6: Even if a single short ID or end-to-end bearer is duplicated in one relay UE, there is no problem. Because the relay UE identifies it based on the L2 ID on the MAC layer. The relay UE needs to store the SL configuration based on the L2 ID on the MAC layer pair.

Observation 7: The global short ID can be assigned by source remote UE.

Observation 8: Using global short ID looks like a simple solution compared to using hop-by-hop short ID. Because the assignment is happened only source remote UE.

Observation 9: If RLF happens in a multi-hop relay operation, using a short ID, e.g., hop-by-hop or globally, is no problem. Relay UE detecting RLF may recover the link. After the recovery the RLF link, the relay UE can assign hop-by-hop short ID or global short ID used before RLF to the new next relay UE. If the global short ID can identify the same pair of source remote UE and target remote UE, the global short ID keeps being used after recovery.

Proposal 9: Using a single short ID is an efficient way in terms of SRAP header overhead.

Proposal 10: After SL unicast is established from the source remote UE to the target remote UE, the direction of delivering the packet is decided based on ingress and egress RLC channel configuration.

Proposal 11: Local ID is just for identifying at the target remote UE side that the packet originated from which source remote UE.

Proposal 12: Even if the single short ID is duplicated at a relay UE, the relay UE can identify the receiving packet based on L2 ID of the MAC layer.

Proposal 13: In terms of local ID assignment, using the global ID looks simpler than using the hop-by-hop local ID.

Proposal 14: Using a global short ID, identifying the pair of the source remote UE and the target remote UE, is preferable.

QoS split and configure procedure

Proposal 15: QoS split is performed only for PDB such as L2 U2N.

Proposal 16: The result of QoS split can be configured by PDB value per RLC channel using RRCReconfigurationSidelink (e.g., SL-RLC-ChannelConfig).

Observation 10: QoS split procedure needs to be combined the SL connection establishment procedure.

Observation 11: To combine QoS split procedure and SL connection establishment procedure, RRCReconfigurationSidelink message includes remaining QoS information per RLC channel and the RRCReconfigurationCompleteSidelink message includes split-QoS value per RLC channel.

Observation 12: The QoS split procedure can be different whether the whole link measurement result is considered or not.

Proposal 17: QoS split procedure needs to combine with the SL connection establishment procedure to reduce signalling handshaking.

Proposal 18: For the U2U relay operation, the RRCReconfigurationSidelink message from the UE source remote UE to relay UE or from relay UE to the target remote UE includes at least the followings:

- SL RLC channel configuration

- The required (/remaining) PDB information per RLC channel ID

Proposal 19: For the U2U relay operation, the RRCReconfigurationCompleteSidelink message from the target remote UE to relay UE or from the relay UE to the source remote UE includes at least the followings:

- The split PDB per RLC channel ID

Proposal 20: If the remote UE or relay UE is in mode-1 resource allocation, the remote UE or relay UE should report the split PDB value configured by another relay UE.

SRAP header structure

Proposal 21: If a single short ID is applied, the SRAP header includes the end-to-end bearer ID and a single short ID similar to the legacy rel-17 U2N.

Path switching from indirect to direct

Proposal 22. Even though not considering the lossless path switching, the remote UE performing U2U relay can switch from the indirect path to the direct path by release and add SLRB via RRCReconfigurationSidelink message.

Proposal 23: If both SL-RSRP, one is for the SL-RSRP between remote UE and relay UE in the 1st-hop and the other is for the SL-RSRP between relay UE and the target remote UE in the 2nd-hop, are above a pre-defined threshold, the remote UE can trigger path switching from indirect path to the direct path.

Measurement configuration for Path switching

Proposal 24: While the source remote UE communicates with the target remote UE via a relay UE, the source remote UE should know whether direct path communication is possible.

Proposal 25: There could be three following methods for the source remote UE to know whether direct path communication is possible.

- The source remote UE sends a discovery message to the target remote UE at any time (by UE implementation).

- The source remote UE configures measurement report to the relay UE to receive 2nd-hop PC5 RSRP using the current SL measurement framework.

- The source remote UE configures measurement report to the target remote UE to receive 2nd-hop PC5 RSRP using the current SL measurement framework.

4.	Reference

- RP-221262, “Revised WID on NR sidelink relay enhancements,” LG Electronics, June, 2022.