3GPP TSG-RAN WG2 Meeting #123		R2-2307933

Toulouse, France, 21st – 25th Aug 2023

Source:	vivo

Title:	Discussion on PUCCH repetition for PUCCH Format 2

Agenda Item:	6.1.3

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss about configurations of PUCCH format to enhance PUCCH repetition.

- Discussion

  - PUCCH repetition enhancements for PUCCH Format 0/2

During Rel-17 URLLC WI, PUCCH repetition enhancements were specified [1].

Regarding PUCCH repetition for PUCCH formats 0 and 2, both subslot-based and slot-based repetitions are supported. Related agreements are listed as below for reference.

Agreement (@ RAN1#104-e): Support PUCCH repetition for PUCCH formats 0 and 2 at least for sub-slot based PUCCH repetition. FFS: Support for slot-based PUCCH repetition Agreement (@RAN1#106-e) Support slot-based PUCCH repetition for PUCCH Format 0 and Format 2 also for single TRP operation. - The support is subject to independent UE capability indication

Besides, for both subslot-based and slot-based repetitions of PUCCH formats 0 and 2, inter-slot and inter-subslot frequency hopping are supported as well.

Agreement (RAN1#106b-e) For PUCCH repetition enhancements: - Support inter-slotFrequencyHopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for slot-based PUCCH configurations. - Support inter-subslot Frequency Hopping for PUCCH repetition operation of PUCCH Formats 0, 1, 2, 3 and 4 for 7OS slot-based PUCCH configurations. - The UE applies the inter-subslot FH operation from sub-slot to sub-slot, if configured with inter-slotFrequencyHopping in the respective PUCCH_config. - (Working Assumption) Support inter-subslot Frequency Hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS slot-based PUCCH configurations. - The UE applies the inter-subslot FH operation from sub-slot to sub-slot, if configured with inter-slotFrequencyHopping in the respective PUCCH_config. - Note: As for Rel-15, the configuration / enabling of inter-slotFrequencyHopping and intraSlotFrequencyHopping is not supported. Agreement (RAN1#107-e) Confirm the following RAN1 working assumption from RAN1#106bis-e with the additional agreement on UE capability (in RED): - (Working Assumption) Support inter-subslot Frequency Hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS slot-based PUCCH configurations. - The UE applies the inter-subslot FH operation from sub-slot to sub-slot, if configured with inter-slotFrequencyHopping in the respective PUCCH_config.. - Support single UE capability indication of inter-subslot FH for PUCCH repetition operation.

However, according to the current field descriptions of PUCCH format configuration, inter-slotFrequencyHopping and nrofSlot are not applicable for PUCCH Format 2.

PUCCH-FormatConfig, PUCCH-FormatConfigExt field descriptions

additionalDMRS If the field is present, the UE enables 2 DMRS symbols per hop of a PUCCH Format 3 or 4 if both hops are more than X symbols when FH is enabled (X=4). And it enables 4 DMRS symbols for a PUCCH Format 3 or 4 with more than 2X+1 symbols when FH is disabled (X=4). The field is not applicable for format 0, 1 and 2. See TS 38.213 [13], clause 9.2.2.

interslotFrequencyHopping If the field is present, the UE enables inter-slot frequency hopping when PUCCH Format 0, 1, 3 or 4 is repeated over multiple slots. For long PUCCH over multiple slots, the intra and inter slot frequency hopping cannot be enabled at the same time for a UE. The field is not applicable for format 2. See TS 38.213 [13], clause 9.2.6.

maxCodeRate Max coding rate to determine how to feedback UCI on PUCCH for format 2, 3 or 4. The field is not applicable for format 0 and 1. See TS 38.213 [13], clause 9.2.5.

maxCodeRateLP Max coding rate to determine how to feedback UCI on PUCCH for format 2, 3 or 4. The field is not applicable for format 0 and 1. This field configures additional max code rate in the second entry of PUCCH-ConfigurationList-r16 for multiplexing low-priority (LP) HARQ-ACK and high-priority (HP) UCI in a PUCCH as described Clause 9.2.5.3 of TS 38.213 [13]. The field is absent for the first entry of PUCCH-ConfigurationList-r16.

nrofSlots Number of slots with the same PUCCH. When the field is absent the UE applies the value n1. The field is not applicable for format 2. See TS 38.213 [13], clause 9.2.6.

pi2BPSK If the field is present, the UE uses pi/2 BPSK for UCI symbols instead of QPSK for PUCCH. The field is not applicable for format 0, 1 and 2. See TS 38.213 [13], clause 9.2.5.

rb-SetIndex Indicates the RB set where PUCCH resource is allocated.

simultaneousHARQ-ACK-CSI If the field is present, the UE uses simultaneous transmission of CSI and HARQ-ACK feedback with or without SR with PUCCH Format 2, 3 or 4. See TS 38.213 [13], clause 9.2.5. When the field is absent the UE applies the value off. The field is not applicable for format 0 and 1.

Observation 1: Either PUCCH Repetitions or Inter-slot frequency hopping when repetition is enabled is not supported for PUCCH Format 2 according to 38.331, against RAN1 agreement.

Therefore, we believe that an amendment on the wording is necessary.

Proposal 1: endorse CR R2-2307467 [3] to enable PUCCH Format 2 with both PUCCH Repetitions or Inter-slot frequency hopping when PUCCH is repeated.

  - NBC issue on inter-slotFrequencyHopping

Regarding the backward compatibility issue, we further make investigation as follow.

Going through 38.306 [4], PUCCH repetition of PUCCH Format 0 and 2 has been included as a UE capability.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| pucch-Repetition-F0-2-r17 Indicates whether the UE supports transmission of a PUCCH format 0 and 2 over multiple slots with the repetition factor 2, 4 or 8. A UE supporting this feature shall also indicate support of pucch-Repetition-F1-3-4. | Band | No | N/A | N/A |

However, there is no such indication on capability of inter-slot frequency hopping for PUCCH.

Based on the above analysis, if there is no change for the current 38.331 [2], a limitation is set for PUCCH Format 2. That is, in the normal situation, gNB would not configure UE with inter-slotFrequencyHopping in PUCCH-FormatConfig. If UE receives such parameters from gNB when the configuration is set up for PUCCH Format 2, it will consider it as a network mal-function.

However, there can be a problem once the amendment is endorsed for Spec. Supposing that a gNB supporting new Specification sends PUCCH Format 2 configuration towards a legacy UE, the gNB can configure inter-slotFrequencyHopping while the UE cannot be configured correctly. As mentioned above, it is a mistake configuration which the UE would release finding out there is no way to implement inter-frequency hopping and repetitions within PUCCH Format 2.

To handle such NBC problem, the basic solution is to introduce a UE capability indication that UE is capable of PUCCH repetition enhancements for PUCCH Format 2. In this way, NW can know whether to configure UE with inter-slot frequency hopping for PUCCH Format 2 or not.

Proposal 2: RAN2 to discuss whether/how to indicate NW about UE capability of being configured with inter-slot frequency hopping for PUCCH Format 2 when PUCCH is repeated.

- Conclusion

We make the following observations based on the above discussion,

Observation 1: Either PUCCH Repetitions or Inter-slot frequency hopping when repetition is enabled is not supported for PUCCH Format 2 according to 38.331, against RAN1 agreement.

We hence propose:

Proposal 1: endorse CR R2-2307467 to enable PUCCH Format 2 with both PUCCH Repetitions or Inter-slot frequency hopping when PUCCH is repeated.

Proposal 2: RAN2 to discuss whether/how to indicate NW about UE capability of being configured with inter-slot frequency hopping for PUCCH Format 2 when PUCCH is repeated.

- Reference

- R1-2306899	Discussion on RRC parameters for PUCCH repetition

- TS 38.331-h50	Radio Resource Control (RRC) protocol specification (Release 17)

- R2-2307467	Correction on PUCCH repetition for PUCCH Format 2

- TS 38.306-h50	User Equipment (UE) radio access capabilities