3GPP TSG RAN WG2 #123    											R2-2307941

Toulouse, France, August 21-25, 2023

Agenda Item:	7.9.4

Source: 	NEC

Title:	Discussion on UP Issues of Multi-path Relaying

Document for:	Discussion

- Introduction

This contribution provides our considerations on UP issues of multi-path relaying and give our preference.

- Discussion

2.1 Considerations on Scenario 1

2.1.1 Data split

MP split bearer was agreed to be supported for multipath relaying in RAN2 in RAN2#119e [1]. For Scenario 1, the adaptation layer is applied over both the Uu link and the PC5 link of the indirect path. As a result, the influence of the SRAP layer should be considered, because the date to be transmitted via the indirect path will be processed first by the SRAP layer.

The essential concepts of the split bearer, such as the primary path and a threshold for selecting a path, can be utilized by MP split bearer. And the essential procedures for enabling date split are as follow:

- First, calculate the total amount of date volume of PDCP layer and RLC layer;

- Next, compare the date volume to a threshold to determine whether to select the primary path or the secondary path.

The indirect path can be configured as the primary path of a MP split bearer, lowering the power consumption of Remote UE. When the indirect path is set as the primary path, the data being processed by the SRAP layer should be counted for the total amount of date volume. So we propose:

For Scenario 1, when the indirect path of a MP split bearer is configured as the primary path, the total amount of date volume includes SRAP data volume.

2.1.2 PDCP duplication

PDCP DRB duplication was agreed to be supported for the MP split bearer based on the existing framework in both Scenario 1 and Scenario 2 in RAN2 119bis-e [2]. The duplicated PDCP PDU discarding is one of the essential functions of the split bearer in NR-DC, which can be supported by the MP split bearer.

- For Scenario 1, the adaptation layer is applied over both the Uu link and the PC5 link of the indirect path. Thus, while discarding the duplicated PDCP Data PDU, the effect of SRAP layer over PC5 link must be considered. For example, if an acknowledgement of one PDCP PDU is received from the AM RLC entity of the direct path, whether the PDCP entity should indicate both the SRAP entity and the (PC5) AM RLC entity of the indirect path in order to delete the acknowledged PDCP PDU? As another example, if one PDCP Data PDU is confirmed at the AM RLC entity of the indirect path, whether to notify the PDCP entity directly by the AM RLC entity or indirectly through the SRAP layer also needs to be discussed and decided.

For Scenario 1, to enable the duplicated PDCP PDU discarding, the influence of the SRAP layer should be considered.

Upon receiving an acknowledgement of one PDCP PDU from the AM RLC entity of the direct path, the PDCP entity should indicate the SRAP entity to delete the corresponding duplicated PDCP PDU because the PDU may still be handle by the SRAP layer. Furthermore, if the corresponding duplicated PDCP PDU has already been forwarded to the (PC5) AM RLC entity of the indirect path, the SRAP entity should indicate the (PC5) AM RLC entity of the indirect path that this PDCP PDU should be deleted.

For Scenario 1, upon receiving an acknowledgement of one PDCP PDU from the AM RLC entity of the direct path, the PDCP entity should indicate the SRAP entity to delete the corresponding duplicated PDCP PDU.

If the corresponding duplicated PDCP PDU has been forwarded to the (PC5) AM RLC entity of the indirect path by the SRAP layer, the SRAP layer should indicate the (PC5) AM RLC entity to delete the corresponding duplicated PDCP PDU.

When the (PC5) AM RLC entity of the indirect path receives a positive acknowledgement for an RLC SDU( for a PDCP PDU) from its peer entity, it should send an indication of successful delivery of the RLC SDU to the PDCP layer directly, because the specification does not require the SRAP layer to store the transmitted PDCP PDU. Therefore we propose:

For Scenario 1, when receiving a positive acknowledgement for an RLC SDU, the (PC5) AM RLC entity of the indirect path should send an indication of successful delivery of the RLC SDU to the SRAP layer directly even if there is an SRAP layer above it.

2.2 Considerations on Scenario 2

2.2.1 Data Split

Data split is a basic function of a split bearer and it is subject to the data volume of the primary path. For Scenario 2, if the indirect path is set as the primary path, it may be difficult for remote UE to collect the total amount of data volume since there is a non-3GPP lower layer. The following solutions can be considered:

- Solution1: Always set the direct path as the primary path.

- Solution2: When the indirect path is set as the primary path, how to perform the date split is up to UE implementation. That is to say, Remote UE should ignore the configured threshold of data split in Scenario 2.

Considering that the UE to UE link is an idea link, we prefer Solution1 and propose:

For Scenario 2, the direct path of MP split bearer is always set as the primary path.

2.2.2 PDCP duplication

To enable the duplicated PDCP PDU discarding:

- For Scenario 2, the adaptation layer is not specified over the UE-to-UE link of the indirect path. Therefore, the solution for Scenario 1 is not suitable for Scenario 2. Moreover, the lower layers of UE-to-UE link are not in 3GPP scope, and whether and how to enable the duplicated PDU discarding can be left to the UE implementation to ease the specification effort.

For Scenario 2, whether and how to enable the duplicated PDCP PDU discarding can be left to the UE implementation.

- Conclusion

We have the following proposals:

Proposal 1:	For Scenario 1, when the indirect path of a MP split bearer is configured as the primary path, the total amount of date volume includes SRAP data volume.

Proposal 2:	For Scenario 1, to enable the duplicated PDCP PDU discarding, the influence of the SRAP layer should be considered.

Proposal 3:	For Scenario 1, upon receiving an acknowledgement of one PDCP PDU from the AM RLC entity of the direct path, the PDCP entity should indicate the SRAP entity to delete the corresponding duplicated PDCP PDU.

Proposal 4:	If the corresponding duplicated PDCP PDU has been forwarded to the (PC5) AM RLC entity of the indirect path by the SRAP layer, the SRAP layer should indicate the (PC5) AM RLC entity to delete the corresponding duplicated PDCP PDU.

Proposal 5:	For Scenario 1, when receiving a positive acknowledgement for an RLC SDU, the (PC5) AM RLC entity of the indirect path should send an indication of successful delivery of the RLC SDU to the SRAP layer directly even if there is an SRAP layer above it.

Proposal 6:	For Scenario 2, the direct path of MP split bearer is always set as the primary path.

Proposal 7:	For Scenario 2, whether and how to enable the duplicated PDCP PDU discarding can be left to the UE implementation.

- References

[1] Chairman’s Notes, RAN2#119e.

[2] Chairman’s Notes, RAN2#119bis-e.