3GPP TSG-RAN WG2 Meeting #123	           R2-2307945

Toulouse, France, August 21th– 25th, 2023

Agenda item:	7.9.3

Source:	China Telecom

Title:	Discussion on the procedure for intra-gNB indirect to indirect path switch

Document for:	Discussion

- Introduction

During RAN3#120 meeting, RAN3 has updated the baseline CR about the message flows and procedure texts for inter-gNB indirect to direct path switch, inter-gNB indirect to indirect path switch and inter-gNB direct to indirect path switch [1]. The procedure and message flow for intra-gNB indirect to indirect path switch are left to RAN2.

In this paper, we would like to discuss and try to describe the message flow and procedure text for the intra-gNB indirect to indirect path switch scenario.

- Discussion

According to the agreements reached in previous meetings, for inter-gNB d2i path switching and intra-/inter-gNB i2i path switching in Rel-18, the network can select a target U2N relay UE in any RRC state, i.e., RRC_CONNECTED/IDLE/ INACTIVE. We describe the possible procedure for the intra-gNB indirect to indirect path switch when the target relay UE in RRC_CONNECTED, as in Figure1.

Figure 1.  intra-gNB i2i path switch procedure for target relay UE in RRC_CONNECTED

Step 1.	The gNB sends the measurement configuration to the L2 U2N Remote UE via the source L2 U2N relay UE. The measurement results from L2 U2N Remote UE are reported when the configured event Z1 is met. The measurement report shall include at least the source L2 ID, serving cell ID, and sidelink measurement quantity results of the source L2 U2N relay UE and one or more candidate L2 U2N relay UEs.

Step 2.	The gNB decides to switch the L2 U2N Remote UE from the source L2 U2N relay UE to a target relay UE, based on the measurement results reported by the L2 U2N Remote UE via the source L2 U2N relay UE.

Step 3.	 The gNB sends RRCReconfiguration message to the L2 U2N Remote UE via the source L2 U2N relay UE. The L2 U2N Remote UE stops UP and CP transmission via L2 U2N Relay UE after reception of RRCReconfiguration message with the path switch configuration.

Step 4.	The gNB sends RRCReconfiguration message to the target L2 U2N relay UE to establish relay bearers for the L2 U2N remote UE.

Step 5. The L2 U2N Remote UE establishes PC5 RRC connection with the target L2 U2N Relay UE.

Step 6. The L2 U2N Remote UE sends the RRCReconfigurationComplete message to the gNB via the target L2 U2N relay UE.

Step 7. The data path is switched from the source indirect path to the target indirect path.

Step 8. The gNB sends RRCReconfiguration message to the source L2 U2N Relay UE to release Uu and PC5 relay RLC channel, and bearer mapping configuration related to the L2 U2N Remote UE.

Step 9. Either the source L2 U2N Relay UE or the L2 U2N Remote UE's AS layer can release PC5-RRC connection and indicate upper layer to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation.

The Rel-17 remote UE oriented solution to trigger the target U2N relay UE to the CONNECTED state is also applicable to the Rel-18 intra-gNB i2i path switch scenario for single-path relay. The procedure in Figure 1 can be also applied for the case that the selected L2 U2N Relay UE for indirect to indirect path switch is in RRC_IDLE or RRC_INACTIVE with the exception that the RRCReconfiguration message is sent from the gNB to the target L2 U2N Relay UE after the target L2 U2N Relay UE enters RRC_CONNECTED state, which happens between step 5 and step 6.

Proposal 1: RAN2 to discuss the procedure for intra-gNB indirect-to-indirect path switch.

Proposal 2: RAN2 to capture the above procedure description and message flow figure in TS 38.300 running CR.

3	Conclusions

Based on the discussion above, we have the following proposals:

Proposal 1: RAN2 to discuss the procedure for intra-gNB indirect-to-indirect path switch.

Proposal 2: RAN2 to capture the above procedure description and message flow figure in TS 38.300 running CR.

4	References

- [1]	R3-233360, (BLCR)Support of SL relay enhancement in TS38.300, CMCC, CATT, Samsung, China Telecom, Nokia, Nokia Shanghai Bell, Ericsson, LG Electronics, ZTE.