3GPP TSG-RAN2 WG2 Meeting #123	R2-2307947

Toulouse, France, August 21-25, 2023

Agenda item:	7.9.4

Source: 	China Telecom

Title:	Discussion on remaining issues of multi-path relaying in scenario 2

WID/SID:	NR_SL_relay_enh-Core

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, multi-path relaying related issues have been widely discussed and much progress has been made. However, there are still some open issues and stage-3 details left for further discussion, for both scenarios 1 and 2.

In this contribution, we would like to share our opinions on the following remaining issues in scenario 2 based on the agreements reached in previous RAN2 meetings.

- whether to support case G in scenario 2

- indirect path failure handling in scenario 2

- whether remote UE reports idle/inactive relay UE ID for indirect path addition in scenario 2

- Discussion

Whether to support indirect path change in scenario 2

In previous RAN2 meeting, it is confirmed that a single procedure for case E and G in scenario 1 can be supported as per the following agreement.

Agreement:

Change of direct path while keeping the indirect path can be done with a release-and-add in a single RRC message.  This does not exclude a gNB implementation from using separate release and add procedures instead.

Change of indirect path while keeping the direct path can be done with a release-and-add in a single RRC message.  This does not exclude a gNB implementation from using separate release and add procedures instead.

Regarding the supported path management cases in scenario 2, whether to support case G (i.e. indirect path change) remains FFS. It has been agreed that the indirect path addition and release cases A and C are supported in previous RAN2 meeting. Technically, case G could be supported by separate release-and-add procedures, i.e. case C+A. However, we may first need to identify whether case G is useful or valid in scenario 2 before making a decision to support it. As agreed in RAN2#119bis-e meeting, the relay UE is restricted to serve only one remote UE in scenario 2, thus it might be difficult for the remote UE to find another suitable relay UE when needed. Besides, in RAN2#119e meeting, RAN2 has already assumed that the relation between remote UE and relay UE in scenario 2 is pre-configured or static, and RAN2 will deprioritize discussion on authorization and association mechanism between remote UE and relay UE in scenario 2. If case G is supported, it may bring potential work on authorization and association mechanism.

In last RAN2 meeting, some company comments that the UE-UE link failure reporting is introduced and the network could fall back to a different relay UE in case of failure, so that case G shall be supported. Considering the target case in scenario 2 is the ideal link, we understand that the motivation to introduce failure reporting of UE-UE link is to help the gNB to release the indirect path. Based on that, we suggest focusing on the basic path management case A and C in Scenario 2 and we can further discuss case G in later release.

Proposal 1: For Scenario 2, the indirect path change case (i.e. case G) is not supported in this release.

Indirect path failure handling in scenario 2

Regarding indirect path failure in scenario 2, it includes UE-UE link failure between remote UE and relay UE or Uu-RLF of relay UE in scenario2. In previous RAN2 meeting, the following agreement and assumption were reached.

Agreement:

If UE-UE link failure is detected on indirect path in Scenario 2, the remote UE can report UE-UE link failure to gNB over direct path.  Details of the reporting mechanism can be further discussed.

WA: For Scenario-1/2, MP remote UE is configured with a single cell group, i.e., MCG, for the direct path, and SL configuration, for the indirect path.

Regarding which message to be used for failure report upon indirect path failure in scenario 2, we think the MCGFailureInformation message can be used since RAN2 has the assumption that MP remote UE is configured with a single cell group. If MCGFailureInformation message is used, additional failure indication might be needed to distinguish the failure of direct path or indirect path.

Proposal 2: For multi-path relaying in Scenario 2, upon detection of UE-UE link failure on the indirect path, MCGFailureInformation message can be used to report the indirect path failure to the gNB via direct path if available.

Proposal 3: If MCGFailureInformation message is agreed for indirect path failure reporting in scenario 2, additional failure indication might be needed to distinguish the failure of direct path or indirect path.

Whether remote UE reports idle/inactive relay UE ID for indirect path addition

In previous RAN2 meeting, the following agreements were reached.

Agreement:

For multi-path Relay Scenario-2, leave it to relay and remote UE implementation on how to trigger the RRC_IDLE/RRC_INACTIVE target relay UE to initiate RRC connection establishment procedure. RAN2 further discuss the solution for Scenario-1.

WA: For scenario 2, remote-UE reports the RRC_CONNECTED relay-UE C-RNTI and serving cell ID (e.g., NCGI) for indirect path addition.

LS to SA3 to check if there is any security concern with the WA.

Based on the above agreements and working assumption, remote UE can trigger the RRC_IDLE/RRC_INACTIVE target relay UE to initiate RRC connection establishment procedure. And during the study phase discussion, RAN2 assumes that the relation between remote UE and relay UE in scenario 2 is pre-configured or static and how the relation is pre-configured or static is out of the 3GPP scope. Then there seems no need to report idle/inactive relay UE ID to the gNB. The remote UE can first trigger the associated relay UE to connected mode, then get the C-RNTI and serving cell ID of the relay UE.

Proposal 4: In scenario 2, do not support remote-UE to report the RRC_IDLE / RRC_INACTIVE relay-UE ID for indirect path addition.

- Conclusion

In this contribution, we discuss several remaining issues on multi-path relaying in scenario 2 based on the agreements reached in previous RAN2 meetings. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: For Scenario 2, the indirect path change case (i.e. case G) is not supported in this release.

Proposal 2: For multi-path relaying in Scenario 2, upon detection of UE-UE link failure on the indirect path, MCGFailureInformation message can be used to report the indirect path failure to the gNB via direct path if available.

Proposal 3: If MCGFailureInformation message is agreed for indirect path failure reporting in scenario 2, additional failure indication might be needed to distinguish the failure of direct path or indirect path.

Proposal 4: In scenario 2, do not support remote-UE to report the RRC_IDLE / RRC_INACTIVE relay-UE ID for indirect path addition.

- Reference

RP-221262	Revised WID on NR sidelink relay enhancements	LG Electronics

3GPP TS 38.331 17.0.0: "NR; Radio Resource Control (RRC) protocol specification".

Report from session on positioning and sidelink relay (RAN2#119e, 119bis-e, 120, 121, 122)