3GPP TSG-RAN2 WG2 Meeting #123	R2-2307948

Toulouse, France, August 21-25, 2023

Agenda item:	7.25.3

Source: 	China Telecom

Title:	Discussion on remaining issues of eNPN in R18

WID/SID:	eNPN_Ph2-NGRAN-Core

Document for:	Discussion and Decision

- Introduction

In the last RAN2 meeting, RAN2 has endorsed the draft 38.304 CR and draft 38.331 CR on introduction of R18 eNPN. Besides, RAN2 discussed the UE AS capability related issue and had the following assumption [1].

R2 assumes that No new AS UE capability (signalled) is needed for supporting R18 eNPN features. FFS any 38.306 impact for conditional UE cap dependent on NAS cap.

In this contribution, we would like to share our opinions on the potential remaining issues in R18 eNPN that may need further discussion in RAN2.

- Discussion

UE capability

Based on the discussion in last RAN2 meeting, no new AS UE capability with signalling is needed for supporting R18 eNPN features. Whether to introduce a conditional UE capability dependent on NAS capability in TS 38.306 remains FFS.

To support equivalent SNPN, an optional UE MM Core Network Capability on Equivalent SNPNs is introduced. For idle mode mobility between SNPNs without new network selection, the UE NAS shall be able to maintain an equivalent SNPN list and provide it to UE AS for cell reselection. Based on the equivalent list, the UE AS will also consider a cell that is part of SNPN of the Equivalent SNPN list of the UE as a suitable cell when the cell (re)selection criteria are fulfilled. It seems natural for a UE capable of Equivalent SNPNs to support cell (re)selection based on the Equivalent SNPN list.

As per draft 38.304 CR (R2-2306801) endorsed in last RAN2 meeting, the UE behaviour of cell (re)selection on support of Equivalent SNPNs is specified. Some company also suggest that the enhancement on cell (re)selection can be specified in TS 38.306 as a conditionally mandatory feature without UE radio access capability parameters. Technically, we see no strong need to introduce a conditional UE capability dependent on NAS capability, which is similar to support of equivalent PLMNs for cell (re)selection. However, considering the concept of Equivalent SNPNs is introduced in a later release, i.e. Rel-18, it brings no harm to have such a conditionally mandatory feature to make the spec clearer if it is the majority view.

Proposal 1: To support equivalent SNPNs for cell (re)selection, a conditionally mandatory feature without UE radio access capability parameters can be introduced in TS 38.306.

Regarding the TP proposed in [2], we think the wording of “Enhanced cell (re)selection for SNPNs” is not quite clear. We suggest making the following changes on the detailed wording. And we also submit a draft 38.306 CR in R2-2307949 for further discussion if Proposal 1 is agreed.

| Features | Condition |
| --- | --- |
| Enhanced cCell (re)selection for within equivalent SNPNs | It is mandatory to support enhanced cell (re)selection for within equivalent SNPNs for UEs in SNPN access mode which support equivalent SNPNs as specified in TS 38.304 [21]. |

Proposal 2: If Proposal 1 is agreed, RAN2 reviews the above TP for 38.306 and if agreeable, endorse the draft 38.306 CR in R2-2307949 as baseline CR.

RAN2 impact to support equivalent SNPN in NR-DC

In RAN3#120 meeting, RAN3 has confirmed to support equivalent SNPN in NR-DC and endorsed the TP in R3-233400 for the draft 38.300 CR of R18 eNPN. The following editor notes in clause 16.6.1 is deleted from the endorsed CR, and no further RAN2 discussion or clarification is needed.

Copied from RAN3 endorsed TP in R3-233400 ### NR-NR Dual Connectivity within a single SNPN or across SNPNs indicated in the mobility restriction list is supported. Editor notes: It will be further checked when RAN3 has consensus on whether and how to support equivalent SNPN in NR-DC, and it may also depend on RAN2.

Proposal 3: From RAN2 perspective, there is no RAN2 impact to support equivalent SNPN in NR-DC.

- Conclusion

In this contribution, we discuss the potential remaining issues that may further discussion in RAN2. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: To support equivalent SNPNs for cell (re)selection, a conditionally mandatory feature without UE radio access capability parameters can be introduced in TS 38.306.

Proposal 2: If Proposal 1 is agreed, RAN2 reviews the above TP for 38.306 and if agreeable, endorse the draft 38.306 CR in R2-2307949 as baseline CR.

Proposal 3: From RAN2 perspective, there is no RAN2 impact to support equivalent SNPN in NR-DC.

- Reference

Draft report of 3GPP TSG RAN2 meeting #122

R2-2305140	Discussion on RAN impacts of further NPN enhancement	Lenovo

Draft report of 3GPP TSG RAN3 meeting #120