3GPP TSG-RAN WG2 Meeting #123 meeting	                      R2-2307953

Toulouse, France, 20th – 25th August, 2023

Agenda item:		7.5.4.2

Source:		Lenovo

Title:		Remaining details on discarding operation for XR

Document for:		Discussion and Decision

# Introduction

- RAN2#122 has agreed the following about the PDU discarding:

Agreement PDU-set discard indication for UL is configured using RRC to handle the PDU Set based discard functionality (i.e. whether UE discards all packets in PDU set when one PDU is discarded). The configuration is per PDCP entity. - Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling. - FFS how/whether to minimize additional UL signalling after this indication. - FFS if the NW indication is a one-shot or also subsequent packets.

In this contribution we further discuss the impacts on the PDU discarding of XR traffic.

# Discussion

### Discarding PDCP SDUs/PDUs of a PDU set

In RAN2#119bis meeting it was agreed that the granularity of the discard operation at PDCP in the transmitter should be the PDU set.

Packets of a PDU set for which the PSDB is exceeded may be depending on the used codec or depending on the application of no use for the receiver, hence shouldn’t be transmitted. Similarly, Packets within a frame have dependency with each other since the application may need all of these packets for decoding the frame. Hence one packet loss will make other associated packets useless even they are successfully transmitted. In some implementation, packets between frames e.g., in a GOP have dependency since the application needs to decode one frame based on another frame. XR applications impose requirements in terms of Media Units (PDU Sets), rather than in terms of single packets/PDUs. Such packet dependencies can be exploited for power saving and capacity improvements by discarding the already or to be scheduled packets whose reception might not be useful by the application.

NR PDCP protocol layer supports the timer-based discard functionality, i.e. by means of a PDCP discard timer. The PDCP discard timer is maintained per PDCP SDU, e.g. discard timer is started at reception of a PDCP SDU from upper layers. However, this mechanism and the discarding decision is based on individual PDCP SDUs/PDUs. As agreed for XR-services it is more suitable to discard an application packet, e.g. PDU set, before starting to transmit it if it is estimated that such PDU set will not meet the PSDB. In other words, all PDCP SDUs/PDUs associated to a PDU set are dropped if such application packet will not meet the agreed QoS e.g. PSDB. In RAN2#121bis-e meeting the following agreement w.r.t PDCP discard timer was reached.

PDU set discard is modelled using the existing PDCP discard timer for the uplink. The timer is in network control.

The following agreement was made in RAN2#122

PDU-set discard indication for UL is configured using RRC to handle the PDU Set based discard functionality (i.e. whether UE discards all packets in PDU set when one PDU is discarded). The configuration is per PDCP entity.

If the PDU Set Integrated handling Indication (PSIHI) provided by the CN indicates that all PDCP SDUs of a PDU set need to be received at the application layer, and UE is accordingly configured, e.g. to discard all packets of a PDU set when one PDU is discarded, UE should consider the PDCP discard timers of all the PDCP SDUs associated with an PDU set as expired for cases when the PDCP discard timer of one PDCP SDU – e.g. first PDCP SDU of the PDU set – expires. Similarly, for a PDU Set in a QoS flow for which the PSIHI is set, UE considers the PDCP discard timers of all the PDU/SDUs belonging to the PDU set as expired when one PDU of that PDU set is known to be lost.

Proposal 1: For a PDU Set of a radio bearer for which PDU set based discarding is configured, UE considers the PDCP discard timer of all the PDCP SDUs associated with the PDU set as expired for cases when the PDCP discard timer of one PDCP SDU expires or when one PDU of the PDU set is known to be lost.

In RAN2#121 meeting it was agreed that PSI can be considered for PDU set discarding in the presence of UL congestion. Therefore, in addition to the timer-based discard mechanism within a given PDCP entity, a PDCP discarding mechanism based on PDU set importance level (PSI) should be introduced.

The detailed discarding mechanism based on the importance level (PSI) of a PDU set in the case of congestion has been initially discussed in RAN2#122 without reaching a consensus for all details yet. However, one general agreement could be achieved which is important for the further design of the discarding procedure. As shown in the agreement below, NW controls the PSI-based discarding at the UE at the presence of congestion. To be more specific, NW  explicitly orders the UE to enable/disable PSI-based PDCP discarding, e.g. NW enables/disables PSI-based discarding, based on a detected congestion. Essentially the NW is responsible for the detection of congestion and UE just follows the NW signalling. Details of the NW indication, i.e. enabling/disabling, are FFS.

Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling. - FFS how/whether to minimize additional UL signalling after this indication. - FFS if the NW indication is a one-shot or also subsequent packets.

In one approach, the NW explicitly activates PDCP discarding of PDU/PDU sets of a certain (e.g. lower) importance level (PSI value) at the UE, e.g. in the presence of a detected congestion at NW. Such indication (PSI-based activation/deactivation), which might be DRB-specific and may contain the PSI level(s), may be for example transmitted within a MAC control element. Similarly, the gNB deactivates the PSI-based discarding mode, when no longer congestion is detected.

Another second approach would be that UE is configured with multiple PDCP discard timer configurations, e.g. one for each importance level. Furthermore, each PDCP discard timer configuration may be comprised of two PDCP discard timer durations, e.g. one for the normal mode of operation and one for the case that congestion has been detected for the air interface. Depending on the indicated mode, e.g. congestion/non-congestion, UE uses the corresponding PDCP discard timer configuration. The PDCP discard timer duration could be for example for a low importance level set to a small value, e.g. 0ms, for the congestion mode. Similar to the first approach, the NW indication, e.g. NW indicating the “mode” of operation (congestion/non-congestion), which determines ultimately the PDCP discard timer configuration to use in the UE, could be indicated within a MAC CE.

Alternatively, each PDCP discard timer configuration may be comprised of only one PDCP discard timer duration. The timer duration can be adjusted by the NW. For example, if congestion has been detected for the air interface, the NW adjusts the time duration with a larger discard timer value for one PSI and may also adjust the timer value to a smaller value for another PSI level. If non-congestion situation has been detected, the NW adjusts the time durations with normal timer values to UE. This would have the advantage to flexibly update the timer duration based on the detected congestion level however at the expense of higher signalling load.

The second approach has the advantage that discarding is done based on the discard timer also for the case of congestion. Therefore, the UE behaviour is basically not changed, e.g. only PDCP discard timer value is changed based on NW indication. This approach was also supported by a number of companies during offline discussion in RAN2#122. Therefore, the second approach would be also our preference.

Proposal 2: RAN2 to agree on the following PSI-based discarding mechanism:

- configuring UE with multiple PDCP discard timer configurations, e.g. one for normal mode of operation and one for congestion. NW indicates the mode of operation, e.g. whether to apply the PDCP discard timer configuration for congestion or for normal mode of operation

- configuring UE with one PDCP discard timer for each PSI. NW updates the timer values based on detected congestion level.

NW explicitly indicates the mode of operation and consequently the PDCP discard timer configuration to apply in the UE for a PDCP entity/LCH. UE should apply the indicated PDCP discarding behaviour until NW changes the mode of operation, e.g. NW explicitly switches the mode from “congestion” to normal. The Indication itself should be signalled by means of a MAC CE according to our understanding. Similar to the PDCP duplication enabling/disabling MAC CE, the indication should be done per DRB/PDCP entity.

Proposal 3: UE applies the PDCP discard timer configuration associated with the indicated mode of operation (congestion/normal mode) until NW explicitly changes the mode of operation, e.g. indicating a different PDCP discard timer configuration.

Proposal 4: MAC CE indicates on a DRB level which discarding mode to apply, e.g. PDCP discarding timer configuration associated with the mode (congestion/normal mode).

Impacts of discarding to L2 protocols (RLC/PDCP)

Discarding a packet at the transmitter side – e.g., due to exceeding the corresponding PSDB or due to a lost packet- may depending on what stage the discarding is done require informing the corresponding receiving entity about the discarded packets. If the to be discarded packets have not been yet submitted to lower layers, the associated L2 (PDCP/RLC) SN could be reused for new packets. However, if packets have been already provided to lower layers for a transmission attempt reassignment of Sequence numbers is not possible, e.g. discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP.

We think that the packet dropping mechanism should be enhanced for XR services. Different to the current specified mechanism, where only those RLC PDUs/SDUs can be discarded which have not yet been submitted to the lower layers for transmission, it may be needed to discard packets even if they have already been submitted to lower layers (MAC layer) for transmission. Discarding will be more often invoked for XR services compared to the legacy discard mechanism – e.g. due to the PDU Set Integrated handling Indication (PSIHI). Therefore, it is beneficial from capacity and transmission power point of view to also allow discarding of packets which have already an associated SN. It should be noted that RAN1 agreed to introduce Dynamic indication of unused CG PUSCH occasion(s) based on UCI for cases when UE doesn’t use the allocated CG PUSCH resources. This unused indication could be also used for cases when PDCP PDUs/SDUs and hence the corresponding TBs are discarded for cases when the TB contains data which is of no use for the receiving entity (due to an exceeded PSDB). Assumption here is that the PDUs of a PDU set, e.g. I/P frame are transmitted on CG PUSCH resources. 
As mentioned before this may though require informing the corresponding receiving entity about the discarded packets. The receiving entity may update its receiving window respectively corresponding timers in RLC/PDCP and for example not requesting RLC retransmissions (when applying RLC AM) based on the provided information on discarded packets.

Proposal 5: RAN2 to discuss enhancements to the discarding mechanism, e.g. informing receiving entity about discarded packets at the transmitter side, which may impact PDCP/RLC window operation.

For UL resource allocation, it would be beneficial if UE provides some assistance information regarding the remaining delay budget of the data pending in its buffer to the gNB. In RAN2#119bis meeting it was agreed that providing delay knowledge of buffered data, consisting of at least remaining time, and distinguishing how much data is buffered for which delay is considered is an useful improvement in order to allow for an enhanced uplink scheduling of XR services.

Similarly, gNB would also benefit from a notification by UE about UL data - which were reported in a BSR previously – which was discarded at the transmitter side before transmission, e.g. for which the delay budget is exceeded. gNB can take such knowledge into account for an efficient resource allocation, e.g. avoiding to issue further UL grants for data pending in UE’s buffer which is of no use for the user.

Proposal 6: RAN2 should discuss UE reporting enhancements to inform gNB about discarded PDU/SDUs at the transmitter side, e.g. when the delay budget is exceeded for data which has been previously reported in a BSR.

# Conclusion

In this contribution, we discuss the discarding of PDUs of a PDU set. We have the following proposals:

Proposal 1: For a PDU Set of a radio bearer for which PDU set based discarding is configured, UE considers the PDCP discard timer of all the PDCP SDUs associated with the PDU set as expired for cases when the PDCP discard timer of one PDCP SDU expires or when one PDU of the PDU set is known to be lost.

Proposal 2: RAN2 to agree on the following PSI-based discarding mechanism:

- configuring UE with multiple PDCP discard timer configurations, e.g. one for normal mode of operation and one for congestion. NW indicates the mode of operation, e.g. whether to apply the PDCP discard timer configuration for congestion or for normal mode of operation; or

- configuring UE with one PDCP discard timer for each PSI. NW updates the timer values based on detected congestion level.

Proposal 3: UE applies the PDCP discard timer configuration associated with the indicated mode of operation (congestion/normal mode) until NW explicitly changes the mode of operation, e.g. indicating a different PDCP discard timer configuration.

Proposal 4: MAC CE indicates on a DRB level which discarding mode to apply, e.g. PDCP discarding timer configuration associated with the mode (congestion/normal mode).

Proposal 5: RAN2 to discuss enhancements to the discarding mechanism, e.g. informing receiving entity about discarded packets at the transmitter side, which may impact PDCP/RLC window operation.

Proposal 6: RAN2 should discuss UE reporting enhancements to inform gNB about discarded PDU/SDUs at the transmitter side, e.g. when the delay budget is exceeded for data which has been previously reported in a BSR.

# References

S2-2301378, Reply LS on PDU Set Handling

[2] RAN2#120-e chairman note.