3GPP TSG-RAN WG2 Meeting #123	R2-2307959

Toulouse, France, August 21-25, 2023

Agenda item:		7.18.2

Source:		Lenovo

Title:		Discussion on remaining CP issues for MT-SDT

Document for:		Discussion and Decision

# Introduction

In RAN2#122 meeting, the following agreements were achieved [1].

Agreements:

- Allow support of only MT-SDT in a cell.  A separate SIB configuration will be introduced.  FFS what is put in there.

- For paging indication signalling, a new list of paging records for MT-SDT indication is optionally included in paging message using non critical extension. Each record in this list optionally includes 1 bit MT-SDT indication. UE identity and access type are not included in paging record of this list.

- gNB may include MT-SDT indication in paging message only if UE’s I-RNTI is included in the paging message (i.e. MT-SDT is only used by RAN initiated paging).

- UE selects '0' as the Access Category when the resumption of the RRC connection is triggered by response to the MT-SDT triggering in a PAGING message

- MT-SDT is only applicable to the legacy MT-Access use case (i.e. it is not applicable to access identities 1, 2 and 11-15).

- SRB2 can be used for MT-SDT (i.e. similar to MO-SDT)

- No additional enhancement is needed specifically for RedCap UE to monitor paging for MT-SDT

- When RRC resume is triggered due to MT-SDT and in the case the condition for paging triggered MT-SDT is not fulfilled, the UE initiates RRC Resume procedure with Resume cause “mt-Access”.

- A separate sdt-RSRP threshold for MT-SDT can be configured, at least in the case where MO-SDT is not configured in the cell.

In this contribution, we discuss remaining CP issues related with the MT-SDT procedure.

# Discussion

CG-SDT triggered by MT-SDT

Figure 1: CG-SDT triggered by NW for DL (MT) data

As shown in the figure 1 which displays some exemplary MT CG-SDT procedure, UE transmits on the selected CG resources, the RRCResumeRequest message (CCCH). Since UE may not have any uplink data available for transmission, it may be beneficial if UE transmits in addition to the RRCResumeRequest message some DL related control information which supports link adaptation/scheduling functionality in the NW, e.g. channel state information (CSI) and/or RSRP information, e.g. RSRP of pathloss reference, and/or beam related information on the selected CG resources. The CSI information and/or RSRP info and/or beam information can be for example transmitted within a new MAC CE.

Proposal 1: The RRCResumeRequest message and some DL related control information can be multiplexed and transmitted on the selected CG resources in response to the MT-SDT if there is no UL data available for transmission.

Proposal 2: The downlink related control information may be transmitted within a new MAC CE.

SDT procedure selection

After the email discussion [2], a note is added to the CR that “Even if the UE is paged, the UE may initiate resume procedure for a mobile originating cause if there is pending UL data or signalling.”.

There could be a case that a paging for MT-SDT message is received at T1; The RRC layer may determine that the RRC connection resume procedure is be initiated since the RRC layer may not identity when the UL data arrives.  Then the UL data arrives after the UE (e.g. the lower layer or RRC layer) determines that conditions for initiating MT-SDT are fulfilled. Is the UE allowed to initiate a MO-SDT instead of the ongoing MT-SDT before the first transmission (CCCH message) of the MT-SDT? Referring to the agreement, we think this procedure can be supported. Otherwise, we can consider only one ongoing SDT is supported besides the agreements on “Assumption is that if the UE has UL data the UE can still check and trigger MO-SDT”.

Proposal 3: The UE is allowed to initiate a MO-SDT instead of an already initiated MT-SDT before the first transmission (CCCH message) of the MT-SDT.

To support the MO-SDT which replaces the MT-SDT, UE would need to make sure that CCCH is ready before transmission, i.e. UE needs to update the resume cause value accordingly.

Proposal 4: The UE needs to update the content in CCCH message according to the latest triggered SDT procedure.

# Conclusion

In this contribution, we discuss the procedure related with MT-SDT and provide the proposals:

Proposal 1: The RRCResumeRequest message and some DL related control information are multiplexed and transmitted on the selected CG resources for MT-SDT if no UL data available.

Proposal 2: The downlink related control information can be transmitted within a new MAC CE.

Proposal 3: The UE is allowed to initiate a MO-SDT instead of an already initiated MT-SDT before the first transmission (CCCH message) of the MT-SDT.

Proposal 4: The UE needs to update the content in CCCH message according to the latest triggered SDT procedure.

# References

RAN2#122 chairman notes.

[Post122][312][MT-SDT] 38.331 Running CR (ZTE)