3GPP TSG-RAN WG2 Meeting #123	R2-2307963

Toulouse, France, 21 - 25 August 2023

Agenda item:	7.4.2.3

Source: 	Huawei, HiSilicon

Title: 	Leftovers related LTM MAC CE and cell switch

Document for:	Discussion and Decision

- Introduction

L1/L2-triggered mobility (LTM) aims at reducing the latency during cell switch [1]. Main steps are: 1) the network provides candidate cells pre-configuration to the UE, 2) the UE reports L1 measurement results, 3) the network triggers a LTM cell switch MAC CE, and 4) the UE switches to the target cell. In this contribution, we further analyse the content of the LTM cell switch MAC CE and the L2 behaviours during cell switch procedure.

- Discussion

  - LTM cell switch MAC CE content and RAN3 LS reply

In RAN2 #122 post email discussion [058], RAN2 is discussing the contents of the LTM cell switch MAC CE. So, for the detailed content, we can wait for the further progress of this email discussion. In this contribution, we discuss some high level principles regarding the kind of information that is not suitable to be put in the LTM cell switch MAC CE. These principles are also related to the RAN3 LS [2], which asks the suggestions from RAN2 about the interaction procedure in inter-DU LTM.

R3-230889, LS on Approaches during execution for inter-DU LTM RAN3 has discussed the following two approaches to support inter-DU LTM cell switch during execution. Approach 1: the serving gNB-DU triggers the execution by transmitting LTM cell switch command to the UE and then informs the gNB-CU of the serving cell switch. Approach 2: the serving gNB-DU first requests information from target DU before triggering LTM cell switch command to the UE. RAN3 would like to get feedback from RAN2 about the above-mentioned approaches, and provide suggestion if there is any other possibility identified.

In LTM, the first step is the pre-configuration. In this step, the source DU can obtain sufficient information from the target DU. Therefore, it is better if the source DU is able to make a cell switch decision without asking the target DU again before sending the LTM cell switch MAC CE. In the above approach 2, through the message interaction between the source DU, the target DU and the CU, it may be feasible to let the source DU know “dynamic” information (e.g., a dynamically selected BWP ID in the target cell), but the benefit is unclear compared with the RRC pre-configured first active BWP ID. In addition, these interactions not only make the procedure more complex, but also defer the triggering of LTM execution with the risk of source link failure. This is not preferred in LTM. Therefore, we think the above approach 1 is preferable. Regarding the content of the LTM cell switch MAC CE, either the source DU directly decides and then optionally informs to the target DU (e.g. beam indication), or the source DU uses the information directly provided from the target DU (e.g. TA value).

Proposal 1: RAN2 replies the LS to RAN3:

- From RAN2 perspective, the information included in the LTM cell switch MAC CE in inter-DU LTM can be (i.e. approach 1 in the RAN3 LS is basically sufficient) :

- either directly decided by the source DU and then optionally informed to the target DU (e.g. beam indication);

- or directly provided from the target DU to the source DU (e.g. TA value).

- RAN2 provides to RAN3 the list of information agreed to be in the LTM cell switch MAC CE in inter-DU LTM, which should be known by the source DU. (up to RAN3 on the signalling design in F1AP)

  - LTM completion: MAC CE vs. C-RNTI

In post email discussion [058], there are two major options for UE to determine the LTM completion in RACH-less.

- Option 2: C-RNTI addressed PDCCH

- Option 3: UE Contention Resolution identify MAC CE

The majority considers that option 2 is sufficient while option 3 is safe but there is no reason to use explicit ACK to UE’s first UL data. However, we see a technical issues to be discussed for option 2.

Either the UE consider LTM to be complete only if the UE confirms that the network has received its first UL data:

- RAN2#121-bis: “For RACH-less LTM, the UE considers that LTM execution procedure is successfully complete when the UE determines the NW has successfully received its first UL data.”

- If we stick to the RAN2 agreement that the LTM completion is determined based on UE is sure that the network has already received its first UL data, option 2 may cause more complicated discussion.

  - There is no explicitly ACK for UL HARQ supported in NR CG transmission, i.e. no way for UE to 100% determine the successful reception by the network

    - If CG are used: the “C-RNTI address PDCCH” should be on the same HARQ process as the one used for first UL data on CG. (most likely successful ACK)

  - If DG is used: the “C-RNTI address PDCCH” should be on the same HARQ process as the one used for first UL data on DG, and NDI has been toggled.

Or the UE considers LTM to be complete as long it received the C-RNTI addressed PDCCH (regardless whether it is before or after its first UL data).

- This means we revert the RAN2#121-bis agreement as UE can declare LTM completion as long as it ensures the target DL link is good to receive the PDCCH (like in BFR case).

Observation 1: In current MAC specification, there is no way to determine that the network has successfully received the UE’s UL transmission (especially using CG) without ambiguity.

Proposal 2: If RAN2 insist to agree on the option 2 (C-RNTI addressed PDCCH) for RACH-less LTM completion, choose one of the two ways forward:

- WF1 (with first UL data successful reception): the UE considers RACH-less LTM to be successfully complete when the UE receives the PDCCH addressed by C-RNTI for UL new transmission (as specified in sub-clause 5.4.2.1 in 38.321), after UE’s first UL data transmission.

- WF2 (without first UL data successful reception): the UE considers RACH-less LTM to be successfully complete when the UE receives the C-RNTI addressed PDCCH from target cell (regardless whether it is before or after its first UL data).

  - MAC PDU processing order

In the MAC PDU (TB) which includes the LTM MAC CE, there could be multiplexed data subPDU(s) from some logical channels. After the UE decodes this TB, if the UE applies the LTM MAC CE before processing other data subPDU, the UE may do L2 reset and the data subPDUs are lost during cell switch, e.g., discarded by RLC. This causes unnecessary data loss or data re-transmission. The UE can deliver the data MAC subPDU(s) to upper layers first before processing the LTM cell switch MAC CE.

Proposal 3: To reduce data loss due to RLC re-establishment, if there is any other MAC SDU(s) in the MAC PDU containing the LTM cell switch MAC CE, the UE MAC layer should first deliver these MAC SDU(s) to upper layers before processing the LTM cell switch MAC CE. FFS if any specification impact.

  - BSR triggering

In legacy L3 handover, all triggered BSRs are cancelled by (full) MAC reset and RLC is re-established. In this case, the RRCReconfigurationComplete message generated after handover triggers a new regular BSR, as new data coming to the empty LCH. Therefore, the target cell knows the UE uplink buffer status timely. In LTM, RAN2 agreed that MAC reset is always performed so the triggered BSR is cancelled. However, in the intra-DU case, the UE may not re-establish RLC and perform PDCP data recovery, so the UE has old data in the UL buffer and its priority could be the same like the priority of the RRCReconfigurationComplete message, in which case the UE does not trigger a regular BSR after LTM cell switch

Observation 2: If the RRCReconfigurationComplete message to be transmitted to the target cell is configured with the same (high) priority like other buffered LCH data (i.e. no higher priority data arrival) and RLC/PDCP is maintained (i.e. not to flush buffered data), a regular BSR may not be triggered timely after LTM execution.

- RAN2 121bis R2 assumes RRCReconfigurationComplete message is always sent at each LTM execution. - RAN2 122 For non-TA parts, we do MAC reset, which overrides earlier agreements on partial MAC reset. As earlier agreed RLC-AM can continue at LTM cell switch (intended for intra-DU).

In order to provide the UE buffer status information to the network early for better uplink scheduling, the UE can trigger a new BSR after LTM cell switch.

Proposal 4: In LTM cell switch, the UE should trigger a new regular BSR at the target cell.

- Conclusion

In this contribution, we have the following proposals:

Observation 1: In current MAC specification, there is no way to determine that the network has successfully received the UE’s UL transmission (especially using CG) without ambiguity.

Observation 2: If the RRCReconfigurationComplete message to be transmitted to the target cell is configured with the same (high) priority like other buffered LCH data (i.e. no higher priority data arrival) and RLC/PDCP is maintained (i.e. not to flush buffered data), a regular BSR may not be triggered timely after LTM execution.

MAC CE content and RAN3 LS reply

Proposal 1: RAN2 replies the LS to RAN3:

- From RAN2 perspective, the information included in the LTM cell switch MAC CE in inter-DU LTM can be (i.e. approach 1 in the RAN3 LS is basically sufficient) :

- either directly decided by the source DU and then optionally informed to the target DU (e.g. beam indication);

- or directly provided from the target DU to the source DU (e.g. TA value).

- RAN2 provides to RAN3 the list of information agreed to be in the LTM cell switch MAC CE in inter-DU LTM, which should be known by the source DU. (up to RAN3 on the signalling design in F1AP)

LTM completion

Proposal 2: If RAN2 insist to agree on the option 2 (C-RNTI addressed PDCCH) for RACH-less LTM completion, choose one of the two ways forward:

- WF1 (with first UL data successful reception): the UE considers RACH-less LTM to be successfully complete when the UE receives the PDCCH addressed by C-RNTI for UL new transmission (as specified in sub-clause 5.4.2.1 in 38.321), after UE’s first UL data transmission.

- WF2 (without first UL data successful reception): the UE considers RACH-less LTM to be successfully complete when the UE receives the C-RNTI addressed PDCCH from target cell (regardless whether it is before or after its first UL data).

MAC PDU processing order

Proposal 3: To reduce data loss due to RLC re-establishment, if there is any other MAC SDU(s) in the MAC PDU containing the LTM cell switch MAC CE, the UE MAC layer should first deliver these MAC SDU(s) to upper layers before processing the LTM cell switch MAC CE. FFS if any specification impact.

BSR trigger

Proposal 4: In LTM cell switch, the UE should trigger a new regular BSR at the target cell.

- References

- RP-221799, Revised WID on Further NR mobility enhancements.

- R3-230889, LS on Approaches during execution for inter-DU LTM.