3GPP TSG-RAN WG2 #123								R2-2307968
Toulouse, France, 21st – 25th August 2023

Agenda Item:	7.14.4 Support of QoE measurements for NR-DC

Source:	Ericsson

Title:	QoE measurements in NR-DC

Document for:	Discussion

# 1	Introduction

As described in the WID on Enhancement on NR QoE management and optimizations for diverse services, NR-DC is one of the important commercial deployment scenarios for 5G networks and it is critical to support it in the NR QoE framework. Therefore, in the Rel-18 WID [1], the following objective is defined:

- Specify to support for QoE in NR-DC, e.g. enable QoE reporting via SN [RAN3, RAN2].

- Specify the QoE configuration, and measurement reporting over MN/SN for NR-DC architecture and specify the QoE measurement reporting over the other DC leg in order to maintain the reporting continuity.

Note 1: The QoE measurements are not performed separately for each leg.

- Support RAN-visible QoE and radio related measurement configuration and reporting in NR-DC scenarios.

- Specify the QoE measurement continuity in mobility scenarios in NR-DC.

- Specify the alignment of QoE measurements (including legacy QoE and RAN visible QoE measurements) and radio related measurement in NR-DC.

Note 4: When the support for QMC in NR-DC is completed, how to reuse the NR-DC solution for supporting QMC in EN-DC should be considered, if time permits.

In RAN2#122, the following agreements related to NR-DC were taken:

1: The network can use explicit indication per QoE config to indicate which SRB is used for the QoE reporting. Details can be discussed in Stage-3.

FFS how to handle the QoE report transmission if there is only one SRB and the QoE report is not (explicitly) configured for that SRB (to be checked if this is possible according to latest RAN3 agreements)

FFS how the above case works if SCG is deactivated or released.

RAN2 will follow RAN3 agreement on QoE config RRC IDs.

2: RAN2 will follow RAN3 agreements (for NR-DC when SRB5 is not configured) on forwarding the received encapsulated QoE reports to the correct recipient (i.e. MCE or SN).

In this contribution, QoE for NR-DC is discussed according to the objective defined in the WID [1].

# 2	Discussion

## 2.1	QoE configuration and reporting for NR-DC

In dual-connectivity, there are two procedures defined for communication between the UE and the SN. One procedure is with MN involvement and the other procedure is without MN involvement. The procedure with MN involvement is typically used when SRB3 is not configured, not available or not supported and the procedure without MN involvement can be used when SRB3 is configured and the reconfiguration does not impact the MN. These two procedures are described in TS 37.340. As an example, reconfiguration in the SN can be done by using one of the procedures for SN Modification:

SN initiated SN Modification with MN involvement

Figure 10.3.2-2: SN Modification procedure - SN initiated with MN involvement

[..]

SN initiated SN Modification without MN involvement

This procedure is not supported for NE-DC.

Figure 10.3.2-3: SN Modification – SN initiated without MN involvement

[..]

For UL messages initiated by the UE, two corresponding procedures are defined. The UE may either send the message directly using SRB3 or transparently via the MN. The procedure for transmitting the messages via the MN is the following:

SN measurement report, failure information report, SN UE assistance information, intra-SN CPC execution completion or IAB other information:

Figure 10.10.2-3: RRC Transfer procedure for SN measurement report, failure information report, SN UE assistance information, intra-SN CPC execution completion or IAB other information

Figure 10.10.2-3 shows an example signaling flow for RRC Transfer in case of the forwarding of the SN measurement report, failure information report, SN UE assistance information, intra-SN CPC execution completion or IAB other information from the UE:

1.	When the UE sends an SN measurement report, failure information report, SN UE assistance information, intra-SN CPC execution completion or IAB other information it sends it to the MN in a container called ULInformationTransferMRDC message as specified in TS 38.331 [4].

2.	The MN initiates the RRC Transfer procedure, in which it transfers the received SN measurement report, failure information, SN UE assistance information, intra-SN CPC execution completion or IAB other information as an octet string.

This procedure is e.g. used for transmitting MeasurementReport to the SN, a very similar case as transmitting MeasurementReportAppLayer message to the SN.

The procedures for transmitting SN messages transparently via the MN were defined so that the procedures work also for the cases where the MN and the SN are from different vendors or of different RATs.

For QoE, the configuration of the UE using a transparent RRCReconfiguration message via the MN was agreed and included in the running RRC CR by a small update in the field description saying that the RRCReconfiguration message from the SN may also include the configuration of QoE. The corresponding UL procedure can be added by minor updates in RRC, allowing the MeasurementReportAppLayer message to be included in the transfer messages:

#### –	ULInformationTransferMRDC

The ULInformationTransferMRDC message is used for the uplink transfer of MR-DC dedicated information (e.g. for transferring the NR or E-UTRA RRC MeasurementReport message, the MeasurementReportAppLayer message, the FailureInformation message, the UEAssistanceInformation message, the RRCReconfigurationComplete message, the IABOtherInformation message or the NR or E-UTRA RRC MCGFailureInformation message).

Signalling radio bearer: SRB1, SRB3, SRB4

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

- ULInformationTransferMRDC message

-- ASN1START

-- TAG-ULINFORMATIONTRANSFERMRDC-START

ULInformationTransferMRDC ::=               SEQUENCE {

criticalExtensions                          CHOICE {

c1                                          CHOICE {

ulInformationTransferMRDC                   ULInformationTransferMRDC-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

ULInformationTransferMRDC-IEs::=           SEQUENCE {

ul-DCCH-MessageNR                           OCTET STRING                    OPTIONAL,

ul-DCCH-MessageEUTRA                        OCTET STRING                    OPTIONAL,

lateNonCriticalExtension                    OCTET STRING                    OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                     OPTIONAL

}

-- TAG-ULINFORMATIONTRANSFERMRDC-STOP

-- ASN1STOP

ULInformationTransferMRDC field descriptions

ul-DCCH-MessageNR Includes the UL-DCCH-Message. In this version of the specification, the field is only used to transfer the NR RRC MeasurementReport, MeasurementReportAppLayer, RRCReconfigurationComplete, UEAssistanceInformation, FailureInformation, and IABOtherInformation messages when sent via SRB1 and to transfer the NR MCGFailureInformation message when sent via SRB3.

ul-DCCH-MessageEUTRA Includes the UL-DCCH-Message. In this version of the specification, the field is only used to transfer the E-UTRA RRC MeasurementReport message when sent via SRB1 and to transfer the E-UTRA MCGFailureInformation message when sent via SRB3.

In RAN2#119bis, RAN2 agreed to use SRB4 as baseline for the QoE reporting and this is an implementation of that agreement. Using the existing procedures for QoE guarantees that QoE works also in multi-vendor (and multi-RAT) scenarios and it also simplifies the implementation. It also means that reports associated with the SN are correctly forwarded to the SN, without any new mechanism having to be introduced.

Reusing existing DC procedures for QoE guarantees support in multi-vendor and multi-RAT scenarios and decreases the complexity of the feature.

A MeasurementReportAppLayer message to the SN can be sent embedded in ULInformationTransferMRDC to the MN.

ULInformationTransferMRDC can be sent using SRB4.

In RAN2#121, the following agreement was taken:

1: RRC configuration determines to which node UE sends the QoE report.  It is possible to change the reporting leg via RRC signalling after it has been configured.

The agreement says that the RRC configuration determines to which node the UE sends the QoE report. Default reporting options were discussed in RAN3#120, but not agreed. Instead RAN3 reached the understanding that explicit configuration is needed:

RAN3 understanding is that the UE can be explicitly instructed whether to report over SRB5 or SRB4.

It was also agreed in RAN3 that the reporting leg can be changed for an ongoing measurement via RRC signalling. The new recipient node needs to acknowledge that it accepts receiving the reports before such a change can be done. The other node may e.g. not be able to receive reports due to overload or lack of support of QoE. This was agreed in RAN3:

The node that currently receives the QoE reports via the Uu can send a request to the peer node, asking that the QoE reporting leg is switched to the peer node.

The leg switch for QoE reporting needs to be approved by both nodes serving the UE.

These agreements mean that a reporting instruction to the UE needs to be defined in RRC. The reporting instruction needs to include the possible options for QoE reporting, i.e. report to MN via SRB4, report to SN transparently via SRB4 and report to SN directly via SRB5. As default reporting options were not agreed, the network needs to indicate a reporting instruction for each QoE configuration. If there is no reporting instruction indicated for the QoE configuration, the configuration can be considered erroneous and the UE may discard the QoE reports.

The RAN3 agreement copied above also means that the UE cannot send the reports to another node than the default node, if the network didn’t explicitly configure another reporting option. This is important as the other node may e.g. be overloaded, may be of a different vendor or have some other reason why it cannot receive QoE reports which the UE isn’t aware of. The reporting instruction needs to be configured per QoE configuration, as both the MN and the SN may configure the UE with QoE and the reporting instruction is likely different for the different configurations.

A reporting instruction is provided for each RRC QoE configuration, i.e. per measConfigAppLayerId, in rel-18.

The RRC reporting instruction for QoE reporting can include the options: 
- report to MN via SRB4, 
- report to SN transparently via SRB4, and 
- report to SN directly via SRB5.

## 2.2	SCG deactivated or released

When a UE is configured with dual connectivity, the network may reconfigure the UE to single connectivity for various reasons, e.g. depending on the amount of data being transmitted. If the SN provided a QoE configuration to the UE while it was in dual connectivity, that configuration needs to be released when the SCG is released. Otherwise, the measurements may remain hanging in the UE.

QoE configurations configured by the SN are released in the UE when the SCG is released.

However, the release of the SCG may be a fast procedure and the UE may not have transmitted all QoE reports when the SCG is released. It may even happen often that the QoE reports have not been transmitted when the SCG is released, as the QoE reports are often transmitted at the end of the session and the AS layer may perform the release faster than the application layer finalizes the reports. Therefore, the network should be allowed to update the reporting instruction when the SCG is released, to indicate to the UE that it may transmit the reports to the MN instead.

A UE can be configured to, upon SN release, send the unsent SN QoE reports to the MN.

The SCG may be deactivated when the UE has a QoE report to send. In other cases when the UE has data to send and the SCG is deactivated, the UE sends a UEAssistanceInformation message to the network, indicating that it has UL data to transmit. This indication can be reused when the UE has a QoE report to transmit.

The UE sends a UEAssistanceInformation message indicating that it has UL data to send (according to existing procedures), if the SCG is deactivated when the UE has a QoE report to send.

Some companies brought up that it may be unnecessary to activate the SCG just to send a QoE report. However, encapsulated QoE reports may be very large and large reports would be reason enough to activate the SCG. RVQoE reports on the other hand are not large. It could be considered if the UE should indicate that it has a QoE/RVQoE report to send, in addition to indicating UL data. The network could then take proper actions and ask the other node if it is able to receive the reports instead and instead configure the UE to use a different reporting option.

The UE can indicate in UEAssistanceInformation that it has a QoE report to transmit.

# Conclusion

RAN2 is kindly asked to discuss the following observations and proposals:

Observation 1	Reusing existing DC procedures for QoE guarantees support in multi-vendor and multi-RAT scenarios and decreases the complexity of the feature.

Proposal 1	A MeasurementReportAppLayer message to the SN can be sent embedded in ULInformationTransferMRDC to the MN.

Proposal 2	ULInformationTransferMRDC can be sent using SRB4.

Proposal 3	A reporting instruction is provided for each RRC QoE configuration, i.e. per measConfigAppLayerId, in rel-18.

Proposal 4	The RRC reporting instruction for QoE reporting can include the options:  - report to MN via SRB4,  - report to SN transparently via SRB4, and  - report to SN directly via SRB5.

Proposal 5	QoE configurations configured by the SN are released in the UE when the SCG is released.

Proposal 6	A UE can be configured to, upon SN release, send the unsent SN QoE reports to the MN.

Proposal 7	The UE sends a UEAssistanceInformation message indicating that it has UL data to send (according to existing procedures), if the SCG is deactivated when the UE has a QoE report to send.

Proposal 8	The UE can indicate in UEAssistanceInformation that it has a QoE report to transmit.

# References

RP-221803, Enhancement on NR QoE management and optimizations for diverse services, China Unicom