3GPP TSG-RAN WG2 #122	R2-2307969

Toulouse, August 21 – 25, 2023

Agenda Item:	 7.14.3 Rel-17 leftover topics for QoE

Source:	Ericsson

Title:	Event based RVQoE reporting

Document for:	Discussion

# 1	Introduction

As some of the features were not finalized in release 17, they are currently of interest in the release 18 as leftovers defined in the WID Enhancement on NR QoE management and optimizations for diverse services [1]. The current status of the leftovers is summarized as the latest RAN2 and RAN3 agreements and discussion points for further discussions in the following. Agreements made in the RAN3#118 meeting were the following:

Turn the WA to agreement: Introduce buffer level as a threshold-based trigger for RVQoE reporting.

Do not introduce the threshold-based trigger for reporting playout delay for media startup.

The final list of topics that are to be discussed in Rel-18:

RVQoE value (pending SA4 reply).

Assistance information for handling of QoE reporting upon RAN overload.

DU activation/deactivation/pause/resume of RVQoE reporting over F1.

DU participation in assembling the RVQoE configuration.

Event-based RVQoE reporting trigger.

In RAN2 #119bis-e the following was agreed:

For easy agreement:

Proposal 1: From RAN2’s perspective, there is no further work for slice-based QoE measurement.

Proposal 2: RAN2 can wait for RAN3 progress on enhancement to per-slice RAN visible QoE.

Proposal 3: RAN2 needs to wait for the progress of RAN3 on RVQoE value.

Proposal 10: The enhancement on UAI message to express the UE’s preference on QoE reporting configurations is not pursued.

Proposal 11: QoE reporting via unlicensed band is out of the WID scope.

Discussion topics agreed in the RAN2#119-bis-e:

Proposal 4: It is proposed RAN2 to discuss possible options for event-based RVQoE, including benefits, spec impacts, and complexities.

Proposal 5: Whether to add the QoS flow ID in the RVQoE report is FFS.

Proposal 6: RAN2 discuss whether to send a LS to CT1/SA4 (related to P5).

Proposal 7: RAN2 to postpone the discussion of the QoE reporting enhancement for overload scenario to the next meeting (based on the progress of RAN3).

Proposal 8: FFS on whether to send the priority information to UE.

Proposal 9: To wait for RAN3 decision on granularity of priority.

One of the other aspects addressed in this meeting in the contributions R2-2209837 and R2-2209845 was which of the layers, UE AS or UE application layer, should be evaluating whether the RVQoE events have happened.

In RAN2#122 the following was discussed and agreed:

2: RAN2 confirms that buffer level threshold-based triggering of RVQoE reporting is triggered by application layer.

3: RAN2 waits for RAN3’s conclusion on the buffer level threshold-based RVQoE reporting.

3: RAN2 will align with RAN3’s agreement on support of radio-related event triggers for RVQoE reporting in Rel-18.

Additionally, RAN3 has agreed that:

The network can configure UE with two types of events for threshold based RVQoE reporting, the RVQoE reporting is triggered when the buffer level is below a threshold or above a threshold. Whether these two types of events for threshold based RVQoE reporting can be configured simultaneously needs to be further discussed.

In this contribution we focus on the remaining issues of BufferLevel reporting and details of the UE application or AS layer could evaluate the thresholds and events for the event and threshold based RVQoE reporting.

# 2	Discussion

## 2.1	Event-based RVQoE

RAN visible QoE (RVQoE) provides a possibility of enabling a real-time radio resource optimization to improve user’s quality of experience while the user is still in an ongoing application session. Currently, the RVQoE reporting is associated with the reporting of the legacy QoE reporting. However, the purposes of legacy QoE and RVQoE are different: the former is meant for the post-session analysis, while the latter is meant for real-time analysis and radio resource optimization. To be more exact, QoE reports are meant for evaluation of the quality of experience in such an entity as an OAM after the session usually is over, while the RVQoE reports are sent to RAN for the real-time improvement of the quality of experience for users in a current session by optimising the radio resources in relation to the results reported in the RVQoE reports. Therefore, RVQoE reporting does not need to be the same as the QoE reporting as the report recipient entity and the aims of the reports differ. By enabling and enhancing the event triggered RVQoE, a faster and more efficient optimization of the radio resources can be achieved directly for users in ongoing sessions enabling improved quality of experience “on the go”. In this section we discuss the importance of event triggered RVQoE reporting.

Extending RVQoE with the events to trigger the reporting, may reduce the report size as the reporting as RVQoE measurements do not have to be continuously logged and stored in the UE AS. Event based RVQoE additionally enables RVQoE to be more independent from the OAM configured QoE measurements and allow real-time optimization of radio resources to improve the quality of experience for the users. Without event triggered RVQoE the reports may continue to contain excessive data in the RVQoE reports that may not be useful to RAN at the time of the report arrival and may result in more difficult and slower metrics’ analysis.

RAN3 agreed to introduce buffer level as a threshold-based trigger for RVQoE reporting, which is in accordance with the RAN2 agreement for a new trigger for RVQoE reporting: the BufferLevel below a configured threshold.

As it was agreed to introduce BufferLevel metric as a threshold-based trigger for RVQoE reporting, another issue to be discussed is format in which the BufferLevel should be reported. It would be the most useful to report the buffer level as a remaining playout time and not as an amount of data (either the collected or remaining in the buffer) due to the data type (video, audio, etc.) may differ. That is, the amount of data for a video to playout will take a shorter time than the same amount of data to playout an audio recording.

Report BufferLevel as remaining playout time in seconds or minutes

SA4 has sent a new LS [3] explaining that the UE application layer may not be the most efficient in evaluating the threshold-based triggering conditions for RVQoE reporting due to the following reasons:

- A higher buffer level may help reduce the probability of a playback stall, but may also increase the playback latency, so it’s a trade-off that needs to be balanced by applications. Different proprietary application mechanisms attempting to strike this balance may result in different application behavior for same buffer level or same content.

- Also, different applications (e.g. low-latency live vs. on-demand video applications) have very different behaviour of maintaining the buffer level.

If the UE application layer is allowed to evaluate whether the RVQoE report triggering conditions  set by the gNB and provided within the RVQoE configuration were fulfilled, it could send only those RVQoE metric values which meet the defined threshold conditions or events directly to UE AS, immediately discarding values which do not meet those conditions, this way reducing the amount of reports collected by the UE AS. UE AS in its turn could directly forward the RVQoE data received from the UE application layer to the RAN. Additionally, the application layer can make a threshold-based decision and submit the RVQoE reports in a timelier manner than UE AS layer, leading to smaller delay between a threshold occurrence and the next scheduled RVQoE report. However, with the above discussion on pros and cons of using the UE application layer to evaluate the threshold-based triggering conditions for RVQoE reporting in mind, it seems to be clear consensus on the matter. Hence, we propose the following:

Wait for RAN3 discussion and/or more analysis from other groups, on which UE layer should be evaluating threshold-based conditions for triggering RVQoE reporting.

RAN2 to agree that the threshold-based conditions are set by gNB and delivered within the RVQoE measurement configuration.

# Conclusion

RAN2 is kindly asked to discuss the following proposals:

Observation 1	RAN3 agreed to introduce buffer level as a threshold-based trigger for RVQoE reporting, which is in accordance with the RAN2 agreement for a new trigger for RVQoE reporting: the BufferLevel below a configured threshold.

Proposal 1	Report BufferLevel as remaining playout time in seconds or minutes

Proposal 2	Wait for RAN3 discussion and/or more analysis from other groups, on which UE layer should be evaluating threshold-based conditions for triggering RVQoE reporting.

Proposal 3	RAN2 to agree that the threshold-based conditions are set by gNB and delivered within the RVQoE measurement configuration.

# References

RP-223488, Revised WID: Enhancement on NR QoE management and optimizations for diverse services, China Unicom https://www.3gpp.org/ftp/TSG_RAN/WG2_RL2/TSGR2_120/Docs/R2-2213054.zip

S4-230684, Reply LS on buffer level threshold-based RVQoE reporting, 3GPP TSG-SA WG4 Meeting #123-e, 17-21 April 2023

S4-231119, Reply LS on buffer level threshold-based RVQoE reporting, 3GPP TSG-SA WG4 Meeting #125