3GPP TSG-RAN WG2 #123	R2-2307971

Toulouse, France, 2023-08-21 - 2023-08-25

Agenda Item:	7.4.3

Source:	Ericsson

Title:	Subsequent CPAC

Document for:	Discussion, Decision

# 1	Introduction

The Rel-18 work on further NR mobility enhancements is described in the WID in [1]. The WID includes an objective related to enhancements for PSCell mobility in case of NR-DC configurations (objective 2).

The following agreements were reached for the objective at the last meeting.

RAN2#122:

For SN-initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC.

FFS if it shall be possible to do something like MN-initiated CPA/CPC where Candidate SN generate execution conditions for subsequent CPC

The UE shall skip the condition evaluation for a candidate which is a current PScell.

The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures.

Will not spend specific efforts for supporting nested configurations for candidate cell configuration.

Terminology is “Subsequent CPAC”

In this contribution we discuss different aspects and open issues for objective 2 in the WID on Rel-18 work on further NR mobility enhancements (in [1]).

# 2	Discussion

## 2.1	Configuration of Subsequent CPAC

At RAN2#121bis it was agreed that the source MN generates the execution conditions for the initial CPAC in case of MN-initiated scenarios and that the source SN generates the execution conditions for the initial CPC in case of SN-initiated scenarios. At RAN2#122 it was further agreed that it is the candidate SN that generates the execution conditions for subsequent CPC in case of SN-initiated Subsequent CPAC.

One question is then how the execution conditions for subsequent CPC (as generated by the candidate SN) should be provided to the UE. To avoid the need for additional signalling at each PSCell change, the UE should receive the execution conditions for subsequent CPC (as generated by the candidate SN) already as part of the initial configuration, i.e. together with the candidate configuration(s) from that candidate SN (for Subsequent CPAC). When the UE executes a candidate configuration to the candidate SN it then already has the execution conditions for subsequent CPC (i.e. for other candidate configurations) stored.

The CPC and CPA candidate configurations are configured using the ConditionalReconfiguration IE, where each candidate configuration is provided to the UE as a CondReconfigToAddMod instance. Since Subsequent CPAC is an enhancement to CPA/CPC, these should be reused for configuration of the candidate PSCell configurations for Subsequent CPAC. Some Rel-18 indication or information need to be included to indicate that the CPC/CPA configuration is for Subsequent CPAC, i.e. that it can be kept after a cell change procedure.

Candidate configurations for Subsequent CPAC are provided with CondReconfigToAddMod in ConditionalReconfiguration, by inclusion of related Rel-18 information.

With the Subsequent CPAC candidates being configured using CondReconfigToAddMod, two options can be considered for provision of execution conditions for the subsequent CPC (i.e. for other candidates):

- within the target configuration (i.e. within condRRCReconfig)

- directly within CondReconfigToAddMod

Since the execution conditions for subsequent CPC concern other candidate configurations, i.e. for other CondReconfigToAddMod instances, it would probably be easier to let them be provided to the UE outside the target configuration (condRRCReconfig), i.e. directly within CondReconfigToAddMod. That way it is e.g. possible to just update or remove the execution conditions without updating the target configuration. By having the updated execution conditions part of the target configuration (i.e. within condRRCReconfig) would rather correspond to that the whole target configuration would need to be resent in case of a change. We therefore slightly prefer to let the execution conditions for subsequent CPC (for other candidates) be included directly under CondReconfigToAddMod.

The network can, in the CondReconfigToAddMod for the executed candidate configuration, indicate updates of execution conditions for the other candidate configurations at the execution (i.e. for subsequent CPC).

The candidate SN would then need to provide the execution conditions for subsequent CPC to the MN, so that the MN can include them in the configuration to the UE. A high level signalling flow for this can be seen in Figure 1 below where the MN, in the request to the candidate target SN (T-SN), provides information about other candidate target PSCells (for subsequent CPAC). The candidate T-SN generates the corresponding execution conditions (applicable when/if the UE moves to the candidate PSCell in that candidate T-SN) and provides them to the MN together with the associated target configuration for the candidate PSCell in the candidate T-SN. The MN then provides them to the UE with the conditional reconfiguration(s).

Figure 1 Signalling flow for provision of execution conditions from candidate SN to UE (for SN-initiated Subsequent CPAC)

We thus propose that the MN includes information about other Subsequent CPAC candidates to the candidate T-SN in the configuration procedure, and that the candidate T-SN provides the corresponding execution conditions for subsequent CPC to the MN. The provision of the execution conditions could be included in the inter-node RRC message (e.g. in the CG-CandidateList or in the CG-Config). The signalling details should be coordinated with RAN3.

During the configuration of Subsequent CPAC, the MN provides information about other Subsequent CPAC candidates to the candidate T-SN (for subsequent CPC). The candidate T-SN includes the corresponding execution conditions for subsequent CPC in the response inter-node RRC messages to the MN.

At RAN2#122 it was left FFS if the candidate SN can generate the execution conditions for subsequent CPC also in case of MN-initiated CPAC. The same signalling as described above for SN-initiated Subsequent CPAC (between MN and candidate T-SN and towards the UE) can be used also for MN-initiated Subsequent CPAC, i.e. where it is the MN that does the initial configuration. To support that the candidate T-SN can generate the execution conditions also for MN-initiated Subsequent CPAC should thus be possible with only minor additional impact, if any at all.

At RAN2#121 the following assumption was also reached regarding reuse of CPA/CPC configurations:

R2 assumes that a CPA conditional configuration can be used for CPC (but with different triggering conditions)

The MN-initiated scenarios thus include that the MN configures the UE with CPA configurations, which then are to be used for subsequent CPC. The triggering/execution conditions should however be different for CPA and for CPC. By including execution conditions for subsequent CPC together with the Subsequent CPAC configurations, as described above, different execution conditions for CPA and CPC will then also be achieved. A UE would then be configured with CPA configurations (for Subsequent CPAC), with execution conditions that are specific for CPA (A4 events). When applying one of those CPA configuration the UE would then also apply the associated execution conditions for subsequent CPC to the other candidate PSCells, which then would be specific for CPC (A3 or A5). This would be achieved by letting the candidate T-SN generate the execution conditions for subsequent CPC in case of MN-initiated Subsequent CPAC consisting of initial CPA configurations. We thus propose that the candidate SN can generate execution conditions for subsequent CPC also for MN-initiated scenarios.

If the candidate SN generates execution conditions for subsequent CPC in case of MN-initiated CPA, different triggering conditions will be achieved for CPA and CPC.

The candidate SN can generate execution conditions for subsequent CPC also for MN-initiated scenarios.

There is still the following FFS from the RAN2#119bis meeting “FFS if UE keeps all configurations or if those are indicated by the network”. As the UE moves between different serving PSCells the candidate cells for CPC would typically also change. It should therefore be possible to then release candidate configurations for Subsequent CPAC.

There may then also be some candidate PSCells that are not relevant candidates in the specific PSCell where the UE is currently located, but that are likely to be relevant candidates in case the UE later on moves to another PSCell. It would then be preferable to let the UE keep those candidate configurations, to avoid the need to reconfigure them again soon afterwards, but to skip performing measurements and evaluation for them where it they are not considered relevant. It should therefore be possible for the network to indicate that a candidate configuration is not applicable and that the UE does then not perform any evaluation of it.

Similar to the indication of execution conditions for subsequent CPC, as discussed above, also the subsequent handling of the other candidate Subsequent CPAC configurations, i.e. whether they are to be kept or released and, in case a configuration is to be kept, whether it should be considered as applicable or not, should then also be included in CondReconfigToAddMod.

The network should, in CondReconfigToAddMod, be able to indicate whether the UE should keep or release the other stored candidate configurations for subsequent CPC.

It should be possible for the network to indicate that a candidate configuration that is kept should be considered as not applicable. A not applicable candidate configuration should not be evaluated by the UE.

A Text Proposal for the configuration of Subsequent CPAC, including configuration of execution conditions for subsequent CPC and configuration for handling of other candidate configurations when one candidate configuration is executed, is included in Annex A. It is proposed to consider this as baseline for the configuration of Subsequent CPAC in the running CR.

Consider the Text Proposal in Annex A as a baseline for the configuration of Subsequent CPAC.

## 2.2	Intra-SN and inter-SN CPC configurations

Both intra-SN and inter-SN CPC configurations are assumed to be supported for Subsequent CPAC. At RAN2#121bis it was agreed that inter-SN CPC configurations are in MN format:

For inter-SN SCG Selective Activation, the RRC reconfiguration message containing the Rel-18 CPC configurations provided to the UE is in MN format.

A question is then however what format to use for intra-SN configurations. The existing Rel-16 intra-SN CPC configurations are in SN format and can thus not be used by the UE when moving to another SN. They need to be released at least when performing an inter-SN PSCell change.

An intra-SN CPC configuration in SN format cannot be used after an inter-SN PSCell change.

To allow that the same CPC configuration can be used for both intra- and inter-SN scenarios, it would then need to possible to configure it also in MN format.

For Rel-18 Subsequent CPAC, it should be supported to configure CPC for an intra-SN candidate target PSCell in MN format.

## 2.3	Keeping/releasing Subsequent CPAC configurations at other RRC reconfigurations

At the RAN2#121 meeting it was agreed that:

In SCG selective activation, the CPC/CPA configurations of the UE should be released after Pcell change, at least for inter MN (by explicit indication from network, FFS other case).

This agreement allows to simplify the scenario in case of inter-MN handover. In fact, keeping the CPA/CPC configurations at inter-MN handover scenarios would lead to more complex handling since the target MN needs to setup the corresponding link towards all the candidate target SN(s) and CPA/CPC configurations would probably need to be updated.

However, the application of this agreement to the intra-MN handover case seems an unnecessary limitation. In case of intra-MN handover, the connections between MN and the candidate target SNs are already setup, and the CPA/CPC configurations are expected to be still valid. It should therefore be supported that the Subsequent CPAC configurations can be kept at intra-MN handover. This should then be indicated through explicit indications, according to the agreement above.

There are already mechanisms and procedures for releasing conditional reconfigurations, i.e. the condReconfigToRemoveList in ConditionalReconfiguration. The explicit indication from the network to release (or keep) Subsequent CPAC configurations can thus be achieved using this already existing release procedure and there is then no need to introduce additional parameters, with associated new procedures etc.

We thus propose that the existing condReconfigToRemoveList can be used to release any existing Subsequent CPAC configuration also at other RRC reconfigurations.

The UE can be configured to release (or keep) Subsequent CPAC configurations at other RRC Reconfigurations, e.g. at handover, by explicit indication from network. The explicit indication consists of that candidate configurations are released using the condReconfigToRemoveList in ConditionalReconfiguration.

Corresponding procedures for adding or modifying Subsequent CPAC configurations at these other RRC reconfigurations can then also be achieved by using condReconfigToAddModList (and the associated procedures).

## 2.4	Support for delta and reference configurations

To support delta configurations for the candidate configurations for Subsequent CPAC, using a reference configuration, has been discussed with the following agreements at RAN2#121bis-e:

For the reference configuration for SCG Selective Activation, aim at following similar design as LTM.

The following may be included in the initial RRC reconfiguration message containing the Rel-18 CPC configurations:

- Reference SCG configuration (Optionality FFS). Assume as for LTM Reference configuration may be empty.

- FFS whether MCG configuration is included.

- FFS RRC model for the reference configuration.

The aspects related to the RRC model for the reference configuration and whether the reference configuration consists of only an SCG configuration or both an SCG and an MCG configuration have not been agreed yet. Considering that the candidate target cell configuration for CPC (i.e. the condRRCReconfig) consists of an RRCReconfiguration message, it is reasonable that the reference configuration should also consist of an RRCReconfiguration message. The candidate configurations for (Rel-17) inter-SN CPC includes both MCG and SCG configurations to be applied at the PSCell change. It should therefore be possible to include both MCG and SCG parts in the reference configuration.

The reference configuration for Subsequent CPAC should consist of an RRCReconfiguration message, which can include both SCG and MCG configurations.

A further aspect to be considered is how to handle the reference configuration. A reference configuration is not required as such for Subsequent CPAC to work, and the use of reference configuration is thus optional. By defining the reference configuration as an optional IE/field, there is no need to send an “empty” reference configuration when such configuration is not provided: instead, the reference configuration IE/field is then not included in the RRC reconfiguration message.

As agreed at RAN2#121 the candidate target SN can thus include an indication in the SN Addition Request Ack message whether a delta configuration has been used or not. A corresponding indication is then needed towards the UE, i.e. whether the received configuration is a delta configuration towards a reference configuration.

The reference configuration IE/field for candidate cell configurations, for Subsequent CPAC, is optional. It is up to the network whether to provide a reference configuration to the UE.

The UE, when receiving a candidate target cell configuration for Subsequent CPAC, should be aware whether it is a delta configuration on top of a reference configuration or if it is a full configuration.

At RAN2#122 it was agreed that:

The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures.

As discussed above the reference configuration should include both an SCG and an MCG part. The corresponding parts should then be generated by an SN and the MN, respectively. Considering that there may be several SNs involved already during the initial preparation, a question is then which of the SNs that can generate the SCG part of the reference configuration. For the case of CPA configurations there is no current serving SN for the UE. In order to support reference SCG configurations for this case, we therefore see a benefit in allowing that the MN can request an SCG reference configuration from any of the candidate SNs. We therefore propose that the MN should be able to request it from any of the involved SNs. For the SN-initiated case it should however be possible for the S-SN to provide an SCG reference configuration to the MN when initiating the procedure.

The MN initially generates the MCG part of the reference configuration.

The SCG part of the reference configuration is initially generated by one of the SNs. The MN should be able to request an SCG reference configuration from any of the involved SNs.

For the SN-initiated case, the S-SN can provide an SCG reference configuration to the MN when initiating the procedure.

# Conclusion

In the previous sections we made the following observations:

Observation 1	If the candidate SN generates execution conditions for subsequent CPC in case of MN-initiated CPA, different triggering conditions will be achieved for CPA and CPC.

Observation 2	An intra-SN CPC configuration in SN format cannot be used after an inter-SN PSCell change.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Candidate configurations for Subsequent CPAC are provided with CondReconfigToAddMod in ConditionalReconfiguration, by inclusion of related Rel-18 information.

Proposal 2	The network can, in the CondReconfigToAddMod for the executed candidate configuration, indicate updates of execution conditions for the other candidate configurations at the execution (i.e. for subsequent CPC).

Proposal 3	During the configuration of Subsequent CPAC, the MN provides information about other Subsequent CPAC candidates to the candidate T-SN (for subsequent CPC). The candidate T-SN includes the corresponding execution conditions for subsequent CPC in the response inter-node RRC messages to the MN.

Proposal 4	The candidate SN can generate execution conditions for subsequent CPC also for MN-initiated scenarios.

Proposal 5	The network should, in CondReconfigToAddMod, be able to indicate whether the UE should keep or release the other stored candidate configurations for subsequent CPC.

Proposal 6	It should be possible for the network to indicate that a candidate configuration that is kept should be considered as not applicable. A not applicable candidate configuration should not be evaluated by the UE.

Proposal 7	Consider the Text Proposal in Annex A as a baseline for the configuration of Subsequent CPAC.

Proposal 8	For Rel-18 Subsequent CPAC, it should be supported to configure CPC for an intra-SN candidate target PSCell in MN format.

Proposal 9	The UE can be configured to release (or keep) Subsequent CPAC configurations at other RRC Reconfigurations, e.g. at handover, by explicit indication from network. The explicit indication consists of that candidate configurations are released using the condReconfigToRemoveList in ConditionalReconfiguration.

Proposal 10	The reference configuration for Subsequent CPAC should consist of an RRCReconfiguration message, which can include both SCG and MCG configurations.

Proposal 11	The reference configuration IE/field for candidate cell configurations, for Subsequent CPAC, is optional. It is up to the network whether to provide a reference configuration to the UE.

Proposal 12	The UE, when receiving a candidate target cell configuration for Subsequent CPAC, should be aware whether it is a delta configuration on top of a reference configuration or if it is a full configuration.

Proposal 13	The MN initially generates the MCG part of the reference configuration.

Proposal 14	The SCG part of the reference configuration is initially generated by one of the SNs. The MN should be able to request an SCG reference configuration from any of the involved SNs.

Proposal 15	For the SN-initiated case, the S-SN can provide an SCG reference configuration to the MN when initiating the procedure.

# References

RP-223520, Revised WID on Further NR mobility enhancements, MediaTek Inc., Apple, 3GPP TSG RAN Meeting #98-e, December 12-16, 2022

# Annex A – Text Proposal

- Below is a Text Proposal to TS 38.331, 17.3.0

-- ASN1START

-- TAG-CONDRECONFIGTOADDMODLIST-START

CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...,

[[

condExecutionCondSCG-r17         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL     -- Need M

]],

[[

subsequentCondReconfig-r18      SubsequentCondReconfig-r18	OPTIONAL     -- Need M

]]

}

CondReconfigExecCondSCG-r17 ::=  SEQUENCE (SIZE (1..2)) OF MeasId

SubsequentCondReconfig-r18 ::=     SEQUENCE {

condReconfigUpdateAtExecutionList-r18      SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReConfigUpdateAtExecution-r18			OPTIONAL,     -- Need M

...

}

CondReConfigUpdateAtExecution-r18 ::=     SEQUENCE {

condReconfigId-r18               CondReconfigId-r16,

actionAtExecution-r18            ENUMERATED {release, activate, deactivate}       OPTIONAL,

condExecutionCond-r18            SEQUENCE (SIZE (1..2)) OF MeasId    OPTIONAL,    -- Need M

condExecutionCondSCG-r18         SEQUENCE (SIZE (1..2)) OF MeasId    OPTIONAL    -- Need M

}

-- TAG-CONDRECONFIGTOADDMODLIST-STOP

-- ASN1STOP