3GPP TSG-RAN WG2 Meeting #123		R2-2307976

Toulouse, France, 21st – 25th Aug, 2023

Source:	vivo

Title:	Further Discussion on SA2 Reply LS on SL CA

Agenda Item:	7.15.1

Document for:	Discussion and Decision

# Introduction

This contribution is to further discuss the issues on SL CA from the SA2 reply LS [1].

# Discussion

# Answer 1 with question for clarification

The Answer 1 to Question 1 from the SA2 reply LS is shown as below.

- Question 1: According to TS 24.588, V2X layer is only provisioned with a mapping between service identifier and initial L2 address used for unicast. But service identifier is invisible to AS-layer, and the initial L2 ID will only be used in DCR and be replaced by a self-chosen L2 ID in PC5-S link establishment procedure. Then, after L2 ID changes, whether/how can UE's AS layer obtain the mapping between L2 ID and frequencies? - Answer 1 with question for clarification: According to discussion in SA2, most companies' understanding is Question 1 is about how the AS layer can know frequency information associated with the PC5-S messages sent during the PC5 unicast link establishment procedure. Anyhow some companies think that Question 1 is related to the scenario after the PC5 unicast link is established. Therefore, SA2 would like to ask RAN2 about what the scenario Question 1 is related to. SA2 also discussed how to let the AS layer know frequency information associated with the PC5-S messages sent for PC5 unicast link establishment, by providing frequency information together with the PC5-S message to the AS layer from the V2X layer, however there was no corresponding CR approved yet in SA2.

From the highlighted yellow text, it’s observed that companies in SA2 have different understandings on the target scenario that the Question 1 is related to and would like to double check with RAN2. In general, there are two scenarios mentioned in the Question 1 as following:

- Scenario-1: whether/how the AS layer can know frequency information associated with the PC5-S messages sent during the PC5 unicast link establishment

- Scenario-2: whether/how the AS layer can know frequency information after the PC5 unicast link establishment

We think both scenarios are valid for unicast SL CA. So, we should seek answers from SA2 on both scenarios.

Proposal 1	RAN2 replies to SA2 that both of the following Scenarios are related to Question 1.

        - Scenario-1: whether/how the AS layer can know frequency information associated with the PC5-S messages sent during the PC5 unicast link establishment

        - Scenario-2: whether/how the AS layer can know frequency information after the PC5 unicast link establishment

## Scenario-1: during the PC5 unicast link establishment

When looking into the previous discussion in the offline email [2], we think there are two kinds of understanding on the frequency information associated with the PC5-S messages sent during the PC5 unicast link establishment.

- Option-1: NO frequency information together with the PC5-S message to the AS layer from the V2X layer

- Option-2: Frequency information together with the PC5-S message are provisioned to the AS layer from the V2X layer

In Option-1, RAN2 may further discuss whether and how to support SL CA for the PC5-S message transmission by some AS layer mechanism, but without any SA2 impact. However, RAN2 hasn’t reached any conclusions so far.

While in Option-2, we still rely on SA2 to define the frequency mapping rule associated with the PC5-S messages sent during PC5 unicast link establishment. Moreover, According to Figure 5.3.1.31 of TS 24.588, V2X layer is provisioned with three types of information as below:

- V2X service identifier to frequency mapping

- V2X service identifier to destination L2 ID mapping used for broadcast and groupcast.

- V2X service identifier to destination layer-2 ID mapping for unicast initial signalling

Combining the above first and last bullets, it can be concluded that at least for unicast initial signalling (i.e., DCR message), the frequenc(ies) associated with a certain destination L2 ID will be provided by V2X layer to the AS layer. And we think it’s also very straightforward that the frequency mapping rule specified for DCR message can be reused to the subsequent PC5-S messages sent during the PC5 unicast link establishment.

Observation 1	According to TS 24.588, at least for unicast initial signalling (i.e., DCR message), the frequencies associated with a certain destination L2 ID can be provided by V2X layer to the AS layer.

Given the above observations, we think it’s more reasonable to assume Option-2 as the RAN2 understanding on frequency information applicable to Scenario-1, and this understanding can be further checked by SA2.

Proposal 2	For Scenario-1 (during the PC5 unicast link establishment), RAN2 assumes that the frequency information associated with each PC5-S message (including DCR message) will be provided by V2X layer to the AS layer, and to check this assumption with SA2.

## Scenario-2: after the PC5 unicast link establishment

According to RAN2#121bis-e agreement “Based on observation that section 6.1.2.12 of TS 24.587-v18.0.0 has captured V2X layer can be provisioned with service to frequency mapping for unicast. RAN2 assume it is applicable to PC5 unicast SL CA after link has been established. RAN2 notify SA2 this assumption and ask their input on identified questions.”, obviously RAN2 has consensus that the AS layer expects the frequency information provided by V2X layer after the PC5 unicast link establishment. Moreover, from RAN2 perspective, the AS layer can maintain the mapping between destination L2 ID and frequenc(ies) as legacy LTE SL CA.

While the main concern is that, when the PC5 unicast link identifier update procedure is performed in V2X layer, the L2 ID for the PC5 unicast link will be updated. In such a case, it’s not clear whether and how V2X layer can provide the up-to-date frequency information to the AS layer when the L2 ID is updated for the PC5 unicast link.

Proposal 3	For Scenaro-2 (after the PC5 unicast link establishment), RAN2 further clarify that the intention is to ask whether and how the frequency information is updated and provided to the AS layer, when the L2 ID associated with the PC5 unicast link is updated in V2X layer.

# 2.2 Answer 2

The Answer 2 to Question 2 from the SA2 reply LS is shown as below.

- Question 2: According to TS 24.587, PC5 unicast allows UEs to add/modify/remove V2X services/PC5 QoS flows to the same L2 ID pair. Then, given service info is invisible to AS layer, how can the UE ensure the modified V2X services to be transmitted only on the corresponding frequencies mapped in the V2X layer? - Answer 2: Based on the following operations as described in clauses 5.4.1.1.3 and 6.3.3 of TS 23.287, it is considered that the UE can ensure the modified V2X services to be transmitted only on the corresponding frequencies mapped in the V2X layer. i) The AS layer maintains the PC5 unicast link related context for the lifetime of the PC5 unicast link based on the various information provided by the V2X layer, e.g. PC5 Link Identifier, source L2 ID, destination L2 ID, etc. ii) The V2X layer updates the AS layer about PC5 QoS Flow addition/modification/removal for the established PC5 unicast link. For PC5 QoS Flow addition, the V2X layer provides one or more radio frequencies associated with the added PC5 QoS Flow to the AS layer. The V2X layer can determine the radio frequencies based on the mapping of V2X service type(s) associated with the PC5 QoS Flow to V2X frequencies by using the related configuration as specified in clause 5.1.2.1 of TS 23.287 (i.e. "The mapping of V2X service types to V2X frequencies with Geographical Area(s)"). iii) The V2X layer ensures that V2X service types associated with different radio frequencies are classified into distinct PC5 QoS Flows.

## Issue-1: Applicable cast types to the new frequency mapping rule

According to the highlighted yellow text, a new frequency mapping rule i.e., different radio frequencies are classified into distinct PC5 QoS Flows is introduced for NR SL Unicast.

Observation 2	According to SA2’s reply LS, a new frequency mapping rule i.e., different radio frequencies are classified into distinct PC5 QoS Flows is introduced for NR SL Unicast.

Given that PC5 QoS Flow based on QoS modelling is applicable to all cast types, it’s worth thinking about whether the new frequency mapping rule defined for NR SL Unicast can also apply to NR SL Broadcast and Groupcast or not. It’s better to check SA2 views on this question so that RAN2 can have a full picture on the NR SL CA design.

Proposal 4	RAN2 to ask a further question on top of SA2’s Answer 2: whether the new frequency mapping rule i.e., different radio frequencies are classified into distinct PC5 QoS Flows is also applicable to NR SL Broadcast and Groupcast.

## Issue-2: RAN2 impacts on the new frequency mapping rule

NOTE: As there is no conclusion for NR SL Broadcast and Groupcast (according to above Proposal 4), the discussion in this section will be focused on NR SL Unicast for now.

In LTE SL CA, the AS layer just maintains the mapping between Destination L2 ID and frequenc(ies). When it comes to NR SL CA, different frequencies are classified into distinct PC5 QoS Flows for the established PC5 unicast link. As a consequence, the AS layer needs to maintain a finer frequency mapping for a specific Destination L2 ID by taking different PC5 QoS Flows into account. For example, it can be the mapping between {Destination L2 ID, distinct PC5 QoS Flows} and frequenc(ies).

Proposal 5	For NR SL Unicast, RAN2 confirms the understanding that the AS layer maintains the mapping between {Destination L2 ID, distinct PC5 QoS Flows} and frequenc(ies) for the established PC5 unicast link.

If the above understanding (based on Proposal 5) is correct, we think it brings potential RAN2 impacts on how the SLRB is configured/established. Let’s take the followings as an Example (see Figure 1):

- Firstly, UE-A establishes a PC5 unicast link with UE-B.

- Secondly, PC5 QoS flow #1, #2, #3 arrive for the established PC5 unicast link. And V2X layer provides the new frequency mapping rule as below to the UE-A’s AS layer:

- PC5 QoS flow #1, #2 ↔ SL freq-1

- PC5 QoS flow #3 ↔ SL freq-2

- Lastly, UE-A’s AS layer transmits the PC5 QoS flow #1, #2, #3 via SLRB(s) according to the SLRB configuration by the NW configuration/Pre-configuration.

  - In Case 1 (See left in Figure 1), PC5 QoS flows associated with different frequencies are configured with the same SLRB

  - In Case 2 (See left in Figure 1), PC5 QoS flows associated with different frequencies are configured with different SLRBs

Figure 1: SLRB configuration Example for NR SL Unicast (Case 1 not valid, Case 2 valid)

In the above Figure 1, Case 1 is not feasible for NR SL CA operation. The reason behind this argument is that, Case 1 could cause problem for the SL carrier selection in MAC layer. Considering that the LTE SL CA framwork is highly reused, we check MAC specification for LTE SL CA (see highlighted in yellow as below), and find out that the allowed carrier configured by upper layers is judged as per sidelink logical channel. However, the PC5 QoS flow is transparent to sidelink logical channel. If Case 1 happens, i.e., there is more than one PC5 QoS flow delivered to the sidelink logical channel but associated with different frequencies, how can the MAC layer correctly perform the SL carrier selection according to the new frequency mapping rule? No matter what the result for SL carrier selection is e.g., select SL freq-1 or SL freq-2, there would always be some of the logical channel data (PC5 QoS flow) that cannot follow the frequency mapping rule configured by upper layers.

From TS 36.321 #### 5.14.1.5 TX carrier (re-)selection for V2X sidelink communication The MAC entity shall consider a CBR of a carrier to be one measured by lower layers according to TS 36.214 [6] if CBR measurement results are available, or the corresponding defaultTxConfigIndex configured by upper layers for the carrier if CBR measurement results are not available. If the TX carrier (re-)selection is triggered for a Sidelink process according to clause 5.14.1.1, the MAC entity shall: - if there is no configured sidelink grant on any carrier allowed for the sidelink logical channel where data is available as indicated by upper layers (TS 36.331 [8] and TS 24.386 [15]): - for each carrier configured by upper layers associated with the concerned sidelink logical channel: - if the CBR of the carrier is below threshCBR-FreqReselection associated with the priority of the sidelink logical channel: - consider the carrier as a candidate carrier for TX carrier (re-)selection for the concerned sidelink logical channel. - else: - for each sidelink logical channel, if any, where data is available and that are allowed on the carrier for which Tx carrier (re-)selection is triggered according to clause 5.14.1.1: - if the CBR of the carrier is below threshCBR-FreqKeeping associated with priority of the sidelink logical channel: - select the carrier and the associated pool of resources. - else: - for each carrier configured by upper layers on which the sidelink logical channel is allowed, if the CBR of the carrier is below threshCBR-FreqReselection associated with the priority of the sidelink logical channel; - consider the carrier as a candidate carrier for TX carrier (re-)selection. The MAC entity shall: - if one or more carriers are considered as the candidate carriers for TX carrier (re-)selection: - for each sidelink logical channel allowed on the carrier where data is available and Tx carrier (re-)selection is triggered: - select one or more carrier(s) and associated pool(s) of resources among the candidate carriers with increasing order of CBR from the lowest CBR.

On the other hand, if we guarantee that PC5 QoS flows associated with different frequencies are configured with different SLRBs (see Case 2 of Figure 1), the potential issue on SL carrier selection as Case 1 can be avoided.

Observation 3	The case that PC5 QoS flows associated with different frequencies configured within the same SLRB (see left in Figure 1) can cause problem to the SL carrier selection operation in MAC layer.

Observation 4	RAN2 needs to study some AS layer mechanism to guarantee that for a specific SLRB, only PC5 QoS flows associated with the same frequenc(ies) are delivered on this SLRB.

When the SL TX UE is in RRC_CONNECTED, it’s the simplest way to adopt a NW based solution i.e., the UE reports the new frequency mapping information to the serving gNB to assist the SLRB configuration. Then it can rely on smart gNB implementation to figure out how many sets of SLRB configuration are needed to classify the PC5 QoS flows according to both of the QoS requirement and frequency mapping requirement. The generated SLRB configuration can be sent via dedicated RRC as legacy.

Proposal 6	For NR SL Unicast, if the SL TX UE is in RRC_CONNECTED, it can report the mapping information between {Destination L2 ID, distinct PC5 QoS Flows} and frequenc(ies) to its serving gNB. FFS Stage 3 singalling design on the reporting.

When the SL TX UE is in RRC_IDLE, RRC_INACTIVE or OOC, the UE reporting solution as Proposal 6 is not applicable. Since the RRC_IDLE/RRC_INACTIVE SL TX UE follows the SIB and the OOC SL TX UE follows SL pre-configuration for SLRB configuration, we believe that SIB/SL pre-configuration may not guarantee the correct SLRB configuration as for RRC_CONNECTED UE. Therefore, we suggest that RAN2 to further study on the expected UE behavior from SLRB addition/modification perspective, in order guarantee that only PC5 QoS flows associated with the same frequenc(ies) are established for the concerned SLRB.

Proposal 7	For NR SL Unicast, if the SL TX UE is in RRC_IDLE, RRC_INACTIVE or OOC, FFS the UE behavior on adding or modifying a SLRB so that only PC5 QoS flows associated with the same frequenc(ies) are established for the concerned SLRB.

# Conclusion

This contribution further discussed the issues on SL CA from the SA2 reply LS. The contribution concludes with:

Observation 1	According to TS 24.588, at least for unicast initial signalling (i.e., DCR message), the frequencies associated with a certain destination L2 ID can be provided by V2X layer to the AS layer.

Observation 2	According to SA2’s reply LS, a new frequency mapping rule i.e., different radio frequencies are classified into distinct PC5 QoS Flows is introduced for NR SL Unicast.

Observation 3	The case that PC5 QoS flows associated with different frequencies configured within the same SLRB (see left in Figure 1) can cause problem to the SL carrier selection operation in MAC layer.

Observation 4	RAN2 needs to study some AS layer mechanism to guarantee that for a specific SLRB, only PC5 QoS flows associated with the same frequenc(ies) are delivered on this SLRB.

Proposals related to SA2’s Answer 1

Proposal 1	RAN2 replies to SA2 that both of the following Scenarios are related to Question 1.

        - Scenario-1: whether/how the AS layer can know frequency information associated with the PC5-S messages sent during the PC5 unicast link establishment

        - Scenario-2: whether/how the AS layer can know frequency information after the PC5 unicast link establishment

Proposal 2	For Scenario-1 (during the PC5 unicast link establishment), RAN2 assumes that the frequency information associated with each PC5-S message (including DCR message) will be provided by V2X layer to the AS layer, and to check this assumption with SA2.

Proposal 3	For Scenaro-2 (after the PC5 unicast link establishment), RAN2 further clarify that the intention is to ask whether and how the frequency information is updated and provided to the AS layer, when the L2 ID associated with the PC5 unicast link is updated in V2X layer.

Proposals related to SA2’s Answer 2

Proposal 4	RAN2 to ask a further question on top of SA2’s Answer 2: whether the new frequency mapping rule i.e., different radio frequencies are classified into distinct PC5 QoS Flows is also applicable to NR SL Broadcast and Groupcast.

Proposal 5	For NR SL Unicast, RAN2 confirms the understanding that the AS layer maintains the mapping between {Destination L2 ID, distinct PC5 QoS Flows} and frequenc(ies) for the established PC5 unicast link.

Proposal 6	For NR SL Unicast, if the SL TX UE is in RRC_CONNECTED, it can report the mapping information between {Destination L2 ID, distinct PC5 QoS Flows} and frequenc(ies) to its serving gNB. FFS Stage 3 singalling design on the reporting.

Proposal 7	For NR SL Unicast, if the SL TX UE is in RRC_IDLE, RRC_INACTIVE or OOC, FFS the UE behavior on adding or modifying a SLRB so that only PC5 QoS flows associated with the same frequenc(ies) are established for the concerned SLRB.

# References

- R2-2307060 (S2-2307794), Reply LS on carrier mapping for unicast SL CA, SA2.

- R2-2304232, [AT121bis-e][507][V2XSL] Applicability of carrier mapping from V2X layer to UC (Apple).