3GPP TSG-RAN WG2 Meeting #R2-123		R2-2307986

Toulouse, France, Aug 21 – 25, 2023

Agenda Item:	7.4.2.1

Source: 	Rakuten Symphony Inc.

Title:  	TA Acquisition before LTM Serving cell change

Document for:	Discussion and agreement

# Introduction

In RAN2#121-bis-e and RAN2#119-bis-e meeting, the following agreements were made:

- #121-bis-e - From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order). - RRC RACH configuration for early TA acquisition (e.g., including whether RAR needs to be received) is specific per target cell and is signalled separately (separate IEs) from the candidate cell configuration (the part that need to be applied at cell switch). #119-bis-e - RAN2 assumes that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported. RACH-less if the UE doesn’t need to acquire TA during the cell switch. RAN2 understands that the feasibility of RACH-less may depend on RAN1, and expect that RAN1 is working on this. - RAN2 assumes RACH resource for CFRA for L1 L2 dynamic switch may be provided in RRC configuration (or potentially by MAC CE FFS). - FFS if the MAC CE can indicate TCI state(s) (or other beam info) to activate for the target Cell(s), dep on RAN1 progress.

In this contribution, we analyse a couple of key factors that influence the mobility latency, discuss how they can be addressed.

# Discussion

Based on the above highlighted agreements/working assumptions, we consider the possibility of the UE performing TA acquisition of the candidate/target cell before execution of L1/L2 triggered mobility (LTM) to realize RACH-less HO and its implications on the network and UE.

## UL synchronization before LTM execution

Figure 1: A UE performing RACH (UL sync) procedure with a candidate/target gNB-DU to acquire TA, while connected to serving gNB-DU

NOTE: The message names are exemplary and signify functionality only.

A UE is expected to perform RACH to an indicated target cell whenever the timing advance (TA) of the UE in target cell is different from that of the source cell. The UE acquiring TA of the target cell any time before the execution of LTM has latency gains during LTM execution. Hence it was agreed to investigate solutions to perform UL sync after the cell switch decision has been made. A UE can have multiple LTM target cells at any point of time and all of them may not be eligible for cell switch simultaneously. So, it is sensible for the network to indicate to the UE, the candidate target cell(s) to which UL sync needs to be performed, regardless of the cell switch decision.

The agreements highlighted above indicate that the RACH configuration for TA acquisition before LTM HO is provided over RRC. But it is also clear that the TA acquisition is performed by the UE based on a PDCCH order by the gNB-DU. Since TA acquisition may have to be performed by the UE more than once, for eg: if the TA timer expires, two aspects need clarification.

- Whether the same RACH configuration provided to the UE while at the time of performing TA acquisition first time, can be used subsequently.

- The inclusion of the candidate/target cell ID or a corresponding index in the PDCCH order, with which the UE has to perform TA acquisition.

Observation 1: For LTM, the handover latency can be reduced, if network (gNB-DU) decides that the UE may perform RACH (UL sync) to a target cell in advance of the LTM SCC and indicates using a PDCCH order, the target cell(s) to which the RACH is to be performed.

Proposal 1: RAN2 discusses and clarifies whether the RACH configuration provided during first TA acquisition can be re-used in a subsequent attempt and how to indicate the candidate/target cell ID to perform TA acquisition.

Impacts to the UE

For both intra-CU intra-DU and intra-CU inter-DU scenarios, performing UL sync consumes latency for the UE during handover. Since the proposal here is for the UE to perform UL sync while still being connected to serving cell, it may impact downlink scheduling of the UE in the serving cell. The UE may not be able to receive DL data while performing UL sync with the target cell.

Observation 2: DL data transmission at the serving cell is impacted if the UE performs UL sync with a target cell, while still being connected to serving cell.

## Re-use of Timing Advances post LTM cell switch

A serving gNB-DU may ask a UE to acquire timing advances of more than one LTM candidate cell before performing LTM cell switch. In any case, the cell switch can happen to only one target cell. If the TA thus acquired at the old serving cell has to be re-acquired at the new serving cell, despite the TAT still active, it causes unnecessary overhead on both UE and the network. Hence, there has to be a way to carry forward and continue the use of the acquired timing advances of different candidate cells at the new serving gNB-DU, unless the TAT has expired. For example, the acquired timing advances and their corresponding candidate cell Ids can be either sent by the UE to the new serving gNB-DU post LTM cell switch or from old serving gNB-DU to the new serving gNB-DU via the gNB-CU.

Proposal 2: RAN2 discusses and agrees a mechanism to continue use of acquired timing advances of different candidate cells at the new serving gNB-DU, post LTM cell switch.

## Security for LTM execution

Several companies have expressed security concerns since the execution of LTM is agreed to be performed using a DL MAC CE which is not encrypted and hence unsecured.

A UE can have multiple LTM candidate/target cells configured at any point and since the target cell configuration is indicated to the UE using RRC, we think each candidate/target cell can be assigned an index by the gNB-CU, which can be further signalled to both UE and the gNB-DU. By including only the index of the candidate/target cell in the DL MAC CE command, the LTM cell switch command can be secured.

Observation 3: Indexing the LTM candidate/target cells for each individual UE and using the candidate/target cell Index rather than the cell identity during LTM cell switch execution addresses security concerns.

The index to a LTM target cell could be allocated as and when a new LTM target cell is configured for the UE. Removal of a target cell will imply freeing up the index as well. Index allocation can be done till the max number of LTM target cells and subsequently re-used.

Proposal 3: In order to address security concerns, candidate/target cells are indexed and index allocation to LTM candidate/target cells can be on a first-cum-first-serve basis till the max number of target cells and subsequently re-used.

# Conclusion

We have the following observations:

Observation 1: For LTM, the handover latency can be reduced, if network (gNB-DU) decides that the UE may perform RACH (UL sync) to a target cell in advance of the LTM SCC and indicates using a PDCCH order, the target cell(s) to which the RACH is to be performed.

Observation 2: DL data transmission at the serving cell is impacted if the UE performs UL sync with a target cell, while still being connected to serving cell.

Observation 3: Indexing the candidate LTM target cells for each individual UE and using the target cell Index rather than the cell identity during LTM execution addresses security concerns.

We have the following proposals:

Proposal 1: RAN2 discusses and clarifies whether the RACH configuration provided during first TA acquisition can be re-used in a subsequent attempt and how to indicate the candidate/target cell ID to perform TA acquisition.

Proposal 2: RAN2 discusses and agrees a mechanism to continue use of acquired timing advances of different candidate cells at the new serving gNB-DU, post LTM cell switch.

Proposal 3: In order to address security concerns, candidate/target cells are indexed and index allocation to LTM candidate/target cells can be on a first-cum-first-serve basis till the max number of target cells and subsequently re-used.

# Reference

- RP-222332, “Revised WID on Further NR mobility enhancements,” MediaTek