3GPP TSG-RAN WG2 Meeting #123	R2-2307993

Toulouse, France, August 21th – 25th, 2023	(Rev. of R2-2305687)

Agenda item:		7.15.4

Source:		Lenovo

Title:		Other remaining issue for NR SL-U

Document for:		Discussion and Decision

- Introduction

This paper will discuss the other remaining issue for NR SL-U, which includes leftovers of CAPC for RRC inactive/idle/ooc UE, autonomous retransmission for CG, and SL DRX issues. The corresponding agreement [2] in RAN2#121b-e meeting is as following

Agreement on SL CAPC:

For ‘best-match’ issue, UE may determine it based on closest PDB, and capture it in stage-2 spec only. Detailed wording can be discussed in running CR phase. FFS on whether to consider default priority as well.

Agreement on SL CG

Confirm the working assumption: Not to support CG retransmission timer in SL-U.

The corresponding agreement [2] in RAN2#122 meeting is as following

Agreements on best-matched rule for non-standardized PQI

1: 	Priority is not considered in best-matched rule.

Agreements on SL CAPC when CAPC of the default SLRB is not configured

1: 	UE selects the lowest CAPC priority level (highest CAPC value) among the associated QoS flows.

- Working assumptions are confirmed at least for UC.

Agreements on multiple PSFCH occasions

1: 	Working assumption “In case of multiple PSFCH occasion per PSCCH/PSSCH, if HARQ A/N is successfully transmitted in one PSFCH occasion, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback.” is agreed at least for UC.

2: 	Working assumption “In case of multiple PSFCH occasion per PSCCH/PSSCH, if LBT failure happens in all PSFCH occasions, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the last PSFCH occasion for the SL HARQ feedback.” is agreed at least for UC.

Agreements on SL DRX active time

1: 	Working assumption “Not define shared COT as SL DRX active time” is agreed. If RAN1 introduces additional ID, we can revisit it.

This paper discusses the remaining issue of CAPC, CG, and SL-DRX.

- Remaining issues for CAPC

In RAN2#121b-e meeting, it was agreed UE in RRC inactive/idle state or out-of-coverage may determine CAPC for non-standardized PQI based on closest PDB of standardized PQI. For a non-standardized PQI, UE will find a standard PQI which has the closest PDB with the non-standardized PQI, and using the CAPC of this standard PQI for the non-standardized PQI. With this scheme, all UEs will determine the same CAPC for the same non-standardized PQI no matter the UE is IC/OOC, or in which RRC state.

However, when UE determines the standardized PQI with closest PDB, there may be multiple standardized PQIs with the same closest PDB, having different CAPC. For example, as analysed in [3], when PDB of non-standardized PQI is 300ms, PQIs with closest PDB are PQI 25, 26 and 61, which has CAPC of 2/1/3. In this case, if left to UE implementation to select CAPC, there still happens unaligned CAPC among UEs. So it is better to specify a simple rule, e.g. select PQI with minimum CAPC value if multiple standardized PQIs are determined as the same closest PDB.

Proposal 1: For ‘best match’ issue, if multiple standardized PQIs with same closest PDB are determined, specify a simple rule to select CAPC among these standardized PQIs e.g. select PQI with minimum/maximum CAPC value

- CAPC indication by gNB

In NR-U the CAPC value is either selected by the gNB, e.g. for dynamically scheduled PUSCH transmissions, or selected by the UE autonomously, i.e. for CG PUSCH transmissions. It needs to be discussed how the CAPC value is selected for SL transmissions when operating in shared spectrum channel access considering that SL resource allocation can either done by the gNB (mode 1) or by the Tx UE autonomously (mode 2).

For sidelink UE in mode 1 transmission, one may argue that the case is very similar as for legacy NR-U, and hence the CAPC for a SL TB transmission can be configured by gNB directly. For SL transmission scheduled by gNB, gNB can indicate CAPC for SL transmission in DCI for SL grant. However, the assumption for NR-U was that gNB is aware of which data (e.g. which logical channels/MAC CEs) is multiplexed in a UL grant. This assumption however doesn’t hold for sidelink, e.g. gNB cannot really predict which data Tx UE multiplexes on a SL grant since gNB cannot know which transmission parameters the UE selects. Even though there are differences between NR-U and SL-U with respect to Mode 1 CAPC handling, we think reusing the NR-U principles would be sufficient. It seems anyway that this is the assumption. Therefore, we have the following proposal:

Proposal 2: RAN2 to confirm that SL DCI allocating SL resources to a UE indicates a CAPC index/value within the DCI which is to be used for when performing a LBT for the transmission of the TB generated according to the allocated SL resources

- CG impact

In NR-U, CG is enhanced considering the DL feedback/retransmit scheduling might be missed because of LBT failure. Autonomous retransmission is introduced to avoid UL retransmission of CG waiting too long. Further, autonomous retransmission of CG can utilize other CG configuration with same TB size, thus HARQ process id indication in CG-UCI is introduced, and NR-U UE can select HARQ process id by itself according to the retransmission on CG resource. For SL-U, it is necessary to discuss whether NR-U CG feature e.g. autonomous retransmission and UE selection HARQ process id needs to be introduced or not.

For autonomous retransmission, which purpose is to handling feedback missing or retransmission schedule missing because of LBT failure. When considering SL-U, for SL-U UE in mode 1 resource allocation, if PSSCH transmission can’t be performed due to LBT failure, no HARQ feedback will be received for cases that PSFCH is configured. According to the current specified procedure UE will not deliver a NACK to the corresponding sidelink HARQ entity. Only for cases when the PSSCH transmission was performed and no feedback is no acknowledgement is obtained from the physical layer, UE will deliver a NACK.

The MAC entity shall for each PSSCH transmission: 1> if an acknowledgement corresponding to the PSSCH transmission in clause 5.22.1.3.1a is obtained from the physical layer: 2> deliver the acknowledgement to the corresponding Sidelink HARQ entity for the Sidelink process; 1> else: 2> deliver a negative acknowledgement to the corresponding Sidelink HARQ entity for the Sidelink process; <omit>

We think that UE should treat a LBT failure for a PSSCH transmission as if no HARQ feedback is received for the corresponding Sidelink HARQ process. We see some benefit if UE triggers an autonomous retransmission for this case. So we have the following proposal

Proposal 3: Support autonomous retransmission if SL CG transmission is dropped due to LBT failure

The feature of HARQ process ID and RV selection for CG transmission is linked to the configuration of the CGRT timer, e.g. support of autonomous retransmissions due to LBT failures. Since in NR-U, there might not be a resource for autonomous retransmission, or the resource is in next period and very late for retransmission, UE can also perform resources of another CG configuration as long as the TB sizes match,, i.e. . If cg-RetransmissionTimer is configured, retransmissions with the same HARQ process may be performed on any configured grant configuration if the configured grant configurations have the same TBS. The selected HARQ process ID is indicated in the CG-UCI. In NR SL, for one SL CG configuration, multiple resource can be reserved for both initial transmission and retransmission, whether it is necessary to use resource of other CG configuration needs to be further discussed. And consider the multiple consecutive PSSCH transmission is still studied in RAN1, whether to introduce the feature of UE autonomous select HARQ process id for SL CG transmission can wait for RAN1 progress.

Proposal 4: The discussion of necessity to introduce the feature of UE autonomous select HARQ process id for SL CG transmission can wait for RAN1 progress.

In legacy NR SL, for SL CG transmission, to determine when to flush HARQ buffer, Tx UE is configured with the parameter sl-MaxTransNum, and Tx UE will flush HARQ buffer when the transmission number during one SL CG period reached sl-MaxTransNum. In SL-U, if the transmission is failure because of LBT failure, the transmission number of MAC PDU should not be increased, otherwise sl-MaxTransNum will be easily reached and HARQ buffer will be flushed.

Proposal 5: For UE configured with mode 1 and CG, a PSSCH transmission dropped by LBT failure does not count as a transmission and increase the transmission number for sl-MaxTransNum comparison

- Remaining issue for SL-DRX

In last meeting, for multiple PSFCH case, the sl-drx-HARQ-RTT-Timer start position was discussed and corresponding working assumptions were confirmed for unicast case. However, for groupcast case, it still FFS. Since for groupcast case, different Rx UE in the same group may response with different HARQ feedback, agreed starting position for unicast case cannot be directly reused, otherwise different Rx UE may start sl-drx-HARQ-RTT-Timer at different position. So for groupcast case, a unified start position which is uncorrelated with HARQ feedback value is preferre

Proposal 6: for Groupcast case, Rx UEs start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot following the last PSFCH occasion for SL HARQ feedback

- Conclusion

In this contribution, the following observations and proposals are made:

Proposal 1: For ‘best match’ issue, if multiple standardized PQIs with same closest PDB are determined, specify a simple rule to select CAPC among these standardized PQIs e.g. select PQI with minimum/maximum CAPC value

Proposal 2: RAN2 to confirm that SL DCI allocating SL resources to a UE indicates a CAPC index/value within the DCI which is to be used for when performing a LBT for the transmission of the TB generated according to the allocated SL resources

Proposal 3: Support autonomous retransmission if SL CG transmission is dropped due to LBT failure

Proposal 4: The discussion of necessity to introduce the feature of UE autonomous select HARQ process id for SL CG transmission can wait for RAN1 progress.

Proposal 5: For UE configured with mode 1 and CG, a PSSCH transmission dropped by LBT failure does not count as a transmission and increase the transmission number for sl-MaxTransNum comparison

Proposal 6: for Groupcast case, Rx UEs start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot following the last PSFCH occasion for SL HARQ feedback

- References

RP-230077, WID revision: NR sidelink evolution

RAN2#121bis-e meeting chairman minutes

RAN2#122 meeting chairman minutes

R2-2303914	Discussion on CAPC for non-standardized PQI to decide 'best match', vivo, Lenovo, InterDigital, ASUSTeK, Huawei, HiSilicon

R2-2302622, Consideration on CAPC and LBT Impacts, CATT