3GPP TSG-RAN WG2 Meeting #123	R2-2308003

Toulouse, France, August 21-25, 2023

Agenda item:		7.4.2.1

Source:		Lenovo

Title:		Re-acquisition and forwarding of early TA

Document for:		Discussion and Decision

- Introduction

Some agreements related to early TA acquisition have been achieved in the previous meeting [1].

Dynamic grant can be used for RACH-less LTM, for the first UL data transmission to the target cell:

- - the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch.

- - upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant.

Configured grant can be used for RACH-less LTM, for the first UL data transmission to the target cell, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM MAC CE (as set by source cell). FFS further optimization

- For early TA acquisition for candidate Cells

For PDCCH ordered early TA acquisition without RAR, there is no need for UE to maintain the TA timer for candidate cell (i.e. it is NW implementation to determine the TA validity), TA is given in the cell switch MAC CE (when available in the network).

RAN2 doesn’t see a need for a solution with RAR in for Rel-18.

Observation: Without RAR (without UE maintaining TA), the UE will need to do RACH for link recovery and/or conditional (if supported), which is acceptable in Rel-18

The UE determines to trigger RACH-less cell switch in MAC layer, if the LTM cell switch MAC CE provides the TA value (no RAR is assumed).

We send LS to R1

In this contribution, we discuss several remaining issues e.g early TA re-acquisition, early TA forwarding.

- Discussion

Regarding early TA acquisition without RAR, after UE receives the PDCCH order from source cell, UE transmits preamble to the corresponding candidate cell based on the RACH resource for early TA acquisition. Once the candidate cell receives the preamble, the candidate cell will calculate TA value and transmit early TA value to source cell/DU via CU. If the candidate cell (DU) does not receive preamble in the configured RO, the candidate cell can determine the failure of early TA acquisition. The candidate cell/DU transmits the message and indicates this to the source cell (DU). This will enable the source cell to determine if it would like the UE to re-attempt Early TA acquisition and when so, source cell may request dedicated PRACH resources from the candidate cell (DU).

Proposal 1: If the candidate cell (DU) does not receive preamble in the configured RO, the candidate cell determines the failure of TA acquisition and indicates this to the source cell (DU).

If candidate cell can calculate the early TA based on the received preamble, the candidate cell (DU) will transmit the early TA value to source cell (DU). The source cell will store it. It was agreed in RAN2 that it is NW implementation to determine the early TA validity. If source cell determines early TA invalid e.g based on timer or RSRP change, the source cell may transmit the PDCCH order for triggering early TA re-acquisition.

Proposal 2: Source cell (DU) may transmit PDCCH order again for triggering early TA re-acquisition once it determines that the TA is not valid anymore.

Source cell (DU) may store early TA value list for some candidate cells. Source cell will transmit LTM command MAC CE to UE once the source cell determines to switch to a candidate cell which may be located in another DU. After candidate cell receives the RRC reconfiguration complete message from UE, the candidate cell may transmit indication to source cell. In subsequent-LTM, UE will continue keeping the LTM candidate configuration after completing cell switching. If source cell (DU) does not transfer the stored early TA value list, UE will be expected to get early TA value again based on RACH. Therefore, source cell may provide the early TA value(s) of candidate cell(s) to target cell when receiving successful handover confirmation from it (i.e., upon UE’s arrival on the target side).

Proposal 3: Source cell may provide the stored early TA value(s) of candidate cell(s) to target cell when receiving successful handover confirmation from it (i.e., upon UE’s arrival on the target side).

- Conclusion

In this contribution, we have the following proposals:

Proposal 1: If the candidate cell (DU) does not receive preamble in the configured RO, the candidate cell determines the failure of TA acquisition and indicates this to the source cell (DU).

Proposal 2: Source cell (DU) may transmit PDCCH order again for triggering early TA re-acquisition once it determines that the TA is not valid anymore.

Proposal 3: Source cell may provide the stored early TA value(s) of candidate cell(s) to target cell when receiving successful handover confirmation from it (i.e., upon UE’s arrival on the target side).

References

RAN2 chairman notes for RAN2#122.