3GPP TSG-RAN WG2 Meeting #123	R2-2308004

Toulouse, France, August 21-25, 2023

Agenda item:		7.4.2.2

Source:		Lenovo

Title:		RRC issues for LTM

Document for:		Discussion and Decision

- Introduction

More agreements have been achieved in the previous meeting [1]. There are still some remaining RRC issues. In this contribution, we discuss several RRC issues for LTM.

- Discussion

We still have an FFS on whether to add a specific failure cause for LTM within the SCG failure information message. More specifically, UE performs LTM for SCG and starts a LTM timer. LTM for SCG could be performed based on RA or RACH-less. SCGfailureinformation procedure will be triggered upon the timer expiry. In legacy SCGfailureinformation message, failure type will be indicated. It seems current failure types (see below) are not suitable for LTM. Therefore, a new failure type is needed for SCG LTM.

Proposal 1: A new failure type of SCGFailureInformation message is needed for SCG LTM.

It was agreed in last meeting that UE may perform early decoding and early validity check. However, what content should be checked is not clear. According to the RAN2 agreement, the reference configuration is managed separately and always explicitly signaled. Candidate cell configuration can be provided as delta configurations on top of a reference configuration. In the RRC procedures, the candidate delta configuration is applied on top of the reference configuration to form a complete candidate configuration. A UE stores the reference configuration as a separate configuration before complete candidate configuration is formed. The time point to form complete candidate configuration can be upon the reception of the LTM configuration or the reception of cell switch command.

Proposal 2: Compliance check includes reference configuration, LTM candidate cell configuration.

Proposal 3: Compliance check for reference configuration and candidate delta configuration can be performed upon the reception of the configuration or the reception of cell switch command.

In traditional handover or condition-based handover, UE will perform recovery procedure when compliance check failure happens. Regarding LTM, UE also can consider reconfiguration failure and perform recovery procedure once the failure of compliance check happens. More specification, if candidate cell is PCell, UE will perform re-establishment procedure. If candidate cell is PSCell, UE will perform SCG failure information procedure. On the contrary, UE also can report compliance check failure to source serving cell instead of triggering recovery procedure. Then, the source cell may reconfigure/release the corresponding candidate cell.

Proposal 4: RAN2 to discuss which option should be supported upon compliance check failure for the case that candidate cell is PCell or PSCell.

- Option 1: UE performs recovery procedure upon compliance check failure. Re-establishment or SCGfailureinformation will be triggered for candidate PCell or candidate PSCell, respectively.

- Option 2: UE reports compliance check failure to source serving cell. Then, the source cell may reconfigure/release the corresponding candidate cell.

- Conclusion

In this contribution, we have the following proposals:

Proposal 1: A new failure type of SCGFailureInformation message is needed for SCG LTM.

Proposal 2: Compliance check includes reference configuration, LTM candidate cell configuration.

Proposal 3: Compliance check for reference configuration and candidate delta configuration can be performed upon the reception of the configuration or the reception of cell switch command.

Proposal 4: RAN2 to discuss which option should be supported upon compliance check failure for the case that candidate cell is PCell or PSCell.

- Option 1: UE performs recovery procedure upon compliance check failure. Re-establishment or SCGfailureinformation will be triggered for candidate PCell or candidate PSCell, respectively.

- Option 2: UE reports compliance check failure to source serving cell. Then, the source cell may reconfigure/release the corresponding candidate cell.

References

RAN2 chairman notes for RAN2#122.

RAN2 chairman notes for RAN2#121bis.